3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230839

# Online, , 13th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3183 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC R16 SON-MDT testcase 8.1.6.1.4.5 in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 8.1.6.1.4.5 to the 5G/NR5GC ATS for FR2 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.6.1.4.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	3

4	Execution Log Files	3

4.1	Qualcomm Snapdragon x70 5G Modem RF System UE	3

5	References	3

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.4.5 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

Shaun, Harry

Shaun.Harry@keysight.com

## Verification Test Summary

Test Case: 	8.1.6.1.4.5

ATS Version:	iwd-TTCN3-B2022-09_D23wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR, Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm Snapdragon x70 5G Modem RF System UE

Verification Status:	PASS

# Corrections required

None

# Branches executed

The test case was executed on NR band n257 using NR ciphering nea1 and integrity algorithm nia1

# Execution Log Files

## Qualcomm Snapdragon x70 5G Modem RF System UE

The Qualcomm Snapdragon x70 5G Modem RF System UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\TC_8_1_6_1_4_5_LOG.xml

Keysight\TC_8_1_6_1_4_5_LOG.html

- PICS/PIXIT parameter file:

Anritsu\TC_8_1_6_1_4_5_PIXIT.txt

Keysight\TC_8_1_6_1_4_5_PIXIT.xml

# References

| [1] | R5s230655: Supporting information for addition of NR5GC R16 SON-MDT testcase 8.1.6.1.4.5 in FR2 |
| --- | --- |