3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230065

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2905 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the ASP definition for Symbol Timing: type union SymbolTimingInfo_Type { /* EUTRA, NBIOT: * REQ ASPs: TTCN shall set the SymbolTimingInfo to "FirstSymbol" for EUTRA, NBIOT addressing the whole subframe * IND ASPs: TTCN shall ignore the SymbolTimingInfo sent by the SS for EUTRA, NBIOT * NR: * REQ ASPs: Any:=true only if the symbol number is not relevant, * in which case the SS may choose the next available symbol of the given slot * IND ASPs: Any:=true only if there is no symbol information available for the particular kind of indication */ SymbolNumber_Type SymbolOffset, /* to address a particular symbol in a slot */ Null_Type FirstSymbol, /* to address the first symbol (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT */ Null_Type Any /* for IND ASPs in EUTRA and NBIOT or if symbol number is not relevant or not available */ }; In the TC, the Symbol is set as cs_SymbolTimingInfo_FirstSlot as a default assignment. However, when receiving UL data for PUSCH Aggregation, it is needed to change the Symbol: = cs_SymbolTimingInfo_Any as symbol number is not relevant here |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced a template cs_SymbolTimingInfo_Any to set the Symbol to “Any” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.9.NR5GC, 7.1.1.3.9.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

| Function name | f_TC_7_1_1_3_9_NR_TestBody () |
| --- | --- |
| Reason for change | According to the ASP definition for Symbol Timing: type union SymbolTimingInfo_Type { /* EUTRA, NBIOT: * REQ ASPs: TTCN shall set the SymbolTimingInfo to "FirstSymbol" for EUTRA, NBIOT addressing the whole subframe * IND ASPs: TTCN shall ignore the SymbolTimingInfo sent by the SS for EUTRA, NBIOT * NR: * REQ ASPs: Any:=true only if the symbol number is not relevant, * in which case the SS may choose the next available symbol of the given slot * IND ASPs: Any:=true only if there is no symbol information available for the particular kind of indication */ SymbolNumber_Type SymbolOffset, /* to address a particular symbol in a slot */ Null_Type FirstSymbol, /* to address the first symbol (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT */ Null_Type Any /* for IND ASPs in EUTRA and NBIOT or if symbol number is not relevant or not available */ }; In the TC, the Symbol is set as cs_SymbolTimingInfo_FirstSlot as a default assignment. However, when receiving UL data for PUSCH Aggregation, it is needed to change the Symbol: = cs_SymbolTimingInfo_Any as symbol number is not relevant here. |
| Summary of change | Introduced a template cs_SymbolTimingInfo_Any to set the Symbol to “Any” |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted on the ground that the symbol need not be checked in that test case. See proposed implementation below. |

Before Change:

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { <CODE SKIPPED> // @siclog "Step 4" siclog@ //Check: Does the UE transmit a MAC PDU including one RLC SDU, in HARQ process 1 and in slot n+4 and repeats in following pusch-AggregationFactor-1 slots with same resource allocation but different redundancy version (Note 3), if the slot can be used for uplink transmission (Note 4) //UL timing checked as per R5w210109 <CODE SKIPPED>

After Change:

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC {

<CODE SKIPPED> // @siclog "Step 4" siclog@ //Check: Does the UE transmit a MAC PDU including one RLC SDU, in HARQ process 1 and in slot n+4 and repeats in following pusch-AggregationFactor-1 slots with same resource allocation but different redundancy version (Note 3), if the slot can be used for uplink transmission (Note 4) //UL timing checked as per R5w210109 //WA# v_TimingUL.Symbol := valueof(cs_SymbolTimingInfo_Any); <CODE SKIPPED>

New Template

template (value) SymbolTimingInfo_Type cs_SymbolTimingInfo_Any := { Any := true };

## MCC160 proposed implementation

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC

{

var NR_MAC_SDU_Type v_EncodedRlcPdu;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL;

var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL;

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var boolean v_FDD := f_NR_CellInfo_GetIsFDD(nr_Cell1);

var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1);

var default v_IgnoreNRSR := null;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL;

var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);   //@sic R5s221322 sic@

…

// @siclog "Step 3" siclog@

//The SS allocates an UL Grant for  HARQ process 1, sufficient for one RLC SDU to be looped back in a slot n, and NDI indicates new transmission and DCI scheduling the PUSCH indicates rvID = 0.

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits

f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, cs_HarqProcessConfig(1));

// @siclog "Step 4" siclog@

//Check: Does the UE transmit a MAC PDU including one RLC SDU, in HARQ process 1 and in slot n+4 and repeats in following pusch-AggregationFactor-1 slots with same resource allocation but different redundancy version (Note 3), if the slot can be used for uplink transmission (Note 4)

//UL timing checked as per R5w210109

select (v_SubcarrierSpacing) {

case (kHz15) {

if ( v_FDD ) {

v_TimingUL.Subframe.Number := 5;

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot), v_MAC_PDUListUL));

v_TimingUL.Subframe.Number := 6;

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot), v_MAC_PDUListUL));

v_TimingUL.Subframe.Number := 7;

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot), v_MAC_PDUListUL));

v_TimingUL.Subframe.Number := 8;

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot), v_MAC_PDUListUL));

} else {

v_TimingUL.Subframe.Number := 4;

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot), v_MAC_PDUListUL));

v_TimingUL.Subframe.Number := 9;

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, v_TimingUL.HSFN.Number, v_TimingUL.Slot), v_MAC_PDUListUL));

}

}

case(kHz30){

v_TimingUL.Subframe.Number := 4;

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, v_TimingUL.HSFN.Number, v_TimingUL.Slot), v_MAC_PDUListUL));

v_TimingUL.Slot.SlotOffset.Numerology1 := 1;

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, v_TimingUL.HSFN.Number, v_TimingUL.Slot), v_MAC_PDUListUL));

}

case(kHz120){

v_TimingUL.Subframe.Number := 0;

v_TimingUL.Slot.SlotOffset.Numerology3 := 4;

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, v_TimingUL.HSFN.Number, v_TimingUL.Slot), v_MAC_PDUListUL));

v_TimingUL.Subframe.Number := 1;

v_TimingUL.Slot.SlotOffset.Numerology3 := 1;

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, v_TimingUL.HSFN.Number, v_TimingUL.Slot), v_MAC_PDUListUL));

}

case else {FatalError (__FILE__, __LINE__, "SCS value not handled");}

}

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

//@sic R5s221024 sic@

deactivate(v_IgnoreNRSR); //UE shall not transmit SR for retransmissions @sic R5s220332 sic@

//@siclog "Step 5" siclog@

//SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 4.

f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id);

}