3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230097

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2933 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MAC CA testcases 7.1.1.7.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The LCID is constant and set to 5 (‘0101’B) for the MAC PDUs and does not consider when IMS PDN is setup which would use DRB 1 and LCID = 4 - Implementation of updated Step 15 of the Prose CR R5-227414 seems to have been missed in 22wk49. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Used the variable v_LCID instead of the constant tsc_LCID_SCG_Bearer - Implemented R5-227414 for Step 15 to transmit an periodic UL grant suitable for transmitting loop back PDU in case of pc_UL_NR_CA_2CC on NR SCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.7.1.1.NR5GC, 7.1.1.7.1.1.ENDC, 7.1.1.7.1.2.NR5GC, 7.1.1.7.1.2.ENDC, 7.1.1.7.1.3.NR5GC, 7.1.1.7.1.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_7_1_1_7_1_NR_TestBody () |
| --- | --- |
| Reason for change | - The LCID is constant and set to 5 (‘0101’B) for the MAC PDUs and does not consider when IMS PDN is setup which would use DRB 1 and LCID = 4 - The Prose CR R5-227414 has not been implemented in IWD 22wk49 |
| Summary of change | - Used the variable v_LCID instead of the constant tsc_LCID_SCG_Bearer - Implemented R5-227414 for Step 15 to transmit an periodic UL grant suitable for transmitting loop back PDU in case of pc_UL_NR_CA_2CC on NR SCell |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | - Implemented as per R5s230080 - Accepted |

Before Change:

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex , DRB_Identity p_NR_DRB_Id, boolean p_UL_CA) runs on NR_BASE_PTC { //Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer <CODE SKIPPED> // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) }; //@sic R5s210893 sic@ v_EncodedRlcPdu4 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListUL1:= { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer,v_EncodedRlcPdu4) }; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL1) } v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) }; v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) }; //@sic R5s210893 sic@ v_EncodedRlcPdu5 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListUL2:= { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer,v_EncodedRlcPdu5) }; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL2) } v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) }; v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) }; v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit, cs_NR_MAC_Padding_SubPDU('00000000'O)) }; // 1 byte sub headers +4byte Padding header= 40 bits v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP); v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case <CODE SKIPPED> //@siclog "Steps 15" siclog@ //@sic R5s210865 sic@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(v_CellID_UL, v_TimingDL, 0); // 40 ms later f_NR_ULGrantTransmissionOnSR(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2); //@sic R5s211654r2 sic@ //@siclog "Steps 17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 30); //@sic R5s211302 R5s211654 sic@ //@sic R5s211654 R5-221467 - R5w220022 sic@ if (p_UL_CA){ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SPCell,cs_TimingInfo_NR(v_TimingDL),crs_MacBearerRouting(p_SCell)); } else { f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SPCell,cs_TimingInfo_NR(v_TimingDL),crs_MacBearerRouting(p_SPCell)); } <CODE SKIPPED>

After Change:

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex , DRB_Identity p_NR_DRB_Id, boolean p_UL_CA) runs on NR_BASE_PTC { //Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer <CODE SKIPPED> // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) }; //WA# v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) }; //@sic R5s210893 sic@ v_EncodedRlcPdu4 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListUL1:= { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID,v_EncodedRlcPdu4) }; //WA# v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL1) } v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu2) }; //WA# v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) }; //@sic R5s210893 sic@ v_EncodedRlcPdu5 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListUL2:= { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID,v_EncodedRlcPdu5) }; //WA# v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL2) } v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu3) }; //WA# v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) }; v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit, cs_NR_MAC_Padding_SubPDU('00000000'O)) }; // 1 byte sub headers +4byte Padding header= 40 bits v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP); v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case <CODE SKIPPED> //@siclog "Steps 15" siclog@ //@sic R5s210865 sic@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(v_CellID_UL, v_TimingDL, 0); // 40 ms later if (p_UL_CA) { //WA# Prose CR R5-227414 f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2, 20, 1); } else { f_NR_ULGrantTransmissionOnSR(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@sic R5s211654r2 sic@ } //@siclog "Steps 17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 30); //@sic R5s211302 R5s211654 sic@ //@sic R5s211654 R5-221467 - R5w220022 sic@ if (p_UL_CA){ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SPCell,cs_TimingInfo_NR(v_TimingDL),crs_MacBearerRouting(p_SCell)); } else { f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SPCell,cs_TimingInfo_NR(v_TimingDL),crs_MacBearerRouting(p_SPCell)); } <CODE SKIPPED>