3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230161

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2977 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function fl_Check_dl_SchedulingOffset_PDSCH_TypeA_FRX_Diff |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of wk49, Function fl_Check_dl_SchedulingOffset_PDSCH_TypeA_FRX_Diff check is not in line with below specification snippets 38.306 4.2.1 Introduction set all fields of UE-NR/MRDC-Capability except fdd-Add-UE-NR/MRDC/Sidelink-Capabilities, tdd-Add-UE-NR/MRDC/Sidelink-Capabilities, fr1-Add-UE-NR/MRDC-Capabilities and fr2-Add-UE-NR/MRDC-Capabilities, to include the values applicable for all duplex mode(s) and frequency range(s) that the UE supports 38.331-h20/ga0 fr1-fr2-Add-UE-NR-Capabilities This instance of UE-NR-CapabilityAddFRX-Mode does not include any other fields than csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework. based on the above statements it is proposed to make Note 18 to be in line with below behavior . If FR1 and FR2 both supported by UE If dl-SchedulingOffset-PDSCH-TypeA is supported for FR1 and FR2 – UE will signal the capability in phy-ParametersFRX-Diff Else if dl-SchedulingOffset-PDSCH-TypeA is supported only for FR1 and not for FR2 - UE will signal the capability in fr1-Add-UE-NR-Capabilities Else if dl-SchedulingOffset-PDSCH-TypeA is supported only for FR2 and not for FR1 - UE will signal the capability in fr2-Add-UE-NR-Capabilities If FR1 only supported by UE If dl-SchedulingOffset-PDSCH-TypeA is supported for FR1 - UE will signal the capability in phy-ParametersFRX-Diff If FR2 only supported by UE If dl-SchedulingOffset-PDSCH-TypeA is supported for FR2 - UE will signal the capability in phy-ParametersFRX-Diff |  |  |  |  |  |  |  |  |
| Summary of change: |  | fl_Check_dl_SchedulingOffset_PDSCH_TypeA_FRX_Diff implementation is changed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC, 8.2.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Overview	4

2	Corrections required	4

2.1	Correction to the Function fl_Check_dl_SchedulingOffset_PDSCH_TypeA_FRX_Diff	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function fl_Check_dl_SchedulingOffset_PDSCH_TypeA_FRX_Diff

| Function name | fl_Check_dl_SchedulingOffset_PDSCH_TypeA_FRX_Diff |
| --- | --- |
| Reason for change | In the current TTCN implementation of wk49, Function fl_Check_dl_SchedulingOffset_PDSCH_TypeA_FRX_Diff check is not line with below specification 38.306 4.2.1 Introduction set all fields of UE-NR/MRDC-Capability except fdd-Add-UE-NR/MRDC/Sidelink-Capabilities, tdd-Add-UE-NR/MRDC/Sidelink-Capabilities, fr1-Add-UE-NR/MRDC-Capabilities and fr2-Add-UE-NR/MRDC-Capabilities, to include the values applicable for all duplex mode(s) and frequency range(s) that the UE supports 38.331-h20/ga0 fr1-fr2-Add-UE-NR-Capabilities This instance of UE-NR-CapabilityAddFRX-Mode does not include any other fields than csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework. based on the above statements it is proposed to make Note 18 to be in line with below behavior . If FR1 and FR2 both supported by UE If dl-SchedulingOffset-PDSCH-TypeA is supported for FR1 and FR2 – UE will signal the capability in phy-ParametersFRX-Diff Else if dl-SchedulingOffset-PDSCH-TypeA is supported only for FR1 and not for FR2 - UE will signal the capability in fr1-Add-UE-NR-Capabilities Else if dl-SchedulingOffset-PDSCH-TypeA is supported only for FR2 and not for FR1 - UE will signal the capability in fr2-Add-UE-NR-Capabilities If FR1 only supported by UE If dl-SchedulingOffset-PDSCH-TypeA is supported for FR1 - UE will signal the capability in phy-ParametersFRX-Diff If FR2 only supported by UE If dl-SchedulingOffset-PDSCH-TypeA is supported for FR2 - UE will signal the capability in phy-ParametersFRX-Diff |
| Summary of change | fl_Check_dl_SchedulingOffset_PDSCH_TypeA_FRX_Diff implementation is changed |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#98 |

Before Change:

function fl_Check_dl_SchedulingOffset_PDSCH_TypeA_FRX_Diff(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-227571 R5-227572 R5-227573 sic@ var boolean v_Phy_ParametersFRX_Diff_dl_SchedulingOffset_PDSCH_TypeA := false; var boolean v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA := false; var boolean v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA := false; var boolean v_FR1_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var Phy_ParametersFRX_Diff v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_Common; var Phy_ParametersFRX_Diff v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_fr1_fr2; var Phy_ParametersFRX_Diff v_FR1_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA; var Phy_ParametersFRX_Diff v_FR2_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA; if (ispresent(v_ReceivedCapabilityMsg.phy_Parameters.phy_ParametersFRX_Diff)) { //case common IE phy_ParametersFRX_Diff v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_Common := v_ReceivedCapabilityMsg.phy_Parameters.phy_ParametersFRX_Diff; } if (ispresent(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff)) { //case fr1-fr2 v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_fr1_fr2 := v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff; } if (ispresent(v_ReceivedCapabilityMsg.fr1_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff)) { //case fr1 v_FR1_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA := v_ReceivedCapabilityMsg.fr1_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff; } if (ispresent(v_ReceivedCapabilityMsg.fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff)) { //case fr2 v_FR2_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA := v_ReceivedCapabilityMsg.fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff; } if(pc_dl_SchedulingOffset_PDSCH_TypeA) { // check indication of Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_Common if (ispresent(v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_Common.dl_SchedulingOffset_PDSCH_TypeA)) { v_Phy_ParametersFRX_Diff_dl_SchedulingOffset_PDSCH_TypeA := true; } // check indication of FR1 IE if (ispresent(v_FR1_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA.dl_SchedulingOffset_PDSCH_TypeA)) { v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA := true; } // check indication of FR2 IE if (ispresent(v_FR2_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA.dl_SchedulingOffset_PDSCH_TypeA)) { v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA := true; } // check indication of fr1_fr2 IE if (ispresent(v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_fr1_fr2.dl_SchedulingOffset_PDSCH_TypeA)) { v_FR1_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA := true; } if ((not f_NR_FreqRangeFr1AndFR2()) and ((not v_Phy_ParametersFRX_Diff_dl_SchedulingOffset_PDSCH_TypeA) or v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA or v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA or v_FR1_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA)) { // fail cases on Single frequency range f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent with pc_dl_SchedulingOffset_PDSCH_TypeA on Single frequency range"); } else if(f_NR_FreqRangeFr1AndFR2()) //UE in FR1&FR2 case { if (v_Phy_ParametersFRX_Diff_dl_SchedulingOffset_PDSCH_TypeA) //common IE phy_ParametersFRX_Diff can not be signalled in FR1&FR2 case { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of Phy_Parameters.phy_ParametersFRX_Diff IE is wrong supporting both frequency ranges"); } if (not v_FR1_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA) { if (v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA and v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA) // fail case: both FR1 and FR2 capabilities are reported but not in fr1_fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FR1/FR2_Add_UE_NR_Capabilities IEs are inconsistent"); } if ((not v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA) and (not v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA)) // inconclusive case: UE does not report any IE for dl_SchedulingOffset_PDSCH_TypeA but pc_dl_SchedulingOffset_PDSCH_TypeA is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_dl_SchedulingOffset_PDSCH_TypeA is set but dl_SchedulingOffset_PDSCH_TypeA IEs are not reported by the UE"); } } else if (v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA or v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA) // fail cases: if IE in fr1_fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff is reported but FR1 and/or FR2 capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FR1/FR2 IEs are reported by the UE if (ispresent(v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_Common.dl_SchedulingOffset_PDSCH_TypeA) or ispresent(v_FR1_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA.dl_SchedulingOffset_PDSCH_TypeA) or ispresent(v_FR2_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA.dl_SchedulingOffset_PDSCH_TypeA) or ispresent(v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_fr1_fr2.dl_SchedulingOffset_PDSCH_TypeA)) { f_SetVerdict(inconc, __FILE__, __LINE__, "setting of pc_dl_SchedulingOffset_PDSCH_TypeA to false is inconsistent with reported capabilities for a Rel-15 and above UE"); } } }

After Change:

function fl_Check_dl_SchedulingOffset_PDSCH_TypeA_FRX_Diff(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-227571 R5-227572 R5-227573 sic@ var boolean v_Phy_ParametersFRX_Diff_dl_SchedulingOffset_PDSCH_TypeA := false; var boolean v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA := false; var boolean v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA := false; var boolean v_FR1_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var Phy_ParametersFRX_Diff v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_Common; var Phy_ParametersFRX_Diff v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_fr1_fr2; var Phy_ParametersFRX_Diff v_FR1_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA; var Phy_ParametersFRX_Diff v_FR2_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA; // if UE is FR1 and FR2 Supported (or) FR1 only Supported (or) FR2 only Supported --> indicated in phy_ParametersFRX_Diff //WA#WI=1053615 if (ispresent(v_ReceivedCapabilityMsg.phy_Parameters.phy_ParametersFRX_Diff)) { //case common IE phy_ParametersFRX_Diff v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_Common := v_ReceivedCapabilityMsg.phy_Parameters.phy_ParametersFRX_Diff; } //below field cant be used as per 38.331-h20/ga0 /*if (ispresent(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff)) { //case fr1-fr2 v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_fr1_fr2 := v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff; }*/ //WA#WI=1053615 // if UE is FR1 and FR2 but TypeA is supported only for FR1 //WA#WI=1053615 if (ispresent(v_ReceivedCapabilityMsg.fr1_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff)) { //case fr1 v_FR1_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA := v_ReceivedCapabilityMsg.fr1_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff; } // if UE is FR1 and FR2 but TypeA is supported only for FR2 //WA#WI=1053615 if (ispresent(v_ReceivedCapabilityMsg.fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff)) { //case fr2 v_FR2_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA := v_ReceivedCapabilityMsg.fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff; } if(pc_dl_SchedulingOffset_PDSCH_TypeA) { // check indication of Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_Common // this is set if UE is either FR1 only / FR2 only / both FR1 + FR2 if (ispresent(v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_Common.dl_SchedulingOffset_PDSCH_TypeA)) { v_Phy_ParametersFRX_Diff_dl_SchedulingOffset_PDSCH_TypeA := true; v_FR1_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA := true; //WA#WI=1053615 } // check indication of FR1 IE if (ispresent(v_FR1_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA.dl_SchedulingOffset_PDSCH_TypeA)) { v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA := true; } // check indication of FR2 IE if (ispresent(v_FR2_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA.dl_SchedulingOffset_PDSCH_TypeA)) { v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA := true; } // check indication of fr1_fr2 IE //below field cant be used as per 38.331-h20/ga0 /*if (ispresent(v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_fr1_fr2.dl_SchedulingOffset_PDSCH_TypeA)) { //WA#WI=1053615 v_FR1_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA := true; }*/ // If UE is FR1 only / FR2 only then v_Phy_ParametersFRX_Diff_dl_SchedulingOffset_PDSCH_TypeA has to be set // v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA and v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA cant be set if ((not f_NR_FreqRangeFr1AndFR2()) and ((not v_Phy_ParametersFRX_Diff_dl_SchedulingOffset_PDSCH_TypeA) or v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA or v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA )) { // fail cases on Single frequency range //WA#WI=1053615 f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent with pc_dl_SchedulingOffset_PDSCH_TypeA on Single frequency range"); } else if(f_NR_FreqRangeFr1AndFR2()) //UE in FR1&FR2 case { /*if (v_Phy_ParametersFRX_Diff_dl_SchedulingOffset_PDSCH_TypeA) //common IE phy_ParametersFRX_Diff can not be signalled in FR1&FR2 case //WA#WI=1053615 { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of Phy_Parameters.phy_ParametersFRX_Diff IE is wrong supporting both frequency ranges"); }*/ // if UE is FR1+FR2 but not supporting TypeA in both FR1 and FR2 then fall in if if (not v_FR1_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA) { if (v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA and v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA) // fail case: both FR1 and FR2 capabilities are reported but not in fr1_fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FR1/FR2_Add_UE_NR_Capabilities IEs are inconsistent"); } if ((not v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA) and (not v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA)) // inconclusive case: UE does not report any IE for dl_SchedulingOffset_PDSCH_TypeA but pc_dl_SchedulingOffset_PDSCH_TypeA is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_dl_SchedulingOffset_PDSCH_TypeA is set but dl_SchedulingOffset_PDSCH_TypeA IEs are not reported by the UE"); } } // if UE is FR1+FR2 and supporting TypeA in both FR1 and FR2 then fall in else else if (v_FR1_Add_dl_SchedulingOffset_PDSCH_TypeA or v_FR2_Add_dl_SchedulingOffset_PDSCH_TypeA) // fail cases: if IE in fr1_fr2_Add_UE_NR_Capabilities.phy_ParametersFRX_Diff is reported but FR1 and/or FR2 capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FR1/FR2 IEs are reported by the UE if (ispresent(v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_Common.dl_SchedulingOffset_PDSCH_TypeA) or ispresent(v_FR1_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA.dl_SchedulingOffset_PDSCH_TypeA) or ispresent(v_FR2_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA.dl_SchedulingOffset_PDSCH_TypeA) or ispresent(v_Phy_ParametersFRX_dl_SchedulingOffset_PDSCH_TypeA_fr1_fr2.dl_SchedulingOffset_PDSCH_TypeA)) { f_SetVerdict(inconc, __FILE__, __LINE__, "setting of pc_dl_SchedulingOffset_PDSCH_TypeA to false is inconsistent with reported capabilities for a Rel-15 and above UE"); } } }