3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230251

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3006 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ENDC MAC BWP test case 7.1.1.8.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 7.1.1.8.1 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to ENDC test case 7.1.1.8.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	4

4	Execution Log Files	4

4.1	Qualcomm Snapdragon X70 5G Modem-RF System	4

5	References	4

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of ENDC test case 7.1.1.8.1 which is part of the 5G ENDC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.1.1.8.1.ENDC

ATS Version:	iwd-TTCN3-B2022-09_D22wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm Snapdragon X70 5G Modem-RF System

Verification Status:	PASS

# Corrections required

Changes from R5s230220 (addition of 7.1.1.8.1.NR5GC) are also requried except change 1.

## Change 1

| Function name | f_TC_7_1_1_8_1_ENDC_NR |
| --- | --- |
| Reason for change | - Based on R5-231578 change 2, the sr-TransMax=n16 is too small to allow delays defined in steps 17 and 18, therefore change to n64 is proposed. - Based on R5-231578 change 7, the PDSCHTimeDomainResourceAllocationList is modified in the SIB1 |
| Summary of change | - Changed sr-TransMax to n64 - Implemented 38.523-1 Table 7.1.1.8.1.3.3-0A |
| TTCN module | MAC_BWP_ENDC_NR |
| MCC160 Comment | Accepted, see MCC160 implementation below |

Before Change

function f_TC_7_1_1_8_1_ENDC_NR() runs on ENDC_NR_PTC {//@sic R5-227417 sic@ var integer v_Lrbs := 6; var DRB_Identity v_DrbId; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) ServingCellConfig.downlinkBWP_ToAddModList v_DedicatedBwpListDL; var template (value) UplinkConfig.uplinkBWP_ToAddModList v_DedicatedBwpListUL; var template (value) NR_DownlinkBWP_List_Type v_DownlinkBWP_List; var template (value) NR_UplinkBWP_List_Type v_UplinkBWP_List; var template (value) NR_ResourceAllocation_Type v_ResourceAllocationUL := cs_NR_ResourceAllocation(v_Lrbs, tsc_NR_DefaultGrant_Imcs); var template (value) NR_FreqDomainResourceAssignmentDL_Type v_FreqDomainSchedulDL := cs_NR_FreqDomainResourceAssignmentDL_Automatic(0, v_Lrbs); f_ENDC_NR_Init(); f_NR_CellConfig_Def(nr_Cell1); // Create and configure NR cell 1 f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); // Set null cipher Algorithm v_DrbId := tsc_NR_DRB2; v_SS_Drb_ConfigList := { cs_NR_SS_RadioBearer_RlcBearer_TestMode(v_DrbId, cs_NR_RLC_TestModeConfig_AM(f_NR_38508_SetSNFieldLengthAM())) }; /* MAC: normal mode, RLC: test mode (transparent, AM 18bit-SN), PDCP, SDAP: not configured */ f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, v_SS_Drb_ConfigList); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Start TestBody"); // Synchronise with the EUTRA PTC f_NR_TestBody_Set(true);

After Change

import from NR_Parameters all; function f_TC_7_1_1_8_1_ENDC_NR() runs on ENDC_NR_PTC {//@sic R5-227417 sic@ var integer v_Lrbs := 6; var DRB_Identity v_DrbId; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) ServingCellConfig.downlinkBWP_ToAddModList v_DedicatedBwpListDL; var template (value) UplinkConfig.uplinkBWP_ToAddModList v_DedicatedBwpListUL; var template (value) NR_DownlinkBWP_List_Type v_DownlinkBWP_List; var template (value) NR_UplinkBWP_List_Type v_UplinkBWP_List; var template (value) NR_ResourceAllocation_Type v_ResourceAllocationUL := cs_NR_ResourceAllocation(v_Lrbs, tsc_NR_DefaultGrant_Imcs); var template (value) NR_FreqDomainResourceAssignmentDL_Type v_FreqDomainSchedulDL := cs_NR_FreqDomainResourceAssignmentDL_Automatic(0, v_Lrbs); var template (value) PDSCH_TimeDomainResourceAllocationList v_PDSCH_TimeDomainResourceAllocationList; var SubcarrierSpacing v_SubcarrierSpacing; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; f_ENDC_NR_Init(); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); select (v_SubcarrierSpacing) { case(kHz15) { if (pc_bwp_SwitchingDelay_Type1) { v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),cs_NR_PDSCH_TimeDomainResourceAllocation(1, typeA, 72)}; } else if (pc_bwp_SwitchingDelay_Type2) { //FFS prose issue - waiting prose CR to change to 6 v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),cs_NR_PDSCH_TimeDomainResourceAllocation(3, typeA, 72)}; } } case(kHz30) { if (pc_bwp_SwitchingDelay_Type1) { v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),cs_NR_PDSCH_TimeDomainResourceAllocation(2, typeA, 72)}; } else if (pc_bwp_SwitchingDelay_Type2) { //FFS prose issue - waiting prose CR to change to 6 v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),cs_NR_PDSCH_TimeDomainResourceAllocation(6, typeA, 72)}; } } case(kHz60) { if (pc_bwp_SwitchingDelay_Type1) { v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),cs_NR_PDSCH_TimeDomainResourceAllocation(3, typeA, 72)}; } else if (pc_bwp_SwitchingDelay_Type2) { v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),cs_NR_PDSCH_TimeDomainResourceAllocation(9, typeA, 72)}; } } case(kHz120) { if (pc_bwp_SwitchingDelay_Type1) { v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),cs_NR_PDSCH_TimeDomainResourceAllocation(6, typeA, 72)}; } else if (pc_bwp_SwitchingDelay_Type2) { v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),cs_NR_PDSCH_TimeDomainResourceAllocation(18, typeA, 72)}; } } } f_NR_Set_TimeDomainAllocation(nr_Cell1,v_PDSCH_TimeDomainResourceAllocationList); f_NR_CellConfig_Def(nr_Cell1); // Create and configure NR cell 1 f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); // Set null cipher Algorithm v_DrbId := tsc_NR_DRB2; v_SS_Drb_ConfigList := { cs_NR_SS_RadioBearer_RlcBearer_TestMode(v_DrbId, cs_NR_RLC_TestModeConfig_AM(f_NR_38508_SetSNFieldLengthAM())) }; /* MAC: normal mode, RLC: test mode (transparent, AM 18bit-SN), PDCP, SDAP: not configured */ f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, v_SS_Drb_ConfigList); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Start TestBody"); // Synchronise with the EUTRA PTC f_NR_TestBody_Set(true);

MCC160 proposed implementation

function f_TC_7_1_1_8_1_ENDC_NR() runs on ENDC_NR_PTC

{

/* Bandwidth Part (BWP) operation UL/DL */

//@sic R5-227417 sic@

var integer v_Lrbs := 6;

var DRB_Identity v_DrbId;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var template (value) ServingCellConfig.downlinkBWP_ToAddModList v_DedicatedBwpListDL;

var template (value) UplinkConfig.uplinkBWP_ToAddModList v_DedicatedBwpListUL;

var template (value) NR_DownlinkBWP_List_Type v_DownlinkBWP_List;

var template (value) NR_UplinkBWP_List_Type v_UplinkBWP_List;

var template (value) NR_ResourceAllocation_Type v_ResourceAllocationUL := cs_NR_ResourceAllocation(v_Lrbs, tsc_NR_DefaultGrant_Imcs);

var template (value) NR_FreqDomainResourceAssignmentDL_Type v_FreqDomainSchedulDL := cs_NR_FreqDomainResourceAssignmentDL_Automatic(0, v_Lrbs);

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var template (value) PDSCH_TimeDomainResourceAllocationList v_PDSCH_TimeDomainResourceAllocationList; //@sic R5s230220 R6-231578 sic@

f_ENDC_NR_Init();

f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1);

//@sic R5s0220 R5-231578 sic@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1);

v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64;

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

//@sic R5s230220 R6-231578 sic@

v_PDSCH_TimeDomainResourceAllocationList := fl_NR_InitialisePDSCH_TimeDomainResourceAllocationList(nr_Cell1); //@sic R5s230220 R6-231578 sic@

f_NR_CellInfo_SetSIB1_TimeDomainAllocation(nr_Cell1,v_PDSCH_TimeDomainResourceAllocationList);

f_NR_CellConfig_Def(nr_Cell1);                                      // Create and configure NR cell 1

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only);

f_NR_AS_CipheringAlgorithm_Set(nea0);                               // Set null cipher Algorithm

v_DrbId := tsc_NR_DRB2;

v_SS_Drb_ConfigList := { cs_NR_SS_RadioBearer_RlcBearer_TestMode(v_DrbId, cs_NR_RLC_TestModeConfig_AM(f_NR_38508_SetSNFieldLengthAM())) };  /* MAC: normal mode, RLC: test mode (transparent, AM 18bit-SN), PDCP, SDAP: not configured */

...

}

/*

* @desc      Initialise PDSCH_TimeDomainResourceAllocationList as per Table 7.1.1.8.1.3.3-0A and Table 7.1.1.8.1.3.3-2CA

* @param     p_NR_CellId

*/

function fl_NR_InitialisePDSCH_TimeDomainResourceAllocationList(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return template(value) PDSCH_TimeDomainResourceAllocationList

{ //@sic R5s230220 sic@

var template (value) PDSCH_TimeDomainResourceAllocationList v_PDSCH_TimeDomainResourceAllocationList;

var SubcarrierSpacing v_SubcarrierSpacing;

v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(p_NR_CellId);

select (v_SubcarrierSpacing) {

case(kHz15) {

if (pc_bwp_SwitchingDelay_Type1) {

v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),

cs_NR_PDSCH_TimeDomainResourceAllocation(1, typeA, 72)};

} else if (pc_bwp_SwitchingDelay_Type2) {

v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),

cs_NR_PDSCH_TimeDomainResourceAllocation(6, typeA, 72)};

}

}

case(kHz30) {

if (pc_bwp_SwitchingDelay_Type1) {

v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),

cs_NR_PDSCH_TimeDomainResourceAllocation(2, typeA, 72)};

} else if (pc_bwp_SwitchingDelay_Type2) {

v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),

cs_NR_PDSCH_TimeDomainResourceAllocation(6, typeA, 72)};

}

}

case(kHz60) {

if (pc_bwp_SwitchingDelay_Type1) {

v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),

cs_NR_PDSCH_TimeDomainResourceAllocation(3, typeA, 72)};

} else if (pc_bwp_SwitchingDelay_Type2) {

v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),

cs_NR_PDSCH_TimeDomainResourceAllocation(9, typeA, 72)};

}

}

case(kHz120) {

if (pc_bwp_SwitchingDelay_Type1) {

v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),

cs_NR_PDSCH_TimeDomainResourceAllocation(6, typeA, 72)};

} else if (pc_bwp_SwitchingDelay_Type2) {

v_PDSCH_TimeDomainResourceAllocationList := {cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53),

cs_NR_PDSCH_TimeDomainResourceAllocation(18, typeA, 72)};

}

}

}

return v_PDSCH_TimeDomainResourceAllocationList;

}

# Branches executed

This ENDC TC was executed in Band combination “DC_3A_n41A” using ciphering algorithm nea2 and integrity algorithm nia2

# Execution Log Files

## Qualcomm Snapdragon X70 5G Modem-RF System

The Qualcomm Snapdragon X70 passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_1_8_1_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_1_8_1_PIXIT.xml

# References

| [1] | R5s230252: Supporting information for addition of ENDC MAC BWP test case 7.1.1.8.1 in FR1 |
| --- | --- |