3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230305

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3038 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 9.1.11.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test,NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 7, security is not activated. No security protection is applied for DL message(Authenticaiton reject). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed Security protection status for Authenitcation reject to tsc_SHT_NoSecurityProtection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.11.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_9_1_11_3_TestBody () |
| --- | --- |
| Reason for change | At step 7, security is not activated. No security protection is applied for DL message(Authenticaiton reject). |
| Summary of change | Changed Security protection status for Authenitcation reject to tsc_SHT_NoSecurityProtection |
| TTCN module | SNPN_NAS_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_9_1_11_3_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); ----------Skipping code--------- v_NID := bit2hex(v_NID_ASN); //@siclog "Step 5" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, // @sic R5s230229 sic@ f_Get_EAP_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, v_NID)))); // @sic R5s230229 sic@ //@siclog "Step 6" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected, tsc_SHT_NoSecurityProtection)))) -> value v_ReceivedAsp; // @sic R5s230229 sic@ if (f_Check_EAPAKA_AuthenticationResponse(v_NAS_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } //@siclog "Step 7" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected, // @sic R5s230229 sic@ cs_NG_AUTHENTICATION_REJECT(cs_EAP_Message(bit2oct(encvalue(cs_EAP_Failure))))))); //@siclog "Step 8" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 9" siclog@ //Check: Does the UE transmit an RRCSetupRequest message for intial registration procedure within the next 30 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 30.0, cr_NR_EstablishmentCause_Any, "Step 9"); //@siclog "Step 10" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_DEREGISTERED); //@siclog "Step 11" siclog@ f_NR5GC_SwitchOnAndResetIMS(); ---------skipping code------ //@siclog Step 26 siclog@ f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA, -, TESTMode_OFF, noRrcConnectionRelease); }

After Change:

function fl_TC_9_1_11_3_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); ----------Skipping code--------- v_NID := bit2hex(v_NID_ASN); //@siclog "Step 5" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, // @sic R5s230229 sic@ f_Get_EAP_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, v_NID)))); // @sic R5s230229 sic@ //@siclog "Step 6" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected, tsc_SHT_NoSecurityProtection)))) -> value v_ReceivedAsp; // @sic R5s230229 sic@ if (f_Check_EAPAKA_AuthenticationResponse(v_NAS_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } //@siclog "Step 7" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, // @sic R5s230229 sic@ cs_NG_AUTHENTICATION_REJECT(cs_EAP_Message(bit2oct(encvalue(cs_EAP_Failure))))))); //@siclog "Step 8" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 9" siclog@ //Check: Does the UE transmit an RRCSetupRequest message for intial registration procedure within the next 30 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 30.0, cr_NR_EstablishmentCause_Any, "Step 9"); //@siclog "Step 10" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_DEREGISTERED); //@siclog "Step 11" siclog@ f_NR5GC_SwitchOnAndResetIMS(); ---------skipping code------ //@siclog Step 26 siclog@ f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA, -, TESTMode_OFF, noRrcConnectionRelease); }