3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230883

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3282 | rev | - | Current version: | 17.8.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GS / EPS Fallback test case 11.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS38.523-1 Table 11.1.7.3.3-10, EPS bearer identity is an additional EPS Bearer Id different from default EPS Bearer Id or/and any mapped EPS bearer. In current TTCN implementation, EPS Bearer Id 6 is used in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message at step15 but the eps id in QOS flow is still 5. A conformant UE can fail this test case. In TS38.508-1 Table 4.8.2.3-2: Reference QoS flow #2, it also says EPS bearer identity is any not yet assigned value different from ‘5’ for EmergencySession AND Interworking_with_EPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass EPS Bearer Id to f_EUTRA38_NR_AdditionalProtocolConfigOptions for QOS flow2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

# Corrections required

## Change 1

| Function name | f_EUTRA38_NR_AdditionalProtocolConfigOptions |
| --- | --- |
| Reason for change | According to TS38.523-1 Table 11.1.7.3.3-10, EPS bearer identity is an additional EPS Bearer Id different from default EPS Bearer Id or/and any mapped EPS bearer. In current TTCN implementation, EPS Bearer Id 6 is used in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message at step15 but the eps id in QOS flow is still 5. A conformant UE can fail this test case. In TS38.508-1 Table 4.8.2.3-2: Reference QoS flow #2, it also says EPS bearer identity is any not yet assigned value different from ‘5’ for EmergencySession AND Interworking_with_EPS. |
| Summary of change | Pass EPS Bearer Id to f_EUTRA38_NR_AdditionalProtocolConfigOptions for QOS flow2. |
| TTCN module | EUTRA38_CommonProcedures.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_EUTRA38_NR_AdditionalProtocolConfigOptions(EUTRA_CellId_Type p_CellId, boolean p_IMS_PDN_ConnEstab, EPS_BearerIdentity p_EpsBearerId := 'C'H, boolean p_InterworkWithEPS := true, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, PDU_PDN_DNN_Type p_PDN_Type := None) runs on EUTRA_PTC return template (value) NAS_ProtocolConfigOptions_Type { var template (value) NAS_ProtocolConfigOptions_Type v_AdditionalProtocolConfigOptions := {}; var template (value) QoS_RuleList v_QoS_RuleList; var template (value) QoS_FlowList v_QoS_Flows; var template (value) NAS_ProtocolConfigOptions_Type v_ProtocolConfigOptions := {}; if (not (p_IwkN26 == SUPPORTED and p_PDN_Type == Emergency_PDN)) { // @sic R5s220883 sic@ // Don't add anything if w/o N26 supported and emergency PDN v_ProtocolConfigOptions := { cs_ProtocolContainer('001B'O, '01'O & f_Asn2Nas_PlmnId (f_EUTRA_CellInfo_GetGutiPLMN (p_CellId))), // @sic R5s201102, R5-204488 sic@ cs_ProtocolContainer('001B'O, '02'O & f_Asn2Nas_PlmnId (f_EUTRA_CellInfo_GetGutiPLMN (p_CellId))), // @sic R5-221467 default now contains 4 SSTs sic@ cs_ProtocolContainer('001B'O, '03'O & f_Asn2Nas_PlmnId (f_EUTRA_CellInfo_GetGutiPLMN (p_CellId))), cs_ProtocolContainer('001B'O, '04'O & f_Asn2Nas_PlmnId (f_EUTRA_CellInfo_GetGutiPLMN (p_CellId))) }; if (p_IwkN26 == NOT_SUPPORTED) { // @sic R5-221406 sic@ if (p_IMS_PDN_ConnEstab) { v_QoS_RuleList := { f_BuildDefaultQoSRule ("2") }; // QoS rules according to TS 38.508-1 [67] Table 4.8.2.1-2 // @sic R5-202585 sic@ v_QoS_Flows := { cs_QoS_Flow2 (p_InterworkWithEPS, '50'O)}; // @sic R5-204488 sic@ Qos flow descriptions refer to TS 38.508-1 [67] 4.8.2.3-2. BearerId hardcoded to 5 for IMS } else { v_QoS_RuleList := { f_BuildDefaultQoSRule ("1") }; // QoS rules according to TS 38.508-1 [67] Table 4.8.2.1-1 // @sic R5-202585 sic@ v_QoS_Flows := { cs_QoS_Flow1 (p_InterworkWithEPS, hex2oct(p_EpsBearerId & '0'H))}; // @sic R5-204488 sic@ Qos flow descriptions refer to TS 38.508-1 [67] 4.8.2.3-1 } v_AdditionalProtocolConfigOptions := { cs_ProtocolContainer('001D'O, bit2oct(encvalue(cs_Session_AMBR(omit, '05'O, '0004'O, '05'O, '0004'O)))), cs_ProtocolContainer('001C'O, bit2oct(encvalue(v_QoS_RuleList))), cs_ProtocolContainer('001F'O, bit2oct(encvalue(v_QoS_Flows)))}; } } v_ProtocolConfigOptions := v_ProtocolConfigOptions & v_AdditionalProtocolConfigOptions; // restructured to avoid tool error return v_ProtocolConfigOptions; }

After Change:

function f_EUTRA38_NR_AdditionalProtocolConfigOptions(EUTRA_CellId_Type p_CellId, boolean p_IMS_PDN_ConnEstab, EPS_BearerIdentity p_EpsBearerId := 'C'H, boolean p_InterworkWithEPS := true, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, PDU_PDN_DNN_Type p_PDN_Type := None) runs on EUTRA_PTC return template (value) NAS_ProtocolConfigOptions_Type { var template (value) NAS_ProtocolConfigOptions_Type v_AdditionalProtocolConfigOptions := {}; var template (value) QoS_RuleList v_QoS_RuleList; var template (value) QoS_FlowList v_QoS_Flows; var template (value) NAS_ProtocolConfigOptions_Type v_ProtocolConfigOptions := {}; if (not (p_IwkN26 == SUPPORTED and p_PDN_Type == Emergency_PDN)) { // @sic R5s220883 sic@ // Don't add anything if w/o N26 supported and emergency PDN v_ProtocolConfigOptions := { cs_ProtocolContainer('001B'O, '01'O & f_Asn2Nas_PlmnId (f_EUTRA_CellInfo_GetGutiPLMN (p_CellId))), // @sic R5s201102, R5-204488 sic@ cs_ProtocolContainer('001B'O, '02'O & f_Asn2Nas_PlmnId (f_EUTRA_CellInfo_GetGutiPLMN (p_CellId))), // @sic R5-221467 default now contains 4 SSTs sic@ cs_ProtocolContainer('001B'O, '03'O & f_Asn2Nas_PlmnId (f_EUTRA_CellInfo_GetGutiPLMN (p_CellId))), cs_ProtocolContainer('001B'O, '04'O & f_Asn2Nas_PlmnId (f_EUTRA_CellInfo_GetGutiPLMN (p_CellId))) }; if (p_IwkN26 == NOT_SUPPORTED) { // @sic R5-221406 sic@ if (p_IMS_PDN_ConnEstab) { v_QoS_RuleList := { f_BuildDefaultQoSRule ("2") }; // QoS rules according to TS 38.508-1 [67] Table 4.8.2.1-2 // @sic R5-202585 sic@ v_QoS_Flows := { cs_QoS_Flow2 (p_InterworkWithEPS, hex2oct(p_EpsBearerId & '0'H))}; // @sic R5-204488 sic@ Qos flow descriptions refer to TS 38.508-1 [67] 4.8.2.3-2. BearerId hardcoded to 5 for IMS } else { v_QoS_RuleList := { f_BuildDefaultQoSRule ("1") }; // QoS rules according to TS 38.508-1 [67] Table 4.8.2.1-1 // @sic R5-202585 sic@ v_QoS_Flows := { cs_QoS_Flow1 (p_InterworkWithEPS, hex2oct(p_EpsBearerId & '0'H))}; // @sic R5-204488 sic@ Qos flow descriptions refer to TS 38.508-1 [67] 4.8.2.3-1 } v_AdditionalProtocolConfigOptions := { cs_ProtocolContainer('001D'O, bit2oct(encvalue(cs_Session_AMBR(omit, '05'O, '0004'O, '05'O, '0004'O)))), cs_ProtocolContainer('001C'O, bit2oct(encvalue(v_QoS_RuleList))), cs_ProtocolContainer('001F'O, bit2oct(encvalue(v_QoS_Flows)))}; } } v_ProtocolConfigOptions := v_ProtocolConfigOptions & v_AdditionalProtocolConfigOptions; // restructured to avoid tool error return v_ProtocolConfigOptions; }

## Change 2

| Function name | f_EUTRA38_NR_InitialRegistration |
| --- | --- |
| Reason for change | According to TS38.523-1 Table 11.1.7.3.3-10, EPS bearer identity is an additional EPS Bearer Id different from default EPS Bearer Id or/and any mapped EPS bearer. In current TTCN implementation, EPS Bearer Id 6 is used in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message at step15 but the eps id in QOS flow is still 5. A conformant UE can fail this test case. In TS38.508-1 Table 4.8.2.3-2: Reference QoS flow #2, it also says EPS bearer identity is any not yet assigned value different from ‘5’ for EmergencySession AND Interworking_with_EPS. |
| Summary of change | Pass EPS Bearer Id to f_EUTRA38_NR_AdditionalProtocolConfigOptions for QOS flow2. |
| TTCN module | EUTRA38_CommonProcedures.ttcn |
| MCC160 Comment | Not needed. Not used for emergency PDN. |

Before Change:

function f_EUTRA38_NR_InitialRegistration(EUTRA_CellId_Type p_CellId, IDLEUPDATED_STATE_Type p_State, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (omit) PLMN_List p_EquivalentPlmnList := omit, template (omit) AbsoluteCellPower_Type p_AbsoluteCellPowerLevel_FR1 := omit, template (omit) AbsoluteCellPower_Type p_AbsoluteCellPowerLevel_FR2 := omit, boolean p_UserPlaneIntegrity := false) runs on EUTRA_5GS_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel_FR1 := tsc_ServingCellRS_EPRE; var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel_FR2 := tsc_ServingCellRS_EPRE_FR2; var template (omit) AdditionalUpdateType v_AdditionalUpdateType; var NAS_UL_Message_Type v_PdnConnectivityRequest; var ProcedureTransactionIdentifier v_EPS_TI; var template (value) ProtocolConfigOptions v_PcoToUE; var template (omit) ProtocolConfigOptions v_PcoFromUE := omit; // may be omit var template (omit) ExtdProtocolConfigOptions v_ExtdPcoToUE := omit; var template (omit) ExtdProtocolConfigOptions v_ExtdPcoFromUE := omit; var boolean v_IPv4AllocationViaNasFlag := f_CheckPCOforIPallocationViaNas(v_PcoFromUE); // See TS 36.508 v.8.2.2 Table 4.7.3_6 var APNandPCOs_Type v_APNandPCOs; var PDN_Index_Type v_PdnIndex := f_EUTRA_InitialRegistrationPdnIndex(firstPDN); var template (value) NAS_ProtocolConfigOptions_Type v_AdditionalProtocolConfigOptions_FirstPDN := {}; var template (value) NAS_ProtocolConfigOptions_Type v_AdditionalProtocolConfigOptions_SecondPDN := {}; var boolean v_IMS_PDN_ConnEstab_FirstPDN; var boolean v_IMS_PDN_ConnEstab_SecondPDN; var template (omit) EUTRA_NR_PduSessionInfo_Type v_PduSessionInfo := omit; var template (omit) integer v_SessionId := omit; var template (omit) integer v_QFI := omit; var NR_Configuration_Type v_NR_ConfigType := f_EUTRA38_NR_Config_Get(); timer t_GuardTimer := tsc_GuardTimePreamble; t_GuardTimer.start; if (ispresent(p_AbsoluteCellPowerLevel_FR1) and ispresent(p_AbsoluteCellPowerLevel_FR2)) { v_AbsoluteCellPowerLevel_FR1 := valueof(p_AbsoluteCellPowerLevel_FR1); v_AbsoluteCellPowerLevel_FR2 := valueof(p_AbsoluteCellPowerLevel_FR2); } // First switch cell back on f_EUTRA38_SetCellPower(p_CellId, v_AbsoluteCellPowerLevel_FR1, v_AbsoluteCellPowerLevel_FR2); // Switch on UE f_EUTRA_SwitchOnUEandStartIP(p_CellId); // Now continue with the procedure v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4(p_CellId, PREAMBLE, NORMAL); // @sic R5s194877 do extra checks now on Attach Req message sic@ if (not ispresent (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present for ENDC"); } select (v_NR_ConfigType) { // @sic R5s200421 sic@ case (EN_DC) { if (not match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_EUTRA_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for ENDC"); } } case else { // other options FFS if (not match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for NR"); } else { /* @sic R5-198996 - Assumption: additional PCOs only for IRAT test cases sic@ */ v_IMS_PDN_ConnEstab_FirstPDN := tsc_EUTRA_Registration_PDN1isIMS; v_IMS_PDN_ConnEstab_SecondPDN := not v_IMS_PDN_ConnEstab_FirstPDN; f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_IpcanInd); // @sic R5-199103: S1-N1-mode change; IMS registration at EUTRA side sic@ v_AdditionalProtocolConfigOptions_FirstPDN := f_EUTRA38_NR_AdditionalProtocolConfigOptions(p_CellId, v_IMS_PDN_ConnEstab_FirstPDN); // @sic R5s201102 sic@ v_AdditionalProtocolConfigOptions_SecondPDN := f_EUTRA38_NR_AdditionalProtocolConfigOptions(p_CellId, v_IMS_PDN_ConnEstab_SecondPDN); // @sic R5s201102 sic@ } } } … }

After Change:

function f_EUTRA38_NR_InitialRegistration(EUTRA_CellId_Type p_CellId, IDLEUPDATED_STATE_Type p_State, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (omit) PLMN_List p_EquivalentPlmnList := omit, template (omit) AbsoluteCellPower_Type p_AbsoluteCellPowerLevel_FR1 := omit, template (omit) AbsoluteCellPower_Type p_AbsoluteCellPowerLevel_FR2 := omit, boolean p_UserPlaneIntegrity := false) runs on EUTRA_5GS_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel_FR1 := tsc_ServingCellRS_EPRE; var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel_FR2 := tsc_ServingCellRS_EPRE_FR2; var template (omit) AdditionalUpdateType v_AdditionalUpdateType; var NAS_UL_Message_Type v_PdnConnectivityRequest; var ProcedureTransactionIdentifier v_EPS_TI; var template (value) ProtocolConfigOptions v_PcoToUE; var template (omit) ProtocolConfigOptions v_PcoFromUE := omit; // may be omit var template (omit) ExtdProtocolConfigOptions v_ExtdPcoToUE := omit; var template (omit) ExtdProtocolConfigOptions v_ExtdPcoFromUE := omit; var boolean v_IPv4AllocationViaNasFlag := f_CheckPCOforIPallocationViaNas(v_PcoFromUE); // See TS 36.508 v.8.2.2 Table 4.7.3_6 var APNandPCOs_Type v_APNandPCOs; var PDN_Index_Type v_PdnIndex := f_EUTRA_InitialRegistrationPdnIndex(firstPDN); var template (value) NAS_ProtocolConfigOptions_Type v_AdditionalProtocolConfigOptions_FirstPDN := {}; var template (value) NAS_ProtocolConfigOptions_Type v_AdditionalProtocolConfigOptions_SecondPDN := {}; var boolean v_IMS_PDN_ConnEstab_FirstPDN; var boolean v_IMS_PDN_ConnEstab_SecondPDN; var template (omit) EUTRA_NR_PduSessionInfo_Type v_PduSessionInfo := omit; var template (omit) integer v_SessionId := omit; var template (omit) integer v_QFI := omit; var NR_Configuration_Type v_NR_ConfigType := f_EUTRA38_NR_Config_Get(); timer t_GuardTimer := tsc_GuardTimePreamble; t_GuardTimer.start; if (ispresent(p_AbsoluteCellPowerLevel_FR1) and ispresent(p_AbsoluteCellPowerLevel_FR2)) { v_AbsoluteCellPowerLevel_FR1 := valueof(p_AbsoluteCellPowerLevel_FR1); v_AbsoluteCellPowerLevel_FR2 := valueof(p_AbsoluteCellPowerLevel_FR2); } // First switch cell back on f_EUTRA38_SetCellPower(p_CellId, v_AbsoluteCellPowerLevel_FR1, v_AbsoluteCellPowerLevel_FR2); // Switch on UE f_EUTRA_SwitchOnUEandStartIP(p_CellId); // Now continue with the procedure v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4(p_CellId, PREAMBLE, NORMAL); // @sic R5s194877 do extra checks now on Attach Req message sic@ if (not ispresent (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present for ENDC"); } select (v_NR_ConfigType) { // @sic R5s200421 sic@ case (EN_DC) { if (not match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_EUTRA_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for ENDC"); } } case else { // other options FFS if (not match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for NR"); } else { /* @sic R5-198996 - Assumption: additional PCOs only for IRAT test cases sic@ */ v_IMS_PDN_ConnEstab_FirstPDN := tsc_EUTRA_Registration_PDN1isIMS; v_IMS_PDN_ConnEstab_SecondPDN := not v_IMS_PDN_ConnEstab_FirstPDN; f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_IpcanInd); // @sic R5-199103: S1-N1-mode change; IMS registration at EUTRA side sic@ if (v_IMS_PDN_ConnEstab_FirstPDN) { v_AdditionalProtocolConfigOptions_FirstPDN := f_EUTRA38_NR_AdditionalProtocolConfigOptions(p_CellId, v_IMS_PDN_ConnEstab_FirstPDN,'5'H); // @sic R5s201102 sic@ } else { v_AdditionalProtocolConfigOptions_FirstPDN := f_EUTRA38_NR_AdditionalProtocolConfigOptions(p_CellId, v_IMS_PDN_ConnEstab_FirstPDN); // @sic R5s201102 sic@ } if (v_IMS_PDN_ConnEstab_SecondPDN) { v_AdditionalProtocolConfigOptions_FirstPDN := f_EUTRA38_NR_AdditionalProtocolConfigOptions(p_CellId, v_IMS_PDN_ConnEstab_SecondPDN,'5'H); // @sic R5s201102 sic@ } else { v_AdditionalProtocolConfigOptions_SecondPDN := f_EUTRA38_NR_AdditionalProtocolConfigOptions(p_CellId, v_IMS_PDN_ConnEstab_SecondPDN); // @sic R5s201102 sic@ } } } } … }