3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240293

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4807 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE DRX testcase 7.1.6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With the current TTCN implementation, proposed in TTCN CR R5s220066, the timing calculated at Step 3 for setting up the Test Mode config as “Normal” and the timing for the following Data transmission are the same. This does not guarantee the transmission of the data with the desired Test Mode Config. This problem can be solved using different timing for CAT-M devices (as specified in R5s220066) and “Normal” devices. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added alternative timing to set up the Test mode config depending on pc_ue_CategoryDL_M1 usage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required:	3

2.1	Correction to function f_TC_7_1_6_2_EUTRA	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required:

## Correction to function f_TC_7_1_6_2_EUTRA

| Function name | f_TC_7_1_6_2_EUTRA |
| --- | --- |
| Reason for change | With the current TTCN implementation, proposed in TTCN CR R5s220066, the timing calculated at Step 3 for setting up the Test Mode config as “Normal” and the timing for the following Data transmission are the same. This does not guarantee the transmission of the data with the desired Test Mode Config. This problem can be solved using different timing for CAT-M devices (as specified in R5s220066) and “Normal” devices. |
| Summary of change | Added alternative timing to set up the Test mode config depending on pc_ue_CategoryDL_M1 usage. |
| TTCN module | MAC_716 |

Before Change:

function f_TC_7_1_6_2_EUTRA ( ) runs on EUTRA_PTC { /* L2/MAC * 7.1.6.2 : DRX Operation / Parameters (short cycle not configured) / DRX command MAC control element reception */ //@sic R5s110173 R5-113679 sic@ const integer tsc_LongDRXCycle := 640; const integer tsc_DrxStartOffset := 4; //@sic R5s110692 sic@ const integer tsc_OnDurationTimer := 20; const integer tsc_DRXRetranTimer := 6; const integer tsc_SubFrameX := 6; const integer tsc_SubFrameY := 1; //@sic R5s210585 R5s220066 sic@ const integer tsc_SubFrameZ := 2; <<SKIPPED CODE>> /////////////////////////////////////////////////////////////////////////////////// // Test Body ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_TestBody_Set(true); f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1, cs_TimingInfo_SubFrame(8), //@sic R5s220066 sic@ cs_DciInfo_CcchDcchDtchUL_Explicit(4, 4), // Imcs = Itbs =4, Nprb Results in TB size of 328 selected to be neither too big nor too small cs_UplinkTimeAlignment_Synch_Auto_Def, //@sic R5s110618 sic@ cs_UL_GrantConfig_ContPeriodic(5)); // periodic grant every 5ms (80% of 6ms the Inactivity Timer) // Reconfigure RRC Connection: Step 0A and 0B f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig (eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_DrxL(tsc_DrxStartOffset)); //@sic R5s110698 sic@ <<SKIPPED CODE>> //@siclog "Step 3" siclog@ v_Timing := f_EUTRA_TimingInfoAdd(v_Timing1, 1); f_SS_ConfigCRC_ErrorMode(eutra_Cell1, cs_DL_SCH_CRC_C_RNTI_Error(Normal), cs_TimingInfo_EUTRA_NB(v_Timing)); v_NormalSF := f_DRX_NormalSF(v_Timing1.Subframe.Number, tsc_SubFrameY, v_EUTRA_FDD_TDD_Mode); v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing1, v_NormalSF); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing2), cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@ cs_HarqProcessAssignment_Specific(2))); <<SKIPPED CODE>> //@siclog "Step 7" siclog@ v_Timing := f_EUTRA_TimingInfoAdd(v_Timing3, 1); f_SS_ConfigCRC_ErrorMode(eutra_Cell1, cs_DL_SCH_CRC_C_RNTI_Error(Normal), cs_TimingInfo_EUTRA_NB(v_Timing)); v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer); if (v_EUTRA_FDD_TDD_Mode == FDD) //@sic R5s120658 sic@ { v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, tsc_DRXRetranTimer - tsc_SubFrameZ, v_EUTRA_FDD_TDD_Mode); } else if (v_EUTRA_FDD_TDD_Mode == TDD) { v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode); } v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU ( eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing4), cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@ cs_HarqProcessAssignment_Specific(4))); <<SKIPPED CODE>> //@siclog "Step 9" siclog@ /* v_EncodedRlcPdu := f_RLC_PDU_Encvalue (cs_RLC_AMD_1PDU(tsc_NoPoll, int2bit (2,10), f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN2, crs_PDCP_SDU_17B))))); */ if (v_EUTRA_FDD_TDD_Mode == FDD) //@sic R5s120658 sic@ { v_NormalSF := f_DRX_NormalSF(v_Timing4.Subframe.Number, tsc_SubFrameZ - 1, v_EUTRA_FDD_TDD_Mode); v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF); } else if (v_EUTRA_FDD_TDD_Mode == TDD) { v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer); v_NormalSF := f_DRX_NormalSF(v_Timing5.Subframe.Number, 5, v_EUTRA_FDD_TDD_Mode); v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing5, v_NormalSF); } <<SKIPPED CODE>>

After Change:

function f_TC_7_1_6_2_EUTRA ( ) runs on EUTRA_PTC { /* L2/MAC * 7.1.6.2 : DRX Operation / Parameters (short cycle not configured) / DRX command MAC control element reception */ //@sic R5s110173 R5-113679 sic@ const integer tsc_LongDRXCycle := 640; const integer tsc_DrxStartOffset := 4; //@sic R5s110692 sic@ const integer tsc_OnDurationTimer := 20; const integer tsc_DRXRetranTimer := 6; const integer tsc_SubFrameX := 6; var integer v_SubFrameY; var integer v_SubFrameZ; var integer v_NormalSF := 0; var SubFrameTiming_Type v_Timing := valueof(cs_SubFrameTiming(0, 0, 0)); var SubFrameTiming_Type v_Timing1 := v_Timing; var SubFrameTiming_Type v_Timing2 := v_Timing; var SubFrameTiming_Type v_Timing3 := v_Timing; var SubFrameTiming_Type v_Timing4 := v_Timing; var SubFrameTiming_Type v_Timing5 := v_Timing; var integer v_HARQRTTTimer := 0; var octetstring v_EncodedRlcPdu; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; timer t_Watchdog := 5.0; if(pc_ue_CategoryDL_M1){ v_SubFrameY := 1; v_SubFrameZ := 2; } else { v_SubFrameY := 3; v_SubFrameZ := 4; } <<SKIPPED CODE>> /////////////////////////////////////////////////////////////////////////////////// // Test Body ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_TestBody_Set(true); if (pc_ue_CategoryDL_M1) { f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1, cs_TimingInfo_SubFrame(8), //@sic R5s220066 sic@ cs_DciInfo_CcchDcchDtchUL_Explicit(4, 4), // Imcs = Itbs =4, Nprb Results in TB size of 328 selected to be neither too big nor too small cs_UplinkTimeAlignment_Synch_Auto_Def, //@sic R5s110618 sic@ cs_UL_GrantConfig_ContPeriodic(5)); // periodic grant every 5ms (80% of 6ms the Inactivity Timer) } else { f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1, cs_TimingInfo_SubFrame(4), cs_DciInfo_CcchDcchDtchUL_Explicit(4, 4), // Imcs = Itbs =4, Nprb Results in TB size of 328 selected to be neither too big nor too small cs_UplinkTimeAlignment_Synch_Auto_Def, //@sic R5s110618 sic@ cs_UL_GrantConfig_ContPeriodic(5)); // periodic grant every 5ms (80% of 6ms the Inactivity Timer) } // Reconfigure RRC Connection: Step 0A and 0B f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig (eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_DrxL(tsc_DrxStartOffset)); //@sic R5s110698 sic@ <<SKIPPED CODE>> //@siclog "Step 3" siclog@ v_Timing := f_EUTRA_TimingInfoAdd(v_Timing1, 1); f_SS_ConfigCRC_ErrorMode(eutra_Cell1, cs_DL_SCH_CRC_C_RNTI_Error(Normal), cs_TimingInfo_EUTRA_NB(v_Timing)); v_NormalSF := f_DRX_NormalSF(v_Timing1.Subframe.Number, v_SubFrameY, v_EUTRA_FDD_TDD_Mode); v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing1, v_NormalSF); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing2), cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@ cs_HarqProcessAssignment_Specific(2))); <<SKIPPED CODE>> //@siclog "Step 7" siclog@ v_Timing := f_EUTRA_TimingInfoAdd(v_Timing3, 1); f_SS_ConfigCRC_ErrorMode(eutra_Cell1, cs_DL_SCH_CRC_C_RNTI_Error(Normal), cs_TimingInfo_EUTRA_NB(v_Timing)); v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer); if (v_EUTRA_FDD_TDD_Mode == FDD) //@sic R5s120658 sic@ { v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, tsc_DRXRetranTimer - v_SubFrameZ, v_EUTRA_FDD_TDD_Mode); } else if (v_EUTRA_FDD_TDD_Mode == TDD) { v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode); } v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU ( eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing4), cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@ cs_HarqProcessAssignment_Specific(4))); <<SKIPPED CODE>> //@siclog "Step 9" siclog@ /* v_EncodedRlcPdu := f_RLC_PDU_Encvalue (cs_RLC_AMD_1PDU(tsc_NoPoll, int2bit (2,10), f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN2, crs_PDCP_SDU_17B))))); */ if (v_EUTRA_FDD_TDD_Mode == FDD) //@sic R5s120658 sic@ { v_NormalSF := f_DRX_NormalSF(v_Timing4.Subframe.Number, v_SubFrameZ - 1, v_EUTRA_FDD_TDD_Mode); v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF); } else if (v_EUTRA_FDD_TDD_Mode == TDD) { v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer); v_NormalSF := f_DRX_NormalSF(v_Timing5.Subframe.Number, 5, v_EUTRA_FDD_TDD_Mode); v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing5, v_NormalSF); } <<SKIPPED CODE>>