3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250277

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4918 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Rel-12 LTE RRC ACB test case 8.1.2.13a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-07-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To verify LTE RRC testcase 8.1.2.13a using iWD D25wk24 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to LTE RRC testcase 8.1.2.13a required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.13a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to function f_TC_8_1_2_13x	3

3	Branches executed	11

4	Execution Log Files	11

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.2.13a which is part of the LTE_A_R12 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendra Kalahasti
                         Narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.2.13a

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6897 UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_2_13x |
| --- | --- |
| Reason for change | - After setting SIB2 of Cell 11 during initialisation it is not necessary to call the function f_EUTRA_SS_ConfigureSysinfo, since cell isn’t configured & created yet. - PLMNid set in the SIB1 of cell 11 is not aligned with Table 8.1.2.13a.3.3-1, TS 36.523-1 - SIB1 modifications done during the initialisation of testcase will not be effective since the function f_EUTRA_CellConfig_Def is called before the SIB1 modifications. - At step 7, in case of else condition, SS doesn’t transmit paging to indicate systemInfo modification. |
| Summary of change | - Remove the function f_EUTRA_SS_ConfigureSysinfo for cell 11 - Modify the PLMNid the match to that of prose - Move the functions to create & configure the cells after the initial SIB modifications - Move the functions to update systemInfoValueTag & transmit paging outside the if-else condition |
| TTCN module | \LTE_A_R12\8_1\RRC_ConnEst_R12.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_2_13x (boolean p_ACB_Per_PLMN)runs on EUTRA_PTC { var GutiParameters_Type v_GutiParameters; var NAS_MSG_Indication_Type v_NasInd; var template (value) CellPowerList_Type v_CellPowerList_AtT1; var template (value) CellPowerList_Type v_CellPowerList_AtT2; var float v_TimerValue; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod_s; var NAS_KsiValue v_KsiValue; var boolean v_RrcMsgReceived; var boolean v_DataPduReceived; var PDCP_SDUList_Type v_PDCP_SDUList_DRB1 := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; timer t_Wait; f_EUTRA_Init(c1); v_GutiParameters := f_EUTRA_CellInfo_GetGuti (eutra_Cell1); //Set cell power level for cell 11 to be used at creation f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell11, -79); // Set Sys Info for cell 11 to be used at creation if (not p_ACB_Per_PLMN){ f_EUTRA_CellInfo_SetSysInfo_AccessBarring(eutra_Cell11, cs_AccessBarringInformation_Def(false, cs_AC_BarringConfig(p00, s512, '11111'B), omit));//@sic R5s240444 sic@); } else { f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell11, 1, false,omit, omit, cs_AC_BarringConfig(p00, s512, '11111'B)); f_EUTRA_SS_ConfigureSysinfo (eutra_Cell11); } //Calculate length of System Information Modification Period v_ModificationPeriodFrames := f_CalculateModificationPeriod(eutra_Cell1); //@sic R5-113705 sic@ //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell11); if (p_ACB_Per_PLMN){ f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell1, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_001_02, //PLMNid 2 (index 1) -, true, 1); f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell11, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_002_11, //PLMNid 2 (index 1) -, true, 1); } // The UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18]. f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeB); f_EUTRA_RbEst_Def(eutra_Cell1); f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1, '05'O); //Cell power levels at T1 v_CellPowerList_AtT1 := { cs_CellPower(eutra_Cell11, -79) }; //Cell power levels at T2 v_CellPowerList_AtT2 := { cs_CellPower(eutra_Cell11, -91) }; f_EUTRA_TestBody_Set(true); //@siclog "Step 1" siclog@ //The SS transmits an RRCConnectionRelease message on Cell 1. f_EUTRA_RRC_ConnectionRelease( eutra_Cell1); //@siclog "Step 2" siclog@ //Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. f_Delay( 5.0); //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T1" in table 8.1.2.13.3.2-1. f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message within 40s? (Note 1) v_TimerValue := 40.0; if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell11, v_TimerValue)) { // @sic R5s110507 Additional Changes sic@ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.1.2.13 Step 4"); } //@siclog "Step 5" siclog@ //The SS transmits a Paging message including a matched identity allocated in Cell 1. f_EUTRA_UE_Page(eutra_Cell1, cs_NasPaging_OneRecord(cs_PagingUE_Identity_S_TMSI(v_GutiParameters.MME_Code, v_GutiParameters.M_TMSI))); //@siclog "Step 6" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message on Cell 1 within 30s? (Note 2) v_TimerValue := 30.0; if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell1, v_TimerValue)) { // @sic R5s110507 Additional Changes sic@ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.1.2.13 Step 6"); } //@siclog "Step 7" siclog@ //The SS changes the SystemInformationBlockType1 and SystemInformationBlockType2 on Cell 1 according to Table 8.1.2.13.3.3-1, -2, and -2A. if (not p_ACB_Per_PLMN){ f_EUTRA_CellInfo_SetSysInfo_AccessBarring(eutra_Cell1, cs_AccessBarringInformation_Def(false, cs_AC_BarringConfig(p00, s8, '11111'B), cs_AC_BarringConfig(p00, s512, '11111'B))); //@sic R5s240444 sic@); f_EUTRA_SetSB1_ValueTag(eutra_Cell1, true); f_EUTRA_CellInfo_IncrementValueTagSI_SIB2(eutra_Cell1); //@sic R5-171539 sic@ f_EUTRA_ModifySysinfo(eutra_Cell1); //@sic R5s110285 sic@ } else{ f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false,omit, omit, cs_AC_BarringConfig(p00, s8, '11111'B), cs_AC_BarringConfig(p00, s512, '11111'B)); f_EUTRA_SS_ConfigureSysinfo (eutra_Cell11); } //@siclog "Step 7A" siclog@ //Wait for 2.1* modification period (Note 5) to allow the new system information to take effect. v_ModificationPeriod_s := int2float( v_ModificationPeriodFrames)/100.0; //Modification period in seconds //@sic R5-113705 sic@ f_Delay( 2.1 * v_ModificationPeriod_s); //@sic R5-113705 sic@ // SKIPPED CODE }

After Change

function f_TC_8_1_2_13x (boolean p_ACB_Per_PLMN)runs on EUTRA_PTC { var GutiParameters_Type v_GutiParameters; var NAS_MSG_Indication_Type v_NasInd; var template (value) CellPowerList_Type v_CellPowerList_AtT1; var template (value) CellPowerList_Type v_CellPowerList_AtT2; var float v_TimerValue; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod_s; var NAS_KsiValue v_KsiValue; var boolean v_RrcMsgReceived; var boolean v_DataPduReceived; var PDCP_SDUList_Type v_PDCP_SDUList_DRB1 := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; timer t_Wait; f_EUTRA_Init(c1); v_GutiParameters := f_EUTRA_CellInfo_GetGuti (eutra_Cell1); //Set cell power level for cell 11 to be used at creation f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell11, -79); // Set Sys Info for cell 11 to be used at creation if (not p_ACB_Per_PLMN){ f_EUTRA_CellInfo_SetSysInfo_AccessBarring(eutra_Cell11, cs_AccessBarringInformation_Def(false, cs_AC_BarringConfig(p00, s512, '11111'B), omit));//@sic R5s240444 sic@); } else { f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell11, 1, false,omit, omit, cs_AC_BarringConfig(p00, s512, '11111'B)); //REMOVED f_EUTRA_SS_ConfigureSysinfo (eutra_Cell11); } //Calculate length of System Information Modification Period v_ModificationPeriodFrames := f_CalculateModificationPeriod(eutra_Cell1); //@sic R5-113705 sic@ if (p_ACB_Per_PLMN){ f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell1, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_001_02, //PLMNid 2 (index 1) -, true, 1); f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell11, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_001_02, //PLMNid 2 (index 1) -, true, 1); } //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell11); // The UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18]. f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeB); f_EUTRA_RbEst_Def(eutra_Cell1); f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1, '05'O); //Cell power levels at T1 v_CellPowerList_AtT1 := { cs_CellPower(eutra_Cell11, -79) }; //Cell power levels at T2 v_CellPowerList_AtT2 := { cs_CellPower(eutra_Cell11, -91) }; f_EUTRA_TestBody_Set(true); //@siclog "Step 1" siclog@ //The SS transmits an RRCConnectionRelease message on Cell 1. f_EUTRA_RRC_ConnectionRelease( eutra_Cell1); //@siclog "Step 2" siclog@ //Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. f_Delay( 5.0); //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T1" in table 8.1.2.13.3.2-1. f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message within 40s? (Note 1) v_TimerValue := 40.0; if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell11, v_TimerValue)) { // @sic R5s110507 Additional Changes sic@ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.1.2.13 Step 4"); } //@siclog "Step 5" siclog@ //The SS transmits a Paging message including a matched identity allocated in Cell 1. f_EUTRA_UE_Page(eutra_Cell1, cs_NasPaging_OneRecord(cs_PagingUE_Identity_S_TMSI(v_GutiParameters.MME_Code, v_GutiParameters.M_TMSI))); //@siclog "Step 6" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message on Cell 1 within 30s? (Note 2) v_TimerValue := 30.0; if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell1, v_TimerValue)) { // @sic R5s110507 Additional Changes sic@ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.1.2.13 Step 6"); } //@siclog "Step 7" siclog@ //The SS changes the SystemInformationBlockType1 and SystemInformationBlockType2 on Cell 1 according to Table 8.1.2.13.3.3-1, -2, and -2A. if (not p_ACB_Per_PLMN){ f_EUTRA_CellInfo_SetSysInfo_AccessBarring(eutra_Cell1, cs_AccessBarringInformation_Def(false, cs_AC_BarringConfig(p00, s8, '11111'B), cs_AC_BarringConfig(p00, s512, '11111'B))); //@sic R5s240444 sic@); } else{ f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false,omit, omit, cs_AC_BarringConfig(p00, s8, '11111'B), cs_AC_BarringConfig(p00, s512, '11111'B)); // REMOVED f_EUTRA_SS_ConfigureSysinfo (eutra_Cell11); } f_EUTRA_SetSB1_ValueTag(eutra_Cell1, true); f_EUTRA_CellInfo_IncrementValueTagSI_SIB2(eutra_Cell1); //@sic R5-171539 sic@ f_EUTRA_ModifySysinfo(eutra_Cell1); //@sic R5s110285 sic@ //@siclog "Step 7A" siclog@ //Wait for 2.1* modification period (Note 5) to allow the new system information to take effect. v_ModificationPeriod_s := int2float( v_ModificationPeriodFrames)/100.0; //Modification period in seconds //@sic R5-113705 sic@ f_Delay( 2.1 * v_ModificationPeriod_s); //@sic R5-113705 sic@ // SKIPPED CODE }

# Branches executed

The test case was executed on LTE primary FDD band 3 using LTE ciphering eea1 and integrity algorithm eia1.

# Execution Log Files

  - Mediatek MT6897 UE

Mediatek MT6897 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_2_13a_LOG.xml

- PICS/PIXIT parameter file:

TC_8_1_2_13a_PIXIT.txt

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250278: Supporting information for agreement of TC 8.1.2.13a |
| --- | --- |