3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250353

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3740 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_Capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In case of MFBI, the UE_CapabilityRequestFilterNR needs to filter the supported band and not the other overlapping on-supported band, otherwise UE may return empty capability list and test fails unfairly during capability check in preamble. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For MFBI the supported overlapping band is populated under the CapabilityRequestFilterNR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may return empty capability list during preamble and fail an MFBI test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | MFBI tests (6.1.2.23.NR5GC) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact: 	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NR_Capability |
| --- | --- |
| Reason for change | In case of MFBI, the UE_CapabilityRequestFilterNR needs to filter the supported band and not the other overlapping on-supported band, otherwise UE may return empty capability list and test fails unfairly during capability check in preamble. |
| Summary of change | For MFBI the supported overlapping band is populated under the CapabilityRequestFilterNR |
| TTCN module | NR5GC_RRCSteps.ttcn |

Before Change:

function f_NR_Capability(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var integer v_NRBandInd := f_NR_CellInfo_GetBandIndicator(p_NR_CellId); var template (omit) FreqBandIndicatorNR v_MFBI_Band := f_NR_CellInfo_GetMFBI_Band(p_NR_CellId); var UE_NR_Capability v_NrCapability; var integer i; var boolean v_Found1stBand := false; var boolean v_Found2ndBand := false; var template (value) FreqBandInformation v_FirstBand := cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(p_NR_CellId)); // @sic R5s200933 sic@ var template (value) FreqBandInformation v_SecondaryBand; var template (value) FreqBandList v_FreqBandList := { v_FirstBand }; var octetstring v_EncodeCapRequestNR; var boolean v_CA_InterBand := false; var boolean v_NR_DC_Band := false; var template (omit) octetstring v_Enquiry_v1560 := omit; var octetstring v_EncodeCapRequestEUTRANR; var template (value) DL_DCCH_Message v_DL_DCCH_Message; var integer v_EUTRA_Band; var boolean v_NEDC_Band := false; var UE_MRDC_Capability v_NrCapability_MRDC; if (isvalue (v_MFBI_Band)) { // @sic R5s200826 R5s201305 sic@ v_NRBandInd := valueof(v_MFBI_Band); } if (f_GetTestcaseAttrib_NR_CA(testcasename())) { //@sic R5-204397 sic@ if (f_NR_GetCA_CombinationType(px_NR_CA_BandCombination) == CA_InterBand) { //For CA interBand includes 2 entries as per 38.508-1 Table 4.6.4-21: FreqBandList v_CA_InterBand := true; // @sic R5-204397 sic@ v_SecondaryBand := cs_NR_BandInformationNR(f_NR_GetCA_SecondaryBand(px_NR_CA_BandCombination)); v_FreqBandList := { v_FirstBand, v_SecondaryBand }; } } if (f_GetTestcaseAttrib_NR_DC(testcasename())) { // @sic R5-215679 sic@ v_NR_DC_Band := true; v_SecondaryBand := cs_NR_BandInformationNR(f_NR_GetNR_DC_SecondaryBand (px_NR_DC_BandCombination)); v_FreqBandList := { v_FirstBand, v_SecondaryBand }; // Table 4.6.1-31: UECapabilityEnquiry with condition NR-DC - Include UE-CapabilityRequestFilterCommon with consition NR-DC v_Enquiry_v1560 := bit2oct(encvalue( cs_UECapabilityEnquiry_v1560_Def(cs_UE_CapabilityRequestFilterCommon))); //@sic R5s253066 sic@ } if (f_GetTestcaseAttrib_NEDC(testcasename())) { // @sic R5-221422 sic@ v_NEDC_Band := true; v_EUTRA_Band := f_NEDC_GetEUTRA_Band(px_NEDC_BandCombination); v_FreqBandList := { v_FirstBand }; v_EncodeCapRequestEUTRANR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR( {cs_NR_BandInformationEUTRA(v_EUTRA_Band), v_FirstBand}))); } v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR(v_FreqBandList) )); // @sic R5-213580 R5-221422 sic@ if (v_NEDC_Band) { v_DL_DCCH_Message := cs_38508_UECapabilityEnquiryMultipleEntries (tsc_NR_RRC_TI_Def, {cs_UE_CapabilityRAT_Request( nr, v_EncodeCapRequestNR), cs_UE_CapabilityRAT_Request(eutra_nr, v_EncodeCapRequestEUTRANR)}, v_Enquiry_v1560); } else { v_DL_DCCH_Message := cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR, v_Enquiry_v1560); } SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, -, v_DL_DCCH_Message)); // @sic R5s200933 R5-210414 R5-221422 sic@ SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def))) -> value v_ReceivedAspForUeCapabilityInfo; // Decode and store the UE-CapabilityRAT-Container if (v_NEDC_Band) { //@sic R5-221422 sic@ if (v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].rat_Type == nr) { // First entry is nr, second enry is eutra_nr v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); v_NrCapability_MRDC := f_NR_DecodeNrCapMsg_MRDC(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[1].ue_CapabilityRAT_Container); } else { // First entry is eutra_nr, second enry is nr v_NrCapability_MRDC := f_NR_DecodeNrCapMsg_MRDC(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[1].ue_CapabilityRAT_Container); } } else { v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); } f_NR_MobileInfo_NRCap_Set(v_NrCapability); // Check the RedCap fl_NR_CheckRedCapUEIndicator(v_NrCapability); // @sic R5s240258 sic@ // Check the UE release f_NR_CheckReleaseIndicator(v_NrCapability.accessStratumRelease); select (v_NrCapability.accessStratumRelease) { case (rel15, rel16, rel17, rel18) { // @sic R5s201387 BASELINE MOVING 2020 R5s221179 BASELINE MOVING 2022 rel-17 R5s240531 BASELINE MOVING 2024 rel-18 sic@ } case else{ FatalError (__FILE__, __LINE__, "accessStratumRelease not yet considered"); } } for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_NRBandInd)) { v_Found1stBand := true; break; } } if (v_CA_InterBand) { // @sic R5-204397 sic@ for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_SecondaryBand.bandInformationNR.bandNR)) { v_Found2ndBand := true; break; } } } if (v_NR_DC_Band) { // @sic R5-215679 sic@ for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_SecondaryBand.bandInformationNR.bandNR)) { v_Found2ndBand := true; break; } } } if (v_NEDC_Band) { //@sic R5-221422 sic@ // Check MRDC capabilities for (i := 0; i < lengthof(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter); i := i+1) { if (match(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter[i].bandInformationEUTRA.bandEUTRA, v_EUTRA_Band ) and match(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter[i].bandInformationNR.bandNR, v_NRBandInd)) { v_Found2ndBand := true; break; } } } if (not v_Found1stBand or ((v_NR_DC_Band or v_CA_InterBand or v_NEDC_Band) and not v_Found2ndBand)) { // @sic R5s190109, R5-204397, R5-215679 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NR Freq Band Ind must match value sent"); } }

After Change:

function f_NR_Capability(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var integer v_NRBandInd := f_NR_CellInfo_GetBandIndicator(p_NR_CellId); var template (omit) FreqBandIndicatorNR v_MFBI_Band := f_NR_CellInfo_GetMFBI_Band(p_NR_CellId); var UE_NR_Capability v_NrCapability; var integer i; var boolean v_Found1stBand := false; var boolean v_Found2ndBand := false; var template (value) FreqBandInformation v_FirstBand := cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(p_NR_CellId)); // @sic R5s200933 sic@ var template (value) FreqBandInformation v_SecondaryBand; var template (value) FreqBandList v_FreqBandList := { v_FirstBand }; var octetstring v_EncodeCapRequestNR; var boolean v_CA_InterBand := false; var boolean v_NR_DC_Band := false; var template (omit) octetstring v_Enquiry_v1560 := omit; var octetstring v_EncodeCapRequestEUTRANR; var template (value) DL_DCCH_Message v_DL_DCCH_Message; var integer v_EUTRA_Band; var boolean v_NEDC_Band := false; var UE_MRDC_Capability v_NrCapability_MRDC; if (isvalue (v_MFBI_Band)) { // @sic R5s200826 R5s201305 sic@ v_NRBandInd := valueof(v_MFBI_Band); v_FreqBandList[0].bandInformationNR.bandNR := v_NRBandInd; } if (f_GetTestcaseAttrib_NR_CA(testcasename())) { //@sic R5-204397 sic@ if (f_NR_GetCA_CombinationType(px_NR_CA_BandCombination) == CA_InterBand) { //For CA interBand includes 2 entries as per 38.508-1 Table 4.6.4-21: FreqBandList v_CA_InterBand := true; // @sic R5-204397 sic@ v_SecondaryBand := cs_NR_BandInformationNR(f_NR_GetCA_SecondaryBand(px_NR_CA_BandCombination)); v_FreqBandList := { v_FirstBand, v_SecondaryBand }; } } if (f_GetTestcaseAttrib_NR_DC(testcasename())) { // @sic R5-215679 sic@ v_NR_DC_Band := true; v_SecondaryBand := cs_NR_BandInformationNR(f_NR_GetNR_DC_SecondaryBand (px_NR_DC_BandCombination)); v_FreqBandList := { v_FirstBand, v_SecondaryBand }; // Table 4.6.1-31: UECapabilityEnquiry with condition NR-DC - Include UE-CapabilityRequestFilterCommon with consition NR-DC v_Enquiry_v1560 := bit2oct(encvalue( cs_UECapabilityEnquiry_v1560_Def(cs_UE_CapabilityRequestFilterCommon))); //@sic R5s253066 sic@ } if (f_GetTestcaseAttrib_NEDC(testcasename())) { // @sic R5-221422 sic@ v_NEDC_Band := true; v_EUTRA_Band := f_NEDC_GetEUTRA_Band(px_NEDC_BandCombination); v_FreqBandList := { v_FirstBand }; v_EncodeCapRequestEUTRANR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR( {cs_NR_BandInformationEUTRA(v_EUTRA_Band), v_FirstBand}))); } v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR(v_FreqBandList) )); // @sic R5-213580 R5-221422 sic@ if (v_NEDC_Band) { v_DL_DCCH_Message := cs_38508_UECapabilityEnquiryMultipleEntries (tsc_NR_RRC_TI_Def, {cs_UE_CapabilityRAT_Request( nr, v_EncodeCapRequestNR), cs_UE_CapabilityRAT_Request(eutra_nr, v_EncodeCapRequestEUTRANR)}, v_Enquiry_v1560); } else { v_DL_DCCH_Message := cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR, v_Enquiry_v1560); } SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, -, v_DL_DCCH_Message)); // @sic R5s200933 R5-210414 R5-221422 sic@ SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def))) -> value v_ReceivedAspForUeCapabilityInfo; // Decode and store the UE-CapabilityRAT-Container if (v_NEDC_Band) { //@sic R5-221422 sic@ if (v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].rat_Type == nr) { // First entry is nr, second enry is eutra_nr v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); v_NrCapability_MRDC := f_NR_DecodeNrCapMsg_MRDC(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[1].ue_CapabilityRAT_Container); } else { // First entry is eutra_nr, second enry is nr v_NrCapability_MRDC := f_NR_DecodeNrCapMsg_MRDC(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[1].ue_CapabilityRAT_Container); } } else { v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); } f_NR_MobileInfo_NRCap_Set(v_NrCapability); // Check the RedCap fl_NR_CheckRedCapUEIndicator(v_NrCapability); // @sic R5s240258 sic@ // Check the UE release f_NR_CheckReleaseIndicator(v_NrCapability.accessStratumRelease); select (v_NrCapability.accessStratumRelease) { case (rel15, rel16, rel17, rel18) { // @sic R5s201387 BASELINE MOVING 2020 R5s221179 BASELINE MOVING 2022 rel-17 R5s240531 BASELINE MOVING 2024 rel-18 sic@ } case else{ FatalError (__FILE__, __LINE__, "accessStratumRelease not yet considered"); } } for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_NRBandInd)) { v_Found1stBand := true; break; } } if (v_CA_InterBand) { // @sic R5-204397 sic@ for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_SecondaryBand.bandInformationNR.bandNR)) { v_Found2ndBand := true; break; } } } if (v_NR_DC_Band) { // @sic R5-215679 sic@ for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_SecondaryBand.bandInformationNR.bandNR)) { v_Found2ndBand := true; break; } } } if (v_NEDC_Band) { //@sic R5-221422 sic@ // Check MRDC capabilities for (i := 0; i < lengthof(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter); i := i+1) { if (match(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter[i].bandInformationEUTRA.bandEUTRA, v_EUTRA_Band ) and match(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter[i].bandInformationNR.bandNR, v_NRBandInd)) { v_Found2ndBand := true; break; } } } if (not v_Found1stBand or ((v_NR_DC_Band or v_CA_InterBand or v_NEDC_Band) and not v_Found2ndBand)) { // @sic R5s190109, R5-204397, R5-215679 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NR Freq Band Ind must match value sent"); } }