3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250365

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3745 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC eCall over IMS test case 11.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 11.5.4 is failing at step-7 because the timer T300 started by TTCN for parallel behavior at step-4 is 300s, but the test case uses default value of T300 which is defined in TS 38.508-1, Table 4.6.3-200 as 1 sec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the value of timer T300 from 300s to 1s. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function fl_TC_11_5_4_NR5GC_TestBody() runs on NR_BASE_PTC |
| --- | --- |
| Reason for change | The test case 11.5.4 is failing at step-7 because the timer T300 started by TTCN for parallel behavior at step-4 is 300s, but the test case uses default value of T300 which is defined in TS 38.508-1, Table 4.6.3-200 as 1 sec. |
| Summary of change | Changed the value of timer T300 from 300s to 1s. |
| TTCN module | ECall_IRAT_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_5_4_NR5GC_TestBody() runs on NR_BASE_PTC { timer t_Wait := 300.0; //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 2" siclog@ // The UE is Switched ON f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state. f_Delay(60.0); // @siclog "Step 4" siclog@ //An automatic eCall is initiated. (Note 1) f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); f_UT_InitiateECall(UT, AUTOMATIC); //@siclog "Step 5" siclog@ //The UE transmits preamble on PRACH. //exception:The preamble and no response below are repeated for the duration of T300. t_Wait.start; alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) { repeat; } []t_Wait.timeout{} }; // @siclog "Step 6" siclog@ // Change the power level in EUTRA side f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); // Now tell EUTRA test is finished f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

After Change:

| function fl_TC_11_5_4_NR5GC_TestBody() runs on NR_BASE_PTC { timer t_Wait := 1.0; //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 2" siclog@ // The UE is Switched ON f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state. f_Delay(60.0); // @siclog "Step 4" siclog@ //An automatic eCall is initiated. (Note 1) f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); f_UT_InitiateECall(UT, AUTOMATIC); //@siclog "Step 5" siclog@ //The UE transmits preamble on PRACH. //exception:The preamble and no response below are repeated for the duration of T300. t_Wait.start; alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) { repeat; } []t_Wait.timeout{} }; // @siclog "Step 6" siclog@ // Change the power level in EUTRA side f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); // Now tell EUTRA test is finished f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); } |  |
| --- | --- |