3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250437

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4938 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the current Prose, Cell12 will not be applicable until Step101. Hence the UE may not include IE registeredMME at Step 10, because this has not changed from earlier value. This is also inline with 36.331 cl 5.3.3.4 : Quote 2> if upper layers provide the 'Registered MME', include and set the registeredMME as follows: 3> if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers: 4> include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers; Unquote Thus, the content of rrcConnectionSetupComplete at step 10 needs to be updated so that the registeredMME should not be checked. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to not checking the registeredMME at step 10. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change to the fl_TC_22_5_4_Body	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the fl_TC_22_5_4_Body

| Function name | fl_TC_22_5_4_Body () |
| --- | --- |
| Reason for change | As per the current Prose, Cell12 will not be applicable until Step101. Hence the UE may not include IE registeredMME at Step 10, because this has not changed from earlier value. This is also inline with 36.331 cl 5.3.3.4 : Quote 2> if upper layers provide the 'Registered MME', include and set the registeredMME as follows: 3> if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers: 4> include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers; Unquote Thus, the content of rrcConnectionSetupComplete at step 10 needs to be updated so that the registeredMME should not be checked. |
| Summary of change | It is proposed to not checking the registeredMME at step 10 |
| TTCN module | NBIOT_NAS_Attach.ttcn |

Before Change:

function fl_TC_22_5_4_Body(GutiParameters_Type p_GutiOldParams) runs on NBIOT_PTC { // ========================================================================== // initialize constant test case data structures // ========================================================================== var NAS_MSG_Indication_Type v_NasInd; var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B); var template (present) NAS_UL_Message_Type v_ESMMessage := ?; var GutiParameters_Type v_Guti_Params := p_GutiOldParams; // @sic R5s250130 sic@ var NAS_PlmnId v_NAS_PlmnId := f_Asn2Nas_PlmnId(v_Guti_Params.PLMN_Identity); var template (value) NB_CellPowerList_Type v_CellPowerList; var NBIOT_SecurityParams_Type v_SecurityParams; // @sic R5-253059 sic@ var float v_T3346Val; //@siclog "Step 1" siclog@ //Void // @sic R5-251349 sic@ …… //@siclog "Step 9" siclog@ //SS transmits an RRCConnectionSetup-NB. f_NBIOT_RRC_ConnectionSetup_Def(); //@siclog "Step 10" siclog@ //EXCEPTION: Steps 10a1 and 10b1 describe behaviour that depends on UE capabilities; //the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. //IF px_DoAttachWithoutPDN THEN //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message containing an ATTACH REQUEST message (indicating GUTI, last visited registered TAI and KSI as per the registration which happened in the preamble), and, an ESM DUMMY MESSAGE? //ELSE //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message containing an ATTACH REQUEST message (indicating GUTI, last visited registered TAI and KSI as per the registration which happened in the preamble), and, a PDN CONNECTIVITY REQUEST? v_NasInd := f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1, // @sic R5-251349 sic@ cr_508_RRCConnectionSetupComplete_NB(-, -, -, ?) cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cdr_CIOT_ATTACH_REQUEST_WithGuti(cr_AdditionalUpdateType_CIOT, ?, ?, // @sic R5-253059 sic@ ?), // @sic R5-253059 sic@ v_ESMMessage)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Step 11-20b1" siclog@ //Steps 5-14b1 from the Generic procedure 'NB-IoT UE Attach, Connected mode (State 2-NB)' as described in TS 36.508 [18], clause 8.1.5.2 take place. f_NBIOT_InitialRegistration_Step5_14 (nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NasInd); // @sic R5-251349 sic@ …… }

After Change:

function fl_TC_22_5_4_Body(GutiParameters_Type p_GutiOldParams) runs on NBIOT_PTC { // ========================================================================== // initialize constant test case data structures // ========================================================================== var NAS_MSG_Indication_Type v_NasInd; var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B); var template (present) NAS_UL_Message_Type v_ESMMessage := ?; var GutiParameters_Type v_Guti_Params := p_GutiOldParams; // @sic R5s250130 sic@ var NAS_PlmnId v_NAS_PlmnId := f_Asn2Nas_PlmnId(v_Guti_Params.PLMN_Identity); var template (value) NB_CellPowerList_Type v_CellPowerList; var NBIOT_SecurityParams_Type v_SecurityParams; // @sic R5-253059 sic@ var float v_T3346Val; //@siclog "Step 1" siclog@ //Void // @sic R5-251349 sic@ …… //@siclog "Step 9" siclog@ //SS transmits an RRCConnectionSetup-NB. f_NBIOT_RRC_ConnectionSetup_Def(); //@siclog "Step 10" siclog@ //EXCEPTION: Steps 10a1 and 10b1 describe behaviour that depends on UE capabilities; //the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. //IF px_DoAttachWithoutPDN THEN //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message containing an ATTACH REQUEST message (indicating GUTI, last visited registered TAI and KSI as per the registration which happened in the preamble), and, an ESM DUMMY MESSAGE? //ELSE //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message containing an ATTACH REQUEST message (indicating GUTI, last visited registered TAI and KSI as per the registration which happened in the preamble), and, a PDN CONNECTIVITY REQUEST? v_NasInd := f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1, // @sic R5-251349 sic@ cr_508_RRCConnectionSetupComplete_NB, //WA#WI cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cdr_CIOT_ATTACH_REQUEST_WithGuti(cr_AdditionalUpdateType_CIOT, ?, ?, // @sic R5-253059 sic@ ?), // @sic R5-253059 sic@ v_ESMMessage)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Step 11-20b1" siclog@ //Steps 5-14b1 from the Generic procedure 'NB-IoT UE Attach, Connected mode (State 2-NB)' as described in TS 36.508 [18], clause 8.1.5.2 take place. f_NBIOT_InitialRegistration_Step5_14 (nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NasInd); // @sic R5-251349 sic@ …… }