3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250360

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4936 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_CheckNotSupportedBand |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-09-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TTCN CR R5s250295 submitted by Anritsu, the following issue has been highlighted: When a band > 64 is passed on to the function f_EUTRA_CheckNotSupportedBand it will lead to runtime error, since a UE which doesn’t support any band > 64 may not report rf_Parameters_v9e0 IE and TTCN tries to assign the contents of this IE to a variable without checking if the IE has any value. To address the issue, they suggested verifying whether the rf_Parameters_v9e0 Information Element (IE) contains any value before proceeding with the assignment of the variable v_SupportedBandListEUTRA_v9e0. This solution appears to be a valid modification. However, to implement this change they have modified the variable SupportedBandListEUTRA_v9e0 as template having an omit value. This is causing Runtime error during compilation when this template is compared against an integer variable p_Band in later part of the code. Hence this needs to be resolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to revert the template SupportedBandListEUTRA_v9e0 as variable. Moreover, a further check if SupportedBandListEUTRA_v9e0 is present or not before comparing this variable against p_Band inside the loop. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.19, 6.1.2.20, 6.1.2.21, 6.1.2.22, 6.2.3.34, 6.2.3.35 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:		Bhise Parikshit

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_EUTRA_CheckNotSupportedBand |
| --- | --- |
| Reason for change | In TTCN CR R5s250295 submitted by Anritsu, the following issue has been highlighted: When a band > 64 is passed on to the function f_EUTRA_CheckNotSupportedBand it will lead to runtime error, since a UE which doesn’t support any band > 64 may not report rf_Parameters_v9e0 IE and TTCN tries to assign the contents of this IE to a variable without checking if the IE has any value. To address the issue, they suggested verifying whether the rf_Parameters_v9e0 Information Element (IE) contains any value before proceeding with the assignment of the variable v_SupportedBandListEUTRA_v9e0. This solution appears to be a valid modification. However, to implement this change they have modified the variable SupportedBandListEUTRA_v9e0 as template having an omit value. This is causing Runtime error during compilation when this template is compared against an integer variable p_Band in later part of the code. |
| Summary of change | It is proposed to revert the template SupportedBandListEUTRA_v9e0 as variable. Moreover, a further check if SupportedBandListEUTRA_v9e0 is present or not before comparing this variable against p_Band inside the loop. |
| TTCN module | EUTRA_IdleMode.ttcn |
| MCC160 Comment | Accepted in principle. MCC160 implementation see below. |

Before Change

function f_EUTRA_CheckNotSupportedBand(integer p_Band, template (omit) octetstring p_EUTRA_Cap := omit) runs on EUTRA_PTC { var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); var UE_EUTRA_Capability v_EUTRA_Cap; var SupportedBandListEUTRA v_EUTRA_BandListReported; var template SupportedBandListEUTRA_v9e0 v_SupportedBandListEUTRA_v9e0 := omit; var UE_EUTRA_Capability_v9a0_IEs v_UE_EUTRA_Capability_v9a0_IEs; var bitstring v_UE_EUTRA_Capability_v9a0_IEsContainer; var boolean v_BandReported := false; var integer i:=0; var integer v_Result; if (isvalue(p_EUTRA_Cap)) { //@sic R5s150635 change 2 sic@ v_EUTRA_Cap := f_EUTRA_DecodeEutraCapMsg(valueof(p_EUTRA_Cap)); } else { v_EUTRA_Cap := f_EUTRA_MobileInfo_GetUECapability(); } v_EUTRA_BandListReported := v_EUTRA_Cap.rf_Parameters.supportedBandListEUTRA; //@sic R5s150635 change 2 sic@ if (p_Band > 64 ) { //@sic R5s200365 sic@ if (isvalue(v_EUTRA_Cap.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension)) { v_UE_EUTRA_Capability_v9a0_IEsContainer := oct2bit(v_EUTRA_Cap.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension); v_Result := decvalue(v_UE_EUTRA_Capability_v9a0_IEsContainer, v_UE_EUTRA_Capability_v9a0_IEs); if (v_Result != 0) { FatalError(__FILE__, __LINE__, "UE_EUTRA_Capability_v9a0_IEs cannot be decoded"); } v_SupportedBandListEUTRA_v9e0 := v_UE_EUTRA_Capability_v9a0_IEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v9e0.supportedBandListEUTRA_v9e0; } else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE_EUTRA_Capability_v9a0_IEs not provided"); } } //Check the band support against the PICS if ((v_SuppEutraBandsAccToPICS[p_Band-1] == '1'B)) { f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Band is wrongly reported as supported in the PICS"); } //Check the band support against the UE capabilities if ( p_Band < 64){ for (i:=0; (i < lengthof(v_EUTRA_BandListReported) and v_BandReported==false) ; i:=i+1) { if (v_EUTRA_BandListReported[i].bandEUTRA == p_Band){ v_BandReported:= true; } } } else { //Band > 64 @sic R5s200365 sic@ for (i:=0; (i < lengthof(v_SupportedBandListEUTRA_v9e0) and v_BandReported==false) ; i:=i+1) { if (v_SupportedBandListEUTRA_v9e0[i].bandEUTRA_v9e0 == p_Band){ v_BandReported:= true; } } } if (v_BandReported) { f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Band is wrongly reported as supported in UE capabilities"); } }

After Change

function f_EUTRA_CheckNotSupportedBand(integer p_Band, template (omit) octetstring p_EUTRA_Cap := omit) runs on EUTRA_PTC { var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); var UE_EUTRA_Capability v_EUTRA_Cap; var SupportedBandListEUTRA v_EUTRA_BandListReported; var SupportedBandListEUTRA_v9e0 v_SupportedBandListEUTRA_v9e0; var UE_EUTRA_Capability_v9a0_IEs v_UE_EUTRA_Capability_v9a0_IEs; var bitstring v_UE_EUTRA_Capability_v9a0_IEsContainer; var boolean v_BandReported := false; var integer i:=0; var integer v_Result; if (isvalue(p_EUTRA_Cap)) { //@sic R5s150635 change 2 sic@ v_EUTRA_Cap := f_EUTRA_DecodeEutraCapMsg(valueof(p_EUTRA_Cap)); } else { v_EUTRA_Cap := f_EUTRA_MobileInfo_GetUECapability(); } v_EUTRA_BandListReported := v_EUTRA_Cap.rf_Parameters.supportedBandListEUTRA; //@sic R5s150635 change 2 sic@ if (p_Band > 64 ) { //@sic R5s200365 sic@ if (isvalue(v_EUTRA_Cap.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension)) { v_UE_EUTRA_Capability_v9a0_IEsContainer := oct2bit(v_EUTRA_Cap.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension); v_Result := decvalue(v_UE_EUTRA_Capability_v9a0_IEsContainer, v_UE_EUTRA_Capability_v9a0_IEs); if (v_Result != 0) { FatalError(__FILE__, __LINE__, "UE_EUTRA_Capability_v9a0_IEs cannot be decoded"); } if (isvalue(v_UE_EUTRA_Capability_v9a0_IEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v9e0)){ v_SupportedBandListEUTRA_v9e0 := v_UE_EUTRA_Capability_v9a0_IEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v9e0.supportedBandListEUTRA_v9e0; } } else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE_EUTRA_Capability_v9a0_IEs not provided"); } } //Check the band support against the PICS if ((v_SuppEutraBandsAccToPICS[p_Band-1] == '1'B)) { f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Band is wrongly reported as supported in the PICS"); } //Check the band support against the UE capabilities if ( p_Band < 64){ for (i:=0; (i < lengthof(v_EUTRA_BandListReported) and v_BandReported==false) ; i:=i+1) { if (v_EUTRA_BandListReported[i].bandEUTRA == p_Band){ v_BandReported:= true; } } } else { //Band > 64 @sic R5s200365 sic@ if (ispresent (v_SupportedBandListEUTRA_v9e0)) { for (i:=0; (i < lengthof(v_SupportedBandListEUTRA_v9e0) and v_BandReported==false) ; i:=i+1) { if (v_SupportedBandListEUTRA_v9e0[i].bandEUTRA_v9e0 == p_Band){ v_BandReported:= true; } } } } if (v_BandReported) { f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Band is wrongly reported as supported in UE capabilities"); } }

MCC160 implementation

function f_EUTRA_CheckNotSupportedBand(integer p_Band, template (omit) octetstring p_EUTRA_Cap := omit) runs on EUTRA_PTC { var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); var bitstring v_SuppEutraBandsAccToPICS_v9e0 := f_DeriveSuppEutraBandsFromPics_v9e0(); //@sic R5s250360 sic@ var UE_EUTRA_Capability v_EUTRA_Cap; var SupportedBandListEUTRA v_EUTRA_BandListReported; var template(omit) SupportedBandListEUTRA_v9e0 v_SupportedBandListEUTRA_v9e0; //@sic R5s250360 sic@ var UE_EUTRA_Capability_v9a0_IEs v_UE_EUTRA_Capability_v9a0_IEs; var bitstring v_UE_EUTRA_Capability_v9a0_IEsContainer; var boolean v_BandReported := false; var integer v_BandEUTRA_v9e0; var integer i:=0; var integer v_Result; if (isvalue(p_EUTRA_Cap)) { //@sic R5s150635 change 2 sic@ v_EUTRA_Cap := f_EUTRA_DecodeEutraCapMsg(valueof(p_EUTRA_Cap)); } else { v_EUTRA_Cap := f_EUTRA_MobileInfo_GetUECapability(); } v_EUTRA_BandListReported := v_EUTRA_Cap.rf_Parameters.supportedBandListEUTRA; //@sic R5s150635 change 2 sic@ if (p_Band > 64 ) { //@sic R5s200365 sic@ if (isvalue(v_EUTRA_Cap.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension)) { v_UE_EUTRA_Capability_v9a0_IEsContainer := oct2bit(v_EUTRA_Cap.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension); v_Result := decvalue(v_UE_EUTRA_Capability_v9a0_IEsContainer, v_UE_EUTRA_Capability_v9a0_IEs); if (v_Result != 0) { FatalError(__FILE__, __LINE__, "UE_EUTRA_Capability_v9a0_IEs cannot be decoded"); } if (isvalue(v_UE_EUTRA_Capability_v9a0_IEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v9e0.supportedBandListEUTRA_v9e0)){ //@sic R5s250295 R5s250360 sic@ v_SupportedBandListEUTRA_v9e0 := v_UE_EUTRA_Capability_v9a0_IEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v9e0.supportedBandListEUTRA_v9e0; } } else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE_EUTRA_Capability_v9a0_IEs not provided FFS"); } } //Check the band support against the PICS if ((v_SuppEutraBandsAccToPICS[p_Band-1] == '1'B)) { f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Band is wrongly reported as supported in the PICS"); } //Check the band support against the PICS (extended bands) @sic R5s250360 sic@ if ((v_SuppEutraBandsAccToPICS_v9e0[p_Band-65] == '1'B)) { // maxFBI + 1 f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Band is wrongly reported as supported in the PICS"); } //Check the band support against the UE capabilities if ( p_Band < 64){ for (i:=0; (i < lengthof(v_EUTRA_BandListReported) and v_BandReported==false) ; i:=i+1) { if (v_EUTRA_BandListReported[i].bandEUTRA == p_Band){ v_BandReported:= true; } } } else { //Band > 64 @sic R5s200365, R5s250360 sic@ if (isvalue(v_SupportedBandListEUTRA_v9e0)) { for (i:=0; (i < lengthof(v_SupportedBandListEUTRA_v9e0) and v_BandReported==false) ; i:=i+1) { if (isvalue(v_SupportedBandListEUTRA_v9e0[i].bandEUTRA_v9e0)) { v_BandEUTRA_v9e0 := valueof(v_SupportedBandListEUTRA_v9e0[i].bandEUTRA_v9e0); if (v_BandEUTRA_v9e0 == p_Band) { v_BandReported:= true; } } } } } if (v_BandReported) { f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Band is wrongly reported as supported in UE capabilities"); } }

Additional change 1 from MCC160

function f_DeriveSuppEutraBandsFromPics_v9e0 was updated to account for bands 253 and 254.

function f_DeriveSuppEutraBandsFromPics_v9e0(B1_Type p_BitValuseForUndefinedBand := '0'B ) return bitstring { //@sic R5s170598: added p_BitValuseForUndefinedBand sic@ var bitstring v_Bitstring := f_ConvertBoolToBit(pc_eBand65_Supp) & //@sic R5-162652 sic@ f_ConvertBoolToBit(pc_eBand66_Supp) & //@sic R5-163090 sic@ … p_BitValuseForUndefinedBand &//Band 252 not yet defined p_BitValuseForUndefinedBand &//Band 253 not yet defined p_BitValuseForUndefinedBand &//Band 254 not yet defined f_ConvertBoolToBit(pc_eBand253_Supp) & //@sic R5s250360 sic@ f_ConvertBoolToBit(pc_eBand254_Supp) & //@sic R5s250360 sic@ f_ConvertBoolToBit(pc_eBand255_Supp) & //@sic R5-231563 sic@ f_ConvertBoolToBit(pc_eBand256_Supp); //@sic R5-231563 sic@ return v_Bitstring; }