3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250376

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0987 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR5GC_eCallSteps20A_23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Newly introduced function f_NR5GC_eCallSteps20A_23 in the D25wk38 TTCN that is a fragment of already existing f_NR5GC_ECallOnly_Steps3_23 function contains part (specifically the function f_NR5GC_Registration) that is being called by both these functions, i.e. effectively it is called twice. This causes the test to expect the registration twice and results in test failure during preamble for conformant UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the f_NR5GC_Registration from the f_NR5GC_eCallSteps20A_23. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE fails eCall tests during test preamble |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.x eCall tests |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact: 	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NR5GC_eCallSteps20A_23 |
| --- | --- |
| Reason for change | Newly introduced function f_NR5GC_eCallSteps20A_23 in the D25wk38 TTCN that is a fragment of already existing f_NR5GC_ECallOnly_Steps3_23 function contains part (specifically the function f_NR5GC_Registration) that is being called by both these functions, i.e. effectively it is called twice. This causes the test to expect the registration twice and results in test failure during preamble for conformant UE. |
| Summary of change | Removed the f_NR5GC_Registration from the f_NR5GC_eCallSteps20A_23. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
|  | Accepted |

Before Change:

function f_NR5GC_eCallSteps20A_23(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var boolean v_InitialRxd := false; var boolean v_EmgRxd := false; var default v_MyDefaultVar; // Steps 3-20 // Steps 2 to 15 of generic procedure 4.5.2.2-2 // @sic R5s230685 sic@ f_NR5GC_Registration(p_NR_CellId, -, Initial_NoSecurity); // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ // Then activate store for any new msgs received - this works for Modification Reqs too v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); …

After Change:

function f_NR5GC_eCallSteps20A_23(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var boolean v_InitialRxd := false; var boolean v_EmgRxd := false; var default v_MyDefaultVar; // Steps 3-20 // Steps 2 to 15 of generic procedure 4.5.2.2-2 // @sic R5s230685 sic@ //REMOVED f_NR5GC_Registration(p_NR_CellId, -, Initial_NoSecurity); // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ // Then activate store for any new msgs received - this works for Modification Reqs too v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); …