3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250381

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3752 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the 25wk38 IWD, IRAT testcases that start with a Eutra preamble are failing when run with a UE that configures Internet PDU first, followed by IMS PDU. For example, in testcase 11.1.2, following is the flow for the Eutra PTC: f_TC_11_1_2_NR5GC_EUTRAf_EUTRA38_NR5GC_PreambleAndSwitchOfff_EUTRA38_NR5GC_Preamblef_EUTRA38_MultiPDN_InitialRegistrationf_EUTRA38_MultiPDN_InitialRegistration_Step5_18b1f_EUTRA38_MultiPDN_InitialRegistration_Step5_16. At this point, for the configuration specified above (Internet first/IMS second), condition select (f_EUTRA38_MultiPDN_GetPDNType(v_APNandPCOs.accessPointName)) evaluats to Internet_DNN, which results in v_WaitForUserplaneSignalling := dontWaitForIMS. After this, the flow continues to f_EUTRA_NAS_AttachCompletef_EUTRA_DelayForUserPlaneSignallingf_IPCAN_DelayForUserPlaneSignalling, where following code is executed : case (dontWaitForIMS) { t_WaitForUPlaneSignalling.start(1.2); alt { [] t_WaitForUPlaneSignalling.timeout { // just continue } [] p_Port.receive(cmr_IMS_IPCAN_Trigger) { f_SetVerdictInconc(__FILE__, __LINE__, "IMS Registration even though loopback mode is activated"); /* @sic R5-153971 sic@ */ } } } However, at the same time when the IMS PTC is being initialized, following execution flow is observed : f_TC_11_1_2_NR5GC_IMS1fl_IMSFallback_RegistrationToMOSpeechCallfl_IMSFallback_PreambleRegistrationsf_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); The highlihted coordination message is received on the Eutra PTC during the highlihted code execution. This results in the testcase failning with verdict "IMS Registration even though loopback mode is activated" This needs to be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to check for the case Internet_DNN whether the PDN2 is IMS, and based on that assign the value for v_WaitForUserplaneSignalling. Basically this is the Anritsu solution proposed in R5s250339. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2 and other IRAT testcases that start with a preamble on Eutra and run with a UE configuration of Internet_PDN first/IMS_PDN second |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:		Bhise Parikshit

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 |
| --- | --- |
| Reason for change | In the 25wk38 IWD, IRAT testcases that start with a Eutra preamble are failing when run with a UE that configures Internet PDN first, followed by IMS PDN. For example, in testcase 11.1.2, following is the flow for the Eutra PTC: f_TC_11_1_2_NR5GC_EUTRAf_EUTRA38_NR5GC_PreambleAndSwitchOfff_EUTRA38_NR5GC_Preamblef_EUTRA38_MultiPDN_InitialRegistrationf_EUTRA38_MultiPDN_InitialRegistration_Step5_18b1f_EUTRA38_MultiPDN_InitialRegistration_Step5_16. At this point, for the configuration specified above (Internet first/IMS second), condition select (f_EUTRA38_MultiPDN_GetPDNType(v_APNandPCOs.accessPointName)) evaluats to Internet_DNN, which results in v_WaitForUserplaneSignalling := dontWaitForIMS. After this, the flow continues to f_EUTRA_NAS_AttachCompletef_EUTRA_DelayForUserPlaneSignallingf_IPCAN_DelayForUserPlaneSignalling, where following code is executed : case (dontWaitForIMS) { t_WaitForUPlaneSignalling.start(1.2); alt { [] t_WaitForUPlaneSignalling.timeout { // just continue } [] p_Port.receive(cmr_IMS_IPCAN_Trigger) { f_SetVerdictInconc(__FILE__, __LINE__, "IMS Registration even though loopback mode is activated"); /* @sic R5-153971 sic@ */ } } } However, at the same time when the IMS PTC is being initialized, following execution flow is observed : f_TC_11_1_2_NR5GC_IMS1fl_IMSFallback_RegistrationToMOSpeechCallfl_IMSFallback_PreambleRegistrationsf_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); The highlihted coordination message is received on the Eutra PTC during the highlihted code execution. This results in the testcase failning with verdict "IMS Registration even though loopback mode is activated" This needs to be fixed. |
| Summary of change | It is proposed to check for the case Internet_DNN whether the PDN2 is IMS, and based on that assign the value for v_WaitForUserplaneSignalling. Basically this is the Anritsu solution proposed in R5s250339. |
| TTCN module | CommonDefs.ttcn |
| MCC160 Comment | Accepted in principle but re-worked the solution for R5s250200 – set to noDelay if pc_noOf_PDNsSameConnection > 0, but always set to waitForIMS for the IMS DNN |

Before Change

<<SKIPPED CODE>> if (p_FallbackSignalling) { // @sic R5-206296, R5s220177, R5s250200 sic@ /* In case of another PDN connectivity request this may be sent immediately by the UE => we cannot wait after the ATTACH_COMPLETE for any trigger from the IMS_PTC but need to do this when all PDNs are configured */ v_WaitForUserplaneSignalling := noDelay; } else { select (f_EUTRA38_MultiPDN_GetPDNType(v_APNandPCOs.accessPointName)) { // @sic R5-206296 sic@ case (IMS_DNN) { v_WaitForUserplaneSignalling := waitForIMS; } case (Internet_DNN) { v_WaitForUserplaneSignalling := dontWaitForIMS; } } } <<SKIPPED CODE>>

After Change

<<SKIPPED CODE>> if (p_FallbackSignalling) { // @sic R5-206296, R5s220177, R5s250200 sic@ /* In case of another PDN connectivity request this may be sent immediately by the UE => we cannot wait after the ATTACH_COMPLETE for any trigger from the IMS_PTC but need to do this when all PDNs are configured */ v_WaitForUserplaneSignalling := noDelay; } else { select (f_EUTRA38_MultiPDN_GetPDNType(v_APNandPCOs.accessPointName)) { // @sic R5-206296 sic@ case (IMS_DNN) { v_WaitForUserplaneSignalling := waitForIMS; } case (Internet_DNN) { //WA#WI1453687 COMMENTED OUT v_WaitForUserplaneSignalling := dontWaitForIMS; if (tsc_EUTRA_Registration_PDN2isIMS){ //WA#WI1453687 This is basically Anritsu solution for R5s250339 put back. v_WaitForUserplaneSignalling := noDelay; } else{ v_WaitForUserplaneSignalling := dontWaitForIMS; } } } } <<SKIPPED CODE>>

#### MCC160 Implementation

function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16(EUTRA_CellId_Type p_CellId,

NR_TESTMODE_STATE_Type p_State,

NAS_MSG_Indication_Type p_NAS_Ind,

template (omit) PLMN_List p_EquivalentPlmnList := omit,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduSessionInfoList := omit,

boolean p_FallbackSignalling := false,

boolean p_PreserveIPaddress := true) runs on EUTRA_Multi5GS_PTC

{

var template (omit) AdditionalUpdateType v_AdditionalUpdateType;

var NAS_UL_Message_Type v_PdnConnectivityRequest;

var ProcedureTransactionIdentifier v_EPS_TI;

var template (omit) ProtocolConfigOptions v_PcoFromUE := omit;  // may be omit

var template (omit) ExtdProtocolConfigOptions v_ExtdPcoFromUE := omit;

var EPS_BearerIdentity v_EpsBearerId;

var boolean v_IPv4AllocationViaNasFlag;

var APNandPCOs_Type v_APNandPCOs;

var NasCount_Type v_NasCountUL;

var NR_Configuration_Type v_NR_ConfigType := f_EUTRA38_NR_Config_Get();

var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := dontWaitForIMS; // Default: no IP signalling possible

var template (omit) UERadioCapId v_UERadioCapIdToSend := omit; // @sic R5s220478 sic@

if (not ispresent (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present");

}

select (v_NR_ConfigType) {

case (EN_DC) {

if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_EUTRA_UENetworkCap(omit))){

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for ENDC");

}

}

case else { // other options FFS

if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for EUTRA38");

}

}

}

v_AdditionalUpdateType := p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.addUpdateType;

v_PdnConnectivityRequest := p_NAS_Ind.Pdu.PiggybackedPduList[0].Msg;

v_EPS_TI := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier;

v_PcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions;

v_ExtdPcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.extdProtocolConfigurationOptions;

// Steps 5-10b6

v_NasCountUL := f_EUTRA_MultiPDN_InitialRegistration_Step5_8 (p_CellId, p_NAS_Ind);  // @sic R5-216280, R5-222047 sic@

v_APNandPCOs := f_EUTRA38_MultiPDN_InitialRegistration_Step9_13 (p_CellId, p_State, p_NAS_Ind, v_NasCountUL);

if (ispresent(v_APNandPCOs.protocolConfigurationOptions)) {

v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions;

} else if (ispresent(v_APNandPCOs.extdProtocolConfigurationOptions)) {

v_ExtdPcoFromUE := v_APNandPCOs.extdProtocolConfigurationOptions;

}

if (isvalue(v_PcoFromUE)) {

v_IPv4AllocationViaNasFlag := f_CheckPCOforIPallocationViaNas(v_PcoFromUE);

} else {

v_IPv4AllocationViaNasFlag := f_CheckExtdPCOforIPallocationViaNas(v_ExtdPcoFromUE);

}

if (pc_EPC_RACS) { // @sic R5-216280 sic@

v_UERadioCapIdToSend := f_EUTRA_MobileInfo_GetRACS_IdToSend(); // @sic R5s220478 sic@

}

v_EpsBearerId := f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(p_CellId,

NORMAL,

v_AdditionalUpdateType,

v_EPS_TI,

v_IPv4AllocationViaNasFlag,

v_PcoFromUE,

v_APNandPCOs.accessPointName,

cs_GprsTimer_v_deact,

omit,

omit,

p_EquivalentPlmnList,

0,

omit,

-,

v_ExtdPcoFromUE,

p_PduSessionInfoList,  // @sic R5s210149 sic@

v_UERadioCapIdToSend,  // @sic R5-216280 sic@

-, p_PreserveIPaddress); // @sic R5s240679 sic@

// Step 10b7

if (pc_noOf_PDNsSameConnection > 0 or p_FallbackSignalling) {  // @sic R5-206296, R5s220177, R5s250200 sic@

/* In case of another PDN connectivity request this may be sent immediately by the UE

=> we cannot wait after the ATTACH_COMPLETE for any trigger from the IMS_PTC but need to do this when all PDNs are configured */

v_WaitForUserplaneSignalling := noDelay;

}

select (f_EUTRA38_MultiPDN_GetPDNType(v_APNandPCOs.accessPointName)) { // @sic R5-206296 sic@

case (IMS_DNN) {

v_WaitForUserplaneSignalling := waitForIMS;

}

}

f_EUTRA_NAS_AttachComplete(p_CellId, v_EpsBearerId, v_WaitForUserplaneSignalling);

}