3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250392

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3755 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC eCall over IMS testcase 11.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 11.5.2 to the IWD_25wk24 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.5.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to testcase 11.5.2	4

2.1	Change 1	4

2.2	Change 2	5

2.3	Change 3	11

2.4	Change 4	12

3	Branches executed	13

4	Execution Log Files	14

4.1	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.	14

5	References	14

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 11.5.2 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk24.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:

Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

  - Verification Test Summary

Test Case: 	11.5.2.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	R&S® 5G Protocol Conformance Test platform.

UE used:	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.

Verification Status:	PASS

- Corrections required to testcase 11.5.2

  - Change 1

| Function name | fl_TC_11_5_2_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | 1. The function f_EUTRA38_TrackingAreaUpdateFromN1 will expect by default that UE came from EUTRA cell, however in this case it was NR cell instead. 2. UE previously registered in NR shall have mapped security context during step 54 procedure. 3. Test defines power levels in step 53 for E-UTRA Cell 1 as "Non-suitable "off" cell". |
| Summary of change | 1. Corrected 3rd parameter in the f_EUTRA38_TrackingAreaUpdateFromN1 2.Function f_EUTRA38_NR5GC_S1ToN1_IdleMode is called with 2nd parameter “true” to reflect mapped security context. 3. E-UTRA Cell 1 power level changed to "Non-suitable "off" cell". |
| TTCN module | ECall_IRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment | - Accepted. - Accepted. - Accepted. |

Before Change:

function fl_TC_11_5_2_NR5GC_EUTRA_TestBody()runs on EUTRA_Multi5GS_PTC { // @siclog "Step 1" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Cell Power"); // @siclog "Step 59" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Serving cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); // @siclog "Step 60" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_EUTRA38_TrackingAreaUpdateFromN1(eutra_Cell1, tsc_RRC_TI_Def); // @siclog "Step 61" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableCellRS_EPRE, tsc_NonSuitableCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 62" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1); } // End of fl_TC_11_5_2_NR5GC_EUTRA_TestBody()

After Change:

function fl_TC_11_5_2_NR5GC_EUTRA_TestBody()runs on EUTRA_Multi5GS_PTC { // @siclog "Step 1" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Cell Power"); // @siclog "Step 59" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Serving cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); // @siclog "Step 60" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_EUTRA38_TrackingAreaUpdateFromN1(eutra_Cell1, tsc_RRC_TI_Def, omit); // @siclog "Step 61" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 62" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1, true); } // End of fl_TC_11_5_2_NR5GC_EUTRA_TestBody()

  - Change 2

| Function name | fl_TC_11_5_2_Body |
| --- | --- |
| Reason for change | - Procedure in steps 6-14 is the initial procedure without any security established prior these steps. - If set to default value true, the 10sec v_WaitIfIMS inside f_NR5GC_RRC_Idle_Steps16_20 f_NR5GC_PDUSessionEstablishment will block the PDU Session Modification for Voice Bearers and test will eventually fail on unexpected new UE attempt for test call. - The wait for trigger is already inside f_NR5GC_CallRelease so needs to be removed from test body - During steps 55-56 the SRB2 has not been established, so the REG COMPLETE in step 49 should be expected on SRB1. - Correction of NR cell ID is needed in step 59. - At step 63, UE should be in idle state, then previous to reregistration, UE needs to establish a RRC connection. |
| Summary of change | - Security set to Initial_NoSecurity for steps 6-14. - Pass parameter p_NormalIMSOperation=false - Remove wait for trigger coordination message - Changed SRB2 to SRB1 for step 57. - Changed NRcell11 to NRcell1 in step 59 - Reworked reregistration steps. Expecting a RRC connection request previous to reregistration message. |
| TTCN module | ECall_IRAT_NR5GC_NR.ttcn |
| MCC160 Comment | 1.Accepted. 2.Accepted. 3.Accepted. 4. Accepted. 5. Accepted. 6. Accepted. |

Before Change:

// @siclog "Steps 6-14" siclog@ // Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), TESTMode_OFF, Initial_Secure); // @siclog "Step 15" siclog@ // The SS transmits a REGISTRATION ACCEPT message and assigns value of 7 hours for the timer T3512. v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}), -, -, -, -, -,-, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Step 16" siclog@ // The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. f_NR5GC_Registration_Complete(nr_Cell1); // @siclog "Step 17" siclog@ // Step 19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] is performed with 'connected without release'. // Check: The UE transmits an ULInformationTransfer message with Request type set to "initial request" and a PDU SESSION ESTABLISHMENT REQUEST message? // @siclog "Step 17A" siclog@ // The SS transmits an RRCReconfiguration message and a PDU SESSION ESTABLISHMENT ACCEPT // @siclog "Step 17B" siclog@ // The UE transmits an RRCReconfigurationComplete message. f_NR5GC_RRC_Idle_Steps16_20(nr_Cell1, -, -, noRrcConnectionRelease); f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 17 PASS"); // @siclog "Steps 18-31" siclog@ // Void // EXCEPTION: In parallel to the events described in steps 32 to 34 below steps 2 - 3 of eCall Setup and MSD Update / 5GS as defined in TS 34.229-5 [41] annex A.23 take place // @siclog "Steps 32-34" siclog@ // Steps 11 to 13b3 of Table 4.9.15.2.2-1 of the test procedure in TS 38.508-1 [4] are performed. // Wait until need to add voice QoS v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger()); // Call now established // @siclog "Steps 35-36" siclog@ // Release eCall for test service using the generic procedure described in TS 38.508-1 [4] subclause 4.9.18. // void f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall, false); //MT release required // @siclog "Step 37" siclog@ // The SS releases the RRC connection. (Note 2) // Note 2: Timer T3445 of 12hours starts. f_NR_RRCRelease(nr_Cell1); t_T3445_Min.start; // @siclog "Step 38" siclog@ // The SS waits 1 minute. f_Delay(60.0); // @siclog "Steps 39-51" siclog@ // Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed. f_NR5GC_MTSpeechCall(nr_Cell1); // @siclog "Steps 52-53" siclog@ // Generic test procedure for MT release of Voice call in 5GS described in TS 34.229-5 [41] subclause A.8 TS 38.508-1 [4] subclause 4.9.18 takes place. // Void f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger()); f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall, false); //MT release required // @siclog "Step 54" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); t_T3512_Min.start; // @siclog "Step 55" siclog@ // Check: Does the UE transmit a REGISTRATION REQUEST message at the expiry of T3512? (Note 3) // Note 3: Timer T3512 of 7hours starts. t_T3512_Min.timeout; v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Periodic)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 55 PASS"); } // @siclog "Step 56" siclog@ // The SS transmits a REGISTRATION ACCEPT message including T3512 value IE. v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}), -, -, -, -, -,-, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Step 57" siclog@ // The UE transmits a REGISTRATION COMPLETE message f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB2); // @siclog "Step 58" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); // @siclog "Step 59" siclog@ // The SS configures: // - NR Cell 1 as " Non-suitable "off" cell" f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 60" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); // @siclog "Step 61" siclog@ // The SS configures: // NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 62" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, -, -); f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 62 PASS"); // @siclog "Step 63" siclog@ // Check: Does the UE transmit a DEREGISTRATION REQUEST message at expiry of T3445? t_T3445_Min.timeout; SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu, crs_DeregisterType('0'B, '0'B, -))) { // Jun24 @sic R5s240531 Baseline Moving sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 63 PASS"); } } //End of fl_TC_11_5_2_Body()

After Change:

// @siclog "Steps 6-14" siclog@ // Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), TESTMode_OFF, Initial_NoSecurity); // @siclog "Step 15" siclog@ // The SS transmits a REGISTRATION ACCEPT message and assigns value of 7 hours for the timer T3512. v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}), -, -, -, -, -,-, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Step 16" siclog@ // The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. f_NR5GC_Registration_Complete(nr_Cell1); // @siclog "Step 17" siclog@ // Step 19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] is performed with 'connected without release'. // Check: The UE transmits an ULInformationTransfer message with Request type set to "initial request" and a PDU SESSION ESTABLISHMENT REQUEST message? // @siclog "Step 17A" siclog@ // The SS transmits an RRCReconfiguration message and a PDU SESSION ESTABLISHMENT ACCEPT // @siclog "Step 17B" siclog@ // The UE transmits an RRCReconfigurationComplete message. f_NR5GC_RRC_Idle_Steps16_20(nr_Cell1, -, -, noRrcConnectionRelease, -, -, -, -, -, -, -, -, -, -, -, -, false); f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 17 PASS"); // @siclog "Steps 18-31" siclog@ // Void // EXCEPTION: In parallel to the events described in steps 32 to 34 below steps 2 - 3 of eCall Setup and MSD Update / 5GS as defined in TS 34.229-5 [41] annex A.23 take place // @siclog "Steps 32-34" siclog@ // Steps 11 to 13b3 of Table 4.9.15.2.2-1 of the test procedure in TS 38.508-1 [4] are performed. // Wait until need to add voice QoS v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger()); // Call now established // @siclog "Steps 35-36" siclog@ // Release eCall for test service using the generic procedure described in TS 38.508-1 [4] subclause 4.9.18. // void //REMOVED f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall, false); //MT release required // @siclog "Step 37" siclog@ // The SS releases the RRC connection. (Note 2) // Note 2: Timer T3445 of 12hours starts. f_NR_RRCRelease(nr_Cell1); t_T3445_Min.start; // @siclog "Step 38" siclog@ // The SS waits 1 minute. f_Delay(60.0); // @siclog "Steps 39-51" siclog@ // Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed. f_NR5GC_MTSpeechCall(nr_Cell1); // @siclog "Steps 52-53" siclog@ // Generic test procedure for MT release of Voice call in 5GS described in TS 34.229-5 [41] subclause A.8 TS 38.508-1 [4] subclause 4.9.18 takes place. // Void f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger()); f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall, false); //MT release required // @siclog "Step 54" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); t_T3512_Min.start; // @siclog "Step 55" siclog@ // Check: Does the UE transmit a REGISTRATION REQUEST message at the expiry of T3512? (Note 3) // Note 3: Timer T3512 of 7hours starts. t_T3512_Min.timeout; v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Periodic)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 55 PASS"); } // @siclog "Step 56" siclog@ // The SS transmits a REGISTRATION ACCEPT message including T3512 value IE. v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}), -, -, -, -, -,-, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Step 57" siclog@ // The UE transmits a REGISTRATION COMPLETE message f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB1); // @siclog "Step 58" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); // @siclog "Step 59" siclog@ // The SS configures: // - NR Cell 1 as " Non-suitable "off" cell" f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 60" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); // @siclog "Step 61" siclog@ // The SS configures: // NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 62" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, -, -); f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 62 PASS"); // @siclog "Step 63" siclog@ // Check: Does the UE transmit a DEREGISTRATION REQUEST message at expiry of T3445? t_T3445_Min.timeout; v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedMsg.Pdu, crs_DeregisterType('0'B, '0'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 63 PASS"); } } //End of fl_TC_11_5_2_Body()

  - Change 3

| Function name | f_TC_11_5_2_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - Handling of IMS registration is done within f_IMS_ECallOnly_TestCall. - After Step 32-34, coordination message to let know NR PTC that test eCall has been released needs to be sent with matching reason. |
| Summary of change | - Redundant function calls removed - Coordination message with matching reason sent |
| TTCN module | ECall_IRAT_NR5GC_IMS.ttcn |
| MCC160 Comment | 1. Accepted. 2. Accepted. |

Before Change:

function f_TC_11_5_2_NR5GC_IMS1() runs on IMS_PTC { var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_1); f_IMS_Registration(); f_IMS_IPCAN_WaitForTrigger(IPCAN); //EXCEPTION: In parallel to the events described in steps 32 to 34 below steps 2 - 3 of eCall Setup and MSD Update / 5GS as defined in TS 34.229-5 [41] annex A.23 take place // @siclog "Steps 32-34" siclog@ // Steps 11 to 13b3 of Table 4.9.15.2.2-1 of the test procedure in TS 38.508-1 [4] are performed. f_IMS_ECallOnly_TestCall(msdUpdateNotRequired); // @sic R5-251275 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger()); // @siclog "Steps 39-51" siclog@ // Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed. f_IMS_NR5GC_MTSpeechCall(); // @siclog "Steps 52-53" siclog@ // Generic test procedure for MT release of Voice call in 5GS described in TS 34.229-5 [41] subclause A.8 TS 38.508-1 [4] subclause 4.9.18 takes place. // Void f_IMS_IPCAN_WaitForTrigger(IPCAN); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); f_IMS_IpcanReleaseWithOptionalDeregistration(); } // End of f_TC_11_5_2_IRAT_IMS1()

After Change:

function f_TC_11_5_2_NR5GC_IMS1() runs on IMS_PTC { var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_1); //REMOVED f_IMS_Registration(); //REMOVED f_IMS_IPCAN_WaitForTrigger(IPCAN); //EXCEPTION: In parallel to the events described in steps 32 to 34 below steps 2 - 3 of eCall Setup and MSD Update / 5GS as defined in TS 34.229-5 [41] annex A.23 take place // @siclog "Steps 32-34" siclog@ // Steps 11 to 13b3 of Table 4.9.15.2.2-1 of the test procedure in TS 38.508-1 [4] are performed. f_IMS_ECallOnly_TestCall(msdUpdateNotRequired); // @sic R5-251275 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); // @siclog "Steps 39-51" siclog@ // Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed. f_IMS_NR5GC_MTSpeechCall(); // @siclog "Steps 52-53" siclog@ // Generic test procedure for MT release of Voice call in 5GS described in TS 34.229-5 [41] subclause A.8 TS 38.508-1 [4] subclause 4.9.18 takes place. // Void f_IMS_IPCAN_WaitForTrigger(IPCAN); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); f_IMS_IpcanReleaseWithOptionalDeregistration(); } // End of f_TC_11_5_2_IRAT_IMS1()

  - Change 4

| Function name | f_GetTestcaseAttrib_ECallOverIMS |
| --- | --- |
| Reason for change | In order to properly set support of IMS emergency in system information the test needs to be included in f_GetTestcaseAttrib_ECallOverIMS. |
| Summary of change | Added 11.5.2 into f_GetTestcaseAttrib_ECallOverIMS. |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | 1. Accepted. |

Before Change:

function f_GetTestcaseAttrib_ECallOverIMS(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_5_20_NR5GC") { return true; } case ("TC_11_5_21_NR5GC") { return true; } case ("TC_11_5_22_NR5GC") { return true; } case ("TC_11_5_23_NR5GC") { return true; } case ("TC_11_5_24_NR5GC") { return true; } case ("TC_11_5_3_NR5GC") { return true; } case ("TC_11_5_4_NR5GC") { return true; } case ("TC_11_5_7_NR5GC") { return true; } case ("TC_11_5_8_NR5GC") { return true; } case ("TC_11_5_9_NR5GC") { return true; } } return false; }

After Change:

function f_GetTestcaseAttrib_ECallOverIMS(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_5_20_NR5GC") { return true; } case ("TC_11_5_21_NR5GC") { return true; } case ("TC_11_5_22_NR5GC") { return true; } case ("TC_11_5_23_NR5GC") { return true; } case ("TC_11_5_24_NR5GC") { return true; } case ("TC_11_5_2_NR5GC") { return true; } case ("TC_11_5_3_NR5GC") { return true; } case ("TC_11_5_4_NR5GC") { return true; } case ("TC_11_5_7_NR5GC") { return true; } case ("TC_11_5_8_NR5GC") { return true; } case ("TC_11_5_9_NR5GC") { return true; } } return false; }

- Branches executed

This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” and LTE Band “FDD3” using NIA2 Integrity and NEA2 ciphering algorithms.

- Execution Log Files

  - Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.

The Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies passed this test case on R&S® 5G Protocol Conformance Test platform

The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

TC_11_5_2.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

- References

| [1] | R5s250393: Supporting information for addition of NR5GC eCall over IMS testcase 11.5.2 |
| --- | --- |