3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250425

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3759 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR NTN testcase 7.1.1.10.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - According to Table 7.1.1.10.4.3.2-1 in 38.523-1, at step46, The SS transmits Differential Koffset MAC CE with differential Koffset set to 20 ms. According to the Table 6.3.4.2-1: in 38.508-1, for NGSO cell, the CellSpecificKoffset is 5 ms, so the value of differential Koffset (20ms) is not sutible for NGSO. - The value of RTT for NGSO is less than 100ms, at step47, get the next send occasion for DCI of DL transmission, it requires at least 100ms in advance. So for NGSO, get next send occasion with the default value(100ms). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For NGSO cell, at step46, set the value of differential Koffset to 1 ms. - Get the next send occasion for DCI of DL transmission with default value in advance.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.10.4NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	1

1	Overview	3

2	Corrections required to test case 7.1.1.10.4	4

2.1	Change 1	4

# Overview

This document lists all the essencial changes needed to correct problems in the ATS iwd-TTCN3-B2025-10_D25wk38 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required to test case 7.1.1.10.4

## Change 1

| Function name | f_TC_7_1_1_10_4_NR5GC () |
| --- | --- |
| Reason for change | - According to Table 7.1.1.10.4.3.2-1 in 38.523-1, at step46, The SS transmits Differential Koffset MAC CE with differential Koffset set to 20 ms. According to the Table 6.3.4.2-1: in 38.508-1, for NGSO cell, the CellSpecificKoffset is 5 ms, so the value of differential Koffset (20ms) is not sutible for NGSO. - The value of RTT for NGSO is less than 100ms, at step47, get the next send occasion for DCI of DL transmission, it requires at least 100ms in advance. So for NGSO, get next send occasion with the default value(100ms). |
| Summary of change | - For NGSO cell, at step46, set the value of differential Koffset to 1 ms. - Get the next send occasion for DCI of DL transmission with default value in advance. |
| TTCN module | MAC_NTN_NR5GC.ttcn |
| MCC160 Comment | 1. Postponed till the prose CR approval 2. Accepted |

Before Change:

function f_TC_7_1_1_10_4_NR5GC() runs on NR5GC_PTC { // NTN / UE specific TA report / UE specific Koffset var template (value) DL_DCCH_Message v_DL_DCCH_Message; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var SubFrameTiming_Type v_Timing; var integer v_DifferentialKoffset; var template (value) FreqBandInformation v_FirstBand; var octetstring v_EncodeCapRequest; //Init NTN Cell parameters f_NR5GC_Init(NR_29); /*@sic R5-252350 sic@*/ //Table 7.1.1.10.4.3.3-2: NTN-Config f_NR_CellInfo_SetSIB19_NTN_ta_Report_r17(nr_Cell1, enabled); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); //The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_UL_TA_Report(enable)); //@siclog "Step 1" siclog@ //Power on the UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //The UE transmits preamble on PRACH SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 3" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted preamble in step 2. //Preconfigured at SS //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with Timing Advance Report MAC CE included? f_NR_CheckMacPduAndTimingAdvanceReport(nr_Cell1,true,"Step 4",cr_38508_RRCSetupRequest,-,true); /*@sic R5s250260, R5-254990 sic@*/ //@sic Steps 5-22 sic@ //Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed f_NR5GC_RRC_Idle_Steps3_20(nr_Cell1, TESTMode_OFF, noRrcConnectionRelease, Initial_NoSecurity); //@sic Step 23 sic@ //The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4]. f_NR_UE_DeRegisterOnSwitchOff(nr_Cell1, STATE_CONNECTED_3A); f_NR5GC_MobileInfo_Init(); //@sic Step 24 sic@ //SS broadcasts SIB19 with ta-Report-r17 not present. //Table 7.1.1.10.4.3.3-2: NTN-Config f_NR_CellInfo_SetSIB19_NTN_ta_Report_r17(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); //@sic Step 25 sic@ //Power on the UE. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_UL_TA_Report(enable)); /*@sic R5s250260 sic@*/ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 26" siclog@ //The UE transmits preamble on PRACH SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 27" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted preamble in step 26. //Preconfigured at SS //@sic Step 28 sic@ //The UE transmits an RRCSetupRequest message with no Timing Advance Report MAC CE included. f_NR_CheckMacPduAndTimingAdvanceReport(nr_Cell1,false,"Step 28",cr_38508_RRCSetupRequest,-,false); /*@sic R5-254990 sic@*/ f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic Step 29 sic@ //The SS transmits an RRCSetup message configuring offsetThresholdTA. //Table 7.1.1.10.4.3.3-5: v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.tar_Config_r17 := {setup := cs_TAR_Config_r17(ms15, enabled)}; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1, -, f_NR_38508_RRCSetup(nr_Cell1))); /*@sic R5s250260 sic@*/ //@sic Step 30 sic@ //Check: Does the UE transmit an RRCSetupComplete message with Timing Advance Report MAC CE included? v_DedicatedNAS_Message := valueof(f_NR_CheckMacPduAndTimingAdvanceReport(nr_Cell1,true,"Step 30",omit,cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),true)); /*@sic R5-254990 sic@*/ //@sic Step 31-45 sic@ //Steps 5 to 19a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed. if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_DedicatedNAS_Message.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_DedicatedNAS_Message, v_PLMN, TESTMode_OFF); if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment(nr_Cell1,-,tsc_NR_RbId_SRB1); } //@sic Step 46 sic@ //The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 20 ms. v_DifferentialKoffset := 20; v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_MAC_DifferentialKoffset_CE(int2bit(v_DifferentialKoffset,6)))); //Reconfigure SS with a new TA v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 2); // add 2ms f_NR5GC_SS_NTN_ApplyDifferentialKoffset(nr_Cell1,v_DifferentialKoffset, cs_TimingInfo_NR(v_Timing)) ; //@sic Step 47 sic@ //The SS transmits a UECapabilityEnquiry message v_FirstBand := cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)); v_EncodeCapRequest := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({v_FirstBand}))); v_DL_DCCH_Message := cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequest); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, f_NR_CellInfo_GetNTN_UE_gNB_RTT(nr_Cell1)); // add RTT delay to allow UE to apply DifferentialKoffset SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_DL_DCCH_Message)); //@sic Step 48 sic@ //Check: Does the UE transmit a UECapabilityInformation message after applying differential Koffset of 20ms received at step 46? //Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 48"); f_NR_TestBody_Set(false); //Postamble f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); /*@sic R5s250260 sic@*/ }; }

After Change:

function f_TC_7_1_1_10_4_NR5GC() runs on NR5GC_PTC { // NTN / UE specific TA report / UE specific Koffset var template (value) DL_DCCH_Message v_DL_DCCH_Message; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var SubFrameTiming_Type v_Timing; var integer v_DifferentialKoffset; var template (value) FreqBandInformation v_FirstBand; var octetstring v_EncodeCapRequest; var NTN_Satellite_Type v_SatelliteType; //Init NTN Cell parameters f_NR5GC_Init(NR_29); /*@sic R5-252350 sic@*/ //Table 7.1.1.10.4.3.3-2: NTN-Config f_NR_CellInfo_SetSIB19_NTN_ta_Report_r17(nr_Cell1, enabled); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); //The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_UL_TA_Report(enable)); //@siclog "Step 1" siclog@ //Power on the UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //The UE transmits preamble on PRACH SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 3" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted preamble in step 2. //Preconfigured at SS //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with Timing Advance Report MAC CE included? f_NR_CheckMacPduAndTimingAdvanceReport(nr_Cell1,true,"Step 4",cr_38508_RRCSetupRequest,-,true); /*@sic R5s250260, R5-254990 sic@*/ //@sic Steps 5-22 sic@ //Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed f_NR5GC_RRC_Idle_Steps3_20(nr_Cell1, TESTMode_OFF, noRrcConnectionRelease, Initial_NoSecurity); //@siclog "Step 23" siclog@ //The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4]. f_NR_UE_DeRegisterOnSwitchOff(nr_Cell1, STATE_CONNECTED_3A); f_NR5GC_MobileInfo_Init(); //@siclog "Step 24" siclog@ //SS broadcasts SIB19 with ta-Report-r17 not present. //Table 7.1.1.10.4.3.3-2: NTN-Config f_NR_CellInfo_SetSIB19_NTN_ta_Report_r17(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 25" siclog@ //Power on the UE. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_UL_TA_Report(enable)); /*@sic R5s250260 sic@*/ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 26" siclog@ //The UE transmits preamble on PRACH SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 27" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted preamble in step 26. //Preconfigured at SS //@siclog "Step 28" siclog@ //The UE transmits an RRCSetupRequest message with no Timing Advance Report MAC CE included. f_NR_CheckMacPduAndTimingAdvanceReport(nr_Cell1,false,"Step 28",cr_38508_RRCSetupRequest,-,false); /*@sic R5-254990 sic@*/ f_NR_ULGrantConfiguration_Start(nr_Cell1); //@siclog "Step 29" siclog@ //The SS transmits an RRCSetup message configuring offsetThresholdTA. //Table 7.1.1.10.4.3.3-5: v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.tar_Config_r17 := {setup := cs_TAR_Config_r17(ms15, enabled)}; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1, -, f_NR_38508_RRCSetup(nr_Cell1))); /*@sic R5s250260 sic@*/ //@siclog "Step 30" siclog@ //Check: Does the UE transmit an RRCSetupComplete message with Timing Advance Report MAC CE included? v_DedicatedNAS_Message := valueof(f_NR_CheckMacPduAndTimingAdvanceReport(nr_Cell1,true,"Step 30",omit,cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),true)); /*@sic R5-254990 sic@*/ //@siclog "Step 31-45" siclog@ //Steps 5 to 19a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed. if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_DedicatedNAS_Message.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_DedicatedNAS_Message, v_PLMN, TESTMode_OFF); if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment(nr_Cell1,-,tsc_NR_RbId_SRB1); } //@siclog "Step 46" siclog@ //The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 20 ms. v_SatelliteType := f_NR_MobileInfo_GetNTNSatelliteType(); if(v_SatelliteType == ntn_GSO) { v_DifferentialKoffset := 5; }else { v_DifferentialKoffset := 1; } v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_MAC_DifferentialKoffset_CE(int2bit(v_DifferentialKoffset,6)))); //Reconfigure SS with a new TA v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 2); // add 2ms f_NR5GC_SS_NTN_ApplyDifferentialKoffset(nr_Cell1,v_DifferentialKoffset, cs_TimingInfo_NR(v_Timing)) ; //@siclog "Step 47" siclog@ //The SS transmits a UECapabilityEnquiry message v_FirstBand := cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)); v_EncodeCapRequest := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({v_FirstBand}))); v_DL_DCCH_Message := cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequest); if(f_NR_CellInfo_GetNTN_UE_gNB_RTT(nr_Cell1)< 100) { v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); // add RTT delay to allow UE to apply DifferentialKoffset }else { v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, f_NR_CellInfo_GetNTN_UE_gNB_RTT(nr_Cell1)); // add RTT delay to allow UE to apply DifferentialKoffset } SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_DL_DCCH_Message)); //@siclog "Step 48" siclog@ //Check: Does the UE transmit a UECapabilityInformation message after applying differential Koffset of 20ms received at step 46? //Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 48"); f_NR_TestBody_Set(false); //Postamble f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); /*@sic R5s250260 sic@*/ }; }