# 3GPP TSG-SA Meeting #107	 SP-250407

Incheon, South Korea, Mar 12 – 14, 2025	 (revision of SP-250347)

# Title:				Rel-18 SA2 AIMLsys Summary

# Source:				Apple, OPPO, Mediatek

# Document for:				Approval

Agenda Item:				7

Work Item / Release:	FS_AIML_CAL / Rel-19

# 1. Introduction

This paper proposes changes to clauses 5.2.2.5.1 and 5.2.2.5.2 introducing a summary of Rel-18 AIMLsys for completeness of TR 22.850. The proposed description includes summarized versions of the key capabilities described in cl. 5.46 of TS 23.501 as well as TR 21.918.

# 2. Proposal

Incorporate the following change in TR 22.850.

*** First Change ***

#### 5.2.2.5	Rel-18 SA WG2 WID - System Support for AI/ML-based Services (AIMLsys)

##### 5.2.2.5.1	Description

This work item implements the conclusions of the Rel-18 study on the 5GS architectural and functional extensions to enable 5GS to assist the Application AI/ML operations. The normative text is defined based on the agreed conclusions on 6 key issues, ensuring consistency with other 5GS features. The agreed conclusions focus on the following aspects:

-	Monitoring of network resource utilization to support the Application AI/ML operations.

-	Exposure of 5GC information to authorized 3rd party for Application AI/ML operations.

-	Enhancement of external parameter provisioning in 5GC to assist the Application AI/ML operations.

-	Enhancement in 5GC to enable Application AI/ML traffic transport.

-	Enhancement of QoS and Policy control to support Application AI/ML data transport over 5GS.

-	5GS assistance to federated learning operation.

##### 5.2.2.5.2	Activities summary

This work item specifies a list of principles that apply when the 5GS assists the AI/ML operation at the application layer as specified in clause 5.46 of TS 23.501 [22], namely:

-	AF requesting 5GS assistance to AI/ML operations in the application layer shall be authorized by the 5GC using the existing mechanisms.

-	Application AI/ML decisions and their internal operation logic reside at the AF and UE application client and is out of scope of 3GPP.

-	Based on application logic, it is the application decision whether to request assistance from 5GC, e.g. for the purpose of selection of Member UEs that participate in certain AI/ML operation.

The activities of this work item are limited to providing assistance to AI/ML-based applications when the participating UEs are not roaming and the AI/ML operations in the application layer are conducted within a single slice. Policy and charging control as defined in TS 23.503 [24] are assumed to be used for traffic related to application AI/ML operations.

The overall objective of this item is to provide assistance by the 5GC to AI/ML operations in the application layer, which are described in clause 5.2.2.1 and specified in clause 6.40 of TS 22.261 [6]. A brief description of the specified capabilities in this work item can be found below, with further details provided in clause 5.46 of TS 23.501 [22] and clause 11.1 in TR 21.918 [2] and the references therein:

- Planned Data Transfer with QoS: this capability is used to enable the AF to negotiate a variable time window for the planned AI/ML operation e.g. application data transfer with specific QoS requirements and operational conditions via the support of the NEF.

- Enhanced external parameter provisioning: this capability enables an AF hosting an AI/ML based application to provision enhanced Expected UE Behaviour parameters and/or Application-Specific Expected UE Behaviour parameter(s) to the 5GC by including corresponding confidence and/or accuracy levels to the expected parameters, which UDM could check against a threshold.

- Member UE selection assistance functionality: this capability provided by NEF is used to assist the AF to select member UE(s) for AI/ML application operations (e.g. Federated Learning) according to the AF's request including a list of target member UEs and a set of filtering criteria.

- Multi-member AF session with required QoS: this capability enables the NEF to map a request for Multi-member AF session with required QoS to individual requests for AF session with required QoS per UE address, and interact with each of the UE's serving PCFs on a per AF session basis.

- End-to-end data volume transfer time analytics: this analytics provides the consumer (e.g. AF, NEF) with analytics (i.e. statistics, predictions or both) referring to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE. The data volume may be the expected or observed data volume from UE to AF or from AF to UE.

- Enhanced NEF monitoring events: new NEF monitoring events are specified relevant to the operation of AI/ML based operations, namely session inactivity time, traffic volume exchanged between the UE and the AF and UL/DL consolidated data rate which is the aggregated data rate across all traffic flows corresponding to the list of UE addresses of the Multi-member AF session with required QoS.

###### 5.2.2.5.2.1	AI/ML related LCM activities

No AI/ML related LCM activities or functional entities were specified as part of this work. Instead, the activities summarized in clause 5.2.2.5.2 specify 5GS support for AI/ML related LCM activities (e.g. AI/ML model training and inference) assumed to be conducted at the application layer.

*** End of Changes ***