3GPP TSG-WG2 Meeting #168	S2-2504087

# 7 - 11 April, 2025, Goteborg, Sweden									(revison of S2-2504042, 2961)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0703 | rev | 2 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving UE shall have maximum one LCS-UPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA2 #167 it was agreed that in rel18 UE can have maximum one LCS-UPP connection. This CR provides necessary changes to ensure that UE can have maximum one LCS-UPP connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding clarification that UE maintains only one LCS-UPP connection - Adding logic that AMF shall reject LMF inititated connection establishment if AMF user plane context for the UE has already use plane connection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Supporting one LCS-UPP per UE is not defined as workable solution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.5, 5.1, 6.18.1, 6.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

### 4.3.5	UE

A target UE may support positioning according to four different modes:

-	UE assisted mode (the UE obtains location measurements and sends the measurements to another entity (e.g. an LMF) to compute a location);

-	UE based mode (the UE obtains location measurements and computes a location estimate making use of assistance data provided by serving PLMN);

-	standalone mode (the UE obtains location measurements and computes a location estimate without making use of assistance data provided by serving PLMN);

-	network based mode (a serving PLMN obtains location measurements of signals transmitted by a target UE and computes a location estimate).

NOTE:	The transmission of UE signals for network based mode may or may not be transparent to the UE.

Positioning procedures used by a UE for NG-RAN access are described in TS 38.305 [9].

A limited set of UE positioning capabilities and UE user plane positioning capabilities can be transferred to the 5GCN in the 5GMM capability during registration of the UE as described in TS 24.501 [11]. Some of these positioning capabilities may be transferred subsequently to an LMF as described in TS 29.572 [12]. UE positioning capabilities may also be transferred directly to a location server (e.g. LMF).

Additional functions which may be supported by a UE to support location services include the following.

-	Support location requests received from a network for 5GC-MT-LR, 5GC-NI-LR or a deferred 5GC-MT-LR for periodic or triggered location.

-	Support location requests to a network for a 5GC-MO-LR.

-	Support privacy notification and verification for a 5GC-MT-LR or deferred 5GC-MT-LR for periodic or triggered location.

-	Send updated privacy requirements to a serving AMF (for transfer to a UDR via UDM).

-	Support periodic or triggered location reporting to an LMF.

-	Support change of a serving LMF for periodic or triggered location reporting.

-	Support cancelation of periodic or triggered location reporting.

-	Support multiple simultaneous location sessions.

-	Support the reception of unciphered and/or ciphered assistance data broadcast by NG-RAN.

-	Support the reception of ciphering keys for the assistance data from the AMF.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF.

-	Support maintaining only one LCS-UPP connection.

-	Support reporting of location events for a periodic or triggered 5GC-MT-LR over a user plane connection to an LCS Client or AF with periodic cumulative event reports being sent over control plane to the LMF, H-GMLC and LCS Client or AF.

>>>> Start of Changes <<<<

## 5.1	LMF Discovery and Selection

LMF selection functionality is supported by the AMF to determine an LMF for location estimation of the target UE or Ranging/Sidelink Positioning between Target UE and SL Reference UE. The LMF selection functionality is also supported by the LMF if it determines that it is unsuitable or unable to support location for the current UE access network or serving cell for the deferred 5GC-MT-LR procedure for periodic, triggered location events, or Modification of User Plane Connection. The LMF selection functionality may also be supported by the GMLC and the GMLC provides the selected LMF ID to AMF.

LMF reselection is a functionality supported by AMF when necessary, e.g. due to UE mobility.

The LMF selection/reselection may be performed at the AMF or LMF or GMLC based on the locally available information i.e. LMF profiles are configured locally at AMF or LMF or GMLC, or by querying NRF.

The following factors may be considered during the LMF selection:

-	LCS client type.

-	Requested Quality of Service information, e.g.:

-	LCS accuracy,

-	Response time (latency),

-	Access Type (3GPP /N3GPP).

NOTE 1:	Location methods may differ depending on the Access Type, e.g. in the case of WLAN Access Location determination may just correspond to retrieval of IP addressing information from the N3IWF/TNGF; As another example, for Wireline access, Location determination may just correspond to retrieval of geo coordinates corresponding to a GLI as defined in clause 4.7.8 of TS 23.316 [21] or a HFC Node ID.

-	RAT type (e.g. 5G NR, eLTE, or any of the RAT Types specified for NR satellite access) and/or the serving AN node (i.e. gNB or NG-eNB) of the target UE.

-	RAN configuration information.

-	LMF capabilities, including:

-	the support of Uu based positioning as defined in clause 4.3.8;

-	Ranging/Sidelink positioning as defined in clause 4.3.8 of TS 23.586 [40];

-	LMF user plane positioning capabilities (the capability to support LCS-UPP).

-	LMF load.

-	LMF location.

-	Indication of either a single event report or multiple event reports.

-	Duration of event reporting.

-	Network slicing information, e.g. S-NSSAI and/or NSI ID.

-	LMF Service Area consisting of one or more TA(s).

-	Supported GAD shapes.

-	Support LCS when MBSR is involved.

-	Requested UE has maintained user plane connection with certain LMF.

When receiving a NAS message from UE, including an LMF ID together with a LPP message (refer to step 25 in clause 6.3.1 for event reporting for a deferred 5GC-MT-LR), AMF sends the LPP message to the LMF, as indicated by the LMF ID.

NOTE 2:	Description on how UE encapsulates the LMF ID in the NAS message is documented in TS 24.571 [36].

UDM may store the LMF ID in UE subscription data. During the location procedure, GMLC received the LMF ID from the UDM and provides it to AMF.

GMLC may be configured with the following parameters:

-	LMF ID and/or

-	per group ID and its correlating LMF ID.

The AMF may use locally provisioned configuration to determine LMF based on UE identify or its group information.

NOTE 3:	It is AMF implementation specific for the priority of different selection criteria from GMLC, AMF and LMF.

When the GMLC receives a MT location request from LCS client/AF, GMLC determines the LMF ID based on the configured parameters for an LCS Client/AF. In case a group ID is provided or derived from the location request, GMLC determines the correlating LMF ID based on the provisioned group ID.

GMLC may have configuration with one or several dedicated LMF ID(s), irrelevant to any LCS client/AF. When the GMLC receives a MT location request from LCS client/AF, GMLC only determines the LMF ID within the configuration for all LCS client/AF.

NOTE 4:	When AMF can't access the LMF instance of the LMF ID, by default, AMF replies to corresponding error to GMLC. GMLC could retry or fail the request accordingly; with explicit configuration to serve as backup selection in this, AMF can also select different configured LMF instance.

>>>> Next Changes <<<<

### 6.18.1	LMF initiated User Plane Connection

LMF may trigger the user plane connection establishment after receiving a location request from AMF if target UE does not have user plane connection with LMF. AMF may subscribe from LMF the status of LCS user plane connection for the target UE, using a Nlmf_Location_UP Subscribe message if the UE supports user plane positioning. Figure 6.18.1-1 shows a procedure triggered by LMF to support positioning over the user plane connection between UE and LMF.

Figure 6.18.1-1: Positioning via a User Plane Connection between UE and LMF initiated by LMF

NOTE 1:	User Plane protocol (LCS-UPP) to support generic transport between the UE and the LMF is defined in TS 24.572 [48].

1.	Based on UE user plane positioning capability, control plane congestion status (e.g. AMF load status) and other implementation factors, LMF decides whether to use the positioning procedure via a user plane connection between UE and LMF.

LMF may invoke Nnrf_NFDiscovery service operation to retrieve control plane congestion status (e.g. AMF load information). LMF may also invoke Nnrf_NFManagement_NFStatusSubscribe service to subscribe specific AMF load information. Based on AMF load information, LMF may determine to use user plane positioning, if there is available user plane connection between UE and LMF.

Steps 2-8 are skipped if there is already a user plane connection context of the target UE in LMF and LMF determines to utilize the user plane connection for positioning.

NOTE 2:	LMF can select user plane positioning for specific positioning methods (e.g. motion sensor-based method) and it is based on implementation and local configuration to determine which positioning method requires user plane transport.

NOTE 3:	The procedure can also be triggered when LMF receives a location request from AMF via control plane signalling as defined in clause 6.1 and clause 6.3.

2.	[Conditional] If LMF decides to utilize user plane for positioning and there is no established secure user plane connection between the UE and LMF, LMF invokes Namf_communication_N1N2MessageTransfer service operation to send the user plane information to AMF in a NAS container to indicate UE to utilize user plane over TLS for positioning. The user plane information includes the user plane positioning address of the LMF. The LMF allocates LCS-UP binding ID to be used to associate the user plane connection to be established with the target UE and includes this LCS-UP binding ID in the user plane information. The LMF associates the target UE identity (SUPI and/or GPSI) with this LCS-UP binding ID.

If the UE supports the user plane positioning capability and AMF has not subscribed the status of LCS user plane connection, the AMF may subscribe from LMF the status of LCS user plane connection.

If user plane connection context exists for this UE, then AMF rejects the request.

NOTE 4:	Security mechanism to support user plane positioning is defined in Annex Q.2 of TS 33.501 [50].

3.	[Conditional] When AMF receives the user plane information from LMF in step 2, and UE does not have user plane connection context stored in AMF, AMF sends it to UE via a DL NAS TRANSPORT message.

4.	[Conditional] If there is no established applicable PDU session for the user plane positioning, the UE uses the URSP as defined in TS 23.503 [41] which includes user plane positioning related PDU session parameters, e.g. a dedicated DNN and S-NSSAI, to establish the PDU session for user plane positioning. UE establishes a secured user plane connection with LMF. If LMF send its FQDN to the UE, a DNS server/resolver is used to resolve the IP address of LMF (e.g. EASDF or local DNS for local LMF address resolution). After the secured user plane connection been established successfully, the UE sends the LCS-UP binding ID received in step 3 to LMF via the secured user plane connection to enable LMF to perform the correlation of the UE with the secured user plane connection. The LCS-UP binding ID will be released once the correlation is complete.

5.	[Conditional] UE sends an acknowledgement to LMF through AMF to indicate a success of user plane connection establishment for positioning service or a failure to utilize the user plane connection as defined in TS 24.572 [48].

6	[Conditional] AMF sends the acknowledgement received in step 5 to the LMF via Namf_N1messageNotify service.

7.	[Conditional] LMF indicates AMF in the Nlmf_Location_UPNotify message that user plane connection between the UE and LMF has been established.

8.	[Conditional] The AMF stores the LCS-UP connection context as part of UE context.

9.	If LMF or UE determines to utilize the user plane connection for positioning and the secure user plane connection is established, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data. Supplementary services messages including event report messages, periodic triggered invoke messages and MS cancel deferred location messages may also be transferred between LMF and UE via the established user plane connection.

>>>> Next Changes <<<<

### 6.18.2	UE initiated User Plane Connection

UE may trigger the user plane connection establishment if the UE does not have user plane connection. Figure 6.18.2-1 shows a procedure triggered by UE to support positioning over the user plane connection between UE and LMF.

Figure 6.18.2-1: Positioning via a User Plane Connection between UE and LMF, initiated by UE

1.	UE sends a user plane connection establishment request to AMF via NAS Message, if there is no established secure user plane connection between the UE and LMF and UE decides to request a user plane connection for upcoming positioning requests.

2.	[Conditional] If the UE is authorized based on UE Subscription to use the user plane positioning, AMF selects an LMF which capable to establish a user plane session for positioning with the UE. AMF may either query the NRF or based on local configuration to discover and select a proper LMF.

3.	[Conditional] The AMF sends a Nlmf_Location_UPConfig Request towards LMF to request set up of an LCS-UP connection. The AMF shall include the target UE identity (see TS 29.572 [12]) (SUPI and/or GPSI) in the request.

4.	[Conditional] If LMF accepts to utilize user plane for positioning and there is no established secure user plane connection between the UE and LMF, LMF sends a user plane information to AMF to indicate UE to accept and utilize user plane for positioning. The user plane information includes the user plane positioning address of the LMF. The LMF allocates a LCS-UP binding ID to associate the user plane connection to be established with the target UE and includes the LCS-UP binding ID in the user plane information. The LMF associates the target UE identity (SUPI and/or GPSI) with the LCS-UP binding ID.

5.	[Conditional] When AMF receives the user plane information from LMF in step 4, AMF forwards it to UE via a DL NAS TRANSPORT message.

6.	[Conditional] If there is no established secure user plane connection, UE establishes a secured user plane connection with LMF. UE uses the user plane positioning address of the LMF, together with the information in the URSP, to determine the PDU session parameters including DNN+S-NSSAI. UE uses the PDU session parameter to establish PDU session. When SMF receives the request, it selects a proper UPF based on the DNN+S-NSSAI, and establishes the connection between the UPF and LMF.

After the secured user plane connection has been established successfully, the UE sends the LCS-UP binding ID received in step 4 to LMF via the secured user plane connection to enable LMF to perform the correlation of the UE with this secured user plane connection. The LCS-UP binding ID will be released once the correlation is complete.

7.	[Conditional] UE sends an acknowledgement to LMF through AMF to indicate a success of user plane connection establishment for positioning service or a failure to utilize the user plane connection as defined in TS 24.572 [48].

8.	[Conditional] AMF sends the acknowledgement received in step 7 to the LMF via Namf_N1messageNotify service.

9.	[Conditional] LMF responds to AMF that user plane connection between the UE and LMF has been established.

10.	[Conditional] The AMF stores the LCS-UP connection context as part of UE context

11.	[Conditional] After the secure user plane connection is established, if LMF determines to utilize the user plane connection for positioning after receiving a positioning request from AMF, or UE determines to utilize the user plane connection for positioning, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data. Supplementary services messages including event report messages, periodic triggered invoke messages and MS cancel deferred location messages may also be transferred between LMF and UE via the established user plane connection.

>>>> End of Changes <<<<