# 3GPP TSG-SA Meeting #108	SP-250578

Prague, CZ, June 10 – 13, 2025

# Source:	Nokia

# Title:	pCR TR 22.850: Updates to section 5.3.2

# Document for:	Approval

# Agenda Item:	7

# 1	Introduction

This pCR aims to provide additional description for AI/ML related activities in RAN WGs.

# 2	Detailed proposal

***** START OF CHANGES *****

---------------------------------------------------------------------------------------------------------------------------------------------

### 5.3.2	AI/ML related activities

Editor's note:	Description clause will be further clarified whether to be based on WI summaries or WID objectives.

#### 5.3.2.1 Rel-19 RAN WG1/RAN WG4 WID - Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (NR_AIML_air)

##### 5.3.2.1.1	Description

The objective of this work is to provide specification support for the following aspects:

-	AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

-	Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback:

-	Identification related signalling is part of the above objective.

-	Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models.

-	Signalling mechanism of applicable functionalities/models.

-	Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

-	Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams ("BM-Case1").

-	Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams ("BM-Case2").

-	Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any.

-	Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE.

-	Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases.

- CSI feedback enhancement, encompassing [RAN1/RAN2]:

  - CSI prediction (UE-sided model):

    - Functionality-based LCM leveraged from other use cases, when necessary and applicable,

    - Study, and if necessary, specify consistency of training/inference,

    - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

- Core requirements for the above three use cases for AI/ML LCM procedures and UE features [RAN4]:

  - Specify necessary RAN4 core requirements for the above three use cases.

  - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

The objective of Performance part of this work are:

- For Beam Management, Positioning Accuracy enhancement and CSI prediction use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models

  - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases

  - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

Note: the following aspects may be considered:

    - Relation to legacy requirements

    - Performance monitoring and LCM aspects considering use-case specifics

    - Generalization aspects

    - Static/non-static scenarios/conditions and propagation conditions for testing (e.g. CDL, field data, etc.)

    - UE processing capability and limitations

    - Post-deployment validation due to model change/drift

  - RAN5 aspects related to testability and interoperability to be addressed on a request basis

***** NEXT CHANGE *****

#### 5.3.2.X 	Rel-19 RAN WG1/RAN WG4 SID - Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (FS_NR_AIML_air_Ph2)

##### 5.3.2.X.1	Description

The objective of this work is to provide further study on some outstanding issues identified during the prior study. specifically for the following aspects:

- CSI feedback enhancement [RAN1]:

  - For CSI temporal prediction: further update TR 38.843 with additional evaluations.

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

- Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

- Testability and interoperability [RAN4]:

  - Further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least:

    - Relation to legacy requirements

    - Performance monitoring and LCM aspects considering use-case specifics

    - Generalization aspects

    - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.)

    - UE processing capability and limitations

    - Post-deployment validation due to model change/drift

  - RAN5 aspects related to testability and interoperability to be addressed on a request basis

***** NEXT CHANGE *****

#### 5.3.2.Y 	Rel-19 RAN WG3 WID - Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (FS_NR_AIML_NGRAN_enh-Core)

##### 5.3.2.Y.1	Description

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743 (FS_NR_AIML_NGRAN_enh).

The objective of this work is to provide specification support for the following aspects:

-	Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3]

-	Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]:

  - Mobility Optimization for NR-DC

  - Split architecture support for Rel-18 use cases

  - Continuous MDT collection targeting the same UE across RRC states

Note: Coordination with RAN2, SA5 when needed if any.

##### 5.3.2.Y.2	Activities summary

Editor’s note:	Reference to TR 21.919 can be added when the work item summary is made available.

***** END OF CHANGES *****