3GPP TSG-SA WG6 Meeting #66	S6-251319

Göteborg, Sweden, 07th – 11th April 2025	(revision of S6-251077)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0142 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update SEALDD XR transmission connection establishment information flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SEALDD XR transmission connection establishment request and response information flows are missing clarity in the relation between the "SEALDD-UU flow IDs", the “Multi-modal SEALDD flow ID", and the "traffic descriptors" IEs. This paper proposes to restructure the SEALDD-XR transmission connection establishment request and response information flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - In the SEALDD-XR transmission connection establishment request, "SEALDD-UU flow IDs" IE is changed to "SEALDD-UU flow" and restructured to include the sub-elements "SEALDD-UU flow ID" and the "traffic descriptors" (with two mandatory sub-elements: "Addressing info" and "Transport layer protocol"). - In the SEALDD-XR transmission connection establishment response, the "Multi-modal SEALDD flow ID" is changed to "Multi-modal SEALDD info" and restructured to include the sub-elements "Multi-modal SEALDD flow ID" and "list of SEALDD-UU flows" (with a mandatory sub-element: "SEALDD-UU flow ID"). The "Traffic descriptor" with two mandatory sub-elements: "Addressing info" and "Transport layer protocol". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition SEALDD-XR transmission connection establishment request and response information flows are inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.3.3, 9.12.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.12.3.3	SEALDD XR transmission connection establishment request

Table 9.12.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the XR transmission connection establishment.

Table 9.12.3.3-1: SEALDD XR transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client ID | M | Identity of the SEALDD client. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID |
| SEALDD-UU flows info | M | Information of the list of SEALDD-UU flows. |
| > SEALDD-UU flow ID | M | Identity of the SEALDD-UU flow. |
| > SEALDD traffic descriptor | M | SEALDD traffic descriptor for the flow of the SEALDD client side used to establish SEALDD multi-modal XR connection. |
| >> Addressing info | M | Indicates the addressing information (e.g. address, port, URL). |
| >> Transport layer protocol | M | Indicates the transport layer protocol. |
| VAL server ID | O | Identity of the VAL server. |
| VAL service ID | O | Identity of the VAL service. |

* * * Next Change * * * *

#### 9.12.3.4	SEALDD XR transmission connection establishment response

Table 9.12.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the XR transmission connection establishment.

Table 9.12.3.4-1: SEALDD XR transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| Multi-modal SEALDD flow info | M | Information of the Multi-modal SEALDD-UU flow associated with the multi-modal XR connection. |
| > Multi-modal SEALDD flow ID | M | Identity of the Multi-modal SEALDD-UU flow associated with the multi-modal XR connection. |
| > SEALDD-UU flows list | M | Indicates the list of SEALDD-UU flows. |
| >> SEALDD-UU flow ID | M | Identity of the SEALDD-UU flow. |
| >> SEALDD traffic descriptor | M | SEALDD traffic descriptors for the flow of the SEALDD server side used to establish SEALDD multi-modal connection. |
| >>> Addressing info | M | Indicates the addressing information (e.g. address, port, URL). |
| >>> Transport layer protocol | M | Indicates the transport layer protocol. |
| Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |
| Cause | O (See NOTE) | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

* * * END of Changes * * * *