3GPP TSG SA WG 1 Meeting #110 	S1-252450

Fukuoka, Japan, 19 - 23 May 2025	(revision of S1-252100)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0836 | rev | 1 | Current version: | 20.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further consolidation of requirements on service adjustments based on energy-related characteristics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergyServ_Ph2-REQ |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Two requirements of clause 6.15a.2.2 correspond to similar functionality and would better be merged into a single requirement. In addition, these requirements should apply to the whole 5G system, not just the 5G network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Merge two paragraphs of clause 6.15a.2.2 and replace “5G network” by “5G system” in the merged paragraph. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary complexity in the specification. Unnecessary restriction of the solution space. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.15a.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.15a.2.2	Requirements

NOTE:	The following requirements are expected to produce net energy saving in the 5G network i.e. the energy used to operate the network and deliver services based on these requirements is less that the energy saved.

The following requirements are subject to regulatory requirements, in particular to keep delivering critical services such as emergency calls, PWS, MPS and MCS to the extent possible.

Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a means to associate energy consumption information with charging information based on subscription policies for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a mechanism to perform energy consumption credit limit control for services without QoS criteria.

NOTE 1: The result of the credit control is not specified by this requirement.

NOTE 2:	Credit control [49] compares against a credit control limit. It is assumed charging events are assigned a corresponding energy consumption and this is compared against a policy of energy credit limit. It is assumed there can be a new policy to limit energy consumption allowed.

Subject to operator’s policy, the 5G system shall support a means to define subscription policies and means to enforce the policy that define a maximum energy consumption (i.e. quantity of energy for a specified period of time) for services without QoS criteria.

NOTE 3:	The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services.

The 5G system shall provide a mechanism to include Energy related information as part of charging information.

Subject to operator policy and agreement with 3rd party, the 5G system shall provide a mechanism to support the selection of an application server based on energy related information associated with a set of application servers.

Subject to user consent and operator policy, 5G system shall be able to provide means to modify a communication service based on energy related information criteria based on subscription policies.

Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to provide means to operate part or the whole network according to energy consumption requirements, which may be based on subscription policies or requested by an authorized 3rd party.

Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall enable the operator to provide means to degrade service performance (e.g. QoS, bitrate) to meet energy rationing constraints.

NOTE 4: 	This assumes that the degradation in service performance will reduce energy consumption in the 5G network to meet energy rationing constraints.

Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall support subscription policies that include alternative (i.e. degraded) service performance (e.g. QoS parameters, maximum bitrate) of services with QoS criteria for energy saving reasons.

NOTE 5:	This requirement implies that the policies could disallow some network energy saving action to be performed for some service with QoS criteria e.g. based on applicability conditions (e.g. slice, application).

Subject to operator’s policy, the 5G network shall be able to support a means to target per UE energy saving actions, based on subscription policies.

Subject to operators’ policy, regulatory requirements, and user consent, the 5G system shall provide mechanisms to adjust communication service (e.g. user plane path, suitable Service Hosting Environment, defer background traffic delivery) and control the access of UEs to the network (e.g. block traffic, disable or defer specific application traffic) considering the change of energy supply mix of the network as one of the factors or based on the energy-related characteristics of the network.

NOTE 6:	It is assumed that 5G network can obtain energy supply mix information. How to obtain this information is out of scope of this requirement. The adjustment is not assumed to be real-time. It is up to the operators’ policy to define the time difference between obtaining the change on energy supply mix information and the actual adjustment of the communication service.

NOTE 7:  Degradation of service experience resulting from the adjustment is subject to subscription-based user consent.

Subject to operator’s policy and regulatory requirements, the 5G network shall be able to trigger charging events corresponding to an impacted UE when degrading performance of services with QoS criteria (e.g. to a lower bitrate) in order to achieve energy saving.

Subject to user consent, operator policy and regulatory requirements, the 5G network shall be able to assist an authorized 3rd party to identify a set of target UEs for whom to adjust the provided application service, considering criteria such as the current and future (e.g. predicted) energy-related characteristics of their serving network.

NOTE 8:	Future (e.g. predicted) information can refer to the next hour(s) or day(s), to the remaining serving time before shutdown/shortage etc.

END OF CHANGES