3GPP TSG-SA3 Meeting #121	S3-251711

Goteborg, Sweden, 7 – 11 April 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0098 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authentication of ROF and CCF for CAPIF-8 reference point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR proposes the normative text for authentication of ROF and CCF based on the following conclusions for the KI#1.1 of TR 33.700-22: -The authentication of the CCF is based on TLS using the CCF’s certificate. The ROF authentication is to be decided in the normative work. Since ROF may have different formats in different applications, e.g., websites, agent, devices, interfaces to human, it is proposed not to specify ROF authentication used and left for implementation based on different applications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Normative text is provided for authentication of ROF and CCF for the CAPIF-8 reference point. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.11	Security procedures for CAPIF-8 reference point

TLS shall be used to provide integrity protection, replay protection and confidentiality protection.

Authentication of CCF shall be based on the CCF’s certificates following the profiles specified in clauses 6.1.3a and 6.1.4a of the 3GPP TS 33.310 [2].

Authentication of ROF is left to implementation.

* * * End of Changes * * * *