3GPP TSG-SA5 Meeting #164	S5-255052

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1639 | rev | - | Current version: | 18.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.541 YANG stage-3 corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | YANG SS is not matching the approved stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update YANG code to match existing stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and Stage 3 mismatch; interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1957 at commit c2352febb4f6b2737136a853db4cee18d67c031c |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1957 at commit c2352febb4f6b2737136a853db4cee18d67c031c

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-5gc-nrm-amffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-amffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amffunction;

prefix amf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcuupfunction { prefix gnbcuup3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "AMFFunction derived from basic ManagedFunction.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-08-18 { reference CR-1557 ; }

revision 2024-10-06 { reference CR-1389; }

revision 2024-04-04 { reference CR-1139; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 { reference "Ericsson refactoring."; }

revision 2018-08-07 { reference "Initial revision"; }

grouping NTNGlobalRanNodeIDGrp{

list pLMNId {

description "The PLMN Identifier is composed of

a Mobile Country Code (MCC) and

a Mobile Network Code (MNC).";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

choice GlobalRanNodeID{

description "Unique identifier of an NG-RAN node.";

reference "3GPP TS 38.413";

mandatory true;

case gNB {

leaf GlobalgNBID{

description "Unique identifier of a gNB.";

reference "3GPP TS 38.413";

type int64;

mandatory true;

}

}

case ng-eNB{

leaf Globalng-eNBID{

type string{

pattern '^(MacroNGeNB-[A-Fa-f0-9]{5}|LMacroNGeNB-

[A-Fa-f0-9]{6}|SMacroNGeNB

-[A-Fa-f0-9]{5})$';

}

description "This represents the identifier of

the ng-eNB ID as specified in

clause 9.3.1.8 of 3GPP TS 38.413.";

mandatory true;

}

}

case N3IWF{

leaf N3IWFID{

type string{

pattern '^[A-Fa-f0-9]+$';

}

description "This IE shall contain the N3IWF

identifier received over NGAP and

shall be encoded as a string of

hexadecimal characters.";

mandatory true;

}

}

case TNGF{

leaf TNGFID{

type string{

pattern '^[A-Fa-f0-9]+$';

}

description "This represents the identifier of

the TNGF ID as specified in clause

9.3.1.161 of 3GPP TS 38.413 in hexadecimal

representation.";

mandatory true;

}

}

case TWIF{

leaf TWIFID{

type string;

description "This represents the TWIF identification

as specified in clause

9.3.1.163 of TS 38.413";

mandatory true;

}

}

case W-AGF{

leaf W-AGFID{

type string{

pattern '^[A-Fa-f0-9]+$';

}

description "This represents the identifier of

the W-AGF ID as specified in

clause 9.3.1.162 of 3GPP TS 38.413

in hexadecimal representation.";

mandatory true;

}

}

}

}

grouping SatelliteBackhaulInfoGrp{

description "Defines information related to satellite backhaul category

and corresponding information of gNB.";

reference "3GPP TS 23.501";

list nTNGlobalRanNodeID{

description "Unique identifier of an NG-RAN node.";

min-elements 1;

max-elements 1;

leaf GlobalRanNodeIDValue{

type string;

description "this relies on the choice of GlobalRanNodeID";

}

key "GlobalRanNodeIDValue";

uses NTNGlobalRanNodeIDGrp;

}

leaf satelliteBackhaulCategory{

description "Satellite backhaul category refers to the type of the

satellite used in the backhaul. Only a single backhaul category

can be indicated.";

reference "3GPP TS 29.571";

type enumeration{

enum GEO;

enum MEO;

enum LEO;

enum OTHER_SAT;

enum DYNAMIC_GEO;

enum DYNAMIC_MEO;

enum DYNAMIC_LEO;

enum DYNAMIC_OTHER_SAT;

enum NON_SATELLITE;

}

mandatory true;

}

leaf geoSatelliteId{

description "Unique identifier of a GEO satellite.";

reference "3GPP TS 29.571";

type string;

//condition present only if the UPF is deployed on the satellite.

}

}

grouping GUAMInfoGrp {

description "Represents the GUAMInfo datatype.";

list pLMNId {

description "This attribute represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:PLMNId ;

}

leaf aMFIdentifier  {

type int64;

description "The AMFI is constructed from an AMF Region ID, an

AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region,

the AMF Set ID uniquely identifies the AMF Set within the AMF Region,

and the AMF Pointer uniquely identifies the AMF within the AMF Set.

(Ref. 3GPP TS 23.003)";

}

}

grouping N2InterfaceAmfInfoGrp {

description "Represents the N2InterfaceAmfInfo datatype.";

leaf-list ipv4EndpointAddress {

type inet:ipv4-address;

min-elements 1;

description "This attribute represents available AMF endpoint

IPv4 address(es) for N2.";

}

leaf-list ipv6EndpointAddress {

type inet:ipv6-address;

min-elements 1;

description "This attribute represents available AMF endpoint

IPv6 address(es) for N2.";

}

leaf amfName {

type inet:host-name;

description "This attribute represents AMF Name FQDN as defined in

clause 28.3.2.5 of TS 23.003.";

}

}

grouping BlockedLocationInfoGrp {

description "Repreasents the datatype BlockedLocationInfo";

list blockedLocation {

description "This provides the geographical location at which the PLMN

are not allowed in case of NTN.";

min-elements 1;

max-elements 1;

key "mnc mcc";

uses types3gpp:PLMNId ;

}

list blockedDurWindow {

description "This provides the time durations for which the PLMN are

not allowed at a given location in case of NTN";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TimeWindowGrp;

}

list blockedSlice {

description "This provides the slice for which the access is

not allowed at a given location in case of NTN. ";

max-elements 1;

key idx;

unique "sst sd";

leaf idx { type uint32; }

uses types3gpp:SNssai;

}

}

grouping NTNPLMNRestrictionsInfoGrp {

description "Represents the datatype NTNPLMNRestrictionsInfo";

list pLMNId {

description "This attribute represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "mnc mcc";

uses types3gpp:PLMNId ;

}

list blockedLocationInfoList {

description "This defines the information related with the location

for which the access restrictions are to be applied in case of NTN.";

key "idx";

leaf idx { type uint32 ; }

uses BlockedLocationInfoGrp;

}

}

grouping NtnLocationInfoGrp {

description  "Represents the NtnLocationInfo datatype.";

list location {

description "This defines the Location (geographical area) under

consideration to which the satellite coverage info belongs";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:GeoAreaGrp;

}

list availabilityWindows  {

description "This attribute defines the list of time windows at which

the satellite coverage will be available for this location.

Either availabilityWindows or nonAvailabilityWindows shall be present.";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TimeWindowGrp;

}

list nonAvailabilityWindows {

description "This attribute defines the list of time windows at which

the satellite coverage will not be available for this location.

Either availabilityWindows or nonAvailabilityWindows shall be present.";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TimeWindowGrp;

}

}

grouping SatelliteCoverageInfoGrp {

description "Represents the datatype SatelliteCoverageInfo";

leaf nRSatelliteRATtype {

type enumeration {

enum NRLEO;

enum NRMEO;

enum NRGEO;

enum NROTHERSAT;

}

description "This attribute defines the RAT Type for NR satellite

access.";

}

list locationInfo {

description "This attribute defines the information about location and

corresponding time windows for which the satellite coverage will be

available or unavailable.";

key idx;

leaf idx { type uint32 ; }

uses NtnLocationInfoGrp;

}

}

grouping SliceExpiryInfoGrp {

description "Represents the datatype SliceExpiryInfo";

list pLMNInfo {

min-elements 1;  // ???

config false;

description "It defines the PLMN(s) of a Network Function. ";

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:PLMNInfo;

}

leaf expiryTime {

type yang:date-and-time;

config false;

description "This attribute provides information about the time at

which the slice is scheduled to be expired as it is not required

anymore.

This attribute will be set based on the sliceAvailability coming as

part of ServiceProfile.";

}

}

grouping MappedCellIdInfoGrp {

description "Represents the datatype MappedCellIdInfo";

list ntnGeoArea {

description "This attribute indicates a specific geographical location

mapped to Mapped Cell ID(s).";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:GeoAreaGrp;

}

list mappedCellId {

description "This attribute is in format of NCGI to indicate a fixed

geographical area (See subclause 16.14.5 in TS 38.300";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types5g3gpp:NcgiGrp;

}

}

grouping AmfInfoGrp {

description "Represents the data type AmfInfo.";

leaf amfRegionId {

type int64;

mandatory true;

description "It represents the AMF Region ID, which identifies the

region. AllowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 ";

}

leaf amfSetId {

type int64;

mandatory true;

description "It represents the AMF Set ID, which is uniquely identifies

the AMF Set within the AMF Region.

allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003";

}

list taiList {

min-elements 1;

description "The list of TAIs.";

key "idx";

leaf idx { type uint32 ; }

uses gnbcuup3gpp:TAIGrp;

}

list taiRangeList {

min-elements 1;

description "The range of TAIs.";

key "idx";

leaf idx { type uint32 ; }

uses nfp3gpp:TaiRangeGrp;

}

list gUAMIdList {

description "List of supported Globally Unique AMF Ids (GUAMIs).";

config false;

min-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses GUAMInfoGrp;

}

list backupInfoAmfFailure {

description "List of GUAMIs for which the AMF acts as a backup for AMF

failure.";

min-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses GUAMInfoGrp;

}

list backupInfoAmfRemoval {

description "List of GUAMIs for which the AMF acts as a backup for

planned AMF removal.";

min-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses GUAMInfoGrp;

}

list n2InterfaceAmfInfo {

description "This attribute represents the N2 interface information of

the AMF. ";

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses N2InterfaceAmfInfoGrp;

}

leaf amfOnboardingCapability {

type boolean;

default false;

description "This attribute indicates the AMF supports SNPN Onboarding

capability. This is used for the case of Onboarding of UEs for SNPNs

(see TS 23.501, clause 5.30.2.10).

- FALSE: AMF does not support SNPN Onboarding;

- TRUE: AMF supports SNPN Onboarding.";

}

leaf highLatencyCom {

type boolean;

description "This attribute indicates whether the AMF supports High

Latency communication (e.g. for NR RedCap UE). This is used for CP NF

to discover AMF supporting High Latency communication (see TS 23.501,

clause 6.3.5).

- FALSE: AMF does not support High Latency communication e.g. for

NR RedCap UE;

- TRUE: AMF supports High Latency communication e.g. for NR RedCap UE;";

}

}

grouping AMFFunctionGrp {

description "Represents the AMFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNInfoList {

min-elements 1;

description "It defines the PLMN(s) of a Network Function. ";

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:PLMNInfo;

}

container aMFIdentifier {

description "An AMF identifier, comprising an AMF Region ID, an

AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

leaf-list cNSIIdList {

type string;

description "It is a set of NSI ID. NSI ID is an identifier for

identifying the Core Network part of a Network Slice instance when

multiple Network Slice instances of the same Network Slice are deployed,

and there is a need to differentiate between them in the 5GC.

See NSI ID definition in clause 3.1 of TS 23.501 and

subclause 6.1.6.2.7 of  TS 29.531";

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

list amfInfo {

max-elements 1;

description "This attribute represents information of an AMF

NF Instance.";

key "idx";

leaf idx { type uint32 ; }

uses AmfInfoGrp;

}

list nTNPLMNRestrictionsList {

description "This attribute defines the location restrictions per PLMN

that relates to non-terrestrial network access.";

key "idx";

leaf idx { type uint32 ; }

uses NTNPLMNRestrictionsInfoGrp;

}

list satelliteCoverageInfoList {

description "This attribute defines the information related to NR

Satellite RAT type and corresponding information of satellite coverage";

key "idx";

leaf idx { type uint32 ; }

uses SatelliteCoverageInfoGrp;

}

list sliceExpiryInfo {

description "This provides information related to a network slice

validity.";

key "idx";

leaf idx { type uint32 ; }

uses SliceExpiryInfoGrp;

}

list satelliteBackhaulInfoList{

description "Specifies a list of satellite backhaul information. It can be

used by NF and NF services.";

leaf GlobalRanNodeIDValue{

type string;

description "this relies on the choice of GlobalRanNodeID";

}

key "GlobalRanNodeIDValue";

min-elements 1;

uses SatelliteBackhaulInfoGrp;

}

leaf-list mdtUserConsentReqList {

type enumeration {

enum M1;

enum M2;

enum M3;

enum M4;

enum M5;

enum M6;

enum M7;

enum M8;

enum M9;

enum MDT_UE_LOCATION;

}

description "represents a list of MDT measurement names that are

subject to user consent at MDT activation.

Any MDT measurement, whose name is not specified in this list, is not

subject to user consent at MDT activation.";

}

list mappedCellIdInfoList {

description "It provides the list of mapping between GEO area and

Mapped Cell ID.";

key "idx";

leaf idx { type uint32 ; }

uses MappedCellIdInfoGrp;

}

leaf aMFSetRef {

type types3gpp:DistinguishedName;

description "This is the DN of AMFSet";

}

}

augment "/me3gpp:ManagedElement" {

list AMFFunction {

description "This IOC represents the AMF functionality in 5GC.

For more information about the AMF, see 3GPP TS 23.501.

The attribute sliceExpiryInfo is used when the validity information of

a network slice need to be configured. The attribute

sliceExpiryInfo.pLMNInfo indicates the network slice to which the

validity information applies.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 2 ***

*** yang-models/_3gpp-5gc-nrm-ausffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-ausffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ausffunction;

prefix ausf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the AUSF function in 5GC. For more

information about the AUSF, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-22 {reference "initial revision"; }

grouping AusfInfoGrp {

description "Represents the information of an AUSF NF Instance

(see TS 29.510).";

grouping SuciInfoGrp {

description "This data type represents SUCI information containing

Routing Indicator and Home Network Public Key ID. (See TS 29.510)";

leaf-list routingInds {

type string;

min-elements 1;

description "It indicates served Routing Indicator (see TS 23.003,

clause 2.2B). If not provided, the AUSF/UDM

can serve any Routing Indicator.";

}

leaf-list hNwPubKeyIds  {

type int64;

min-elements 1;

description "It indicating served Home Network Public Key

(see TS 23.003, clause 2.2B). If not provided, the AUSF/UDM

can serve any public key.";

}

}

leaf nFSrvGroupId {

type string;

mandatory false;

config false;

yext3gpp:inVariant;

yext3gpp:notNotifyable;

description "This parameter defines identity of the group that is

served by the NF instance (See TS 29.510).";

}

list supiRanges {

description "It represents list of ranges of SUPIs that can be served

by the PCF instance.";

key idx;

leaf idx { type uint32 ; }

uses nfp3gpp:SupiRange;

}

leaf-list routingIndicators {

description "This attribute represents a list of Routing Indicator

information that allows to route network signalling with SUCI

(see TS 23.003 [13]) to the AUSF instance.

If not provided, the AUSF can serve any Routing Indicator.";

type string {

pattern "[0-9]{1,4}";

}

}

list suciInfos {

description "This attribute represents a list of SuciInfo. A SUCI

that matches this information can be served by the AUSF.

A SUCI that matches all attributes of at least one entry in this

array shall be considered as a match of this information.";

key idx;

leaf idx { type uint32 ; }

uses SuciInfoGrp;

}

}

grouping AUSFFuntionGrp {

description "Represents the AUSFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNInfoList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and

a Mobile Network Code (MNC).";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:PLMNInfo;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

list ausfInfo {

description "It represents the information of an AUSF NF Instance

(see TS 29.510)";

key idx;

min-elements 1;

max-elements 1;

leaf idx { type uint32 ; }

uses AusfInfoGrp;

}

}

augment "/me3gpp:ManagedElement" {

list AUSFFunction {

description "5G Core AUSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AUSFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 2 ***

*** START OF CHANGE 3 ***

*** yang-models/_3gpp-5gc-nrm-lmffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-lmffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-lmffunction;

prefix lmf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

description "This IOC represents the LMF function defined in 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-25 { reference "S5-194457 S5193518"; }

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping LMFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id). The PLMN Identifier is composed of

a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list LMFFunction {

description "5G Core LMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses LMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 3 ***

*** START OF CHANGE 4 ***

*** yang-models/_3gpp-5gc-nrm-managed-nfprofile.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-managed-nfprofile {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-managed-nfprofile;

prefix mnfp3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "NF profile class.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 29.510";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-11-06 { reference "initial revision"; }

grouping SPNInfoIdGrp {

description "This data type represents the SNPN identification

consisting of MCC, MNC, and optionally NID.";

leaf mcc {

type string;

description "Mobile Country Code (MCC) of the SNPN.";

}

leaf mnc {

type string;

description "Mobile Network Code (MNC) of the SNPN.";

}

leaf nId {

type string;

description "Network Identifier (NID) of the SNPN.";

}

}

grouping SupiRangeGrp {

leaf start {

description "First value identifying the start ofa SUPI range.To be

used when the range of SUPI's can be represented as a numeric range

(e.g., IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {

description "Last value identifying the end of a SUPI range.To be

used when the range of SUPI's can be represented as a numeric range

(e.g. IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf pattern {

description "Pattern representing the set of SUPI's belonging to

this range.A SUPI value is considered part of the range if and

only if the SUPI string fully matches the regular expression.";

type string;

}

}

grouping ConditionItemGrp {

description "This data type represents a single condition item that

shall be evaluated to determine whether a discovered NF (Service)

Instance shall be selected.";

leaf consumerNfTypes {

description "It represents the NF types of the consumers for

which the conditions included in this ConditionItem apply.

If this attribute is absent, the conditions are applicable

to all NF consumer types.";

type types3gpp:NfType;

}

leaf serviceFeature {

description "It represents a feature number of that NF Service

Instance,under CANARY_RELEASE status. This attribute only

applies when the          selectionConditions, where this

ConditionItem is included, is included in a NF Service Instance

This condition is evaluated to <true> when the service requests

froma consumer of this NF Service Instance require the support

of the indicated feature on the NF Service Instance.";

type uint32; // positive integer, 0 excluded by context

}

leaf vsServiceFeature {

description "It represents a Vendor-Specific feature

number of thatNF Service Instance, under CANARY_RELEASE

status. This attribute only applies when the

selectionConditions, where this ConditionItem is included,

is included in a NF Service Instance.This condition is

evaluated to true when the service requests from a

consumer of this NF Service Instance require the support

of the indicated Vendor-Specific feature on the NF

Service Instance.";

type uint32;

}

list supiRangeList {

description "It represents a set of SUPIs for which

the NF (Service) instance under CANARY_RELEASE status shall

be selected.";

key idx;

leaf idx { type uint32; }

uses SupiRangeGrp;

}

list gpsiRangeList {

description "It represents a set of GPSIs for which the NF

(Service)

instance under CANARY_RELEASE status shall be selected";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses IdentityRange;

}

list impuRangeList {

description "It represents a set of IMS Public Identities

for which the NF (Service) instance under CANARY_RELEASE

status shall be selected";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses IdentityRange;

}

list impiRangeList {

description "It represents a set of IMS Private Identities

for which theNF (Service) instance under

CANARY_RELEASE status shall be selected.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses IdentityRange;

}

leaf peiList {

description "It represents a set of PEIs of the UEs for

which the NF(Service) instance under CANARY_RELEASE status

shall be selected";

type string;

}

list taiRangeList {

description "It represents a set of TAIs where the

NF(Service) instance under CANARY_RELEASE status shall

be selected for a certain UE";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses nfp3gpp:TaiRangeGrp;

}

leaf dnnList {

description " It represents a set of TAIs where the NF

(Service) instance under CANARY_RELEASE status shall be

selected for a certain UE";

type string;

}

}

grouping PlmnSnssai {

list plmnId {

description "PLMN ID for which list of supported S-NSSAI(s)

is provided.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssaiList {

description "The specific list of S-NSSAIs supported

by the given PLMN.";

min-elements 1;

max-elements 1;

key "sst sd";

uses types3gpp:SNssai;

}

leaf nid {

description "Network Identity; Shall be present if PlmnIdNid

identifies an SNPN ";

reference "see clauses 5.30.2.3, 5.30.2.9, 6.3.4,

and 6.3.8 in 3GPP TS 23.501 ";

type string;

}

}

grouping IdentityRange {

leaf start {

description "First value identifying the start of an identity range.

To be used when the range of identities can be represented

as a numeric range (e.g., MSISDN ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {type string; } // dummy

leaf pattern {type string; } // dummy

}

grouping ManagedNFProfileGrp {

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

mandatory true;

type string;

}

leaf nfType {

description "Type of Network Function.";

mandatory true;

type types3gpp:NfType;

}

leaf heartBeatTimer {

description "Time in seconds expected between 2 consecutive

heart-beat messages from an NF Instance to the NRF.

It may be included in the registration request.

When present in the request it shall contain the

heartbeat time proposed by the NF service consumer.";

type uint16;

}

leaf hostAddr {

mandatory true;

type inet:host ;

description "Host address of a NF";

}

leaf authzInfo {

description "This parameter defines NF Specific Service authorization

information. It shall include the NF type (s) and NF realms/origins

allowed to consume NF Service(s) of NF Service Producer ";

reference "TS 23.501";

type string;

}

list allowedPlmns {

description "PLMNs allowed to access the NF instance.

If not provided, any PLMN is allowed to access the NF.";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNPNList {

key "mcc mnc nId";

description "SNPN(s) of the Network Function. This attributeIE shall be

present if the NF pertains to one or more SNPNs.";

reference "TS 29.510";

uses SPNInfoIdGrp;

}

list allowedSNPNs {

description "SNPNs allowed to access the NF instance.

The absence of this attribute in the NF profile indicates that no SNPN,

other than the SNPN(s) registered in the snpnList attribute of the

NF Profile, is allowed to access the service instance";

key "mCC mNC";

uses types3gpp:SNPNIdGrp;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the NF instance.

If not provided, any NF type is allowed to access the NF.";

min-elements 1;

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern (regular expression according to the ECMA-262

dialect [75]) representing the NF domain names within the PLMN of

the NRF allowed to access the NF instance If not provided,

any NF domain is allowed to access the NF.";

type string;

}

list allowedNSSAIs {

description "S-NSSAI of the allowed slices to access the NF instance.

If not provided, any slice is allowed to access the NF.";

key "sst sd";

uses types3gpp:SNssai;

}

leaf locality {

description "Operator defined information about the location

of the NF instance (e.g. geographic location, data center).";

type string;

}

leaf capacity {

description "Static capacity information in the range of 0-65535,

expressed as a weight relative to other NF instances of

the same type; if capacity is also present in the nfServiceList

parameters, those will have precedence over this value.";

type uint16;

}

leaf-list recoveryTime {

description "Timestamp when the NF was (re)started. The NRF shall notify

NFs subscribed to receiving notifications of changes of the NF profile,

if the NF recoveryTime is changed";

type yang:date-and-time;

}

leaf nfServicePersistence {

description "This parameter indicates whether the different service

instances of a same NF Service in the NF instance, supporting a same

API version, are capable to persist their resource state in shared

storage and therefore these resources are available after a new NF

service instance supporting the same API version is selected by a NF

Service Consumer ";

reference "TS 29.510";

type boolean;

}

leaf-list nfSetIdList {

type string ;

min-elements 1;

description "A NF Set Identifier is a globally unique identifier of

a set of equivalent and interchangeable CP NFs from a given network

that provide distribution, redundancy and scalability

(see clause 5.21.3 of 3GPP TS 23.501 [2]).An NF Set Identifier shall

be constructed from the MCC, MNC, NID (for SNPN), NF type and a Set ID.

A NF Set Identifier shall be formatted as the following string:

set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC> for a NF Set in a PLMN, or

set<Set ID>.<nftype>set.5gc.nid<NID>.mnc<MNC>.mcc<MCC> for a NF Set

in a SNPN.At most one NF Set ID shall be indicated per PLMN-ID or SNPN

of the NF.";

}

leaf-list nfProfileChangesSupportInd {

description "This parameter indicates if the NF Service Consumer supports

or does not support receiving NF Profile Changes. It may be present in

the NFRegister or NFUpdate (NF Profile Complete Replacement) request and

shall be absent in the response (see Annex B 3GPP TS 29.510 [23]).  ";

max-elements 1;

type boolean;

}

leaf-list nfProfilePartialUpdateChangesSupportInd {

description "It represents NF Profile Partial Update Changes Support

Indicator. TRUE: the NF Service Consumer supports receiving NF Profile

Changes in the response to an NF Profile Partial Update operation.

FALSE (default): the NF Service Consumer does not support receiving

NF Profile Changes in the response to an NF Profile Partial Update

operation";

max-elements 1;

type boolean;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.

This attribute may contain multiple default subscriptions for a same

notification type; in that case, those default subscriptions are used

as alternative notification endpoints.";

key notificationType;

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

leaf servingScope {

description "This parameter indicates the served geographical areas

of a NF instance.";

type string;

}

leaf-list lcHSupportInd {

description "This parameter indicates whether the NF supports or does

not support Load Control based on LCI Header.";

reference "clause 6.3 of 3gpp TS 29.500";

max-elements 1;

type boolean;

}

leaf-list olcHSupportInd {

description "This parameter indicates whether the NF supports or does

not support Overload Control based on OCI Header .";

reference "clause 6.4 of 3gpp TS 29.500";

max-elements 1;

type boolean;

}

leaf-list nfSetRecoveryTimeList {

description "This parameter contains the recovery time of NF Set(s)

indicated by the NfSetId, where the NF instance belongs.";

type yang:date-and-time ;

max-elements 1;

}

leaf-list serviceSetRecoveryTimeList {

description "This parameter contains the recovery time of NF Service

Set(s) configured in the NF instance, which are indicated by the

NfServiceSetId";

type yang:date-and-time ;

max-elements 1;

}

leaf scpDomains {

description "This parameter shall carry the list of SCP domains the

SCP belongs to, or the SCP domain the NF (other than SCP) or the SEPP

belongs to..";

type string;

}

leaf vendorId {

description "Vendor ID of the NF instance, according to the

IANA-assigned SMI Network Management Private Enterprise Codes [77].

allowedValues: 6 decimal digits; if the SMI code has less than

6 digits, it shall be padded with leading digits 0 to complete

a 6-digit string value.";

type string;

}

leaf-list interPlmnFqdn {

description "If the NF needs to be discoverable by other

NFs in a different PLMN, then an FQDN that is used

for inter-PLMN routing is specified.";

type inet:domain-name;

}

}

}

<CODE ENDS>

*** END OF CHANGE 4 ***

*** START OF CHANGE 5 ***

*** yang-models/_3gpp-5gc-nrm-neffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-neffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-neffunction;

prefix nef3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NEF function in 5GC. For more

information about the NEF, see 3GPP TS 23.501.

Copyright 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-08-18 { reference CR-1557 ; }

revision 2024-05-18 { reference CR-1273 ; }

revision 2024-04-12 { reference CR-1218 ; }

revision 2023-09-18 { reference CR-1103 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-20 { reference "initial revision"; }

grouping DnnInfoItemGrp {

description "This data type represents set of parameters supported by NF

for a given S-NSSAI. (See clause 6.1.6.2.97 of TS 29.510)";

leaf dnn {

type string;

mandatory true;

description "It represents supported DNN or Wildcard DNN if the NF

supports all DNNs for the related S-NSSAI. The DNN shall contain the

Network Identifier and it may additionally contain an Operator

Identifier. If the Operator Identifier is not included, the DNN is

supported for all the PLMNs in the plmnList of the NF Profile.";

}

}

grouping SnssaiInfoItemGrp {

description "This data type represents set of parameters supported by NF

for a given S-NSSAI. (See clause 6.1.6.2.97 of TS 29.510)";

list sNssai {

description "It represents the S-NSSAI the NetworkSlice managed object

is supporting. The S-NSSAI is defined in TS 23.003.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses types3gpp:SNssai;

}

list dnnInfoList{

description "It represents list of parameters supported by the

NF per DNN.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses DnnInfoItemGrp;

}

}

grouping AfEventExposureDataGrp {

description "This data type represents the AF Event Exposure data managed

by a given NEF Instance. (See clause 6.1.6.2.50 TS 29.510";

leaf-list afEvents {  // stage 2 double defined

type string;

min-elements 1;

description "It represents AF Event(s) exposed by the NEF after

registration of the AF(s) at the NEF.";

}

leaf-list afIds {

type string;

min-elements 1;

description "It represents list of application function identifiers of

the managed PFDs.";

}

leaf-list appIds {  // stage 2 shaky

type string;

min-elements 1;

description "It represents list of internal application identifiers.";

}

}

grouping PfdDataGrp {

description "This data type represents the list of Application IDs

and/or AF IDs managed by a given NEF Instance.

(See clause 6.1.6.2.49 TS 29.510)";

leaf-list appIds {  // stage 2 shaky

type string;

min-elements 1;

description "It represents list of internal application identifiers.";

}

leaf-list afIds {

type string;

min-elements 1;

description "It represents list of application function identifiers of

the managed PFDs.";

}

}

grouping UnTrustAfInfoGrp {

description "This data type represents information of an untrusted AF

Instance. (See clause 6.1.6.2.95 TS 29.510)";

leaf afId {

type string;

mandatory true;

description "It represents associated AF id.";

}

list sNssaiInfoList {

description "It represents S-NSSAIs and DNNs supported by the AF.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses SnssaiInfoItemGrp;

}

leaf mappingInd {

type boolean;

yext3gpp:initial-value false;

description "When present, this attribute indicates whether the

AF supports mapping between UE IP address (IPv4 address or IPv6 prefix)

and UE ID (i.e. GPSI).

True: the AF supports mapping between UE IP address and UE ID;

False: the AF does not support mapping between UE IP address

and UE ID.";

}

}

grouping NefInfoGrp {

description "This data type represents information of an NEF Instance.

(See clause 6.1.6.2.48 TS 29.510).";

list taiList {

config false;

description "List of TAIs";

yext3gpp:inVariant;

yext3gpp:notNotifyable;

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TaiGrp;

}

list taiRangelist {

config false;

description "Range of TAIs";

yext3gpp:inVariant;

yext3gpp:notNotifyable;

key idx;

leaf idx { type uint32 ; }

uses nfp3gpp:TaiRangeGrp;

}

leaf nefId {

type string;

mandatory true;

description "It represents the NEF ID.

(see clause 6.1.6.3.2 of TS 29.510)";

}

list pfdData {

config false;

description "It represents PFD data, containing the list of internal

application identifiers and/or the list of application function

identifiers for which the PFDs can be provided.

Absence of this attribute indicates that the PFDs for any internal

application identifier and for any application function identifier

can be provided.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses PfdDataGrp;

}

list afEeData {

config false;

description "It represents the AF provided event exposure data. The NEF

registers such information in the NRF on behalf of the AF.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses AfEventExposureDataGrp;

}

list gpsiRanges {

description "It represents list of ranges of GPSIs whose profile data

is available.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses nfp3gpp:IdentityRange;

}

list externalGroupIdentifiersRanges {

description "It represents list of ranges of external groups whose

profile data is available.";  // stage 2 definition is shaky

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses nfp3gpp:IdentityRange;

}

leaf-list servedFqdnList {

type string;

min-elements 1;

description "It represents pattern (regular expression according to

the ECMA-262 dialect [75]) representing the Domain names served by

the NEF.";

}

leaf-list dnaiList {

type string;

min-elements 1;

description "It represents list of Data network access identifiers

supported by the NEF. The absence of this attribute indicates that

the NEF can be selected for any DNAI.";

}

list unTrustAfInfoList {

description "It represents list of information corresponding to the AFs.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses UnTrustAfInfoGrp;

}

leaf uasNfFunctionalityInd {

type boolean;

yext3gpp:initial-value false;

description "When present, this attribute shall indicate whether the ,

NEF supports UAS NF functionality:

- True: UAS NF functionality is supported by the NEF.

- False (default): UAS NF functionality is not supported by the NEF";

}

leaf multiMemAfSessQosInd {

type boolean;

default false;

description "This attribute indicates whether the NEF supports

Multi-member AF session with required QoS functionality:

- TRUE: Multi-member AF session with required QoS functionality is

supported by the NEF

- FALSE (default): Multi-member AF session with required QoS

functionality is not supported by the NEF.";

}

leaf memberUESelAssistInd {

type boolean;

default false;

description "This attribute indicates whether the NEF supports member

UE selection assistance functionality:

- TRUE: member UE selection assistance functionality is supported by

the NEF

- FALSE (default): member UE selection assistance functionality is

not supported by the NEF.";

}

}

grouping NEFFunctionGrp {

description "Represents the NEFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

leaf-list capabilityList {

description "List of supported capabilities of the NEF.";

reference "3GPP TS 23.003";

type string;

}

leaf isCAPIFSup {

type boolean;

yext3gpp:inVariant;

}

list nefInfo {

description "This attribute represents information of an NEF NF Instance.";

key idx;

leaf idx { type uint32 ; }

uses NefInfoGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NEFFunction {

description "5G Core NEF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NEFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 5 ***

*** START OF CHANGE 6 ***

*** yang-models/_3gpp-5gc-nrm-ngeirfunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-ngeirfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ngeirfunction;

prefix ngeir3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the 5G-EIR function in 5GC. For more

information about the 5G-EIR, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {reference "initial revision"; }

grouping NGEIRFunctionGrp {

description "Represents the NGEIRFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and

a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list NGEIRFunction {

description "5G Core NGEIR Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NGEIRFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 6 ***

*** START OF CHANGE 7 ***

*** yang-models/_3gpp-5gc-nrm-nssffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-nssffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nssffunction;

prefix nssf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NSSF function in 5GC. For more

information about the NSSF, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-08-18 { reference CR-1557; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 { reference "initial revision"; }

grouping NSSFFunctionGrp {

description "Represents the NSSFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least

one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

leaf-list cNSIIdList {

description "NSI ID. NSI ID is an identifier for identifying the Core

Network part of a Network Slice instance when multiple Network Slice

instances of the same Network Slice are deployed, and there is a need

to differentiate between them in the 5GC, see clause 3.1 of TS 23.501

and subclause 6.1.6.2.7 of 3GPP TS 29.531";

type string;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NSSFFunction {

description "5G Core NSSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NSSFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 7 ***

*** START OF CHANGE 8 ***

*** yang-models/_3gpp-5gc-nrm-nwdaffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-nwdaffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nwdaffunction;

prefix nwdaf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NWDAF function in 5GC. For more

information about the NWDAF, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-08-18 { reference CR-1557; }

revision 2024-08-06 { reference CR-1130; }

revision 2024-04-12 { reference CR-1218; }

revision 2023-09-18 { reference CR-1043; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {reference "initial revision"; }

typedef NwdafEvent {

description "The detailed ENUM value for NwdafEvent see the

Table 5.1.6.3.4-1 in TS 29.520";

type union {

type enumeration {

enum SLICE_LOAD_LEVEL;

enum NETWORK_PERFORMANCE;

enum NF_LOAD;

enum SERVICE_EXPERIENCE;

enum UE_MOBILITY;

enum UE_COMMUNICATION;

enum QOS_SUSTAINABILITY;

enum ABNORMAL_BEHAVIOUR;

enum USER_DATA_CONGESTION;

enum NSI_LOAD_LEVEL;

enum DN_PERFORMANCE;

enum DISPERSION;

enum RED_TRANS_EXP;

enum WLAN_PERFORMANCE;

enum SM_CONGESTION;

}

type string;

}

}

grouping NwdafCapabilityGrp {

description "This data type represents the capability supported by the

NWDAF.";

reference "TS 29.510";

leaf analyticsAggregation {

type boolean ;

default false;

description "It indicates whether the NWDAF supports analytics

aggregation";

}

leaf analyticsMetadataProvisioning {

type boolean ;

default false;

description "It indicate whether the NWDAF supports analytics metadata

provisioning:";

}

}

grouping MlAnalyticsInfoGrp {

description "This data type represents ML Analytics Filter information

supported by the Nnwdaf_MLModelProvision service.";

reference "TS 29.510";

leaf-list mlAnalyticsIds {

type NwdafEvent;

ordered-by user;

description "This attribute represents the Analytic functionalities

(identified by nwdafEvent defined in TS 29.520 [85]) of the NWDAF

instance. MnS consumer can configure this attribute to specify

which Analytic functionalities (identified by nwdafEvent) can be

performed the NWDAF instance. If the value of this attribute is not

present, the NWDAF instance can perform any NWDAFEvents

Analytics Id(s) supported by the Nnwdaf_MLModelProvision service,

if none are provided the NWDAF can serve any mlAnalyticsId.";

}

list sNSSAIList {

min-elements 1;

description "List of S-NSSAIs the managed object is capable of

supporting.";

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list trackingAreaList {

description "This attribute represents area of Interest of the ML model,

if none are provided the ML model for the analytics can apply to any

TAIs.

If present, it represents the list of TAIs, it may contain one or

more non-3GPP access TAIs.";

key idx ;

leaf idx { type uint32; }

uses types3gpp:TaiGrp;

}

leaf-list mlModelInterInfo {

type string {

pattern "[0-9]{6}";

}

description "This attribute defines the list of NWDAF vendors that are

allowed to retrieve ML models from the NWDAF containing MTLF. The

absence of this attribute indicates that none of the NWDAF vendors

can retrieve the ML models.

allowedValues: 6 decimal digits; if the SMI code has less than

6 digits, it shall be padded with leading digits '0' to complete a

6-digit string value.";

}

leaf flCapabilityType {

type enumeration {

enum FL_SERVER {

description "NWDAF containing MTLF as Federated Learning Server";

}

enum FL_CLIENT {

description "NWDAF containing MTLF as Federated Learning Client";

}

enum FL_SERVER_AND_CLIENT {

description "NWDAF containing MTLF as Federated Learning Server and

Client.";

}

}

description "This attribute defines the federated learning capability

type supported by NWDAF containing MTLF.";

}

list flTimeInterval {

key idx ;

leaf idx { type uint32; }

description "This attribute defines the time window at which the

indicated flCapabilityType supported by NWDAF MTLF is available.

This attribute shall be present only if flCapabilityType attribute

is present.";

uses types3gpp:TimeWindowGrp;

}

}

grouping NwdafInfoGrp {

description "This data type represents specific data for the NWDAF.";

reference "TS 29.510";

leaf-list nwdafEvents {

type NwdafEvent;

ordered-by user;

description "This attribute represents the Analytic functionalities

(identified by nwdafEvent defined in TS 29.520) of the NWDAF instance.

MnS consumer can configure this attribute to specify which Analytic

functionalities (identified by nwdafEvent) can be performed the NWDAF

instance. If the value of this attribute is not present, the NWDAF

instance can perform any NWDAFEvents.

The detailed ENUM value for NwdafEvent see the Table 5.1.6.3.4-1

in TS 29.520";

}

leaf-list eventIds {

type string ;

min-elements 1;

description "It represents the EventId(s) supported by the

Nnwdaf_AnalyticsInfo service, if none are provided the NWDAF can serve

any eventId. (see clause TS 29.520)";

}

list taiList {

description "The list of TAIs. ";

min-elements 1;

key idx ;

leaf idx { type uint32; }

uses types3gpp:TaiGrp;

}

list taiRangeList {

min-elements 1;

key idx ;

leaf idx { type uint32; }

description "The range of TAIs.";

uses nfp3gpp:TaiRangeGrp;

}

list nwdafCapability {

max-elements 1;

key idx ;

leaf idx { type uint32; }

description "This attribute indicates the capability of the NWDAF.

If not present, the NWDAF shall be regarded with no capability.";

uses NwdafCapabilityGrp;

}

leaf analyticsDelay {

type int32 ;

units seconds;

description "It represents the supported Analytics Delay related to the

eventIds and nwdafEvents.

It is an unsigned integer identifying a period of time in units of

seconds.(see clause 5.2.2 TS 29.571).";

}

leaf-list servingNfTypeList {

type types3gpp:NfType;

min-elements 1;

description "It contains the list of NF type(s) from which the NWDAF NF

can collect data. The absence of this attribute indicates that the

NWDAF can collect data from any NF type.";

}

leaf-list servingNfSetIdList {

type string ;

min-elements 1;

description "It contains the list of NF type(s) from which the NWDAF NF

can collect data. The absence of this attribute indicates that the

NWDAF can collect data from any NF type. (see clause 5.4.2 NfSetId

in TS 29.571)";

}

list mlAnalyticsList {

min-elements 1;

key idx ;

leaf idx { type uint32; }

description "It represents ML Analytics Filter information supported by

the Nnwdaf_MLModelProvision service.";

uses MlAnalyticsInfoGrp;

}

}

grouping NetworkSliceInfoGrp {

description "Represents information of network slice when the NWDAF is

authorized to collect the management data of the network slice. ";

list sNSSAI {

description "It represents the S-NSSAI the NetworkSlice managed object

is supporting. The S-NSSAI is defined in TS 23.003 ";

key idx;

min-elements 1;

max-elements 1;

leaf idx {

type string;

}

uses types3gpp:SNssai;

}

leaf-list cNSIId {

type string;

description "It represents NSI ID which is an identifier for identifying

the Core Network part of a Network Slice instance when multiple

Network Slice instances of the same Network Slice are deployed,

and there is a need to differentiate between them in the 5GC.

See NSI ID definition in clause 3.1 of TS 23.501 and

subclause 6.1.6.2.7 of TS 29.531.";

}

leaf networkSliceRef {

type types3gpp:DistinguishedName;

mandatory true;

description "This holds a DN of the NetworkSlice managed object relating

to the NetworkSlice instance differentiated by sNSSAI and optional

cNSIId.";

}

}

grouping NWDAFFunctionGrp {

description "The NWDAFFunction IOC includes attributes inherited from

ManagedFunction IOC (defined in TS 28.622)";

uses mf3gpp:ManagedFunctionGrp;

list pLMNInfoList {

description "It defines the PLMN(s) of a Network Function.";

min-elements 1;

key "idx";

leaf idx { type uint32; }

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

list networkSliceInfoList {

key idx;

min-elements 1;

description "The attribute specifies a list of NetworkSliceInfo which

is defined as a datatype (see clause 5.3.95). It is used by an

authorized consumer, e.g. NWDAF to facilitate the data collection

from OAM.";

leaf idx {

type string;

}

uses NetworkSliceInfoGrp;

yext3gpp:inVariant;

}

list nwdafInfo {

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32; }

description "It represents specific data for the NWDAF.";

uses NwdafInfoGrp;

}

leaf administrativeState {

type types3gpp:BasicAdministrativeState ;

mandatory true;

description "This attribute determines whether the NWDAF is enabled or

disabled. MnS consumer can configure this attribute to activate or

de-activate the analytic functionalities (identified by nwdafEvent

defined in TS 29.520) of the NWDAF instance.";

}

leaf nwdafLogicalFuncSupported {

type enumeration {

enum NWDAF_WITH_ANLF {

description "Indicates the NWDAF containing Analytics logical

function (AnLF)";

}

enum NWDAF_WITH_MTLF {

description "Indicates the NWDAF containing Model Training logical

function (MTLF).";

}

enum NWDAF_WITH_ANLF_MTLF {

description "Indicates the NWDAF containing both Analytics logical

function (AnLF) and Model Training logical function (MTLF).";

}

}

config false;

description "It represents the logical functions supported by the NWDAF.

If not present, the NWDAF shall be regarded with no logical

decomposition, in that case the NWDAF only supports the analytics

services.";

}

}

augment "/me3gpp:ManagedElement" {

list NWDAFFunction {

description "5G Core NWDAF Function.

For more information about the NWDAF, see TS 23.501. Several

attributes (including 'nwdafInfo', 'administrativeState' and

'ManagedNFProfile.servingScope') are used to control the functionalities

(identified by nwdafEvent defined in TS 29.520) of the NWDAF instance.

The attribute 'ManagedNFProfile.servingScope' is used to represent

specified certain geographical area(s) can be served by the NWDAF

instance.

The attribute 'NwdafInfo.taiList' and 'NwdafInfo.taiRangeList' is used

to represent specified certain tracking area(s) can be served by the

NWDAF instance.";

reference "3GPP TS 28.541 3GPP TS 23.501";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NWDAFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 8 ***

*** START OF CHANGE 9 ***

*** yang-models/_3gpp-5gc-nrm-pcffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-pcffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-pcffunction;

prefix pcf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the PCF function in 5GC. For more

information about the PCF, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-08-18 { reference CR-1557; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-06 { reference "CR-0331"; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-22 { reference "initial revision"; }

grouping PCFFunctionGrp {

description "Represents the PCFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the PCFFunction supports

(is associated to).";

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the PCFFunction supports

(is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list PCFFunction {

description "5G Core PCF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses PCFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 9 ***

*** START OF CHANGE 10 ***

*** yang-models/_3gpp-5gc-nrm-smffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-smffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-smffunction;

prefix smf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "SMFFunction derived from basic ManagedFunction.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-08-18 { reference CR-1557; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 {reference "Ericsson refactoring."; }

revision 2018-08-07 { reference "Initial revision";}

grouping SMFFunctionGrp {

description "Represents the SMFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

min-elements 1;

description "A list of PLMN identifiers (Mobile Country Code and Mobile

Network Code).";

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types5g3gpp:NRTAC;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the SMFFunction supports

(is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the SMFFunction supports

(is associated to).";

}

list dnaiSatelliteMappingList {

description "List of the mapping relationship between

satellite ID and at least one DNAI.";

min-elements 1;

key "satelliteid";

uses dnaiSatelliteMapping;

}

}

grouping dnaiSatelliteMapping {

leaf-list dnaiList {

description "List of Data network access identifiers.";

min-elements 1;

type string;

}

leaf satelliteid {

description "Unique identifier of a GEO satellite.";

type string;

}

}

augment "/me3gpp:ManagedElement" {

list SMFFunction {

description "5G Core SMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 10 ***

*** START OF CHANGE 11 ***

*** yang-models/_3gpp-5gc-nrm-smsffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-smsffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-smsffunction;

prefix smsf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

description "This IOC represents the SMSF function defined in 3GPP TS 23.501.

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {

description "initial revision";

}

grouping SMSFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list SMSFFunction {

description "5G Core SMSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SMSFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 11 ***

*** START OF CHANGE 12 ***

*** yang-models/_3gpp-5gc-nrm-udmfunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-udmfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udmfunction;

prefix udm3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDM function in 5GC. For more

information about the UDM, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-08-18 { reference CR-1557; }

revision 2024-04-04 { reference CR-1139; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 { reference "initial revision";}

grouping UDMFuntionGrp {

description "Represents the UDMFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

leaf-list mdtUserConsentReqList {

type enumeration {

enum M1;

enum M2;

enum M3;

enum M4;

enum M5;

enum M6;

enum M7;

enum M8;

enum M9;

enum MDT_UE_LOCATION;

}

description "represents a list of MDT measurement names that are

subject to user consent at MDT activation.

Any MDT measurement, whose name is not specified in this list, is not

subject to user consent at MDT activation.";

}

}

augment "/me3gpp:ManagedElement" {

list UDMFunction {

description "5G Core UDM Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDMFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 12 ***

*** START OF CHANGE 13 ***

*** yang-models/_3gpp-5gc-nrm-udrfunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-udrfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udrfunction;

prefix udr3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDR function in 5GC. For more information

about the UDR, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-08-18 { reference CR-1557; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 {reference "initial revision"; }

grouping UDRFuntionGrp {

description "Representse the UDRFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

}

augment "/me3gpp:ManagedElement" {

list UDRFunction {

description "5G Core UDR Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDRFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 13 ***

*** START OF CHANGE 14 ***

*** yang-models/_3gpp-5gc-nrm-udsffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-udsffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udsffunction;

prefix udsf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDSF function which can be interacted

with any other 5GC NF defined in 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-08-18 { reference CR-1557; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 { reference "initial revision"; }

grouping UDSFFuntionGrp {

description "Represents the UDSFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

}

augment "/me3gpp:ManagedElement" {

list UDSFFunction {

description "5G Core UDSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDSFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;    }

}

}

<CODE ENDS>

*** END OF CHANGE 14 ***

*** START OF CHANGE 15 ***

*** yang-models/_3gpp-5gc-nrm-upffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-upffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-upffunction;

prefix upf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "UPFFunction derived from basic ManagedFunction.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-11-06 { reference CR-1639 ; }

revision 2025-08-18 { reference CR-1557; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 { reference "Ericsson refactoring."; }

revision 2018-08-07 { reference "Initial revision"; }

grouping UPFFunctionGrp {

description "Represents the UPFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "A list of PLMN identifiers (Mobile Country Code and Mobile

Network Code).";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types5g3gpp:NRTAC;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

description "This data type represents a Profile definition of a

Managed NF ";

reference "See TS 23.501";

uses mnfp3gpp:ManagedNFProfileGrp;

}

leaf-list supportedBMOList {

type string;

description "List of supported BMOs (Bridge Managed Objects) required

for integration with TSN system.";

}

}

augment /me3gpp:ManagedElement {

list UPFFunction {

description "5G Core UPF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UPFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 15 ***