3GPP TSG-SA5 Meeting #164	S5-255091

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.160 | CR | 0080 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.160 Simplify the YANG mapping of defaultValue |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the table about mapping of default values from stage 2 to stage 3 the mandatory field contains "Y/N". It not clearly described when "N" applies. The mapping of default value is unnecessarily complicated. Specifying that default values are only used for simple attributes, the description can be significantly simplified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is specified that defaultValues are only mapped for simple attributes. This allows simplifying the description of the mapping including the table in 6.2.13.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the specification, potentially leading to interopability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.13, 6.2.13.1, 6.2.14.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 6.2.13	defaultValue

#### 6.2.13.1	Introduction

Reference TS 32.156 [3] clause 5.2.1.1.

Default values can only be formally defined for simple attributes or attribute elements. For structured attributes default values are not used.

The 3GPP/UML defaultValue has a different meaning then the YANG "default" statement.

The 3GPP defaultValue could be considered an initialValue as it has effect only at object creation. If the attribute is later deleted the 3GPP defaultValue has no effect. In YANG the "default" is always used whenever a leaf/leaf-list does not have a value: both at creation of the parent object and if the leaf/leaf-list is deleted (set to null in 3GPP operation).

Note: Void

The 3GPP defaultValue, isNullable and multiplicity properties cannot be mapped one-to-one into YANG statements. A combination of these three stage 2 input properties shall result in a combination of the four YANG statements mandatory, min-elements, default,and yext3gpp:initial-value (defined in the YANG module _3gpp-common-yang-extensions.yang). The table below describes the combinations of input properties and the resulting YANG statements.

Table 6.2.13.1—1: Usage of default and initial-value for simple attributes

| Stage 2 properties |  |  | YANG mapping |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| multiplicity | isNullable | Stage-2 default | Attribute mapped to leaf Yang mandatory | Attribute mapped to leaf-list YANG Min-Elements > 0 | YANG default | YANG initial-value |
| 0..1 | True | none | N | N/A | N | N |
|  |  | defined | N | N/A | N | Y |
|  | False | none | N | N/A | N | N |
|  |  | defined | N | N/A | N | Y |
| 1 | True | none | N | N/A | N | N |
|  |  | defined | N | N/A | N | Y |
|  | False | none | Y | N/A | N | N |
|  |  | defined | N | N/A | Y | N |
| 0..* | True | none | N/A | N | N | N |
|  |  | defined | N/A | N | N | Y |
|  | False | none | N/A | N | N | N |
|  |  | defined | N/A | N | N | Y |
| x..* x >= 1 | True | none | N/A | N | N | N |
|  |  | defined | N/A | N | N | Y |
|  | False | none | N/A | Y | N | N |
|  |  | defined | N/A | N | Y | N |

YANG mandatory indicates that the leaf shall have a “mandatory true;” substatement.

YANG min-elements > 0 indicates that the leaf-list shall have a “min-elements” substatement that has an argument that is greater than zero.

YANG default indicates that the leaf shall have a “default” substatement.

YANG initial-value indicates that the leaf should have a “yext3gpp:initial-value” substatement.

==============Next change==============

#### 6.2.14.1	YANG mapping

For simple attributes (attribute elements) mapped to a leaf or leaf-list YANG mandatory, or min-elements statements shall be used as specified in the table in clause 6.2.13.1.

Multiplicity of attributes mapped to a list or leaf-list shall be mapped to the "min-elements" and "max-elements" YANG statements.

Cardinality for containment of classes shall be mapped to "min-elements" and "max-elements" on the list representing the child objects.

Cardinality for reference relationships shall be mapped to "mandatory", "min-elements" and "max-elements" on the reference attributes representing the reference.

==============End of change==============