3GPP TSG-SA5 Meeting #163	S5-254562

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0582 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.623 Clarify usage of notifyFileReady |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Canada Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ReportingCtrl choice supports multiple scenarios each described in a separate paragraph of its description. In the case of CHOICE_3 there is text which unnecessarily refers to CHOICE_1 when describing the notifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the unnecessary reference to CHOICE_1 in the CHOICE_3 description text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description is inconsistent and can lead to confusion when implementing CHOICE_3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge only. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1950 at commit f2661b545dc6f63a35df7665769db897bd0d99c9 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-common-yang-types.yang ***

<CODE BEGINS>

module _3gpp-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-yang-types";

prefix "types3gpp";

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The model defines a YANG mapping of the top level

information classes used for management of 5G networks and

network slicing.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623";

revision 2025-10-07 { reference CR-0582; }

revision 2025-08-31 { reference "CR-0551 CR-0556 CR-0562"; }

revision 2025-02-19 { reference CR-0512; }

revision 2025-02-07 { reference CR-0492; }

revision 2024-11-25 { reference CR-1442; }

revision 2024-11-25 { reference CR-1412; }

revision 2024-11-01 { reference CR-0463; }

revision 2024-08-13 { reference CR-0387; }

revision 2024-05-18 { reference CR-0359; }

revision 2024-04-12 { reference CR-0346; }

revision 2023-11-06 { reference CR-0305; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-08-09 { reference CR-0266; }

revision 2023-05-10 { reference CR-0250; }

revision 2023-02-14 { reference CR-0234; }

revision 2022-11-04 { reference "CR-0194"; }

revision 2022-10-24 { reference CR-0196;  }

revision 2022-07-26 { reference "CR-0180" ; }

revision 2022-02-09 { reference "CR-0144"; }

revision 2021-11-01 { reference "CR-0141"; }

revision 2021-09-30 {

description "Added Longitude, Latitude, TenthOfDegrees, OnOff.";

reference "CR-0138";

}

revision 2020-11-06 {

description "Removed incorrect S-NSSAI definitions.";

reference "CR-0118";

}

revision 2020-03-10 {

description "Removed faulty when statements.";

reference "SP-200229";

}

revision 2019-10-25 {

description "Added ManagedNFProfile.";

reference "S5-194457";

}

revision 2019-10-16 {

description "Added SAP and usageState.";

reference "S5-193518";

}

revision 2019-06-23 {

reference  "Initial version.";

}

typedef EnabledDisabled {

type enumeration {

enum DISABLED ;

enum ENABLED ;

}

}

grouping nameValuePair {

leaf name { type string; }

leaf value { type string; }

}

typedef DayOfWeek {

type enumeration {

enum MONDAY;

enum TUESDAY;

enum WEDNESDAY;

enum THURSDAY;

enum FRIDAY;

enum SATURDAY;

enum SUNDAY;

}

}

typedef DateMonth {

type uint8 {

range 0..12;

}

}

typedef DateMonthDay {

type uint8 {

range 0..31;

}

}

typedef FullTime {

type yang:time-with-zone-offset;

}

grouping DayInYearGrp {

description "This <<dataType>> represents a day in a year.";

leaf month {

description "It indicates the month in a year.";

type DateMonth;

mandatory true;

}

leaf monthDay {

description "It indicates the day in a month.";

type DateMonthDay;

mandatory true;

}

}

grouping TimeWindowGrp {

leaf startTime {

type yang:date-and-time;

yext3gpp:inVariant;

}

leaf endTime {

type yang:date-and-time;

yext3gpp:inVariant;

}

}

grouping ProcessMonitorGrp {

description "Provides attributes to monitor the progress of processes

with specific purpose and limited lifetime running on MnS producers.

It may be used as data type for dedicated progress monitor attributes

when specifying the management representation of these processes.

The attributes in this clause are defined in a generic way.

For some attributes specialisations may be provided when specifying a

concrete process representation.

If a management operation on some IOCs triggers an associated

asynchronous process (whose progress shall be monitored), this should

also result in creating an attribute named 'processMonitor' (of type

'ProcessMonitor') in these IOC(s). The processMonitor attribute may be

accompanied by use-case specific additional data items.

The progress of the process is described by the 'status' and

'progressPercentage' attributes. Additional textual qualifications for

the 'status' attribute may be provided by the 'progressStateInfo' and

'resultStateInfo' attributes.

When the process is instantiated, the 'status' is set to 'NOT_STARTED'

and the 'progressPercentage' to '0'. The MnS producer decides when to

start executing the process and to transition into the 'RUNNING' state.

This time is captured in the 'startTime' attribute. Alternatively, the

process may start to execute directly upon its instantiation. One

alternative must be selected when using this data type.

During the 'RUNNING' state the 'progressPercentage' attribute may be

repeatedly updated. The exact semantic of this attribute is subject to

further specialisation. The 'progressInfo' attribute may be used to

provide additional textual information in the 'NOT_STARTED', 'CANCELLING'

and 'RUNNING' states. Further specialisation of

'progressStateInfo' may be provided where this data type is

used.

Upon successful completion of the process, the 'status' attribute is set

to 'FINISHED', the 'progressPercentage' to 100%. The time is captured in

the 'endTime' attribute. Additional textual information may be provided

in the 'resultStateInfo' attribute. The type of

'resultStateInfo' in this data type definition is 'String'.

Further specialisation of 'resultStateInfo' may be provided

where this data type is used.

In case the process fails to complete successfully, the 'status'

attribute is set to 'FAILED' or 'PARTIALLY_FAILED', the current value of

'progressPercentage' is frozen, and the time captured in 'endTime'. The

'resultStateInfo' specifies the reason for the failure.

Specific failure reasons may be specified where the data type defined in

this clause is used. The exact semantic of failure may be subject for

further specialisation as well.

In case the process is cancelled, the 'status' attribute is first set to

'CANCELLING' and when the process is really cancelled then to 'CANCELLED'.

The transition to 'CANCELLED' is captured in the 'endTime' attribute.

The value of 'progressPercentage' is frozen. Additional textual

information may be provided in the 'resultStateInfo' attribute.

The 'resultStateInfo' attribute is provided only for additional textual

qualification of the states 'FINISHED', 'FAILED', 'PARTIALLY_FAILED' or

'CANCELLED'. It shall not be used for making the outcome, that the

process may produce in case of success, available.

The process may have to be completed within a certain time after its

creation, for example because required data may not be available any

more after a certain time, or the process outcome is needed until a

certain time and when not provided by this time is not needed any more.

The time until the MnS producer automatically cancels the process is

indicated by the 'timer' attribute.";

leaf id {

type string;

mandatory true;

description "Id of the process. It is unique within a single

multivalue attribute of type ProcessMonitor.";

}

leaf status {

type enumeration {

enum NOT_STARTED ;

enum RUNNING ;

enum CANCELLING ;

enum FINISHED ;

enum FAILED ;

enum PARTIALLY_FAILED ;

enum CANCELLED ;

}

config false;

default  RUNNING;

description "Represents the status of the associated process,

whether it fails, succeeds etc.

It does not represent the returned values of a successfully finished

process. ";

}

leaf progressPercentage {

type uint8 {

range 0..100;

}

config false;

description "Progress of the associated process as percentage";

}

leaf-list progressStateInfo {

type string;

config false;

description "Additional textual qualification of the states

'NOT_STARTED', 'CANCELLING' and 'RUNNING'.

For specific processes, specific well-defined strings (e.g. string

patterns or enums) may be defined as a specialisation.";

}

leaf resultStateInfo {

type string;

config false;

description "Additional textual qualification of the states

'FINISHED', 'FAILED', 'PARTIALLY_FAILED and 'CANCELLED'.

For example, in the 'FAILED' or 'PARTIALLY_FAILED' state this

attribute may be used to provide error reasons.

This attribute shall not be used to make the outcome of the process

available for retrieval, if any. For this purpose, dedicated

attributes shall be specified when specifying the representation of

a specific process.

For specific processes, specific well-defined strings (e.g. string

patterns or enums) may be defined as a specialisation.";

}

leaf startTime {

type yang:date-and-time;

config false;

description "Start time of the associated process, i.e. the time when the

status changed from 'NOT_STARTED' to 'RUNNING'.";

}

leaf endTime {

type yang:date-and-time;

config false;

description "Date and time when status changed to 'SUCCESS', 'CANCELLED',

'FAILED' or 'PARTIALLY_FAILED'.

If the time is in the future, it is the estimated time

the process will end.";

}

leaf timer {

type uint32;

units minutes;

description "Time until the associated process is automatically cancelled.

If set, the system decreases the timer with time. When it reaches zero

the cancellation of the associated process is initiated by the

MnS_Producer.

If not set, there is no time limit for the process.

Once the timer is set, the consumer can not change it anymore.

If the consumer has not set the timer the MnS Producer may set it.";

yext3gpp:notNotifyable;

}

}

typedef EutraCellId {

type string {

pattern '^[A-Fa-f0-9]{7}$';

}

description "28-bit string identifying an E-UTRA Cell Id

as specified in clause 9.3.1.9 of 3GPP TS 38.413

in hexadecimal representation. Each character in the

string shall take a value of '0' to '9', 'a' to 'f'

or 'A' to 'F' and shall represent 4 bits.

The most significant character representing the 4 most

significant bits of the Cell Id shall appear first in the

string, and the character representing the 4 least

significant bit of the Cell Id shall appear last in the

string.";

}

typedef NrCellId {

type string {

pattern '^[A-Fa-f0-9]{9}$';

}

description "36-bit string identifying an NR Cell Id

as specified in clause 9.3.1.7 of 3GPP TS 38.413

in hexadecimal representation. Each character in the

string shall take a value of '0' to '9', 'a' to 'f'

or 'A' to 'F' and shall represent 4 bits.

The most significant character representing the 4 most

significant bits of the Cell Id shall appear first in the

string, and the character representing the 4 least

significant bit of the Cell Id shall appear last in the

string.";

}

grouping NpnIdGrp {

description "Represents the NPN supported by the <<IOC>> using this

<<dataType>> as one of its attributes in case of the cell is a

NPN-only cell.";

list plmnId {

key "mcc mnc";

min-elements 1;

description "It specifies the PLMN Id of the NPN network.";

uses types3gpp:PLMNId;

}

choice npnidChoice {

leaf-list cAGIdList {

type CagId;

max-elements 256;

description "It identifies a CAG list containing up to 12

CAG-identifiers per Ue or up to 12 CAG-identifiers per cell,

see TS 38.331.

CAG ID is used to combine with PLMN ID to identify a PNI-NPN.CAG ID is

a hexadecimal range with size 32 bit";

}

leaf-list nIDList {

type Nid;

max-elements 16;

description "It identifies a list of NIDs containing up to 16 NIDs, see

TS 38.331. NID is used to combine with PLMN ID to identify an SNPN.

NID is a hexadecimal range with size 44 bit.";

}

}

}

typedef TenthOfDegrees {

type uint16 {

range 0..3600;

}

units "0.1 degrees";

description "A single integral value corresponding to an angle in degrees

between 0 and 360 with a resolution of 0.1 degrees.";

}

typedef Latitude {

type decimal64 {

fraction-digits 4;

range "-90.0000..+90.0000";

}

description "Latitude values";

}

typedef Longitude {

type decimal64 {

fraction-digits 4;

range "-180.0000..+180.0000";

}

description "Longitude values";

}

typedef Altitude  {

type decimal64 {

fraction-digits 6;

}

units "meters";

description

"Height from a reference 0 value.";

}

grouping GeographicalCoordinates {

description "This datatype represents the geographical coordinates";

reference "3GPP TS 28.538 clause 6.3.8";

leaf latitude {

type Latitude;

mandatory true;

}

leaf longitude {

type Longitude;

mandatory true;

}

}

typedef OnOff {

type enumeration {

enum ON;

enum OFF;

}

}

typedef usageState {

type enumeration {

enum IDLE;

enum ACTIVE;

enum BUSY;

}

description "It describes whether or not the resource is actively in

use at a specific instant, and if so, whether or not it has spare

capacity for additional users at that instant. The value is READ-ONLY.";

reference "ITU T Recommendation X.731";

}

grouping SAPGrp {

leaf host {

type inet:host;

mandatory true;

}

leaf port {

type inet:port-number;

mandatory true;

}

description "Service access point.";

reference "TS 28.622";

}

typedef Mcc {

description "The mobile country code consists of three decimal digits,

The first digit of the mobile country code identifies the geographic

region (the digits 1 and 8 are not used):";

type string {

pattern '[02-79][0-9][0-9]';

}

reference "3GPP TS 23.003 subclause 2.2 and 12.1";

}

typedef Mnc {

description "The mobile network code consists of two or three

decimal digits (for example: MNC of 001 is not the same as MNC of 01)";

type string {

pattern '[0-9][0-9][0-9]|[0-9][0-9]';

}

reference "3GPP TS 23.003 subclause 2.2 and 12.1";

}

grouping PLMNId {

leaf mcc {

mandatory true;

type Mcc;

}

leaf mnc {

mandatory true;

type Mnc;

}

reference "TS 38.413 clause 9.3.3.5";

}

typedef Nci {

description "NR Cell Identity. The NCI shall be of fixed length of 36 bits

and shall be coded using full hexadecimal representation.

The exact coding of the NCI is the responsibility of each PLMN operator";

reference "TS 23.003";

type union {

type string {

length 36;

pattern '[01]+';

}

type string {

length 9;

pattern '[a-fA-F0-9]*';

}

}

}

typedef OperationalState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum DISABLED {

value 0;

description "The resource is totally inoperable.";

}

enum ENABLED {

value 1;

description "The resource is partially or fully operable.";

}

}

}

typedef BasicAdministrativeState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum LOCKED {

value 0;

description "The resource is administratively prohibited from performing

services for its users.";

}

enum UNLOCKED {

value 1;

description "The resource is administratively permitted to perform

services for its users. This is independent of its inherent

operability.";

}

}

}

typedef AdministrativeState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum LOCKED {

value 0;

description "The resource is administratively prohibited from performing

services for its users.";

}

enum UNLOCKED {

value 1;

description "The resource is administratively permitted to perform

services for its users. This is independent of its inherent

operability.";

}

enum SHUTTINGDOWN {

value 2;

description "Use of the resource is administratively permitted to

existing instances of use only. While the system remains in

the shutting down state the manager or the managed element

may at any time cause the resource to transition to the

locked state.";

}

}

}

typedef AvailabilityStatus {

type enumeration {

enum IN_TEST;

enum FAILED;

enum POWER_OFF;

enum OFF_LINE;

enum OFF_DUTY;

enum DEPENDENCY;

enum DEGRADED;

enum NOT_INSTALLED;

enum LOG_FULL;

}

}

typedef CellState {

type enumeration {

enum IDLE;

enum INACTIVE;

enum ACTIVE;

}

}

typedef Nrpci {

type uint32;

description "Physical Cell Identity (PCI) of the NR cell.";

reference "TS 36.211 subclause 6.11";

}

typedef Tac {

type string {

pattern "([A-Fa-f0-9]{4})|([A-Fa-f0-9]{6})" ;

}

description "Tracking Area Code

2 or 3-octet string identifying a tracking area code as specified in

clause 9.3.3.10 of 3GPP TS 38.413 [34], in hexadecimal representation.

Each character in the string shall take a value of

'0' to '9', 'a' to 'f' or 'A' to 'F' and shall represent 4 bits.

The most significant character representing the 4 most significant

bits of the TAC shall appear first in the string, and the character

representing the 4 least significant bit of the TAC shall appear last

in the string.

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

Examples:

A legacy TAC 0x4305 shall be encoded as '4305'.

An extended TAC 0x63F84B shall be encoded as '63F84B'";

reference "TS 23.003 clause 19.4.2.3";

}

grouping TaiGrp {

description "This <<dataType>> defines a Tracking Area Identity (TAI)

as specified in clause 28.6 of TS 23.003, clause 8.2 of TS 38.300

and clause 9.3.3.11 of TS 38.413. It is composed of the PLMN

identifier (PLMN-Id, which is composed of the MCC and MNC) and

the Tracking Area Code (TAC).";

list plmnId {

description "PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf tac {

type Tac;

mandatory true;

description "Tracking Area Code";

}

}

grouping GeoCoordinateGrp {

description "Geographical location on earth";

leaf latitude {

type decimal64 {

fraction-digits 4;

range -90..90 ;

}

mandatory true;

description "Latitude based on World Geodetic System (1984 version)

global reference frame (WGS 84). Positive values correspond to the

northern hemisphere.";

}

leaf longitude {

type decimal64 {

fraction-digits 4;

range -180..180 ;

}

mandatory true;

description "Longitude based on World Geodetic System (1984 version)

global reference frame (WGS 84). Positive values correspond to

degrees east of 0 degrees longitude.";

}

leaf altitude {

type Altitude;

}

}

grouping GeoAreaGrp {

description "This data type defines a geographical area.

The geo-area is defined using a convex polygon in the attribute

'geoPolygon'.";

list geoPolygon {

description "Specifies the geographical area with a convex polygon.

The convex polygon is specified by its corners.";

key "latitude longitude";

min-elements 3;

ordered-by user;

uses GeoCoordinateGrp;

}

}

typedef AmfRegionId {

type union {

type uint8 ;

type string {

length 8;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

typedef AmfSetId {

type union {

type uint16 {

range '0..1023';

}

type string {

length 8;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

typedef AmfPointer {

type union {

type uint8 {

range '0..63';

}

type string {

length 6;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

grouping AmfIdentifier {

leaf amfRegionId {

type AmfRegionId;

}

leaf amfSetId {

type AmfSetId;

}

leaf amfPointer {

type AmfPointer;

}

description "The AMFI is constructed from an AMF Region ID,

an AMF Set ID and an AMF Pointer.

The AMF Region ID identifies the region,

the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and

the AMF Pointer uniquely identifies the AMF within the AMF Set. ";

}

// type definitions especially for core NFs

typedef NfType {

type enumeration {

enum NRF;

enum UDM;

enum AMF;

enum SMF;

enum AUSF;

enum NEF;

enum PCF;

enum SMSF;

enum NSSF;

enum UDR;

enum LMF;

enum GMLC;

enum 5G_EIR;

enum SEPP;

enum UPF;

enum N3IWF;

enum AF;

enum UDSF;

enum BSF;

enum CHF;

}

}

typedef NotificationType {

type enumeration {

enum N1_MESSAGES;

enum N2_INFORMATION;

enum LOCATION_NOTIFICATION;

}

}

typedef Load {

description "Latest known load information of the NF, percentage ";

type uint8 {

range 0..100;

}

}

typedef N1MessageClass {

type enumeration {

enum 5GMM;

enum SM;

enum LPP;

enum SMS;

}

}

typedef N2InformationClass {

type enumeration {

enum SM;

enum NRPPA;

enum PWS;

enum PWS_BCAL;

enum PWS_RF;

}

}

grouping DefaultNotificationSubscription {

leaf notificationType {

type NotificationType;

}

leaf callbackUri {

type inet:uri;

}

leaf interPlmnCallbackUri {

description "It indicates the callback URI to be used by NF Service

Producers located in PLMNs that are different from the PLMN of

the NF consumer.";

type inet:uri;

}

leaf n1MessageClass {

type N1MessageClass;

}

leaf n2InformationClass {

type N2InformationClass;

}

leaf versions {

description "This attribute identifies the API versions

(e.g. v1) supported for the default notification type";

//optional

type N2InformationClass;

}

leaf binding {

description "This attribute shall contain the value of the Binding

Indication for the default subscription notification ";

reference " i.e. the value part of 3gpp-Sbi-Binding header, as

specified in

clause 6.12.4 of 3GPP TS 29.500 76. ";

//optional

type string;

config false;

}

leaf acceptedEncoding {

description "It indicates the content encodings that are accepted by a

NF Service Consumer when receiving a notification related to a default

notification subscription";

reference "IETF RFC 9110 clause 12.5.3 ";

//optional

type string;

config false;

}

leaf supportedFeatures {

description "It is a string, which indicates the features of the service

corresponding to the subscribed default notification, which are supported

by the NF (Service) instance acting as NF service consumer. The string

shall contain a bitmask indicating supported features in hexadecimal

representation:Each character in the string shall take a value

of 0 to 9, a to f or A to F ";

reference "shall represent the support of 4 features as described in

table 5.2.2-3 of TS 29.571 [61].";

//optional

type string;

config false;

}

leaf serviceInfoList {

description "It indicates a list of service specific information. It may

be present when the notification request of the notification type may be

generated by multiple services, i.e. notifications from different

services may be received by the subscription.";

//optional

type string;

config false;

}

leaf callbackUriPrefix {

description "It indicates the optional path segment(s) used to construct

the prefix of the Callback URIs during the reselection of an NF service

consumer, as described in 3GPP TS 29.501";

//optional

type inet:uri;

}

}

grouping Ipv4AddressRange {

leaf start {

type inet:ipv4-address;

}

leaf end {

type inet:ipv4-address;

}

}

grouping Ipv6PrefixRange {

leaf start {

type inet:ipv6-prefix;

}

leaf end {

type inet:ipv6-prefix;

}

}

typedef NsiId {

type string;

}

typedef UeMobilityLevel {

type enumeration {

enum STATIONARY;

enum NOMADIC;

enum RESTRICTED_MOBILITY;

enum FULLY_MOBILITY;

}

}

typedef ResourceSharingLevel {

type enumeration {

enum SHARED;

enum NOT_SHARED;

}

}

typedef TxDirection {

type enumeration {

enum DL;

enum UL;

enum DL_AND_UL;

}

}

grouping AddressWithVlan {

leaf ipAddress {

type inet:ip-address;

}

leaf vlanId {

type uint16;

}

}

typedef Nid {

description "This represents the Network Identifier, which

together with a PLMN ID is used to identify a SNPN.";

type string {

pattern '^[A-Fa-f0-9]{11}$';

}

reference "3GPP TS 23.003 and 3GPP TS 23.501 clause 5.3.2.1.";

}

typedef CagId {

description "This represents the identifier of a Closed Access

Group (CAG), which together with a PLMN ID is used to

identify a PNI-NPN.";

type string {

pattern '^[A-Fa-f0-9]{8}$';

}

reference "3GPP TS 23.003 and 3GPP TS 23.501.";

}

/* DistinguishedName pattern is built up based on the

EBNF in 32.300 clause 7.3  EBNF of DN String Representation

leaf DN { type string {   //  Same pattern as LocalDN

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?(,[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)*';

} }

leaf fullLocalDN { type string {   // LocalRDN , { RDNSeparator , LocalRDN }    RDNSeparator is a single , no space or \R allowed   Me.mykey=1 allowed

//  (fullLocalRDN)(,(fullLocalRDN))*

pattern '(([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?))(,(([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)))*';

} }

leaf LocalDN { type string {   // LocalRDN , { RDNSeparator , LocalRDN }    RDNSeparator is a single , no space or \R allowed

//  LocalRDN(,LocalRDN)*

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?(,[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)*';

} }

leaf fullLocalRDN { type string {   // same as fullLocalDNAttributeTypeAndValue

pattern '([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)';

} }

leaf LocalRDN { type string {   // same as LocalDNAttributeTypeAndValue

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?';

} }

leaf fullLocalDNAttributeTypeAndValue { type string { // LocalDNAttributeType , AttributeTypeAndValueSeparator , RegularAttributeValue

// pattern LocalDNAttributeType=RegularAttributeValue

pattern '([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)';

} }

// limitation: NamesOfClassAndNamingAttributenot supported Me.mykey=1

leaf LocalDNAttributeTypeAndValue { type string {

// ebnf1          LocalDNAttributeType , AttributeTypeAndValueSeparator , RegularAttributeValue

// ebnf2-limited  NameOfClassWithIdAttribute , AttributeTypeAndValueSeparator , RegularAttributeValue

// pattern        NameOfClassWithIdAttribute=RegularAttributeValue

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?';

} }

leaf LocalDNAttributeType { type string {   // NameOfClassWithIdAttribute | NamesOfClassAndNamingAttribute  RDNSeparator is a single , no space or \R allowed

//  NameOfClassWithIdAttribute|NamesOfClassAndNamingAttribute

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*)';

} }

leaf RegularAttributeValue { type string {       // ( AttributeValueChar - SpaceChar ) , [ { AttributeValueChar } , ( AttributeValueChar - SpaceChar ) ]

pattern '([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?' ;

} }

leaf NamesOfClassAndNamingAttribute  { type string {  // ClassName , ClassNamingAttributeSeparator , NamingAttributeName

// pattern: ClassName\.NamingAttributeName

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*' ;

} }

leaf restrictiveClassName { type string {     //

pattern '[a-zA-Z][a-zA-Z0-9-_]*' ;

} }

leaf ClassName { type string {     // CapitalLetterChar , { LocalDNAttributeTypeChar }

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*' ;

} }

leaf NamingAttributeName { type string {   // SmallLetterChar , { LocalDNAttributeTypeChar }

pattern '[a-z][^,=+<>#;\\"\r\n*.]*' ;

} }

*/

typedef DistinguishedName {

type string {

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|'

+ '(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*'

+ '([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?'

+ '(,[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))'

+ '(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*'

+ '([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)*';

}

description "Represents the 3GPP standard for DistinguishedName.

Limitations:

- RDNSeparator: don't allow SpaceChar or CarriageReturnChar

- NullDN: Disallow nullDN that is the same as not providing a DN

- NamesOfClassAndNamingAttribute format not allowed

(eg. ManagedElement.mykey=345436)";

reference  "3GPP TS 32.300";

}

typedef QOffsetRange  {

type int8 {

range "-24 | -22 | -20 | -18 | -16 | -14 | -12 | -10 | -8 | -6 | " +

" -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 8 | 10 | " +

" 12 | 14 | 16 | 18 | 20 | 22 | 24";

}

units dB;

}

grouping ReportingCtrl {

choice reportingCtrl {

mandatory true;

description "

This choice defines the method for reporting collected performance

metrics to MnS consumers as well as the parameters for configuring the

reporting function. It is a choice between the control parameter

required for the reporting methods, whose presence selects the

reporting method as follows:

- When only the fileReportingPeriod attribute is present, the MnS

producer shall store files on the MnS producer at a location selected

by the MnS producer and, on condition that an appropriate subscription

is in place, inform the MnS consumer about the availability of new

files and the file location using the notifyFileReady notification.

In case the preparation of a file fails, 'notifyFilePreparationError'

shall be sent instead.

- When the 'fileReportingPeriod' and 'notificationRecipientAddress'

attributes are present, then the MnS producer shall behave like

described for the case that only the 'fileReportingPeriod' is present.

In addition, the MnS producer shall create on behalf of the MnS

consumer a subscription, using 'NtfSubscriptionControl', for the

notification types 'notifyMOICreation' and 'notifyMOIDeletion' related

to the 'File' instances that will be produced later. In case an

existing subscription does already include the 'File' instances to be

produced, no new subscription shall be created. The

'notificationRecipientAddress' attribute in the created

'NtfSubscriptionControl' instance shall be set to the value of the

'notificationRecipientAddress' in the related 'PerfMetricJob'. This

feature is called implicit notification subscription, as opposed to the

case where the MnS consumer creates the subscription (explicit

notification subscription). When the related 'PerfMetricJob' is

deleted, the 'NtfSubscriptionControl' instance created due to the

request for implicit subscription shall be deleted as well.

- When only the 'fileReportingPeriod' and 'fileLocation' attributes

are present (CHOICE_3), the MnS producer shall store the files at the

location specified by 'fileLocation'. The file location may identify

any entity such as a file server or a MnS consumer. The identified

MnS consumer may or may not be identical to the MnS consumer creating

the 'PerfMetricJob'. The MnS producer may emit 'notifyFileReady' and

'notifyFilePreparationError' notifications to inform subscribers that

a file has been made available at the location specified by 'fileLocation'.

- When only the streamTarget attribute is present, the MnS producer

shall stream the data to the location specified by streamTarget.

For the file-based reporting methods the fileReportingPeriod attribute

specifies the time window during which collected measurements are

stored into the same file before the file is closed and a new file is

opened.";

case file-based-reporting {

leaf fileReportingPeriod {

type uint32 {

range 1..max;

}

units minutes;

mandatory true;

description "For the file-based reporting method this is the time

window during which collected measurements are stored into the same

file before the file is closed and a new file is opened.

The time-period must be a multiple of the granularityPeriod.

Applicable when the file-based reporting method is supported.";

}

choice reporting-target {

case file-target {

leaf fileLocation {

type string ;

description "Applicable and must be present when the file-based

reporting method is supported, and the files are stored on the

MnS consumer.";

}

}

case notification-target {

leaf notificationRecipientAddress {

type string;

description "Must be present when the notification-based reporting

method is supported, and the the files are available as

notifications for the MnS consumer to subscribe to.";

}

}

description "When netiher fileLocation or notificationRecipientAddress

are present, the files are stored and available to the MnS consumer

if the MnS subscribes to the notifyFileReady notification.";

}

}

case stream-based-reporting {

leaf streamTarget {

type string;

mandatory true;

description "Applicable when stream-based reporting method is

supported.";

}

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***