3GPP TSG-SA5 Meeting #163 	S5-254234

Wuhan, China, 13th October 2025 - 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0736 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 performance measurements for QoS flow fulfillment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The incoming LS (S2-2508116) requires monitoring and capturing statistics for downgraded QoS flows. This aligns with TS 23.288, Clause 6.9, which specifies that QoS Sustainability Analytics must provide insights into historical QoS change statistics over a past analytics target period within a given area or predict the likelihood of QoS changes over a future target period in that same area. There are a few possibilities how the QoS fulfillment is provided by gNB - PDU Session Resource Setup Response with QoS Flow Failed to Setup List indicates that list of QoS flow cannot be fulfilled at setup - PDU Session Resource Modify Response with Current QoS Parameters Set Index indicates which alternative QoS profile (from the list in SMF config) reflects currently achieved QoS level. - Initial Context Setup Response with QoS Flow Failed to Setup List indicates that list of QoS flow cannot be fulfilled at setup - PDU Session Resource Notify due to one or more GBR QoS flows fulfilment (i.e., the message contains "QoS Flow Notify List"). indicates that active QoS flows are Not fulfilled (partially or fully lost), or fulfilled to a level as described by a referenced less preferred alternative |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding new measurements on Number of QoS flows fulfilment and non- fulfilment notification - Small corrections on Number of QoS flow failed to setup, Number of Initial QoS flow failed to setup, Number of QoS flows failed to modify; to avoid repeated text from TS32.404. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA2 specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.13.3.3, 5.1.1.13.3.6, 5.1.1.13.4.3, 5.1.1.13.4.x (new), 5.1.1.13.4.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

###### 5.1.1.13.3.3	Number of QoS flow failed to setup

a)	This measurement provides the number of QoS flows failed to setup. The measurement may be split into subcounters per failure cause.

b)	CC.

c)	On transmission by the NG-RAN of a PDU SESSION RESOURCE SETUP RESPONSE message, or transmission by the NG-RAN of a INITIAL CONTEXT SETUP RESPONSE message, or transmission by the NG-RAN of a PDU SESSION RESOURCE MODIFY RESPONSE message, each QoS flow failed to establish is added to the relevant measurement.

d)	Each measurement is an integer value.

e)	QF.EstabFailNbr; or

QF.EstabFailNbr.Cause, 
where Cause identifies the cause resulting in the QoS flow setup failure as specified in TS 38.413 [11].

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

*** START OF NEXT CHANGE ***

###### 5.1.1.13.3.6	Number of Initial QoS flow failed to setup

a)	This measurement provides the number of Initial QoS flows failed to setup. The measurement may be split into subcounters per failure cause.

b)	CC.

c)	On transmission by the NG-RAN of a INITIAL CONTEXT SETUP RESPONSE message, each QoS flow failed to establish is added to the relevant measurement.

d)	Each measurement is an integer value.

e)	QF.InitialEstabFailNbr; or

QF.InitialEstabFailNbr.Cause, 
where Cause identifies the cause resulting in the QoS flow setup failure as specified in TS 38.413 [11].

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

*** START OF NEXT CHANGE ***

###### 5.1.1.13.4.3	Number of QoS flows failed to modify

a)	This measurement provides the number of QoS flows failed to modify. The measurement may be split into subcounters per failure cause.

b)	CC.

c)	On transmission by the gNB of a PDU SESSION RESOURCE MODIFY RESPONSE message (see TS 38.413 [11]), each QoS flow failed to modify is added.

d)	Each measurement is an integer value.

e)	QF.ModNbrFail; or

QF.ModNbrFail.Cause, where Cause identifies the cause as specified in TS 38.413 [11].

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

*** START OF NEXT CHANGE ***

###### 5.1.1.13.4.x	Number of QoS flows fulfillment notification

a)	This measurement provides the number of notifications due to one or more GBR QoS flows fulfillment. The measurement may be split into subcounters per QoS level.

b)	CC

c)	On transmission by the NG-RAN of a PDU SESSION RESOURCE NOTIFY message (see 3GPP TS 38.413 [11]) by gNB, each fulfilled QoS flow is added.

d)	Each measurement is an integer value.

e)	QF.ModNtfNbrFulfilled; or

QF.ModNtfNbrFulfilled.QoS; where QOS represents the mapped 5QI value

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

###### 5.1.1.13.4.y	Number of QoS flows non-fulfillment notification

a)	This measurement provides the number of notifications due to one or more GBR QoS flows non-fulfillment. The measurement may be split into subcounters per QoS level.

b)	CC

c)	On transmission by the NG-RAN of a PDU SESSION RESOURCE NOTIFY message (see 3GPP TS 38.413 [11]) by gNB, each non-fulfilled QoS flow is added.

d)	Each measurement is an integer value.

e)	QF.ModNtfNbrNotFulfilled; or

QF.ModNtfNbrNotFulfilled.QoS; where QOS represents the mapped 5QI value

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

*** END OF CHANGE ***