3GPP TSG-SA5 Meeting #163 	S5-254235

Wuhan, China, 13th October 2025 - 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0251 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.554 performance measurements for QoS flow fulfillment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The incoming LS (S2-2508116) requires monitoring and capturing statistics for downgraded QoS flows. This aligns with TS 23.288, Clause 6.9, which specifies that QoS Sustainability Analytics must provide insights into historical QoS change statistics over past analytics target period within a given area or predict the likelihood of QoS changes over a future target period in that same area. The existing QoS Flow Retainability KPI specifies the frequency of abnormal, premature flow release. Per LS requirements, it will be beneficial for the QoS sustainability analytics to introduce a KPI that captures how often the performance for an end-user QoS non-fulfilment from the agreed QoS requirements of a QoS flow occurred during the time of the QoS flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new QoS flow Retainability KPI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA2 specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

#### 6.5.1.x	QoS Flow Maintainability

a)	QF.QoSMaintain

b)	This KPI shows how often the performance for an end-user QoS non-fulfilment from the agreed QoS requirements of a QoS flow occurred during the time of the QoS flow. It is obtained by number of QoS flows with data in a buffer that were QoS non-fulfilment from the required QoS flow attributes, normalized with the data session time.

b-1)	Integer, NotFullfilled QoS flows per second

b-2)	MEAN

c)	The QoS flow Maintainability for a single QoS level (QoSx) is defined as:

R1QoS_x=QF.RelActNbr.QoSQoS_xQF.SessionTimeQoS.QoSQoS_xQF.QoSMaintainQoSx =QF.ModNtfNbrNotFulfilled.QoSQoSxQF.SessionTimeQoS.QoSQoSx

d)	SubNetwork, NRCellCU

e)	The definition of the service provided by 5GS is QoS flows.

*** END OF CHANGE ***