3GPP TSG-SA4 Meeting #134	S4-252048

# Dallas, TX, USA, 17 – 21 November 2025                                            (revision of S4-251733)

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.264 | CR | 0009 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and examples |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AvCall-MED |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR provides several editorial changes to address typos, improve the clarity of the specification, and define missing structures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections of typos, clarifications, and examples. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Readibility and clarity of the specification will remain impacted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 7.3.2, 7.4.3.1, 7.4.3.2, Annex B.1.8.2, Annex D (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 5.6.3.2	2D Avatar simple profile

The 2D avatar container shall consist of the following mandatory components:

At least one 2D mesh representation that consists of a single planar mesh or quad suitable for texture mapping, conforming to the binary glTF (GLB) format.

At least one static image asset for the base avatar representation that conforms to still image formats as defined in clause 5.5.

At least one LandmarkSet component as defined in [10] for facial animation.

Data items of the 2D avatar should signal no compression or protection schemes.

Support for animation using voice-based animation through a pre-trained and fine- tuned model for the user may be supported.

==============Next change==============

### 6.4.1	General

An AR-MTSI client in terminal that is a compliant device type of TS 26.119 [3] shall support the capabilities requirements for scene description as described in clause 10 of TS 26.119 [3] for its respective device type.

When used in an AR call, the scene description should be the entry point to the AR session (after establishment of a regular call/conference) and shall be exchanged over the data channel as described in TS 26.114 [2]. The Scene Description is exchanged over a stream with a stream id in the range 1 to 1000 and shall be provided by the AR AS through the MF to the AR-MTSI client in terminal. In this case, no web application needs to be downloaded.

Based on the information in the Scene Description, the UE may decide to add additional media streams through a re-INVITE. However, at least the RTP session for the voice of every participant should be present and should be linked to an audio source in the scene description.

NOTE:	Support for advanced audio codecs, such as IVAS, in scene description is for further study.

Each participant should be associated with their own camera node, identified through the node name, which is also provided as part of the SDP through the “sd-nodes” attribute of media session of the data channel that carries the Scene Description.

The “sd-nodes” attribute shall conform to the following ABNF syntax:

att-field			= "sd-nodes"

att-value			= participant-label SP node-desc *("," node-desc)

participant-label	= char-val	; char-val is defined in RFC 7405
node-desc			= node-name ["/avatar"]

node-name			= char-val	; char-val is defined in RFC 7405

The AR MF should apply pose updates from the received pose information of each participant to their respective camera nodes, as negotiated by the SDP sd-nodes attribute.

A scene description of an AR session may be sent from the AR MF/MRF to the AR-MTSI clients in terminal.

An AR MF that supports scene description shall support:

-	The capability to generate a scene description file that conforms to the SD-Rendering-glTF-Core capability as defined in TS 26.119 [3].

-	The capability to generate and update a scene description file that conforms to the SD-Rendering-glTF-Ext1 as specified in TS 26.119 [3].

An AR MF that supports scene description may additionally support the generation of scene description files and updates that conform to the SD-Rendering-glTF-Ext2 capabilities as defined in TS 26.119 [3].

An AR MF that supports scene description may additionally support the generation of scene description files and updates that conform to the SD-Rendering-glTF-Interactive capabilities as defined in TS 26.119 [3].

In addition, an AR MF that supports scene description shall support the referencing of RTP streams in the scene description through the MPEG_media extension as defined in ISO/IEC 23090-14 AMD 2 [7]. The external media shall be RTP media streams supported by an AR-MTSI client and signalled in the SDP.

When scene description is not used as the entry point, the scene description shall be sent by the AR MF/MRF to the AR-MTSI client in terminal over the application data channel. The type of the message containing the scene description shall be set to “urn:3gpp:ar:v1:sd”.

An AR MF that supports scene descriptions should create and distribute the scene for an AR call with audio and video streams based on the visualization space, viewer position and AR media properties. The AR MF should create the scene description for each participant (AR-MTSI client in terminal) such that the shared experience is symmetrical for the different users in the call, e.g., to maintain relative position of users and objects.

==============Next change==============

### 7.3.2	Network Animation and Rendering

When an AR-MTSI client initiates or receives a call with avatar media, the IMS Application Server (IMS AS) shall evaluate whether network-based avatar animation and rendering is required. The IMS AS forwards avatar-related INVITE requests to the Avatar-capable AR AS for capability assessment and media routing decisions.

The AR AS shall invoke network-based animation and rendering through an MF when:

the receiving MTSI client has not registered the "+sip.3gpp-avatar-support" feature tag,

the receiving MTSI client has registered the "+sip.3gpp-avatar-support" feature tag with “avatar-assisted” value,

the receiving MTSI client's registered capabilities indicate insufficient resources for avatar animation,

the offered animation frameworks are not supported by the receiving MTSI client.

When network animation and rendering is invoked, the sending AR-MTSI client shall use the negotiation processes with the AR AS to determine the avatar animation and rendering configuration. The exchange of the configuration information shall take place using an established application data channel. The application data channel established for animation and rendering negotiation of the avatar with SDP offer/answer between the sending AR-MTSI client in terminal and the MF with the sub-protocol “3gpp-avatar-negotiation” and continue to be used for animation and rendering re-negotiation until the end of the avatar call. In this case, the MF shall establish a video stream with the receiving MTSI client using standard video codecs as specified in [2].

When network animation and rendering is invoked, the AR AS shall allocate an MF capable of real-time avatar rendering and configure it with the appropriate rendering parameters based on the receiving UE's video capabilities. The IMS AS shall modify the SDP to route avatar animation data to the MF instead of the receiving UE, effectively inserting the MF into the media path between the sending and receiving MTSI clients.

The MF performing network-based avatar animation and rendering shall fetch the ARF container from the BAR using the reference provided in the SDP and load the avatar model for rendering. The MF shall receive animation streams through the data channel from the sending AR-MTSI client and apply these animation samples to the avatar model in real-time. The animated avatar shall be rendered as a 2D video stream or stereoscopic 3D video stream based on the receiving UE's capabilities.

For the receiving MTSI client, the avatar data channel media description shall be replaced with a video media description including standard video codec negotiation as specified in [2], and avatar-specific attributes that are not applicable to video streams shall be removed.

The MF should ensure lip-sync between avatar animation and associated audio streams.

==============Next change==============

#### 7.4.3.1	Avatar negotiation protocol

Messages sent over the ADC for avatar negotiation shall use the AR metadata message format defined in clause 6.2.

The message type for avatar negotiation messages shall be set to "urn:3gpp:avatar:v1:negotiation". The payload of the messages shall be as defined in Table 7.4-1.

Table 7.4-1: Payload format of the avatar negotiation message type

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| subtype | number | 1..1 | An identifier of the subtype of the message payload. The allowed values are listed in Table 7.4-2. |
| content | object | 1..1 | The payload of the message as defined in 7.4.3.2. |

Table 7.4-2 defines all supported sub-types of the avatar negotiation message type:

Table 7.4-2: Subtypes of avatar negotiation messages

| Value | Message Name |
| --- | --- |
| 1 | REQUEST |
| 2 | ACCEPT |
| 3 | REJECT |
| 4 | ACK |

The REQUEST is sent by the UE or by the MF when acting as the negotiation coordinator.

The ACCEPT and REJECT messages are sent by the UE or MF to indicate that the received REQUEST is accepted or rejected.

The ACK is issued by the DC-AS or MF when it decides to perform animation and rendering on behalf of the receiver.

==============Next change==============

#### 7.4.3.2	Message Formats

The following tables describe the message formats exchanged for avatar negotiation.

The REQUEST message payload is defined in Table 7.4-3.

Table 7.4-3: REQUEST payload

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| avatarProfile | string | 1..1 | Indicates the avatar profile as defined in clause 5.6. NOTE: allowed values for the avatarProfile are FFS. |
| arfContainer | object | 1..1 | Reference to ARF container of the Avatar representation that is offered for the call. The container shall be retrievable from the BAR. |
| animationSourceData | array(URI) | 1..1 | Provides a list of supported animation data for the avatar. The following values are allowed: A subset of the AnimationFrameworks as declared in the ARF container of the base avatar Audio with the URN "urn:3gpp:avatar:v1:animation:audio" Video with the URN "urn:3gpp:avatar:v1:animation:video" At least one animation source data shall be included. |
| renderingLocation | array(string) | 1..1 | Indicates the possible location(s) for animating and rendering of the offered avatar. The possible values are: “sender”, “receiver” or “mf”. An MF may add “mf” in the offered list if it is capable of animating and rendering the offered avatar. |
| animationFallback | array(URI) | 0..1 | A list of URNs indicating the supported animation data to be used as a fallback for animating the avatar when the selected animation stream stops. If the animationFallback is an RTP stream, the MID value of the fallback stream shall be sent as part of the urn:3gpp:avatar:v1:animation:stopped message over ADC. |

The ACCEPT message payload is defined in Table 7.4-4

Table 7.4-4: ACCEPT payload

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| renderingLocation | string | 1..1 | Indicates the selected location for the rendering and animation of the offered avatar. If the location is set to “mf”, it has to be confirmed by the transmission of an ACK message from the MF to the receiver. |
| animationSourceData | array(URI) | 1..1 | Includes the selected subset of the animation frameworks that the receiver prefers to receive for performing the avatar animation. If an audio or a video source is used, the receiver or MF shall use one of the audio or video stream from the IMS session to animate the avatar. |
| animationFallback | URI | 0..1 | Selected URN from the values offered in the animationFallback field of the REQUEST message to be used as a fallback for animating the avatar when the selected animation stream stops. If the animationFallback is an RTP stream, the MID value of the fallback stream shall be sent as part of the urn:3gpp:avatar:v1:animation:stopped message over ADC. |

If a fallback stream is indicated in the animationFallback field of the ACCEPT message and the animation fallback is an RTP stream, then the SDP media description of the fallback RTP stream shall include a MID value.

The ACK message payload is defined in Table 7.4-5.

Table 7.4-5: ACK payload

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| selectedRenderingLocation | string | 1..1 | Indicates the selected rendering location as a final decision. |

The REJECT message payload is defined in Table 7.4-6.

Table 7.4-6: REJECT payload

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| code | string | 1..1 | Machine-readable error code. NOTE 1: If the REJECT message needs to be associated with a particular REQUEST is FFS. NOTE 2: Definition of Error codes is FFS. |
| detail | string | 0..1 | Human-readable diagnostic text. |

The arfContainer type is defined in Table 7.4-7.

Table 7.4-7: Component: arfContainer

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| uri | string | 1..1 | Authorization-bound reference to the base avatar in BAR or an operator repository. Typical schemes: bar://… or https://…. |
| auth | object | 0..1 | Ephemeral authorization for dereferencing URI (e.g., bearer token); present when required by BAR policy. |

The auth object of the arfContainer is defined in Table 7.4-8.

Table 7.4-8: Sub-component: arfContainer.auth

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| scheme | string | 1..1 | Authentication scheme; "bearer" is supported. NOTE: Other HTTP defined authentication schemes can be added and is FFS. |
| token | string | 1..1 | Opaque credential used to access and download the ARF container. |
| expiresAt | number | 0..1 | Expiry time in milliseconds since epoch for the token. |

==============Next change==============

### B.1.8.2	Resource structure

The Avatar Representation Selection API is accessible through the following URL base path:

{apiRoot}/3gpp-mbar-management/{apiVersion}/avatars/{avatarId}/representations/

Table B.1-10 specifies the operations and the corrresopnding HTTP methods that are supported by this API. In each case, the sub-resource path specified in the second column of the table shall be appended to the above URL base path.

Table B.1-10: Operations supported by the Avatar Representations API

| Operation name | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Avatar Representation |  | POST | Creates an Avatar Representation resource in the BAR. |
| Get Avatar Representation | {avatarRepresentationId} | GET | Used to retrieve a previously created Avatar Representation resource in the BAR. |
| Update Avatar Representation |  | PUT, PATCH | Used to update Avatar Representation resource corresponding to an Avatar Representation ID. |
| Delete Avatar Representation |  | DELETE | Removes and deletes an Avatar Representation. |

==============Next change==============

Annex D (informative):
Examples

# D.1	Avatar Call

The following SDP examples depicts an offer for an Avatar Call:

v=0 o=ue1 3735928559 3735928559 IN IP4 203.0.113.1 s=Avatar call t=0 0 a=group:BUNDLE 0 1 2 m=audio 9 UDP/TLS/RTP/SAVPF 111 c=IN IP4 0.0.0.0 a=mid:0 a=rtpmap:111 AMR-WB/16000/1 a=sendrecv m=video 9 UDP/TLS/RTP/SAVPF 96 c=IN IP4 0.0.0.0 a=mid:1 a=rtpmap:96 H264/90000 a=sendrecv m=application 9 UDP/DTLS/SCTP webrtc-datachannel c=IN IP4 0.0.0.0 a=mid:2 a=sctp-port:5000 a=dcmap:0 subprotocol="3gpp-avatar-negotiation",ordered,unordered=false,reliability="reliable" a=dcsa:0 label="3gpp-avatar-proc-gc"

A conformant answer that accepts the ADC and the audio stream but rejects the video stream in favor of the avatar may look like the following example:

v=0 o=ue2 2718281828 2718281829 IN IP4 198.51.100.2 s=Avatar call answer (video rejected, ADC used) t=0 0 a=group:BUNDLE 0 2 m=audio 9 UDP/TLS/RTP/SAVPF 111 c=IN IP4 0.0.0.0 a=mid:0 a=rtpmap:111 opus/48000/2 a=recvonly a=label:3gpp-avatar-proc-cp m=video 0 UDP/TLS/RTP/SAVPF 96 c=IN IP4 0.0.0.0 a=mid:1 a=inactive m=application 9 UDP/DTLS/SCTP webrtc-datachannel c=IN IP4 0.0.0.0 a=mid:2 a=sctp-port:5000 a=dcmap:0 subprotocol="3gpp-avatar-negotiation",ordered,unordered=false,reliability="reliable" a=dcsa:0 label="3gpp-avatar-proc-gc"

Once the data channel is established, the negotiation of the avatar may start. The following is an example of a REQUEST sent by the UE to the MF:

{ "type": "urn:3gpp:avatar:v1:negotiation", "subtype": 1, "content": { "avatarProfile": "r19.basic-3d", "arfContainer": { "barRef": "bar://tenant/avatars/ue1/base-1234" }, "animationSourceData": [ "urn:3gpp:avatar:v1:animation:audio", "urn:3gpp:avatar:v1:animation:video" ], "renderingLocation": ["mf","receiver","sender"], "animationFallback": [ "urn:3gpp:avatar:v1:animation:audio" ] } }

If accepted, the receiving UE will respond with an ACCEPT message via the MF:

{ "type": "urn:3gpp:avatar:v1:negotiation", "subtype": 2, "content": { "renderingLocation": "mf", "animationSourceData": [ "urn:3gpp:avatar:v1:animation:audio" ], "animationFallback": "urn:3gpp:avatar:v1:animation:audio" } }

==============End of change==============