3GPP TSG SA Meeting #82	TD SP-181250

Sorrento, Italy, 12-14 Dec 2018	(revision of SP-181242)

3GPP TSG SA WG-4 Meeting #101 	SA4-181524

Busan, Korea, 19-23 November 2018	(revision of S4-181411)

# Source:	Ericsson LM, Samsung Electronics Co., Ltd., Qualcomm Incorporated, Sony Corporation, KPN N.V., Orange, Deutsche Telekom AG, Rogers Communication Canada Inc., Intel.

# Title:	New WID on 5G Media streaming architecture

# Document for:	Approval

Agenda Item:	18

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 5G Media streaming architecture

## Acronym: 5GMSA

## Unique identifier:	820002

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 760039 | FS_5GMedia_Distribution (5G enhanced Mobile Broadband Media Distribution) | Preceding Study Item |
| 810005 | FS_TyTrac (Typical Traffic Characteristics of Media Services) | Identifies Traffic Characteristics of existing Operator and Third-Party Media Services |
| 800014 | FS_AVPROD (Study on Audio-Visual Service Production) | Service requirements for Rel-17. Only considered for future compatibility of Media architecture. |
| 800008 | eLSTR (Enhancement of LTE for Efficient delivery of Streaming Service) | Service requirements for 4G. |
| 790001 | SMARTER_Ph2 (New Services and Markets Technology Enablers – Phase 2) | Service requirements for 5G. |
| 800001 | E-FLUS (Enhancements to Framework for Live Uplink Streaming) | Live Uplink streaming stage 3. Goal to incorporate the enhanced FLUS architecture into 5GMSA. |

## 3	Justification

The Packet Switched Streaming Architecture in TS 26.233 includes many different streaming aspects. It has been mainly developed based on RTSP streaming and although it was evolved to match latest deployements, it is now seen as unfit for 5G media streaming services.

The TyTrac study item identifies the functions and characteristics of existing operator and third-party media services and motivates a new media streaming architecture.  Classical broadcast and content distribution services are migrating towards modern distribution architectures. The proper integration of such services in 5G Media Distribution is a commonly agreed objective and deserves a revisiting of the 3GPP media architecture.

Rel-15 Framework for Live Uplink streaming stage 3 was developed with an under-specified architecture.

The 5G System offers many features (e.g. Local Break Out, Distributed Compute) and many deployment options for 5G Media Streaming Services. The FS_5GMedia_Distribution (5G enhanced Mobile Broadband Media Distribution) Study Item conclusions are documented in 3GPP TR 26.891 “5G enhanced mobile broadband; Media distribution” and one of the conclusions is that a new 5G media streaming architecture is required to enable differenttypes of deployments of streaming services over 5G.

## 4	Objective

Create a new 5G Media Streaming (5GMSA) architecture specification which supports:

- MNO and 3rd party Media Downlink Streaming Services with relevant functions and interfaces to support:

  - Different collaboration scenarios between third party-providers and mobile network operators for media distribution over 5G;

  - Appropriate service and session definitions in the context of 5G Media Distribution, especially for third-party media services and corresponding network interfaces to establish, announce and discover those;

  - A distribution-independent service establishment and content ingest interface;

  - Relevant functions for operators and third-party service providers in different collaboration scenarios, including but not limited to aspects such as session management, QoS framework, network assistance, QoE reporting, accessibility, content replacement, notification, content rights management, etc.

  - The delivery of 3GPP-defined media formats and profiles as well as third-party formats based on commonly defined packaging formats.

Note:  Support of 5G Media streaming over MBMS with 5GC is not considered in this work item.

- MNO and 3rd party Media Uplink Streaming Services based on the non-IMS FLUS architecture:

  - Specify the non-IMS FLUS entities and interfaces as part of the 5GMSA where the FLUS sink is not in the UE;

  - Enable different collaboration scenarios between third party-providers and mobile network operators for media over 5G.

- Corresponding UE functions and APIs;

- Backwards compatibility for EUTRAN deployments (with and without MBMS)

- Usage of 5G specific features such as network slicing and edge computing.

The new 5G Media Streaming architecture should be functionally decomposed into independent components enabling different deployments with various degrees of integration between 5G MNOs and Content Providers. It should be specified as a set of extensions to TS 23.501 “System Architecture for the 5G System”.

The work should consider input from key industry players on their requirements and usage scenarios for media streaming over 5G. This includes broadcasters and content providers/emerging media service providers, as well as mobile network operators running their own media services.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| TS | TBD | 5G Media Streaming (5GMS); general description and architecture | TSG SA#83 | TSG SA#84 | rapporteur: Frédéric GABIN (frederic.gabin@ericsson.com) WG preference for TS number is 26.501 |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 26.233 | Not maintained from Rel-16 onward | SA#84 |  |

## 6	Work item Rapporteur(s)

Frédéric GABIN (frederic.gabin@ericsson.com)

## 7	Work item leadership

SA4

## 8	Aspects that involve other WGs

SA2 should be given the opportunity to review the architecture prior to approval.

## 9	Supporting Individual Members

Supporting IM name

Ericsson LM

Samsung Electronics Co., Ltd.

Qualcomm Incorporated

Sony Corporation

KPN N.V.

Orange

Deutsche Telekom AG

Rogers Communication Canada Inc.

Intel

NOVAMINT