3GPP TSG-SA Meeting #90e 	SP-201090

EMeeting, 8-14 December, 2020 	(revision of S1-204388)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0478 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service requirements for enhancing service function chaining support by 5G network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, Deutsche Telekom, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | SFChain |  |  |  |  |  |  | Date: |  | 2020-11-19 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enhancing service function chaining in 5G network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add the following aspects for service requirements: - Allowing authorized third parties to request utilisation of specific service function and service function chaining for their applications. - Allowing service function chaining under control of the operator network - Allowing the management of service function and service function chaining. - Addressing charging aspects |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancement of service function chaining in 5G is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1; new clause; 9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR is within the set of agreed CRs (S1-204387, S1-204388, S1-204389) for the agreed WID (S1-204386). |  |  |  |  |  |  |  |  |

*** 1st Change ***

# X	Service Function Chaining

## X.1	Introduction

In order to support enhancement of service function chaining for 5G networks beyond the requirements for FMSS in TS 22.101, the network operator defines service function chaining policies for service function chaining to steer the traffic associated to the application and its users on per UE basis to appropriate ordered service functions.

A service function chain for 5G network contains service functions such as firewall functions, NAT, antimalware, parental control, DDoS protection, TCP proxies, load balancers, KPI monitoring, and video optimization, etc.

NOTE:	these are non-exhaustive examples of service functions. Other service functions can be provided by an operator.

## X.2	General Requirements

The following requirements apply for supporting enhancement of service function chaining for 5G network:

-	The network operator shall be able to define and modify service function chaining policies for steering traffic on per application per UE basis through required service function chaining with ordered service functions to improve the user’s QoE.

-	Service function chaining policies shall be able to distinguish between upstream and downstream traffic.

- 	The coexistence of traffic with and without service function chaining shall be supported.

-	Service function chaining shall provide suitable means for authorized third parties to requesta chain of service functions provided by the network operator based on operator’s service function chaining policies.

-	In case of roaming, the HPLMN shall be able to apply traffic steering policies and service function chaining polices for home routed traffic.

-	In case of roaming with local breakout, the HPLMN shall be able to provide the traffic steering policies and service function chaining policies to the VPLMN providing local breakout with support of service function chaining.

-	Service function chaining shall support deployments where the  HostedServices are provided by the operator and  deployments where the Hosted services  are provided by a third party.

## X.3	Service Function Management

-	The service function management shall allow the operator to create, modify, and delete a service function based on operator’s service function chaining policies.

-	The service function management shall allow the operator to create, configure, and control a chain of service functions per application and its users on per UE basis based on operator’s policy or request from third parties.

-	The service function management shall be able to manage service function chaining for deployments where the Hosted Services are provided by the operator and for deployments where the Hosted Services are provided by a third party.

MORE changes

# 9	Charging aspects

## 9.1	General

The following set of requirements complement the requirements listed in 3GPP TS 22.115 [11]. The requirements apply for both home and roaming cases.

The 5G core network shall support collection of all charging information on either a network or a slice basis.

The 5G core network shall support collection of charging information for alternative authentication mechanisms.

The 5G core network shall support collection of charging information associated with each serving MNO when multi-network connectivity is used under the control of the home operator.

The 5G core network shall support charging for services/applications in an operator’s Service Hosting Environment.

The 5G core network shall support charging for content delivered from a content caching application.

The 5G core network shall support collection of charging information based on the access type (e.g. 3GPP, non-3GPP, satellite access).

The 5G core network shall support collection of charging information based on the slice that the UE accesses.

The 5G core network shall support collection of charging information based on the capacity and performance metrics.

In a 5G system with satellite access, charging call records associated with satellite access(es) shall include the location of the associated UE(s) with satellite access

NOTE: The precision of the location of the UE can be based on the capabilities of the UE or of the network.

The 5G system shall be able to support an indirect network connection even when the UE is in E-UTRAN or NG-RAN coverage.

For service function chaining (see clause X) the collection of charging information associated to the use of service functions and the chain of service functions requested by third parties shall be supported.

*** End of Change ***