TSG-SA Working Group 1 (Services) meeting #4	TSG S1#4 (99)429

Quebec City, Quebec, Canada 5th-9th, July 1999 	 Agenda Item:6.4.2

TSG-SA Working Group 1 (Services) meeting #4	TSG S1#4 (99)xxx

Quebec City, Quebec, Canada 6th-9th, July 1999	 	 Agenda Item:

Source: Telecom Italia Mobile

Title:	Proposed CR to 22.115

Document for:	Discussion	 _________________________________________________________________________________________

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 22.115 |  | Version: |  |  | 3.1.0 |  |  |  |
| Submitted to SA | 4 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  | X |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME |  | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | Telecom Italia Mobile | Date: | 25/06/99 |
| --- | --- | --- | --- |

| Subject: | - Modifications to 3G TS 22.1115 to clarify the mandatory features supported by the standard and the features that may not be supported in release ’99. - Definition of location information for call detail records - Editorial modifications |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | It has been requested to clearly identify the mandatory/optional features of UMTS standard and if the feature may not be supported by UMTS release ’99. |
| --- | --- |

| Clauses affected: | 2, 5, 6, 7, 8 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

3G TS 22.115 3.2.0 (1999-04)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects

Service aspects;

Charging and Billing

(3G TS 22.115 version 3.2.0)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTS/TSGS-0122115U

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Contents

Foreword	5

1	Scope	6

2	Normative references	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Requirements	7

5	Generation of Call Detail Records	7

5.1	Call Detail Record Requirements	7

5.1.1	Information provided by the user	7

5.1.2	Information provided by the serving network	8

5.1.3	Charged Party	8

5.2	Special Cases	8

5.2.1	Long calls	8

5.2.2	Multimedia calls	8

5.2.3	E-Commerce	9

6	Transfer of Charging Information	9

6.1	Integrity, Secrecy and Validation of Content and Receipt of Charging Information	9

7	Accounting and Settlement	9

7.1	Delegation of charging authority	9

7.2	Fraud Control	10

7.2.1	Fraud Control by the Home Environment	10

7.2.2	Fraud Control by the Serving Network	10

7.3	Cost Control	10

7.4	Inter-network Settlement	10

8	Automatic Roaming Agreements	10

8.1	Routing the Registration Request	11

8.2	Settlement of charges	11

Annex A (informative) : Change history	12

History	13

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TS, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version 3.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	Indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the specification;

# 1	Scope

This Technical Specification describes the Service Aspects of  charging and billing of the Universal Mobile Telecommunications System (UMTS).

This standard is not intended to duplicate existing standards or standards being developed by other groups on these topics, and will reference these where appropriate. This standard will  elaborate on the charging requirements described in the Charging Principles in UMTS 22.01 Service Principles. It will allow the generation of accurate charging information to be used in the commercial and contractual relationships between the parties concerned.

# 2	Normative references

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.

-	A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

[1]	3G TS 22.101 “Service aspects; Service Principles”;

[2]	GSM 02.31: "Fraud Information Gathering System (FIGS); Service description - Stage 1";

[3]	GSM 02.78: “Customised Applications for Mobile network Enhanced Logic (CAMEL); Service definition – Stage 1

[4]						GSM 02.86: "Digital cellular telecommunications system (Phase 2+); Advice of charge (AoC) supplementary services - Stage 1"..

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of this TS, the definitions in [1] are supplemented by the following definitions:

Accounting: The process of apportioning charges between the Home Environment, Serving Network  and User.

Billing: A function whereby CDRs generated by the charging function are transformed into bills requiring payment.

Call Detail Record (CDR): A formatted collection of information about a chargeable event (e.g. time of call set-up, duration of the call, amount of data transferred, etc) for use in billing and accounting. For each party to be charged for parts of or all charges of a chargeable event a separate CDR shall be generated, i.e more than one CDR may be generated for a single chargeable event, e.g. because of its long duration, or because more than one charged party is to be charged.

Chargeable Event: An activity utilising telecommunications network infrastructure and related services for user to user communication (e.g. a single call, a data communication session or a short message), or for user to network communication (e.g. service profile administration), or for inter-network communication (e.g. transferring calls, signalling, or short messages), or for mobility (e.g. roaming or inter-system handover), which the network operator wants to charge for. The cost of a chargeable event may cover the cost of sending, transporting, delivery and storage. The cost of call related signalling may also be included.

Charged Party: A user involved in a chargeable event who has to pay parts or the whole charges of the chargeable event, or a third party paying the charges caused by one or all users involved in the chargeable event, or a network operator.

Charging: A function whereby  information related to a chargeable event is formatted and transferred in order to make it possible to determine usage for which the charged party may be billed.

Real time: Time, typically in number of seconds, to perform the on-line mechanism used for fraud control and cost control.

Settlement: Payment of amounts resulting from the accounting process.

Short time: Time, typically in number of minutes, to perform the off-line mechanism used for accounting.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CDR	Call Detail Record

# 4	Requirements

The main new requirements for UMTS charging and accounting are:

-	to provide a call detail record for all charges incurred and requiring settlement between the different commercial roles;

-	to allow fraud control by the Home Environment and the Serving network;

-	to allow cost control by the charged party;

-	to provide at the beginning of a chargeable event an indication to the charged party (if involved in the chargeable event) of the charges to be levied for this event;

-	to allow itemised billing for all services charged to each subscription, including voice and data calls, and services offered by home environments.

These new requirements will allow users more freedom to obtain service when roaming, whilst providing effective cost and credit control for the Home Environment and User.

# 5	Generation of Call Detail Records

The standard shall support the creation and transfer of charging records in order to facilitate:

-	interworking with pre-UMTS systems (e.g. GSM);

-	fraud management procedures;

-	detailed itemised billing.

## 5.1	Call Detail Record Requirements

Call Detail Records shall be generated in the Serving Network to record chargeable User or Mobile Station activity and inter-carrier connections. Some of the information is provided by the user, other information is only available in the network element of the serving network.

Depending on the type of chargeable event some of the information may not be available or might not be required.

### 5.1.1	Information provided by the user

The user incurring the charge shall provide the following information to the serving network:

- User identity used for authentication;

- Home environment identity;

- Terminal Identity and Terminal Class;

- Destination endpoint identifier for service requested (e.g. B number);

- Resource requested (e.g. bandwidth, connectionless);

- QoS parameters (e.g. maximum delay).

### 5.1.2	Information provided by the serving network

The serving network serving the user shall provide the following information to the home environment:

- All of the information listed in section above (Information provided by the user);

- Serving network identity;

- Recording network element identity;

- Universal Time (UT) at which the service request was initiated;

- Universal Time (UT) at which resources were provided for the service;

- Resource allocated to the user;

- Quantity of data transferred by the user;

- QoS provided to the user;

- Location of the userin the standard format used for UMTS location based services (e.g. geographical co-ordinates, Cell ID, Localised Service Area);

- whether GSM Optimal Routing was applied;

- If IN or CAMEL services were applied, the service parameters and the actually used destination number and calling party number identification;

- Time duration covered by this call record to an accuracy of at least 1 second;

- Unique identity of the chargeable event which allows the billing system to correlate all records belonging to the same chargeable event;

- Unique CDR identity (unique per network element in a period of about 100 days).

### 5.1.3	Charged Party

For subscription related chargeable events the CDR shall indicate the charged party, i.e. normally the calling party. As alternative it should be possible to apply reverse charging or to charge the event to a party not involved in the event itself (e.g. a company as VPN subscriber). It should be possible for multiple leg calls (e.g. forwarded, conference or roamed) to be charged to each party as if each leg was separately initiated. However, in certain types of call, the originating party may wish/be obliged to pay for other legs (e.g. SMS MO may also pay for the MT leg.).

It shall be possible to change the chargeable party at the call set-up.

In case of inter-network chargeable events, the CDR usually does not contain the charged party, but it can be derived from network configuration information contained in the CDR.

For each party to be charged for a chargeable event or parts of it a separate CDR shall be generated.

## 5.2	Special Cases

### 5.2.1	Long calls

The advent of packet data calls, which can extend for very long periods of time (days, weeks etc), although at low cost because charges are based on data throughput, may mean that billing records are only output at the end of very long periods.For this reason the serving network shall support the generation of call records also during the life of the packet data call, either when some charge value is reached or some duration or both, to allow for both charging settlement and cost control.

### 5.2.2	Multimedia calls

During one call the user may invoke different services like speech, data transmission, video and audio, which may lead to a separate CDR for each service. If several CDRs will be the result of each multimedia call, the billing system shall be able to correlate these records and to indicate to the user on the bill that they belonged to one call.

### 5.2.3	E-Commerce

- The UMTS system may be used to trade soft goods (e.g. information, video, audio), or hard goods (e.g. books) of high or low value per item between the user and a merchant. It shall be possible for such merchants to charge users directly for services they provide. Electronic payment mechanisms are or shall be made available through other standards (micropayment, credit card payment, etc), and therefore are outside the scope of this specification UMTS shall not prohibit the use of these mechanisms, and, where possible, shall provide the basic communications transport to allow them to be used effectively.

- However, if the serving network acts as merchant of soft goods, it may charge the user directly, creating a CDR as described above or using micropayment mechanisms.

# 6	Transfer of Charging Information

The efficient transfer of charging information between serving networks and from serving networks to home environments requires a standardised interface between these entities. It shall be possible to define different time intervals for the transfer of charging information between serving network and home environment  (e.g. when a chargeable event occurs, when a chargeable event is initiated by the user, when a chargeable event terminates, at regular intervals during a chargeable event)

-

-

-

The format of the charging information exchanged (see 5.1) shall be standardised. It shall be possible for the relevant parties to agree minimum and maximum age of call information tranfered between themselves.

## 6.1	Integrity, Secrecy and Validation of Content and Receipt of Charging Information

The transmission mechanism for charging information collected in 5.1 above shall ensure its integrity and secrecy.

A mechanism to validate the source and integrity of the information shall be provided so that:

-	The home environment shall be able to validate the source and integrity of the charging information supplied by the serving network;

-	The serving network shall be able to validate the source and integrity of the charging information supplied by the user;

-	The serving network shall have proof that services were provided to a specified user.

# 7	Accounting and Settlement

The serving network shall collect and process the charging data generated in its network elements. The record of each individual transaction shall be reported to the home environment in order to provide itemised bills, and to deal with any disputes regarding charges both for users and for other UMTS networks and home environment.

The standard shall support the transfer of charging data at different intervals as required by the Home Environment (e.g. short time intervals, real time, other regular intervals).

## 7.1	Delegation of charging authority

The registration process allows the home environment to authenticate users before they incur any charges. Once authenticated, the home environment then delegates authority to the serving network operator with which he has a direct commercial relationship to incur charges for services supplied to that user. The direct commercial relationship may be with either the serving network operator if known directly by the home environment or a network operator known to the home environment. This procedure uses each network as trusted third parties in a chain of delegation between entities, thus allowing commercial transactions between entities who have no direct commercial dealings. There shall be an authentication procedure between all entities in the UMTS system which have a commercial relationship.

## 7.2	Fraud Control

A mechanism to control fraud shall be provided by the serving networks and the home environment.

### 7.2.1	Fraud Control by the Home Environment

Charging information shall be collected by the home environment from all serving networks which its users are allowed to use. The billing system in the home environment shall process the information and provide the means to set charge thresholds per time interval upon which some actions may be started, such as informing the customer care centre or even barring the user in the HLR.

It shall be possible for the Home Environment to define different time intervals for the collection and the processing of charging information (e.g. real time, short time, other regular intervals)

### 7.2.2	Fraud Control by the Serving Network

Charging information shall be collected from the network elements and processed as required. This will allow the serving network to always be aware of the exposure to visitors. A limit for the accumulated charges for all visitors from one home environment or a limit per visitor may be agreed between the home environment and the serving network.

It shall be possible for the Serving Network to define different time intervals for the collection and the processing of charging information (e.g. real time, short time, other regular intervals)

### Cost Control

- A mechanism shall be standardised providing an indication to the user (if involved in the chargeable event) of the charges to be levied for a chargeable event.

- This mechanism shall be able to handle all possible charging scenarios, and all service and tariff variants that the home environment and the serving network may offer to the user. This requirement may not be part of release ’99. For release 99 the cost control mechanism may be based on the GSM Advice of Charge.

- The user shall be able to set in his home environment a limit for the accumulated charges per time interval. Upon exceeding this limit or prior to incurring a charge which would exceed the limit, certain actions may be desired by the user:

- notification to the user, requesting to extend the limit, or

- Home Environment barring allowing no further originating calls, or

- Home Environment barring cancelling the roaming permission.

## 7.4	Inter-network Settlement

Mechanisms shall also be provided to allow inter-network settlement of charges on a bulk basis. The same mechanisms shall be used between home environments and serving networks. This will allow each of these parties to meter the total input and output of charges and thus determine the payments required on a periodic basis between each of the parties with which they directly interact. The mechanisms used shall allow each of the parties to meter charge flows independently, with the aim of matching the values recorded at both sides of the same interface. The imbalance in charge flow shall be accumulated in short time, such that each entity can be informed when a threshold has been exceeded and determine whether to continue.

# 8	Automatic Roaming Agreements

It is a requirement that UMTS users shall be able to obtain service and use chargeable services with networks with whom neither they nor their home environment have any direct commercial agreement. This shall be enabled by interworking via trusted third parties. Each Home Environment shall interwork with one or more serving network operators, with whom they would negotiate a commercial roaming agreement and test the interworking. Any user wishing to use the services of a particular serving network would register with that serving network, who would either directly or indirectly interwork with the home environment. Fraud and cost control mechanisms shall be used to ensure that charges incurred for UMTS services do not exceed the credit limits set. This can be applied for the user and the other roles involved in commercial dealings. In practice, any serving network shall be capable of operating as a roaming broker.

Figure 1: Registration and Roaming Process

## 8.1	Routing the Registration Request

The same mechanisms used for routing calls and resolving addresses shall be used to route the subscription identity back to its Home Environment . The standard shall support a routing identification mechanism to allow a serving network, which does not maintain its own list of all  known HE, to determine the appropriate route to reach a given HE. A number of alternative routes may be possible, and ideally the system should be capable of determining the lowest cost to the end user.

Typically, smaller networks will only have a limited number of external connections to other networks or clearing houses, but may not know which one to use for an unknown (new) HE. In this case, the serving network may make a number of inquiries for each route to determine the lowest cost route to handle the call.

These requirements may not be part of release ’99.

## 8.2	Settlement of charges

Settlement of charges incurred by a user shall be on a wholesale basis between the different parties involved in the registration link. By authorising a user to register, or a roaming broker to pass that on, each party is in turn authorising charges up to a maximum credit limit with the adjacent party. Any charges levied can then be paid to the adjacent party on a wholesale basis at the end of a mutually agreed accounting period. Funds are thus passed between each party for the services supplied by the network operator in a serial fashion.

Annex A (informative) :
Change history

| Change history |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SMG No. | TDoc. No. | CR. No. | Section affected | New version | Subject/Comments |
| SMG#28 |  |  |  | Version 3.0.0 | Approved |
| SA#3, Yokohama, Japan | SP-99106 | 001r1 |  | Version 3.1.0 | - Short time and real time defined. - For multimedia calls (eg H324M) it is not always possible to identify the different media streams and not possible to define separate CDRs for the different media streams. - 7.2.3 is restructured and the section is renamed - The charging mechanism is also a function of the service and tariff variants in the serving network. - The serving network should also be able to do fraud control. - GSM 02.31 FIGS may be considered. - A mobility event can also be an example of a chargeable event. - It is here interpreted that the chargeable party can be changed at the call set-up. 9. Accumulated charges in the currency of the serving network should not be a CDR requirement. |

# History

| Document history |  |  |
| --- | --- | --- |
| Date | Status | Comment |
| 27 August 1996 | Version 0.0.1 | SMG 1 WPC output draft for editing purposes only |
| 3 February 1997 | Version 0.0.3 | Presented to SMG1 WPC meeting, London Incorporated changes agreed at Dec 96 Meeting including charging model and charge enquiry |
| 20 April 1997 | Version 0.0.4 | Presented to SMG1 WPC meeting in Sophia Antipolis Incorporated text submitted at Feb 97 meeting |
| 4 June 1997 | Version 1.0.0 | Proposed Version 1 incorporating changes discussed at SMG1 WPC meeting in Antwerps, June 97 |
| 23 June 1997 | Version 1.0.2 | Incorporated remaining changes discussed at SMG1 WPC meeting in Antwerps, June 97 |
| 27 Nov 1997 | Version 1.1.0 | Preparation for SMG1 UMTS Helsinki meeting, incorporating text from reports 22.24 and 22.71 |
| 4 Dec 1997 | Version 1.2.0 | Incorporated comments from 22.24, 22.71 developed at SMG1 UMTS Meeting in Helsinki |
| 26 April 1999 | Version 3.1.0 |  |
| (continued) |  |  |

(concluded)

| Document history |  |  |
| --- | --- | --- |
| 8 Dec 1997 | Version 1.2.2 | Format and editorial changes by ETSI Sec for SMG#24 |
| 5 November 1998 | Version 1.3.0 | Incorporate changes discussed at SMG1 Rome, including reflecting changes to 22.01 role model. |
| 12 January 1999 | Version 1.3.1 | Accept Changes and send to editing SMG1 Rome Tdoc 98-0859 Agreed by correspondence. Reviewed by Rapporteur Jan 13, 1999 |
| 27 January 1999 | Version 2.0.0 | To be presented at SMG#28 for Approval |
| February 1999 | Version 3.0.0 | Approved at SMG#28 |
| March 1999 | Version 3.0.1 | Reformatted for TSG SA |
| May 1999 | Version 3.1.0 | SA#3, Yokohama, Japan |