TSG-SA Working Group 1 (Services) meeting #4	TSGS1#4(99)490

Quebec City, Canada, 5-9 July 1999		Agenda 6.6.7

| CHANGE REQUEST No : |  |  |  |  | TMOD03 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.129 |  | Version: |  |  | 3.0.0 |  |  |  |
| Submitted to TSG_SA |  |  | for approval |  |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GSM UMTS Handover |
| --- | --- |

| Source: | Rapporteur | Date: | 24.5.99 |
| --- | --- | --- | --- |

| Subject: | Clean-up of terminology (see below) |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| Only shall be | D Editorial modification | X |  | UMTS 99 | X |
| Marked with an X) |  |  |  |  |  |

| Reason for change: | To consistently use the terms: “inter network”, “intra network” rather than “inter/intra operator” . Also it has been remarked that some clarification of the wording of 4.2.2 would be desirable, and this is contained here. |
| --- | --- |

| Clauses affected: | 7.4 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 1st Editorial Change

### 4.2.2	Inter Network Handover Issues

Handovers between GSM and UMTS networks operated by different operators should remain an optional feature to implement. It is envisaged that handover would take place due to changing radio conditions caused eg by movement of the terminal causing it toleave the coverage area of a network.

The following networks may be involved with an inter-network handover procedure. These concepts are illustrated in Annex A:

-	the user’s home network, i.e. the operator where the user’s subscription may be found;

-	the user’s visited network  where the subscriber user is currently registered, i.e. the network  where the subscriber user has performed the last successful update location procedure. As long as the subscriber user  is roaming roams within the home network, home and visited network are identical.

-	the user's serving network covering the cell that serves the subscriber. After successful completion of the update location update procedure, the serving network is identical with the visited network. After an inter-network handover, the visited network is different from the serving network until a location update procedure has been successfully completed (excepted the case that the subscriber returns into the visited network).

-	the target network covering candidate target cell(s) for inter-network handover. The target network has overlapping radio coverage with the serving network but not necessarily with the visited network.

The minimum requirements for inter network HO are:

-	continuity of an active call across the handover procedurein the same circumstances as required for intra-operator handover;

-	charging, billing and accounting for inter-network handover should be according to the principles defined in UMTS 22.15. For R'99 the mechanisms currently used in GSM should be provided as a minimum (charging for handover leg is based on vistited network tariff, etc., settlement between operators is based on bulk metering, etc.);

-	the ability to check with the home network whether the user is permitted to handover  from the visited network to a target network;

-	the decision whether the handover request is accepted must be taken by the target network;

-	invocation of the handover procedure only occurs if the target network provides the radio channel type required for the respective call;

-	the avoidance of "network hopping", i.e. successive handover procedures between neighbouring networks for the same call;

-	the possibility of user notification of inter network HO (eg possible tariff change) when it occurs.

### 2nd Editorial Change

### 4.2.5	Security

Security requirements relating to handover shall be elaborated in a separate document (UMTS 33.21, security requirements), but should embody the principle that handover shall not compromise the security of: the network providing the new radio resources; the (possibly different) network providing the original radio resources; and the terminal. The security mechanisms should also cater for appropriate authentication processes and meet the requirements of national administrations in terms of lawful interception.

### 3rd Editorial Change

Annex A (informative):
Illustration of elements in inter-network handover

Figure 1 illustrates the above definitions taking an example of European GSM networks. The subscriber's home network is France. The visited network where the subscriber is registered in a VLR is Germany. The signalling connection between HLR and VLR is indicated by dotted lines. The calls for the subscriber are controlled by the MSC collocated to the VLR where the subscriber is registered. This MSC is called "anchor MSC".