Technical Specification Group Services and System Aspects	TSGS1#4(99)495

Meeting #4, Quebec 1999

Source:	T-Mobil

Title:	CR to clarify the usage of SIM and USIM

Document for:	approval

Agenda Item:	6.2.2.4

Due to many discussions on the issue of SIM and USIM usage as access module to 2G or 3G systems and for roaming within these systems, the following CR is proposed for clarification.

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS |  |  |  | 22.101 |  | Version: |  |  | 3.6.0 |  |  |  |
| Submitted to TSG |  |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  | x |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form: crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME |  | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | T-Mobil | Date: | 30.06.99 |
| --- | --- | --- | --- |

| Subject: | Clarification on the usage on 2G SIM and 3G USIM |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | x |
| marked with an X) |  |  |  | UMTS | x |

| Reason for change: | Clarification in figure 3, example of a multifunction UICC including a SIM-application to access 2G GSM systems. Correction of figure 4, roaming users, due to the directory structure of a 2G terminal can not address a USIM application on a UICC. |
| --- | --- |

| Clauses affected: |  |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

3G TS 22.101 3.6.0 (1999-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects

Service aspects;

Service principles

(3G TS 22.101 version 3.6.0)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

# 11	UMTS IC Card, USIM and Terminal

This clause defines the functional characteristics and requirements of the User Service Identity Module (USIM) for use in UMTS. The USIM is an application residing on a UICC.

## 11.1	The USIM and User Profiles

### 11.1.1	The USIM

Every USIM shall have a unique identity and shall be associated with one and only one home environment.

It shall be possible for a home environment to uniquely identify a user by the USIM.

The USIM shall be used to provide security features.

For access to UMTS services, provided via a UMTS home environment, a valid USIM  shall be required.

The USIM shall reside on a UICC, UMTS shall adopt both of the GSM SIM card physical formats. New UMTS terminals may require other  formats also. USIM specific information shall be protected against unauthorised access or alteration.

It shall be possible to update USIM specific information via the air interface, in a secure manner.

### 11.1.2	User Profiles

It shall be possible for a user to be associated with one or a number of user profiles, which the user can select and activate on a per call basis. The user profile contains information which may be used to personalise services for the user.

It shall be possible for one or more  user profiles associated with the same user to be active simultaneously so that the user may make or receive calls associated with different profiles simultaneously. Activation of  profiles shall be done in a secure manner, for example with the use of a PIN.

For terminating calls the correct profile shall be indicated by the user address used (e.g. IMUN), each profile will have at least one unique user address associated with it. For originating calls the user shall be able to choose from the available profiles, the appropriate one for the call. A profile identity will need to be associated with the call for accounting and billing purposes. User profile identities need not be standardised but a standardised means is required for indicating that a particular profile is being used.

Simultaneous use of the same user profile on multiple terminals for the same type of service shall not be allowed.

User profiles associated with different home environments shall not share the same user address.

### 11.1.3	Multiple USIMs per UICC

The standard shall support the simultaneous use of more than one USIM per UICC even when those USIMs are associated with different home environments. For UMTS Phase 1 the support of only one USIM per UICC is required.

The standard must not prevent the coexistence of USIM applications, each associated with different home environments on the same UICC, so long as the security problems which arise from such a coexistence are solved when UMTS terminals and UICC are produced. Nevertheless, in the short term, it is safer to assume that only USIMs associated to one home environment will be stored on one UICC.

## 11.2	The UICC

Physical aspects of the UICC shall be defined by an appropriate committee. However there is a requirement to support access to services via GSM and UMTS with a single UICC.

## UICC usage in 2G Terminals

It shall be possible to use the UICC in 2G terminals as access module to GSM networks. Therefore it shall be possible to store a SIM application on the UICC which shall be accessed via the standard GSM SIM-terminal interface.

### 11.2.1	The UMTS UICC and Applications other than the USIM

It shall be possible for the UICC to host other applications in addition to the USIM, see figure 3. Service providers, subscribers or users may need to establish additional data or processes on the UICC. Each application on an UICC shall reside in its own domain (physical or logical). It shall be possible to manage each application on the card separately. The security and operation of an application in any domain shall not be compromised by an application running in a different domain. Applications may need to use their own security mechanisms which are separate to those specified for UMTS e.g. electronic commerce applications.

Examples of other UICCapplications are: a more practical implementation of GSM Phase 2+ SIM items, off-line user applications like UPT, electronic banking, credit service, etc.

Applications should be able to share some information such as a common address book.

It shall be possible to address applications which reside on the UICC, via the air interface.

Figure 3 Example of a Multifunction UICC

## 11.3	Terminals and Multiple UICCs

The standard shall support multiple registration on a single terminal via insertion of multiple UICC, each with at least one unique USIM. In UMTS phase 1 the support of only one UICC is required. One or  more of the USIMs  may be active at the same time so that the terminal may be used to  engage in more than one simultaneous call, possibly associated with different home environments. In UMTS phase 1 the activation of only one USIM at a time is required.

The standard shall allow a user to obtain services simultaneously from different home environments through a single network. It is understood that it may not always be possible to access all home environments through a given network.

If the UICC is removed from the mobile terminal during a call (except for emergency calls), the call shall be terminated immediately.

# 13	Service environment

The success of UMTS may depend upon its deployment in many regions of the world. Different regions of the world are likely to have widely different market needs for wireless based telecommunications, ranging from low cost provision of POTS (to users whose mobility is to be limited) to the provision of high bit rate and multimedia services (to highly mobile users).

The following scenarios should therefore be considered:

-	use of UMTS to primarily provide wideband services (up to 2Mbps).

-	use of UMTS to provide primarily telephony.

-	use of UMTS for narrow (up to 64kbps) and wideband services.

-	use of multi standard terminals to provide integrated services.

# 14	Evolution

## 14.1	Support of pre UMTS services

The UMTS standard shall be capable of supporting pre UMTS services in a manner which is transparent to the users of these services.

UMTS shall provide some mechanisms which permit pre UMTS users to roam easily onto UMTS and access the services. See Figure 5 for clarification.

UMTS shall provide some mechanisms which permit UMTS users to roam easily onto pre-UMTS systems and access the services.

## 14.2	Provision and evolution of services within UMTS

UMTS may be introduced before a complete set of UMTS standards is available. As one of the identified priority areas, UMTS service related standards need to be available at an early stage. If a phased approach to the completion of standards is adapted then the same general service principals shall apply at an early stage.

UMTS networks shall be capable of providing a specified core set of capabilities. Responsibility for providing this core set of capabilities should lie with the serving network.

The core set of capabilities should permit UMTS home environment to offer a range of distinctive services including those which cannot be implemented on pre-UMTS systems.

UMTS shall provide some mechanism which permits UMTS users to roam easily onto pre UMTS systems, with access to a minimum set of services.

It shall be possible for the home environment to develop services with full roaming capability. It should not be necessary for users to subscribe to more than one home environment in order to receive a particular service. For example a company may market an in car navigation/location system which uses UMTS as the core network. As far as users of the navigation service are concerned, that company is their home environment.

The radio interface should not unnecessarily restrict the development of new services (within physical limitations).

The standard shall provide a mechanism which allows a UMTS terminal to be easily upgraded so that it can access new services which are within the physical limitations of the terminal. Figure 4 shows as an example the support of roaming users between GSM and UMTS.

Figure 4 Roaming Users