3GPP TSG SA WG 1 Meeting #107	S1-242035

Maastricht, The Netherlands, 19 – 23 Aug 2024	(revision of S1-24xxxx)

Source:	Nokia

pCR Title:	New Use case on assisted user feedback for IMS services

Draft Spec:	tbd

Agenda item:	4

Document for:	Discussion

Contact:	Laurent-Walter Goix <laurent-walter.goix@nokia.com>

Abstract: This pCR proposes a new use case on IMS user feedback.

1. Introduction

This pCR proposes a new use case on IMS user feedback.

2. Reason for Change

This pCR is for information to clarify the intention of the related SID on IMS user feedback by providing a sample use case and potential requirements.

3. Conclusions

None.

4. Proposal

If such use case and potential requirements are sound, it is proposed to agree the related SID and discuss this use case in details.

FIRST CHANGE (NEW TEXT)

## 5.x	Use case on assisted user feedback for IMS services

### 5.x.1	Description

Network operators offering IP Multimedia Subsystem (IMS) services have interest in enhancing their IMS services by offering innovative engagement from their users as a potential way to optimize their network, provide better services, reduce churn, or enable new revenue opportunities. For example, the collected user feedback can be used to improve network operations as well as the overall user experience and satisfaction about IMS services.

In the context of current IMS services, including data channel-based services, various scenarios could be addressed, as an opportunity for the network operator to enhance the current experience and possibly to take follow-up actions based on a received user feedback, for example:

- Voice/video service quality reporting, e.g., to help the network improve QoE and customer satisfaction, detect potential network issues in certain areas or caused by parts of the network, or verify the impact on perceived QoE and customer satisfaction in case of underlying network issues detected.

- Robocall/spam callers or messengers reporting to the network, e.g., to help marking/blocking spam numbers.

- Rating of IMS Data Channel applications in the DC app catalogue.

- Reacting to video ringback tone, for example like/dislike/subscribe.

A user feedback opportunity is defined as the ability for the user to provide feedback, without being constrained to do so. Such opportunities are provided by the network to UEs and may be defined in a way to be as objective and inambiguous as possible (e.g., via limited choices, predefined values and avoiding free-form text). Contextual information may be provided together with a feedback opportunity, to assist and/or influence user behavior (eg current number of “likes” or already reported spam calls).

A (actual) user feedback is a set of information provided/reported by a UE on behalf of a user as a result of a feedback opportunity. It contains the type of feedback (spam reporting, content like, call quality rating) together with optional contextual information (reported phone number, content URL, inappropriate content).

### 5.x.2	Pre-conditions

MNO 1 allows its subscribers to report feedback on specific IMS services, whilst ensuring the “right to be forgotten”, “privacy by default” and “complete control over their data” principles.

When deploying user feedback reporting, MNO 1 has informed its subscribers about it ahead of time allowing them to opt-in or opt-out from this service. All features are disabled (opt-out) by default and MNO 1 has clarified that user feedback is anonymous and does not contain any personal identifiable information about the sender. Free-form text feedback is not allowed neither as specific choices are provided to users depending on the feedback type.

The capability is configured as follows:

- Spam reporting / flag as inappropriate:

  - This feature is considered legitimate interest from the operator, allowing it to improve its services to its IMS customers. Subscribers can opt-in (and opt-out) at any time.

  - When providing this opportunity, MNO 1 can also inform about the number of times this phone number was already reported.

  - On an incoming call targeting one of its IMS subscribers, MNO 1 offers the opportunity for that subscriber to report whether this is a spam call.

  - On an outgoing call from one of its IMS subscribers, MNO 1 offers the opportunity for that subscriber to report whether the called party is a spam number (e.g. in case she called back a previously missed call or a wrong number that happened to be spam number).

  - On an incoming message (SMS over IMS), MNO 1 offers the possibility to report it as spam or flag the content as inappropriate.

- Call quality rating:

  - This feature is considered legitimate interest from the operator, allowing it to improve its services to its IMS customers. Subscribers can opt-in (and opt-out) at any time.

  - In case of underlying network issues, or upon random selection for some percentage of calls, at the end of the call, the subscriber is offered the opportunity to rate the call quality by using several rating levels. Rating levels could be “bad”, “sufficient”, “good”, “excellent”.

- Video ringback tone like/dislike indications:

  - When the service is subscribed, MNO 1 further allows to like/dislike the provided content.

  - When providing this opportunity, MNO 1 also informs about the number of likes/dislikes already reported about this content.

### 5.x.3	Service Flows

- Julia is receiving a call from an unknown number. On her screen, besides the incoming phone number and the icons to answer or reject the call, she can see a new icon indicating this is a potential spam with a small number below that shows how many times this was already reported.

- As the number is quite high (>100) she clicks on that icon and the call is automatically rejected, with a report sent to the operator.

- Later she receives a message from another number with some inappropriate content and a doubtful link. There is no indication apparently that this was already reported as spam, but still she can easily click on a button to report this as spam to the operator.

- On the day after she is calling her best friend to plan a night out, but although none of them is moving from their home, her best friend’s voice is clipping from time to time. At the end of the call, she can easily click on an icon to “rate” the call quality and inform the operator of this.

### 5.x.4	Post-conditions

In general, MNO 1 is able to take actions as a result of received user feedbacks.

When MNO 1 receives a very high number of spam reporting for a specific phone number, it starts blocking any further call or message from/to that number.

Bad call quality feedback is further processed within MNO 1 to understand additional potential network issues. As this feature is not enabled per default on all calls but only on suspected and/or random calls, this perceptive QoE feedback from users (e.g. from 1 to 5 stars) is used by the operator to selectively investigate potential issues besides objective measurements.

### 5.x.5	Existing features partly or fully covering the use case functionality

Parts of the functionality described above, based on users’ feedback, is already available via OTT applications for specific types of feedback (e.g. spam protection in India, application rating on Android/Apple platforms, user call quality rating in popular voice apps). However, using OTT applications is outside the control of the operator and the user experience is not standardized across applications and feedback types. Operators typically have no open access to the feedback data reported by users (e.g. the phone numbers of the reported spammers) via these apps and cannot use it to improve network and user experience. Furthermore, younger children may not be allowed in some regions to use some OTT applications (e.g. for voice calls and messaging) that do provide some types of user feedback opportunities (e.g. voice quality rating, block/report), thus being constrained to use IMS services, which currently do not provide such features.

The MCID (Malicious Communication Identification) is an existing supplementary service. However it is related to a missing or incomplete originating identification (asserted identity) that can be challenged by the AS or the UE of the called user, and not as a “spam reporting” feature per se. Similarly for Anonymous Call Rejection (ACR) supplementary service, which could be activated once spam user feedbacks are reported.

In the context of RCS, GSMA has identified “5G new calling” (RCC.71 etc) to “evolve the current voice call experience throughout all phases of a voice call: before, during and after the voice call”. Pre-call experience is also covered for video calls. However, such experiences do not specifically address the case of “user feedback” and the related requirements e.g. in terms of feedback types support, dynamic user consent and handling of feedback information.

GSMA RCC.07 has also identified a spam messaging handling, which is based either on a Spam Block function in the Service provider network, or on a local blacklist processing on the UE. A “Spam Report Message” also allows an RCS user to report a spam or inappropriate content, e.g. based on a “suspicion indicator” from the network via the CPIM “rcs.advised-action” header. A similar mechanism may be used to apply outside of RCS, as well as for other types of user feedback.

Yet in general in its white paper NG.129, GSMA has defined use cases targeting similar scenarios by leveraging IMS Data Channel (DC). Whilst it could technically enable some of these concepts, IMS DC adequateness for user feedback reporting related to messaging services needs to be studied, as it would require a DC session. Furthermore, dedicated and operator-specific DC applications to this end may have limitations in terms of roll-out, experience (e.g. to mix video ringbacks with web content to enable like/dislike) and security/privacy. For example, each DC app could induce user privacy threats and different consideration on personal data and LI.

Some regulators have also put in place some solution that partially address some types of feedback, e.g. SMS spam reporting, but they are not related to IMS services.

In general, the need to enable several user feedback types is established. However, instead of addressing such scenarios individually, which may lead to partial considerations, there is no common approach to all types of direct user feedback related to IMS services with network operators.

### 5.x.6	Potential new requirements needed to support the use case

Editor’s Note: this list of PRs is not exhaustive nor for agreement at this stage. During the study, it may be split and discussed across multiple use cases that clarify their intention.

[PR.5.x.6-1]	Subject to operator’s policy, regulatory requirements and user consent, the IMS network shall be able to provide one or more types of user feedback opportunities to an IMS UE when establishing a session, during a session and when terminating a session.

NOTE 1:	User feedback opportunities can be provided to both calling and called parties.

NOTE 2:	It is assumed that users are not constrained by the network or the UE to provide feedback as a result of receiving feedback opportunities.

[PR.5.x.6-2]	Subject to operator’s policy, regulatory requirements and user consent, the IMS network shall be able to provide one or more types of user feedback opportunities to an IMS UE for non-session-based services (e.g. when receiving a message).

[PR.5.x.6-3]	Subject to operator’s policy and regulatory requirements, the IMS network shall be able to provide contextual information together with a feedback opportunity (e.g. current number of “likes” or already reported spam calls, suspicious indicator, which could influence the user behavior).

[PR.5.x.6-4]	Subject to operator’s policy, regulatory requirements and user consent, the IMS UE shall be able to send user feedback to the network, as a result of receiving one or more user feedback opportunities from the network.

NOTE 3:	It is assumed that the network has provided the UE with the information where and how to send the feedback. Spontaneous user feedback outside of a user feedback opportunity received from the network is not supported in this release.

[PR.5.x.6-4]	Subject to operator’s and subscription policy, regulatory requirements and user consent, the IMS network shall enable users to opt-in or opt-out of all or specific types of feedback opportunities.

NOTE 4:	It is expected by regulatory requirements applying “privacy by default” approaches that subscribers are opt-out of this functionality by default.

[PR.5.x.6-5]	Subject to operator’s policy and regulatory requirements, the IMS network shall be able to support at least the following feedback types in a standard manner and may support additional feedback types:

- Spam reporting of messages and calls

- Call quality rating

- CAT/CRS like/dislike

- Data Channel applications rating

[PR.5.x.6-6]	Subject to operator’s policy, regulatory requirements and user consent, the IMS network shall enable the operator to configure the types of user feedback opportunities and reporting options for each type (e.g. scale of rating, predefined choices, optional user authorization for location information).

NOTE 5:	It is expected that standard reporting options or constraints of reported information for some feedback type (e.g. spam reporting, call quality etc) are defined.

[PR.5.x.6-7]	Subject to operator’s policy, regulatory requirements and user consent, the IMS network shall be able to inform the user about the different purposes for which they retain or process their user feedback if they contain any personal data.

NOTE 6:	By default, it is expected that user feedback does not contain any personal (nor sensitive) data unless strictly necessary, e.g. based on the feedback type.

[PR.5.x.6-8]	Subject to operator’s policy, regulatory requirements and user consent, the IMS network shall be able to delete any user feedback containing personal data.

NOTE 7: For example, deletion can be triggered upon user request, subscription termination, or automatically after a period of inactivity (unless for legitimate reasons, e.g. security or legal).

[PR.5.x.6-9]	Subject to operator’s policy and regulatory requirements, the IMS network shall be able to provide the received user feedback for LI.

Editor’s Note: It is FFS the case in which the feedback relates to a specific subscriber (e.g. spammer or inappropriate content provider), e.g. to inform him, or give the “right to be forgotten” once this is solved.

[PR.5.x.6-10]	Subject to operator’s policy and regulatory requirements, the IMS network shall not expose user feedback to authorized third parties without explicit user consent.

[PR.5.x.6-11]	Subject to operator’s policy, regulatory requirements and user consent, the IMS network shall avoid exposing personal identifiable information when exposing user feedback to authorized third parties.

NOTE 8: The IMS network can restrict user feedback information exposure to the minimum sufficient level of granularity, e.g. via anonymisation techniques.

END OF CHANGES