3GPP TSG-SA WG1 Meeting #109 	S1-250679

Athens, Greece, 17-21 February 2025	(revision of S1-250258)

Source:	InterDigital

New use case title:	Personalized Interactive Immersive Guided Tour

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.5

Document for:	Approval

Contact:	Catalina Mladin, Catalina.Mladin@Interdigital.com

Abstract: Localized (i.e., location-aware) mobile MR services provide users with information and services that are of local relevance. This use case focuses on a personalized interactive immersive guided tour service of a city.

S1-250679 updates:

- Clarifications on pose and space as SA4 XR pose, and XR space terminology

- Renamed “a-syncronicity” as “asynchronicity” which aligns with SA2 and SA4 terminology. Added to EN on the need to align these terms, as well as with “synchronicity threshold” from TS 22.261.

- Added clarifications on IMS

- Clarified PR 4 and PR 5

, a new KPI is proposed: A-synchronicity.

Asynchronicity proposed definition: tolerable negative and positive delay between one media type and another to achieve perceived synchronicity, with higher values being acceptable in passive experiences compared to active ones. This KPI is defined relative to the media types in use and indicating which media type is delayed.

Asynchronicity can be calculated based on (but not limited to) one or more of the following: network delays, transmission & synchronisation delays between data flows  for a user,  rendering delays, sampling frequencies of the media,  buffer and jitter pacer,  processing delays, mismatches between estimated and real positioning; variabilities between UEs for any given user, etc.

## ----------------  Change 1:  Use Case  - all new text   -----------------------

## 9.x	Personalized Interactive Immersive Guided Tour

### 9.x.1	Description

Localized (i.e., location-aware) mobile Mixed Reality (MR) services provide users with information and services that are of local relevance. This use case focuses on a personalized interactive immersive guided tour service of a city (Figure X-1) including outdoor/indoor configurations. The following technical aspects are supported:

Personalization:

Each visitor of the group can select one or multiple 6G connected device(s) he’s willing to wear during the guided tour based on his desired degree of immersion. He may also add, remove or change a 6G device during the guided tour. Therefore, heterogeneous 6G devices such as lightweight Augmented Reality (AR) glasses, tablets/phones with potential tethered devices, eXtended Reality (XR) video or optical see-through headsets, haptic gloves and/or wristbands, wearables, and immersive audio headsets are connected for a group of visitors during the same period.

Each visitor of the group can select a different avatar (and/or personalize a virtual representation) of the remote touristic guide application, possibly with AI techniques for an intuitive and fast avatar personalization.

The touristic information content is adapted simultaneously to the preferences of each visitor of the group (e.g., adapted overlaid text, picture, 3D virtual asset, video, audio).

Interactive immersion:

Each visitor of the group can choose a personalized degree of immersion based on the 6G device(s) he’s wearing. The rendering of different media (e.g., haptics, video, audio) needs to be spatially and timely consistent along the guided tour to maintain a good level of immersion.

An AR Spatial Computing Service is required to ensure a good level of immersion by integrating virtual content into the visitor’s real environment in a seamless manner (e.g., by managing occlusion, maintaining consistent lighting). AI techniques for real-time environment analysis are necessary especially in fast-evolving environments (e.g. crowding). Such techniques may be used for the proper placement of virtual content by ensuring that each group member has a good point of view of the virtual content.

One major requirement for the virtual content placement is the safety of the group of visitors.

The group of visitors can interact in real-time with the remote touristic guide represented by a personalized avatar for each visitor. Interactions may be conversational (i.e., asking/answering a question), gesture-based (e.g., hand shaking with a haptic glove).

AI-based services may be used to analyse the facial expression/gaze of the visitors to trigger content adaptations (e.g., when some visitors are looking to a particular object, additional touristic information related to this object is displayed).

Configurated and maintained  Quality-of-Experience (QoE):

Data flow metrics supporting the QoE for each visitor of the group need to be configured and maintained along the guided tour which may be composed of outdoor and indoor configurations, sparse and dense, crowded areas potentially leading to various fluctuations of 6G connectivity conditions (e.g., capacity/congestion).

XR data flow metrics supporting QoE (e.g., XR pose-to-render-to-photon, roundtrip interaction delay) need to be measured periodically and the XR content and/or the network parameters need to adapt accordingly to changes in 6G connectivity conditions.

- Figure 9.X.1-1: Interactive immersive guided tour

### 9.x.2	Pre-conditions

A family including adults, a teen and young child are visiting Paris. Hank is their remote touristic guide. Each member of the family has configurated a dedicated avatar of Hank for the guide tour.

They all wear different 6G connected devices. The mother wears a 6G smartphone and sometimes lightweight tethered AR glasses, the father wears an immersive audio headset, the teen wears a XR headset and sometimes a haptic glove, and the young kid wears a 6G tablet.

The family is walking in the city and stops to learn more about a place (e.g., a monument). Visits of indoor landmarks (e.g. museum) are also supported.

### 9.x.3	Service Flows

The service flows are described for an interactive immersive experience in one dedicated location of the overall guided tour of Paris.

- The family stops in front of the Palais Garnier and would like to know more. The behavior of the visitors (e.g., captured facial expressions, gestures analyzed by AI) triggers an immersive experience. Hank, the remote guide appears in a way that all family members seem to listen to a single person.

- The avatar positioning is using AR Spatial Computing Service to identify the best position within the real environment for both visualization and safety. Changes in the scene (such as a pedestrian walking toward the avatar position) may result in changing the positioning of the avatar(s).

- The AR Spatial Computing Service has identified the Palais Garnier and the guided tour application server provides XR content composed of a dance performance with volumetric video, immersive audio, haptics and virtual objects. The XR content is placed and rendered at an optimal position regarding the number of people and environment sensing.

- Each family member listens/views the content in a 6 Degrees-of-Freedom (6DoF).

- The young kid asks a question to the remote guide through his avatar. The XR content animation is paused for all the family members and the answer to the question is provided to all the family members.

- Once the dance performance is done, the family and the guide avatar enter the Palais Garnier. Each family member receives personalized touristic information (e.g., overlaid text, picture, 3D virtual asset, video, audio, haptics) linked to the real surrounding objects they discover during the walk.

- Suddenly the 6G connection conditions fluctuate (e.g., changes in capacity and/or congestion)  during a significant period of the presentation. The 6G network may adapt and/or trigger adaptations of the XR content to preserve an acceptable QoE for each family member during that period.

- When exiting the Palais Garnier, the teen replies to a question and shakes hands with the guide, and everyone sees Hank’s avatar doing it with the teenager. The teen kid wears a haptic glove to sense the hand sharing.

### 9.x.4	Post-conditions

With the advancements in interactive immersive XR technologies and 6G capabilities, visitors can acquire knowledge, touristic, and historical information of a city in a comprehensive and personalized way.

### 9.x.5	Existing features partly or fully covering the use case functionality

The following existing features can partly cover some functionalities of this use case.

Features on localized metaverse service for receiving locally relevant XR content are provided in the 3GPP SA1 Mobile Metaverse Services Study (TR 22.856) and Work (TS 22.156) documents. The placement of the XR content in the real environment to ensure an optimal viewing, interaction and safety for each group member may need additional requirements.

Features on avatar-based real-time (including IMS) communication are provided in the 3GPP SA1 Mobile Metaverse Services Study (TR 22.856) and Work (TS 22.156) documents. The time and spatial synchronizations (e.g., coherent gestures, facial expressions, displacements) of the personalized avatars for the group of participants may need additional requirements.

Features on QoE/QoS management of multi-modal flows considering low round-trip latency, and the synchronisation of different media (e.g., haptic, video, avatar, audio) for each group member, is provided in TS 22.156 (e.g., [R-5.1.1-002]). QoS management features using the Network Exposure Function (NEF) API are provided in TS 23.501, TS 23.502 and TS 23.503. A fast dynamic QoE/QoS adaptation to the fluctuations of the 6G connection conditions during a real-time communication may need additional requirements.

To support the heterogeneous 6G devices of this use case, tethered AR glasses (TR 26.998, TR 26.806) and split rendering architecture (TR 26.928, TS 26.565) have been studied with QoE/QoS support. The synchronization of heterogeneous data flows from/to various UEs within a group of participants may need additional requirements.

Performance KPIs and parameters partially covering the functionality include:

- Synchronization threshold between audio and video for VR (TS 22.261 clause 7.6.1), i.e. in the range of [125 ms to 5 ms] for audio delayed and in the range of [45 ms to 5 ms] for audio.

- Asynchronicity threshold to ensure acceptable QoE for support of haptic media in addition to other audio and video media (TS 26.854 clause 10) as follows:

  - in the range of [25 ms to 50 ms] for audio delayed and in the range of [25 ms to 12 ms] for audio advanced.

  - in the range of [15 ms to 20 ms] for video delayed and in the range of [50 ms to 30 ms] for video advanced

Additional performance specifications necessary to address this usecase include:

- A new KPI termed to ensure alignment between “synchronization threshold” and  “asynchronicity” terminology in TR 22.847, TR 22.856, TR 26.854. A definition is proposed for asynchronicity as KPI.

- New asynchronicity KPI ranges are to be specified, e.g. for: XR Pose to Video asynchronicity, XR Pose to other media asynchronicity, avatar animation to scene change asynchronicity.

### 9.x.6	Potential New Requirements needed to support the use case

The following potential new requirements are needed to support the use case:

[PR-9.x.6-1]	Subject to operator policy and privacy considerations, the 6G system (including evolved  IMS) shall provide a means to place XR content in the real environment based on specified criteria and policies (e.g. to ensure an optimal viewing, interaction and safety for each participant of an XR session).

[PR-9.x.6-2]	Subject to operator policy and privacy considerations, the 6G system (including evolved IMS) shall provide a means to synchronize data for rendering in time and in XR space (e.g., coherent gestures, facial expressions, displacements) the personalized avatars of the group of participants.

[PR-9.x.6-3]	Subject to operator policy and privacy considerations, the 6G system (including evolved IMS) shall support a mechanism, including enabling one or more authorized third party(ies) to adapt QoS dynamically based on the fluctuations of the 6G connection conditions and QoE.

[PR-9.x.6-4]	Subject to operator policy and privacy considerations, the 6G system (including evolved IMS) shall provide a means to synchronize heterogeneous data flows from/to a set of UEs associated with a single user and between UEs associated with multiple users in a multimodal communication .

[PR-9.x.6-5]	Subject to operator policy and privacy considerations, the 6G system (including IMS) shall provide a means to keep heterogeneous media flows within an acceptable asynchronicity level whether rendered on a single or multiple UEs , for each user in a multimodal communication .

KPIs

The following potential new KPIs are needed to support the use case:

Editor’s Note: Requirements for the asynchronicity KPI are FFS.

## ----------------  Change 2:  Terms   -----------------------

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Asynchronicity :

Tolerable negative and positive delay between one media type and another to achieve perceived synchronicity, with higher values being acceptable in passive experiences compared to active ones. This KPI is defined relative to the media types in use and indicating which media type is delayed.

NOTE: Asynchronicity can be calculated based on (but not limited to) one or more of the following: network delays, transmission & synchronisation delays between data flows  for a user,  rendering delays, sampling frequencies of the media,  buffer and jitter pacer,  processing delays, mismatches between estimated and real positioning; variabilities between UEs for any given user, etc.

Editor’s Note The definition above is for further refinement and alignment with “synchronization threshold” in TS 22.261 clause 6.43, and “asynchronicity” terminology in TR 22.847, TR 22.856, TR 26.854.