3GPP TSG SA WG 1 Meeting #110 	S1-252140

Fukuoka, Japan, 19-23 May 2025	(revision of S1-25XXXX)

Source:	China Mobile, Huawei, Turkcell

pCR Title:	New Use Case on Optimizing User Experience for GenAI Applications

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com

Shuang Zhang, zhang.shuang2@huawei.com

Abstract: This proposal describes a new use case on optimizing the user experience for emerging GenAI based applications.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3] 	ITU-T Recommendation Y.3090 (02/22): "Digital twin network - Requirements and architecture" (https://www.itu.int/rec/T-REC-Y.3090-202202-I).

[4] 	3GPP TS 22.101: "Service principles".

[5]	ITU-T Recommendation F.703 (11/00): "Multimedia conversational services".

[6]	European Commission: "Shaping Europe's digital future", https://ec.europa.eu/info/strategy/priorities-2019-2024/europe-fit-digital-age/shaping-europe-digital-future_en.

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

…

[X]	Doherty Threshold | Laws of UX. https://lawsofux.com/doherty-threshold/

[Y]	Hello GPT-4o | OpenAI. https://openai.com/index/hello-gpt-4o/

[Z]	I. Burgetová, O. Ryšavý and P. Matoušek, "Towards Identification of Network Applications in Encrypted Traffic," 2024 8th Cyber Security in Networking Conference (CSNet), Paris, France, 2024, pp. 213-221, doi: 10.1109/CSNet64211.2024.10851738.

* * * Second Change (new clause) * * * *

6.x  Use Case on Optimizing User Experience for GenAI Applications

6.x.1	Description

Applications based on generative AI (GenAI) capabilities are becoming popular recently, and as the research and development goes deeper, we see they are offering services from simpler text-based chatting to more complex multi-modal services like audio and video generation. Examples of these new applications and services include real-time translation, image or video recognition / visual assistant and real-life assistance (e.g., home doctor, cooking and game agent). Some of them are already commercialized, and have the potential to be a main type of traffic in the future.

Therefore, it’s important to understand the traffic characteristics of these GenAI applications, so that the 6G network could optimize the service experience of their users, while at the same time improving the efficiency of the network when transmitting these traffic.

Figure 6.x.1-1 traffic characteristic of typical GenAI applications

The figure above shows three kinds of typical GenAI applications, visual assistant, text-to-image generation, and chatbot. Their characteristics are summarized as below:

Common characteristics:

- Burst traffic: high bandwidth requirement during a short time period can be noticed in both situations, while application #1 mainly for uplink and application #2 for downlink;

- Traffic encryption: not shown in the figure, but current GenAI applications usually support end-to-end encryption of the traffic, though it ensures the security and privacy of the user, it also makes the network harder to identify the traffic type and provide proper service quality assurance to these applications;

- Various connection design: applications are using different connection designs, for example, some applications have long-lived connection for each task/session, and some applications use different connections and even different source IPs for each request, and might support parallel transmission (e.g., in image generation case).

Application #1 Visual Assistant:

- Low transmission latency requirement: not shown explicitly in the figure, the real-time service requires low end-to-end latency. For example, according to these following analysis, good interaction experience requires a round-trip latency within 400 ms [X]. And because the current GenAI algorithm usually takes time to generate content (for example, GPT-4o is able to respond to audio inputs within at least 232 ms), the transmission latency boundary is quite limited (e.g., less than 70 ms) [Y];

- Short-lived connection: also not shown explicitly, the image upload sets up a new connection each time.

Application #2 Text-to-image Generation:

- Unpredictable bandwidth requirement: the GenAI application supports multi-modal communication, and could transmit text, audio or video randomly based on user request, and therefore has no periodic traffic pattern;

- High reliability requirement: unlike audio or video processing which are tolerant to some packet loss, the “prompts” used to express user intent are required to be transmitted reliably for correct processing.

Application #3 Chatbot:

- Small packet length: Chatbot applications typically have very small packet length, including both data packets and ACK packets, which would lead to decrease of processing speed considering the current user plane design.

Table 6.x.1-1 KPIs comparison between GenAI application and short video application

| Application | Average packet size (Byte) | Uplink |  |  |  |  |  | Downlink |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Average Data Rate (Mbps) | Packet Percentage | Traffic Percentage | Burst Data Rate (Mbps) | Burst Packet Percentage | Burst Traffic Percentage | Average Data Rate (Mbps) | Burst Data Rate (Mbps) |
| short video | 1144 | 0.007 | 17% | 1% | 0.23 | 17% | 1% | 0.91 | 24 |
| short video live streaming | 921 | 0.034 | 34.7% | 1.6% | 0.23 | 38% | 1.6% | 2 | 13.2 |
| visual assistant | 838 | 1 | 49% | 48% | 67.6 | 51% | 52% | 1.2 | 69.4 |
| text-to-image generation | 987 | 0.03 | 28% | 5.6% | 0.9 | 22% | 2.7% | 0.6 | 34.1 |
| chatbot | 87 | 0.008 | 48% | 22.7% | 0.08 | 48% | 31.8% | 0.11 | 0.14 |
| Note: meanings of KPIs are as follows - average (uplink/downlink) data rate: average data rate of uplink/downlink traffic during the whole transmission - (uplink) packet percentage: number of uplink packets divided by number of all packets during the whole transmission - (uplink) traffic percentage: amount of uplink data traffic divided by amount of all data traffic during the whole transmission - burst (uplink/downlink) data rate: average data rate of uplink/downlink traffic within a burst - burst (uplink) packet percentage: average number of uplink packets within a burst divided by average number of packets within a burst - burst (uplink) traffic percentage: average amount of uplink data traffic within a burst divided by average amount of data traffic within a burst |  |  |  |  |  |  |  |  |  |

Table 6.x.1-1 also shows a statistics example of KPIs of traditional video streaming applications and typical GenAI applications. It’s obvious that GenAI applications have quite different characteristics from traditional applications, for example, noticeably high burst traffic and uplink packet/traffic percentage (more interactive).

To better serve the GenAI applications and services, it’s proposed in this use case that several enhancements could be introduced into the 6G system, including:

- UE and network coordination: UE could help network to identify the application and traffic type more accurately and quickly, and the network could help UE make better flow control

- Better QoS assurance: to ensure the GenAI application’s performance and service reliability, the network need to be aware of the burst and provide assurance for the whole burst; to deal with short-lived connections, QoS configuration needs to be done in a shorter time period; provide end-to-end service assurance by choosing an optimized path to the application server; and make QoS adjustments according to payload type, real-time network status and user feedback

6.x.2	Pre-conditions

User Andy has subscribed to operator B’s services, and uses B’s mobile network to access and utilize an AI image generation service provided by a GenAI service provider DrawYourPicture.

Operator B and GenAI service provider DrawYourPicture have a specific SLA in place that allows the Operator B to treat the traffic to/from DrawYourPicture in an optimised manner.

6.x.3	Services Flows

1.	Andy is writing a blog on his phone and would like use DrawYourPicture’s mobile application service to generate an image as part of his blog.

2. 	DrawYourPicture’s service is e2e encrypted. To ensure the service quality, Andy’s phone coordinates the predicted service requirements to the network before sending the actual request, including for example application type (GenAI/Image generation), size and duration of the burst traffic, required latency, etc.

3.	After receiving these information, the network checks the SLA with DrawYourPicture and tries to prepare an optimized session for Andy. The network may choose an optimized path to DrawYourPicture’s application server and prepare and configure a proper QoS rule utilizing, for example, historical data, AI predictions.

4.	Andy sends some prompts to DrawYourPicture with the content of the blog.

5.	To ensure the service quality, the network need to assure the whole packets related to the prompts are delivered through an optimized path for better performance and avoiding packet loss.

6.	DrawYourPicture’s application server receives the prompts and uses its GenAI algorithm to generate an image.

7.	The network prepares to send the generated image back to Andy, it coordinates with his phone about the workload status of the user plane and provide a recommended bandwidth to it.

8.	Andy checks the generated image, and thinks it doesn’t reflect all his requirements. So he decides to send a sample image searched online to DrawYourPicture, to help it generate a better image.

9.	Before sending the image, Andy’s phone coordinates the new requirements and current QoE to the network, and the network updates the QoS rules and/or routing paths for this image upload accordingly. After that, the phone uploads the image to DrawYourPicture.

10.	Andy receives a newly generated image from DrawYourPicture, and is satisfied with the result.

11. The network summarizes the whole service procedure, and keep the record for self-learning and self-optimization for providing connections to similar image generation applications in the future.

6.x.4	Post-conditions

Andy gets the ideal image for his blog post, and experience an optimized service from operator B.

Operator B fulfils Andy’s service requirements associated with DrawYourPicture service, and improves the network efficiency through coordination with Andy’s phone.

6.x.5	Existing features partly or fully covering the use case functionality

The paper [Z] describes a feasible approach on how to identify encrypted traffic, which requires: 1) the initial packet’s SNI is not encrypted, and 2) need to have packets transmitted in X seconds’ time to help identification. This mechanism is not suitable for identifying GenAI applications which have burst type of traffic.

The current mobile network system supports QoS assurance mechanism. Although the current mechanism is implemented by exchanging signalling within control plane, and it usually takes a lot of signalling exchanges and introduces additional latency, which is not suitable for short-lived connections.

The current mobile network system supports measure QoE of the user, but the analysis is not too accurate because traffic statistics is used as the only approach.

6.x.6	Potential New Requirements

[PR 6.x.6-1] 6G system shall provide efficient QoS assurance mechanisms for different types of GenAI application traffic, with the KPI summarized below:

Table 6.x.6-1: Required KPI for GenAI applications

| Traffic Type | Burst size | Max Allowed latency for a burst | Service bit rate: user-experienced data rate | Delay | Average packet size |
| --- | --- | --- | --- | --- | --- |
| Image based GenAI app | 400 KB | 50 ms | 64 Mbps | 20 ms | > 800B |
| Video based GenAI app | 20 MB | 400 ms | 400 Mbps | 20 ms | > 800B |
| chatbot | 0.5 KB | 20 ms | 200 Kbps | 30 ms | < 800B |
| Note: meanings of some KPIs: - max allowed latency for a burst: max latency for sending out the whole packets within a burst - delay: transmission latency of a packet |  |  |  |  |  |

[PR 6.x.6-2] Subject to operator’s policy, agreement with authourized 3rd party and user consent, 6G system shall be able to enable the network and UE coordination for identifying the type of application traffic (e.g., visual assistant, text-to-image generation, chatbot), and adjusting the QoS configurations accordingly.

[PR 6.x.6-3] Subject to operator’s policy, agreement with authourized 3rd party and user consent, 6G network shall be able to be aware of burst in traffic and provide mechanisms to optimize resource efficiency and assure user experience when handling such traffic.

[PR 6.x.6-4] Subject to operator’s policy and agreement with authourized 3rd party, 6G network shall be able to adjust the QoS configurations dynamically when the payload type changes (e.g., from text to image), or the pay load type is predicted/notified to be changed during a session.