3GPP TSG SA WG 1 Meeting #110 	S1-252155

19-23 May 2025, Fukuoka, Japan	(revision of S1-252025)

Source:	6G Rapporteurs

pCR Title:	Pseudo-CR Clause 7 (ISAC) Editorial Clean up

Draft Spec:	3GPP TR 22.870v0.2.1

Agenda item:	8.1.4

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Editorial clean up on ISAC Use Cases (Clause 7)

1. Introduction

While reviewing the TR, the rapporteurs found several editorial points that could impact readability and consistency.  This pCR proposed some editorial modifications to Clause 7 (ISAC) to improve readability, consistency, and usability.

2. Reason for Change

Clause 7 clean up included:

- Removed single use acronyms (PPDR, ETWS, MCS)

- Align terminology usage (primarily “operator” to “Mobile Network Operator” or “MNO” (when the term “mobile” was present) and network operator when the term “mobile” was not present, and “UAV operator” to “UAV Service Operator”)

- Added Table & Figure Titles (where missing) and aligned formats

  - Rapporteur suggested changes have been removed due to the proposed titles in existing contributions:

    - Table 7.8.6-1 (to be provided in TDOC 2043)

    - Figures 7.6.3-1 and -2 (to be provided in TDOC 2048)

- Removed extra spaces, corrected typos (e.g., UTM, not UTIM or UMT)

- Corrected/aligned capitalizations (“Use case” not “Use Case”in clause titles)

- Revervsed the proposed alignment to it is “LiDAR” not “LIDAR”)

- Aligned to “monostatic”, “bistatic”, and “multistatic” per TS 22.137 (i.e., not “bi-static” or “multi-static”)

- Aligned to “AI/ML” not “AIML”

- Aligned “automated” to “autonomous” for AMR usage – there was no sustained objections to this change on the reflector.

Note:  no changes to “third party” was made at this time.

Revisions are based on discussions in the 24 April SA1 call.

Further provisions made based on reflection discussions, to include minor editorial changes proposed by Siemens and posted on the reflector.

Table 7.5.6-1:  Moved/integrated the NOTES from outside the Table to one cell at the bottom row of the table and formatted it per TR 22.801, clause 6.6.5.6

Note:  The “NOTES” for Table 7.13.6-1 are missing and need to be supplied.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870v0.2.1.

* * * First Change * * * *

## 7.1	Use case on coordination of search and rescue missions in large disaster areas

### 7.1.1	Description

In large disaster areas, a high degree of coordination for the search and rescue operations is essential. In such situations, sensing can play an important role in providing helpful information to the Public Protection and Disaster Relief authorities and first responders by providing an integrated platform for real-time monitoring and coordination which will support the efficient allocation of resources and the facilitation of decision-making in challenging environments.

Collecting data from a disaster area(s) involves the use of special equipment and devices supporting sensing to capture real-time information about the affected area. Also, the base stations in the disaster areas, or temporarily deployable and tactical base stations, can be used for providing the required sensing services.

The collected environmental and devices sensing data can be used to generate real-time maps for the affected areas allowing officials to prioritize and to direct the rescue efforts more efficiently. Furthermore, these maps can be used to monitor the evacuation processes, as well as the situations at the evacuation centres (e.g. to detect overcrowding at an evacuation centre).

### 7.1.2	Pre-conditions

A) After the earthquake struck, a wide disaster area emerged.

Base stations can provide sensing services even after a disaster, or new base stations can be installed to ensure the required capacity and/or coverage over areas where the mobile networks are down.

Rescue teams are equipped to use special equipment and devices supporting sensing.

B) People are expected to gather at evacuation centres (rescue points).

The data on people flow is needed at evacuation centres to distribute disaster relief supplies.

### 7.1.3	Service Flows

A) Coordination of search and rescue:

- 	Rescue teams start searching using sensing devices.

- 	Sensing technology is used to analyse the environment, structure, etc. of the disaster areas and generate real-time maps.

- 	The generated maps will reflect severely affected areas, searched areas, and unsearched areas.

- 	The rescue command centre can use the real-time map information to optimize the dispatch of rescue teams.

B) People evacuate and gather at designated evacuation centres:

- 	By using sensing technologies, data about people flow can be monitored and provided to rescue teams.

- 	The rescue teams will provide the necessary relief supplies based on the level of crowding at the evacuation centres and distribute them accordingly.

- 	In the event of overcrowding at an evacuation centre, additional facilities will be installed (e.g. portable toilets, drinking water points) or a secondary evacuation centre will be set up to and people will be sent there.

### 7.1.4	Post-conditions

A) The Rescue Command Centre ensures the optimal allocation of resources for more efficient rescue operations.

B) The real-time monitoring of crowds helps in providing the required supplies. In the case of overcrowding, appropriate measures can be taken.

### 7.1.5	Existing features partly or fully covering the use case functionality

None.

### 7.1.6	Potential New Requirements needed to support the use case

[PR 7.1.6-1]	Based on operator policy, regional and/or national regulations, the 6G network in the Earthquake and Tsunami Warning System Notification Area shall provide secure mechanisms of collecting the sensing results with a specified level of accuracy that can be used to generate real-time maps.

Editor's Note: 	Definition of KPIs for the above requirements is FFS.

* * * Second Change * * * *

## 7.2	Use case on safety assistance for vulnerable pedestrians

### 7.2.1	Description

In a presentation by 5GAA at the 6G Use Case Workshop [7], a positioning accuracy of 1 meter was identified as necessary for vulnerable road users (VRUs). Additionally, their technical report [8] noted that VRUs might carry various devices to enhance pedestrian safety. Global Navigation Satellite System (GNSS)-based positioning alone is insufficient due to issues such as slow convergence, multipath in urban jungles, and susceptibility to jamming or spoofing. Therefore, rapid sensing through communication with roadside base stations and other infrastructure is essential. For instance, if a pedestrian begins moving toward a crosswalk at a red light, it is crucial to quickly detect this movement and send a warning to the UE they carry. This situation may occur even with typical pedestrians; those looking at their smartphones while waiting for the light often mistakenly interpret movements around them as an indication that the signal has changed and start walking. Furthermore, assessing whether a pedestrian can fully cross before the light turns red requires an instantaneous, quantitative understanding of their walking speed. With this data, it becomes possible to determine, based on the road width and remaining green-light time, whether the pedestrian can cross safely.

While speed detection can be achieved through side imaging, using radio waves with high rectilinearity, such as millimetre wave, to detect variations of the propagation time or to measure the Doppler shift creates a robust system that does not demand extensive computing power. Although pedestrians are the primary VRU group, the aging population has led to increasingly varied walking speeds, making it impractical to assume a single, typical walking speed. Real-time measurement in each instance is essential. In the case of wheelchair users braking can be applied more quickly and reliably without human intervention. However, applying the optimal braking force to prevent forward pitching requires knowledge of the speed before braking, making it particularly important to accurately capture low-speed movements.

### 7.2.2	Pre-conditions

Good partnership and cooperation are established between the road supervision department and Mobile Network Operator A in City B. At the request of the supervision department, suitable base stations or small cells equipped with directional antennas are selected and installed near crossings. This setup enables MNO A to continuously detect the presence of VRUs who have subscribed to safety assistance services.

### 7.2.3	Service Flows

1. Bill, who lives in City B, is now 75 years old. Although he remains active, he can no longer conceal the decline in his mobility. At his family's suggestion, he has signed up for the safety assistance service option with MNO A.

2. One summer day, after enjoying a conversation with an old friend at his favourite bar, Bill started walking home. However, his thoughts were so absorbed in replaying the conversation that he attempted to cross the street at a red light without realizing it.

3. At that moment, a warning sound emitted from his mobile phone, stopping him in his tracks. A young person standing next to him, who happened to be looking at their smartphone while waiting for the light, also noticed the warning and helped him stop.

4. Just as Bill was thanking the young person, the light changed to green without him realizing. The young person quickly crossed the street, and Bill started crossing as well, but his phone emitted another warning sound.

5. This second alert made Bill realize that, at his current walking speed, he would not be able to make it across during the current green light. He decided to wait until the light turned green again. As soon as it did, he started crossing, and this time there was no warning sound, allowing him to cross safely.

### 7.2.4	Post-conditions

Thanks to the capability to detect and evaluate the mobility of vulnerable pedestrians and to alert them via their mobile devices, the safety level of highly vulnerable individuals is enhanced. Network-based sensing provides rapid, accurate, and robust results, which can also support autonomous vehicle driving by detecting and reporting the movement of these pedestrians.

### 7.2.5	Existing features partly or fully covering the use case functionality

GNSS can estimate a pedestrian's position with an accuracy of approximately several meters while they are walking.

By taking GNSS positioning measurements at multiple locations over several seconds or ten seconds, an approximate speed can be calculated.

Positioning technology using carrier phase for centimetre-level accuracy is currently being discussed for indoor applications

### 7.2.6	Potential New Requirements needed to support the use case

[PR 7.2.6-1]	The 6G system shall be able to support the following KPIs.

Table 7.2.6-1: KPIs for safety assistance for vulnerable pedestrians

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Safety Assistance for vulnerable pedestrians | Outdoor (Crossing) | [95] | [1] | N/A | [0.5] | N/A | [0.2] | [0.5] x N/A | ≤[500] | ≤[0.1] | ≤[5] | ≤[5] |

* * * Third Change * * * *

## 7.3	Use case for high-resolution topographical maps

### 7.3.1	Description

Some 5G sensing use cases in [9] and the associated functional requirements and performance requirements in [6], capture the use of mobile network sensing measurements for services such as topographical maps, through the exposure of the sensing results to an application server connected to the mobile network. The application server may combine this sensing information with other non-3GPP sensing data e.g. light detection and ranging (LiDAR), camera data, and/or with other non-3GPP data e.g. localization, synchronisation data, to improve the detail of the application server output by enhancing the output data/product beyond that obtainable from the mobile network sensing measurement data alone.

Enhancements in 6G mobile network sensing will provide for enhanced sensing performance for example in terms of measurement accuracy, measurement latency, measurement confidence level and measurement efficiency through enhanced wave form design, enhanced integration of 6G mobile network access node connectivity and coordinated resource utilisation across triggered sensing transmitters.

6G mobile network sensing in the RAN will be provided by sensing transmitters and receivers which may be capable of enhanced sensing operation.

This use case is distinguished from previous use cases in [9] not only by the 6G mobile network sensing using more advanced and reliable sensing measurements but also due to sensing transmitters and receivers being able to provide enhanced sensing measurements with higher QoS and performing to higher KPIs that enable the application server to provide enhanced or higher-resolution topographical maps.

### 7.3.2	Pre-conditions

An application server capable of producing topological maps from mobile network sensing measurements and data and other non-mobile network sensing data is connected to a mobile network.

The mobile network supports the application server through the acquisition of sensing measurements and data from its connected 6G capable sensing transmitters and receivers, through the authorisation of the application server and the identification of suitable sensing transmitters and receivers capable of supporting the application servers request for high accuracy sensing assistance data. The application server request may include an indication of a required sensing measurement accuracy in order to ensure production of high-resolution topological maps.

### 7.3.3	Service Flows

1. An application server used for the production of topographical maps, activates a service request for the mobile network to provide sensing measurement data of an enhanced QoS, in accordance with the required measurement accuracy, to support the production of high-resolution topological map for a specific area.

2. The mobile network receives the server request and identifies for a specified area the authorised and authenticated mobile network sensing transmitters and receivers capable of providing 6G sensing measurement data to support the enhanced QoS in accordance with the application server request.

3. The mobile network configures the identified 6G sensing measurement transmitters and receivers for the enhanced sensing measurement and with the sensing mode e.g. monostatic, bistatic or multistatic, in order to obtain the sensing measurements in accordance with the requested service enhanced QoS. The 6G sensing measurement data is collected and transferred via the mobile network to the application server, for the production of high-resolution maps for the specific area and for a time period as specified.

### 7.3.4	Post-conditions

The application server processes the mobile network sensing data produced from the selected mobile network sensing transmitters and receivers, to produce high resolution topological map.

In one use of the high-resolution topographical map, the map is forwarded to vehicles capable of high levels of autonomous driving capability e.g. L3-L5 autonomy [10], where higher levels of topological map accuracy are required.

Production of high-resolution topographical maps can be used for many other services e.g. civil engineering, for outdoor pursuits, assist emergency services, road traffic control, hazard prevention, etc.

### 7.3.5	Existing features partly or fully covering the use case functionality

Combining non-3GPP generated High Definition (HD) sensing measurements with mobile network measurements e.g. [9] use case 5.28, may provide some limited ability to produce topological maps. However, the resolution, repeatability and complexity in achieving and maintaining such a solution is unreliable. In particular it addresses challenges with the reliability and repeatability in configuring, synchronising and processing data from both the mobile network sensing results with sensing results from other disparately connected HD sensors of varying sensing types, in order to produce reliable results whilst minimizing impacts on service quality and latency.

### 7.3.6	Potential New Requirements needed to support the use case

[PR 7.3.6-1] The 6G system shall be able to support the following KPIs.

Table 7.3.6-1:  KPIs for high-resolution topographical maps

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| High topology mapping | Outdoor | N/A | [0.10] | [0.10] | [0.12] | [ - ] | [0.4] | [≤ 0.6] | [50] | [≤ 0.2] | [≤10] | [<1] |
| Editor's Note: KPIs derived from FFS (reference to be confirmed). |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Fourth Change * * * *

## 7.4	Use case on low-altitude UAV supervision

### 7.4.1	Description

With the development of Uncrewed Aerial Vehicles (UAV) technologies, light and small civilian UAVs have played a great role in aerial photography, agriculture, mapping and other fields. Various commercial UAV applications are now becoming a reality. These UAVs typically operate at low altitudes and may produce a series of safety control problems, e.g. UAV illegal intrusion and UAV collision. Thus, how to realize the low-altitude UAV supervision is important and challenging in 6G.

In some scenarios, these UAVs may follow carefully planned routes that ensure efficient, regulated, and safe operation in designated airspace. To perform tasks like package delivery, surveillance, or environmental monitoring, commercial UAVs operate based on pre-determined flight paths that dictate their altitude, speed, and direction. For instance, a UAV delivering goods will follow a direct route from the dispatch location to the recipient, while a UAV assigned to environmental monitoring will travel from its station to a specific target area for data collection. Route design and optimization are crucial for safe and efficient UAV operations. These flight routes which are approved by UAV service operators, prioritize the shortest flight path, avoid restricted airspace, and ensure safe distances from obstacles such as buildings, trees, or other UAVs. Following a strict route minimizes the risk of accidents and enhances the reliability of UAV services.

Although commercial UAVs are equipped with sensors to assist with real-time navigation, these sensors can be affected by environmental conditions like lighting, weather, or geographical obstructions. Such limitations can impair a UAV's ability to accurately determine its position, altitude, or velocity, which may lead to deviations from the approved flight path. Furthermore, for services such as good delivery, where a number of UAVs are involved in a given area, UAV collision might happen due to sensor limitations and lead to safety issues.

The existing UAV tracking technologies, such as ground-based radar systems and dedicated surveillance equipment, provide route monitoring. However, the widespread deployment of these systems faces challenges due to high installation and maintenance costs and limited availability of suitable installation sites.

As illustrated in Figure 7.4.1-1 [9], UEs connected to 6G Radio Access Network (RAN) entities can be configured to support sensing operations. This configuration enhances sensing coverage, provides additional positioning reference points for sensing measurements, and improves the accuracy and reliability of sensing results. These improvements are due to the higher density of UEs compared to base stations, which increases the likelihood that some UEs are positioned closer to the UAV than the 6G RAN entities (for example, with a UAV located between two 6G RAN entities and a UE located directly beneath the UAV). Additionally, certain UEs may be placed in reflection directions that provide a larger radar cross section (RCS) for the UAV, taking into account the UAV's RCS variations in different incident/reflection angles.

NOTE:	The term “6G Radio Access Network” does not imply any architectural assumption, e.g. whether 6G core network is a new or evolved core network (compared to 5G).

The 6G sensing processing unit can gather sensing data from one or multiple network infrastructures. Upon request, the 6G mobile network operator can provide UAV flight trajectory tracking services to trusted third-party applications, such as UAV service operators, regulatory agencies, Uncrewed Aerial System Traffic Management (UTM) systems, UAV itself, etc.

Figure 7.4.1-1: Low-altitude UAV trajectory tracing by 6G system

Thus, how to realize the low-altitude UAV supervision (e.g. UAV intrusion detection, UAV trajectory tracking) is important and challenging in 6G.

Sensing is an efficient technology for object detection by means of 6G radio signals, e.g. monitoring UAV illegal flying in a specific area. 6G network could provide sensing service by collecting sensing data, transmitting sensing data, processing sensing data, storing sensing data and support sensing result exposure to the third application platform. These sensing data has the following characteristics:

-	These data may not necessarily belong to a specific UE while these data may be produced by the relationship between the network and the physical environment.

-	These data may be produced by a UE or a base station in order to complete a specific task which needs multi-dimensional cooperation.

-	These data may have a relationship with time and space which needs efficient on-demand transmission, storage and collection.

-	These data may be collected from non-3GPP sensing sources which needs unified management in the 6G network.

To address these new data characteristics, the 6G network capability of data processing needs to be extended based on the 5G network including multi-source heterogeneous data collection, efficient and guaranteed large-scale data transmission, efficient data processing within the network, and unified data storage to support multi-node cooperative sensing and multi-node information convergence.

In the low-altitude UAV supervision scenario, the 6G network could be used for sensing the UAV intrusion such as a UAV illegally flying in a restricted area including government and company regions. In this scenario, the network security and data security need to be guaranteed due to the privacy issue. Furthermore, the historical data may be used to identify an illegal UAV. In addition to the communication property of the detected UAV itself, the data management in this low-altitude sensing scenario must be operated within the network.

In a word, the 6G network will break through the "pipeline" capability and go towards to a new type of information service network by realizing diversified data collection, transmission, processing, storage and exposure within the core network.

### 7.4.2	Pre-conditions

In this use case, a UAV Service Operator/UTM provides package delivery services within an area covered by a 6G network.

Network Operator NN provides 6G sensing service for UAV flight assistance service, including illegal UAV intrusion detection, UAV flight trajectory tracing, UAV collision prediction and etc. NN can make use of wireless base station to sense the airspace within their coverage area and report the sensing information (including tracked UAV and the environment around the UAV) to the USS (Uncrewed Aerial System Service Supplier)/UTM.

The Company MM uses the USS/UTM to supervise the low-altitude UAVs and manage potential illegal intrusion into the restricted areas. MM has proved its restricted area information to the USS/UTM.

The USS/UMT uses 6G sensing service provided by the 6G Network Operator NN to detect potential UAV illegal intrusion and UAV collision prediction.

The UAV Service Operator/UTM provides specific details to the 6G Network Operator NN, including the characteristics of the UAV that will be tracked, along with details about the time and location for flight tracing. This information includes regulated flight paths as well as potential areas where the UAV might temporarily deviate from its route.

The 6G Network Operator NN can realize 6G data collection, 6G data transmission, 6G data processing, 6G data storage within the network, and provide sensing results to the USS/UMT/UAV.

### 7.4.3	Service Flows

For illegal UAV intrusion:

1.	The Company MM requests 6G sensing service for illegal UAV intrusion detection in the restricted area from the USS/UTM.

2.	The USS/UTM transmits the request to the 6G Network Operator NN.

3.	The 6G Network Operator NN selects the base stations located in the restricted area to collect and process initial sensing data by collaborative sensing. The selected 6G base station constantly collect sensing data of the location of UAVs near the restricted area and sends the sensing data to the 6G core network with a defined frequency to obtain the sensing result (i.e., the distance between the UAV and the border or motion trail).

NOTE:	The term “6G core network” does not imply any architectural assumption, e.g. whether 6G core network is a new or evolved core network (compared to 5G).

4.	The 6G Network aggregates, forwards and transmits the data generated by the base stations, and processes the data to obtain the sensing results.

5.	The 6G Network exposes the sensing result to the USS/UTM. The USS/UTM could trigger to send warning messages to the UAV or intercept the illegal UAV directly based on the sensing results.

For UAV flight trajectory tracing:

1.	When the scheduled time for tracking begins, the 6G Network Operator activates the UAV trajectory tracing service within the designated area until the tracking session ends. The UAV Service Operator then launches UAV#1, which takes off from the delivery source and heads toward the destination, following a pre-set flight path.

2.	Using radio sensing, a network of 6G base stations and connected devices (UEs) detect UAV#1 and continuously gather data on its position and movement, such as distance, velocity and angle. These metrics, also known as 3GPP sensing data, are sent to a 6G processing unit for real-time analysis.

3.	During the flight, if UAV#1 leaves the coverage range of one base station and enters a new coverage zone, the 6G system could let the old base station stop radio sensing, and switch to a new base station for sensing UAV#1 until it is out of coverage. This transition is based on the UAV's estimated position and velocity, which the 6G processing unit calculates. The network can automatically adjust the sensing operations at base stations depending on this data or based on a pre-defined time frame. In certain cases, sensing handover may be triggered to maintain continuous coverage. For instance, if the current base station's connection weakens or if another nearby base station can offer better coverage for UAV#1, the system proactively shifts the sensing function to this new station to ensure uninterrupted tracking.

4.	The 6G processing unit can aggregate sensing data from multiple sources, including RANs and UEs, to estimate UAV#1's location and velocity. Similar approach can also be applied to UAV#2 - UAV#N, in the case there are multiple UAVs providing service in the area. This real-time information is then transmitted to the UAV Service Operator and/or UTM, who monitors the UAV's trajectory.

5.	If UAV#1 - UAV#N deviate from prescribed routes, the UAV Service Operator and/or UTM receives alerts, allowing them to take corrective action and redirect the UAVs as necessary.

### 7.4.4	Post-conditions

For illegal UAV intrusion: The illegal UAVs are moved away from the restricted area. Potential privacy risks are avoided. Thanks to the wide-area and constant sensing capability of the 6G base stations, and the efficient data transmission, processing and storage by the 6G core network, the safety supervision of the low-altitude space of Company MM is improved.

NOTE:	The term “6G core network” does not imply any architectural assumption, e.g. whether 6G core network is a new or evolved core network (compared to 5G).

For UAV flight trajectory tracing: UAV#1 follows the tracked flight route to deliver the package to its destination;  any off-route movements are detected.

For UAV collision prediction: UAV#1 flies efficiently to its destination.

### 7.4.5	Existing features partly or fully covering the use case functionality

In TS 22.137 [6], there are requirements specified for 5G system on integration of sensing and communication.

### 7.4.6	Potential New Requirements needed to support the use case

[PR 7.4.6-1] Subject to operator policy, the 6G network shall enable the base station to send sensing measurement data to the core network, and enable the core network to aggregate, collect, process, and store sensing measurement data from base stations.

[PR 7.4.6-2] The 6G system should support energy-efficient sensing operations.

[PR 7.4.6-3] The 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity.

NOTE:	The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11].

* * * Fifth Change * * * *

## 7.5	Use case on environment object reconstruction

### 7.5.1	Description

Environmental object reconstruction offers significant potential with great societal and business impact, representing opportunities across wide range of sectors, from smart factories, homes, and transportation to components of/entire smart cities and countries. The 3GPP ISAC provides a non-invasive dual-functionality of communication and sensing data collection of environmental features for reconstruction. This sensing data collection is expected to unlock new services and support enhanced performance, deployment utilization, energy, and spectral efficiencies due to the nature of ISAC and its wide-area coverage and radio-signal availability. For static environmental objects, such as outdoor building/city construction and indoor machinery/wall, 3GPP wireless sensing is effective to provide a wealth of low-cost environmental information for both wide-areas and certain areas. Additionally, for monitoring dynamic targets of interest, such as vehicles, this is essential for enhancing environmental perception, especially in scenarios where non-line-of-sight (NLOS) conditions or poor visibility may limit traditional sensing methods.

The use case of environmental object reconstruction in 3GPP demands finer characterization of surrounding targeted objects based on wireless sensing signal to facilitate industrial innovation. Some applications include:

-	Smart Transportation: The impact of autonomous driving is expected to be significant in terms of safety and comfort, and potentially high efficiencies with respect to traffic, logistics, energy, etc. Such application requires advanced knowledge of moving target detection and its trajectory with detection-to-track association [12], within a multi-object tracking context, including perception of micro-features of the surrounding objects such as micro-Doppler effects, precise classification, and accurate dimensions/orientations, etc. Rough localization of these objects is insufficient for ensuring transport safety and public confidence. Therefore, environmental object reconstruction by 3GPP wireless sensing provides assistance for interpreting the complex traffic scenarios and making rapid decisions.

-	Smart City: a smart city can demand survey/digital twinning for a component/system of an entire city, which has motivated a number of smart city initiatives [13] to capture the dynamic nature of society. Initially based on cameras, these digital twin models, can be greatly enhanced by environmental object reconstruction enabled by 3GPP wireless sensing.

-	Smart Home: there are increasing interests for innovative applications of smart home, e.g. for fall detection, provided with better sensing privacy protection and reliability. Environmental object reconstruction at home is expected for finer sensing information of home objects in order to improve service stability for better consumer experience.

Typical environmental objects to be reconstructed can include the following:

Table 7.5.1-1: Representative dimensions of an environment object

| Object Type | Dimensions |
| --- | --- |
| Building | Approximately ~400 m x ~200 m x ~20 m (L x W x H), static |
| Vehicle | Truck: 13 m x 2.6 m x 3 m (L x W x H), up to 120 km/h Passenger vehicle: 5 m x 2 m x 1.6 m (L x W x H), up to 120 km/h |

The 3GPP ISAC is expected to offer more detailed characteristics of an environmental object in the spatial domain. The sensing target in the corresponding KPI table refers to a [segment/part] of the target object to be detected and/or tracked, whereas the size of each [segment/part] is comparable to corresponding spatial resolution. The percentage of missed detection/false alarm represent missed/falsely reconstruction of [segments/parts] of the object statistically. In summary, the closeness of environmental object reconstruction is categorized jointly in the sensing result by the estimation of positioning/velocity accuracy, missed detection and false alarm over [segments/parts] of a target object.

Figure 7.5.1-1: Segments/parts of a target object to be detected and/or tracked

### 7.5.2	Pre-conditions

Map Provider A is a third-party service provider which can render and virtualize detected/tracked surrounding environmental objects within its application, offer real-time 3D virtualization of objects (e.g. via 3D glasses), and display alerts for object-related warnings and information. Examples of alerts include "a car is approaching from the left street corner in 3 seconds".

Good partnership and cooperation are established between Map Provider A and Mobile Network Operator B in City C. Requested by Map Provider A for sensing service, suitable sensing transmitter and/or sensing receiver deployed in City #C are selected by MNO B to constantly sense environmental objects of City #C including buildings and vehicles. The sensing signal emitted from a sensing transmitter arrives at an environmental object whose micro-objects will reflect/diffract the signal to be detected by selected sensing receivers.

NOTE:	For the ease of elaboration, base station or UE is acting as sensing transmitter and/or sensing receiver. Other sensing modes are not excluded and can be useful for environmental object reconstruction.

Charlie is a subscriber of MNO B and also a subscriber of Map Provider A for real-time 3D virtualisation of his surrounding objects. Charlie would like to navigate unfamiliar city streets while wearing 3D glasses, which display surrounding objects in real-time, for his interest and safety.

### 7.5.3	Service Flows

Figure 7.5.3-1:  Notional environmental changes over time

1. Charlie is a tourist, who is driving a car to enjoy the view around the City #C. He would like to navigate unfamiliar city streets while wearing 3D glasses, which display surrounding objects in real-time, for her interest and safety. Map Provider A initiates a sensing request to MNO B for the latest information of environment reconstruction in the vicinity of Charlie, who is the subscriber of both Map Provider A MNO B. The sensing objects, required by Map Provider A, include vehicles, city architecture, etc., around Charlie up to a certain range.

2. MNO B selects and configures the sensing transmitters and sensing receivers, and any applicable non-3GPP sensors, to facilitate the sensing operations. Both 3GPP and non-3GPP sensing data will be collected, aggregated, and processed by MNO B's network in accordance with environment reconstruction requirements.

3. Based on environment reconstruction requirements, MNO B is expected to preform further sensing operations dedicated to each individual object, to provide fine characteristics, in order to determine object type/dimension, monitor micro-motions like car turning, etc. Corresponding sensing operations may require cooperative and dedicated sensing resource allocations across the 3GPP system for Charlie. Thereafter sensing results of each object are delivered to Map Provider A.

4. The sensing results generated by MNO B, with more detailed characteristics of environment objects, are exposed to Map Provider A. This data enables Map Provider A to update Charlie's localized map, reflecting changes such as ongoing construction of the university campus, shapes/dimensions of targets, and real-time tracking of movement direction/speed etc.

5. Charlie receives a real-time hazard display (e.g. a car is approaching from the left street corner in 3 seconds) from Map Provider A virtualized in his glass, tailored to Charlie's trajectory. This information is transmitted through MNO B's network, adhering to pre-defined latency requirements.

6. Luckily Charlie has a chance to visit City# C again. MNO B keeps detecting and tracking ongoing renovation by sensing for local university campus changes including new classroom buildings. During his visit, such renovation is completed roughly. By partnering with MNO B, Map Provider A can integrate and reconstruct the buildings into the real-time 3D map. Charlie decides to pay a visit because of his interest and unique building design.

### 7.5.4	Post-conditions

User Charlie can safely travel in a new city with enjoyable information associated with his surroundings and necessary travel advice.

### 7.5.5	Existing features partly or fully covering the use case functionality

TR 22.837 [9] has described use cases to monitor micro doppler effect by ISAC caused by chest rise/fall during sleeping. The sensing results represent the human respiration rate.

In this use case, 3GPP ISAC is expected to detect and track more comprehensive characteristics of individual environmental object, e.g. for a building, vehicle, robot, etc., with sufficient and accurate sensing information per object type.

### 7.5.6	Potential New Requirements needed to support the use case

[PR 7.5.6-1] The 6G system shall be able to provide sensing service by detecting and tracking the characteristics of individual environmental object as described in the following Table 7.5.6-1.

Table 7.5.6-1: KPIs for environment object reconstruction

| Scenario | Object Type | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Spatial resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Environment object reconstruction | Building (Note 1) | 95 | [0.5-5] (Note 2) | [0.5-5] (Note 2) | N/A | N/A | [0.5] | N/A | [10,000-600,000] | [10-60] | [1-5] | [1-5] |
|  | Vehicle (Note 1) | 95 | 0.5 | 0.5 | 1.5 | N/A | [0.5] | [0.5] | 100 | 0.1 | 5 | 5 |
| NOTE 1: A sensing target in above KPI refers to a [segment/part] of the target object to be detected and/or tracked, whereas the size of each part is comparable to corresponding spatial resolution of object reconstruction. The percentage of missed detection/false alarm represent missed/falsely reconstruction of parts of the object statistically. NOTE 2: Considering a variety of dimensions, shapes, and functionalities of urban buildings, a range of KPI values is needed to measure and provide the flexibility of reconstruction accuracy. |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE 1: a sensing target in above KPI refers to a [segment/part] of the target object to be detected and/or tracked, whereas the size of each part is comparable to corresponding spatial resolution of object reconstruction. The percentage of missed detection/false alarm represent missed/falsely reconstruction of parts of the object statistically.

NOTE 2: Considering a variety of dimensions, shapes, and functionalities of urban buildings, a range of KPI values are needed to measure and provide the flexibility of reconstruction accuracy.

* * * Sixth Change * * * *

## 7.6	Use case on road digitalization

### 7.6.1	Description

Many transportation and urban applications require real-time and citywide traffic flow estimation, which is the basis for transportation planning and traffic control. Estimated traffic flow is generally represented by the number of cyclists/vehicles/pedestrians passing a reference location per unit of time and can be virtualized by a third-party application.

In order to enable real-time navigation for automatic driving or traffic flow monitoring, it is necessary to define the upper limit of sensing object detection/tracking required by a 3GPP sensing service, since any 3GPP based sensing detection/tracking coexists with communication services and is not cost-free for operators by sharing. 3GPP system shall allocate limited but sufficient spectrum resources, power, base stations and/or road side units and etc for sensing operations. Such kind of necessity can be represented by sensing target density.

For example, the number of vehicles that needs to be simultaneously detected and tracked at a crossroad may be up to 1000 cars per [km2], including all stationary and moving vehicles occupying that cross-road temporally.

The assumptions of this use case are described in the Table 7.6.1-1.

Figure 7.6.1-1: Sensing area

Table 7.6.1-1: Assumptions

| Sensing target | Size (Length x Width x Height) | Velocity | Distance between vehicles[96] | Service area (Note 1) |
| --- | --- | --- | --- | --- |
| Vehicle | 5 m x 1.75 m x 1.5 m | Up to 5km/h(static/moving) | 3 m | Crossroad (NOTE 1) |
|  |  | Up to 120 km/h | 66 m | Highway (NOTE 2) |
| NOTE 1: One crossroad area is approximated by 1000 m dual three-lane carriageway(24 m), by assuming 250 m per direction at the crossroad. NOTE 2: Based on an assumption of dual three-lane carriageway (1000 m x 24 m). |  |  |  |  |

### 7.6.2	Pre-conditions

Good partnership and cooperation are established between Traffic Department A and Mobile Network Operator B. Traffic Department A subscribes to the 3GPP wireless sensing service from MNO B for the real-time road digitalization.

In order to monitor traffic volume constantly, MNO B has deployed and activated base stations and road side units around traffic intensive areas, such as crossroads, urban roads and highways, in order to provide a wide sensing coverage and capability to detect and track all moving objects, including vehicles, bicycles and pedestrians effectively.

NOTE:	For the ease of elaboration, the base station is acting as sensing transmitter and/or sensing receiver in this case. Other sensing modes can also be feasible and useful as well.

### 7.6.3	Service Flows

Figure 7.6.3-1:  Title is needed

Figure 7.6.3-2:  Title is needed

1.	Traffic Department A receives a lot of complaints from local residents about heavy traffic at crossroads and highways due to ongoing city construction or bad route planning.

2.	Traffic Department A requests MNO B to detect and track at least [100] sensing objects simultaneously including cyclists, pedestrians and vehicles at crossroads shown in Figure 7.6.3-1, and at least [50] sensing objects simultaneously including high-speed vehicles at overpasses shown in Figure 7.6.3-2. According to preferred sensing target density, base stations and road side units deployed by MNO B along these areas constantly monitor the road situation by appropriate sensing operations according to these requests.

3.	MNO B collects 3GPP sensing data from base stations and road side units, and non-3GPP sensing data from sensors (e.g. radar, cameras) if available. Then MNO B provides Traffic Department A with real-time traffic flow information, including vehicle volume of different directions within a unit time (e.g. every 10 minutes, every half hour), average queuing time for a vehicle/pedestrian as the crossroad, etc.

4.	For more demanding traffic monitoring, Traffic Department A requests MNO B to detect and track more objects simultaneously at given crossroads and overpasses during rush hours. MNO B adapts to the request by updating the configuration parameters of sensing operations.

5.	Based on sensing results exposed by MNO B for road digitalization, Traffic Department A figures out solutions to improve the utilization of public roads at real-time by finer spatial/temporal traffic control and traffic divergence.

### 7.6.4	Post-conditions

Thanks to the network-wide coverage, the bird's-eye-view of the base station, road digitalization is enabled by capturing real-time information of the road environment.

### 7.6.5	Existing features partly or fully covering the use cast functionality

The capability to provide secure means for a trusted third-party to request the sensing target density object tracking is not covered.

### 7.6.6	Potential New Requirements needed to support the use case

[PR 7.6.6-1]	Subject to regulation and operator's policy, 6G system shall efficiently support sensing target density requested by the third party for a given sensing service.

[PR 7.6.6-2] The 6G system shall be able to support the following KPIs:

Table 7.6.6-1:  KPIs for road digitalization

| Scenario | Service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Sensing target density （NOTE 3） | Sensing target Velocity | Max sensing service latency [s] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |
| Road digitalization | Outdoor | 95 | 0.5 (vehicle) 1（pedestrian） | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | ≤ 750 vehicles Per 1000 m x 24 m ≤100 pedestrians per 1000 m x 24 m NOTE 1 | Static/moving, up to 5 km/h | 1~5 | ≤ 0.1 | ≤5 | ≤5 |
|  | Outdoor | 95 | 3 | N/A | 3 | N/A | 1 | 1m/s | ≤85 vehicles per 1000 m x 24m NOTE 2 | Moving, up to 120 km/h | 1~5 | ≤ 0.1 | ≤5 | ≤5 |
| NOTE 1: One crossroad area is approximated by 1000 m dual three-lane carriageway (24m), by assuming 250 m per direction at the crossroad. NOTE 2: Based on an assumption of dual three-lane carriageway (1000 m x 24 m). NOTE 3: The definition of sensing target density is described in Clause 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s Note: the KPI values are FFS.

* * * Seventh Change * * * *

## 7.7	Use case on intelligent leveraging nearby entities for real-time awareness

### 7.7.1	Description

In our daily living environments, numerous situations arise where a physical entity—such as a device, vehicle, or piece of equipment—needs to be aware of events or changes occurring or anticipated to occur in the vicinity where it will soon be present. For instance, a delivery robot navigating urban streets can benefit from timely information about a traffic jam forming on its route, while an autonomous vehicle entering a parking area could benefit from real-time occupancy updates. The quality of such information, including aspects like timeliness, accuracy, and relevance, is often significantly enhanced through collaboration with nearby entities—devices or systems with similar sensing and processing capabilities—that are closer to the event or location of interest. By sharing real-time data or situational updates, these nearby entities can help each other access more accurate, immediate information, ultimately improving responses and decision-making processes in dynamic environments.

### 7.7.2	Pre-conditions

In an indoor (or outdoor) jobsite environment, there occasionally exist certain sensing area of interest, which is shared by human workers and autonomous mobile robots (AMRs) working and moving around.

AMR A (which is a UE) is able to observe and monitor an area of interest (e.g., zone X) using its 3GPP sensing feature to certain extent; however, the quality of observing and monitoring is not enough due to some reasons: for example,

because the sensing performance is poor (e.g., due to relatively long distance); and/or

because zone X is often not directly observable via sensing service used by some AMRs, such as AMR A, that need to know of the situation at the zone.

AMR B (which is a UE) is closer to zone X, the area of interest for AMR A, and is able to observe and monitor that zone more accurately or reliably.

### 7.7.3	Service Flows

Figure 7.7.3-1:  AMR A (UE) is able to obtain more reliable and accurate information on the situation that is happening and will be happening (predictive information) with the help of another UE (i.e., AMR B) in a timely manner. The nearby node that helps AMR A can be a UE (primary focus) or a base station.

AMR A is in motion along its planned trajectory in an outdoor environment.

The trajectory shares an area with human (e.g., pedestrians, human workers) where human workers walk in, stay and walk out.

While the speed of AMR A is limited to speed level A if AMR has limited information on its proximity (e.g., to an extent of tens of meters), AMR is allowed to increase the speed up to level B if AMR is able to obtain live information on its proximity and if the particular area of interest is considered clear. This specific area is inside the area covered/reachable by AMR B that is capable of joint communication and sensing.

[Case 1] AMR B (UE) is aware that certain AMR(s) is approaching and responds by sending the situational information (e.g., humans or objects are present) using 3GPP sensing service so that the AMR(s) can prepare their necessary actions (e.g., reducing speed, re-planning their routes).

[Case 2] AMR B (UE) periodically broadcasts the situational information (e.g., humans or objects are present) using 3GPP sensing service so that the AMR(s) can prepare their necessary actions (e.g., reducing speed, re-planning their routes).

NOTE:	The situational information can include: sensing result, processed data from sensing result, or inferred information from sensing result (e.g., indication that “objects” are present, or indication of the level of “crowdedness”). The situational information can be prepared by the AMR itself (e.g., on-device AI/computing) or by an edge server or a group of servers.

AMR A receives the information that AMR B has sent.

AMR A decides what to do (e.g., reduce speed, prepare to stop, re-plan the route e.g., due to “over-crowded blah…”).

AMR B sends the situational information, e.g., the number of humans or objects is increasing or decreasing in the particular area of interest, so that AMR A can manage its maneuver.

Also, this information can be utilized by the 6G network to adjust the network resources needed for stable operation of AMRs.

### 7.7.4	Post-conditions

AMR A (UE) is able to gather more accurate information about the scene of interest through the collaboration from AMR B (UE).

AMR A is able to plan ahead to reduce speed to a suitable level of deceleration (e.g., to remain balance of physical loads in forklift), to re-plan the travel route in the job site, or to prevent collision.

### 7.7.5	Existing features partly or fully covering the use case functionality

The following includes some examples:

- The 5G system supports the network exposure to an authorized third party (e.g., TS 23.501 [140], TS 23.502 [30], and TS 23.503 [141]).

- The 5G system supports the assistance to AI/ML Operations in the Application Layer (e.g., TS 23.501 [140], TS 23.502 [30], and TS 23.503 [141]).

- There are sensing related service requirements for 5G system specified in TS 22.137 [6].

- There are AI/ML related service requirements for 5G system specified in TS 22.261 [14].

- Support of QoS prediction information for intelligent physical systems (such as UE supporting V2X applications, automated mobile robots) in TS 22.186 [66] and TS 22.104 [64].

- 5G system features that support the network data analytics (predictions or statistics) services as specified in some of stage-2 specifications (e.g., TS 23.228 [142], TS 23.501 [140], TS 23.502 [30], and TS 23.503 [141]).

### 7.7.6	Potential New Requirements needed to support the use case

[PR 7.7.6-1] 6G system shall provide a means to activate and deactivate providing prediction information about the situation of a particular sensing area of interest at a particular time of interest to nearby AMRs (UEs), if requested by an application.

NOTE 1:	This requirement is intended to describe multiple AMRs (UEs) collaborating to provide useful prediction information for each other in areas shared by human and AMRs. Prediction information is made from sensing operation by nearby AMRs (UEs) of a particular AMR (UE). Examples of prediction information about the situation include the following with the location information: “a plural of people are present in zone X now” (e.g., zone shared by human workers and robots) or “a plural of people are approaching zone X shortly” (e.g., in X second), or “a human worker is approaching in zone Z shortly” (e.g., in X second).

NOTE 2:	This requirement is based on the scenario where an AMR (UE) performing detecting situational information (e.g., through sensing or other mechanisms) requests an edge server (with a time-stamp and location and orientation) to process and provide prediction information (can be “to that AMR” or “to AMR(s) already in or to be in the zone of interest).

Editor’s Note: This requirement is FFS.

[PR 7.7.6-2] 6G system shall be able to provide a means for scalable and efficient use of network resources needed for stable sensing operation of AMRs (UEs) with prediction information when a large number of AMRs (UEs) are present.

NOTE 3:	This requirement focuses on the need of optimizing network resources for integrated sensing and communication when there exist a large number of objects of interest and especially when the number is dynamically varying over time.

Editor’s Note: This requirement is FFS.

[PR 7.7.6-3] 6G system shall be able to provide a means to ensure a latency upper-bound, requested by the application, when providing prediction information based on sensing service to nearby AMRs (UEs).

NOTE 4:	The latency depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest.

* * * Eighth Change * * * *

## 7.8	Use case on detection of ships on the coast or in rivers

### 7.8.1	Description

Maritime and inland waterway transportation are the primary means of cargo transport worldwide. Over two-thirds of global international trade volume and approximately 90% of China's import and export freight are transported by sea and river routes.

Ship detection plays an important role in port management, ship traffic, maritime rescue, cargo transportation an so on. There are following important issues on the ship detection:

- Size of ship differs a lot. The radar cross section (RCS) of ships differs a lot among different types and sizes of ships, which has important effect on the capability of ships to be detected. 6G system shall be able to detect different ships with different RCS value and sizes.

- Ships are widely distributed in the river, requiring large sensing coverage area.

In summary, ship detection in nearshore waters and rivers is a crucial scenario. Since base stations are primarily land-based, large sensing coverage is required to ensure effective ship detecting at nearshore areas.

Figure 7.8.1-1: Shipping monitoring

### 7.8.2	Pre-conditions

Network Operator MM provides “smart ship detection and tracking” service with 6G system.

Network Operator MM has deployed 6G base stations along the river between City A and City B to monitor ships, collaborating with the river management department of government.

Henry subscribes to this smart ship detection and tracking service from Network Operator MM.

### 7.8.3	Service Flows

1. Henry frequently operates a ship on the river connecting City A and City B to transport goods.

2. The river management department has temporarily established a forbidden zone for ships on the river to protect the ecology of river, because the noise and disturbing made by ships would cause severe damage to creatures in the river. The information of the forbidden zone is shared to Network Operator MM.

3. The 6G base stations on the sides of the river, deployed by Network Operator MM, could transmit and receive the sensing signal. The 6G network could generate the sensing results based on the measurements of 6G base stations on the sides of the river.

4. Based on the sensing results, 6G network could sense and track Henry’s ship. Within the sensing service with high accuracy and resolution, 6G network could derive the position and velocity information of Henry’s ship.

5. When Henry is operating his ship near the forbidden zone, an alert is sent by Network Operator MM because he is too close to the forbidden zone. Then Henry notices the alert and steers his ship away from the forbidden zone.

### 7.8.4	Post-conditions

Henry delivers the goods safely and the ecology of the river is protected. Thanks for the 6G system with ship tracking and ship recognition function, any ship would not enter the forbidden zone, and the ecology of the river could be protected.

### 7.8.5	Existing features partly or fully covering the use case functionality

In TS 22.137 [6], the basic procedure of sensing has been described, including capability reporting, data collection, data transmission, data processing, and capability exposure, listed below:

- The 5G system shall be able to provide sensing service to detect, and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s).

- Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

- Subject to operator’s policy and regulation, the 5G system shall be able to provide secure means for a trusted third-party to receive sensing results with contextual information.

### 7.8.6	Potential New Requirements needed to support the use case

[PR 7.8.6-1] The 6G system shall be able to provide sensing service with larger coverage as described in the following table:

Table 7.8.6-1:  Title is needed

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Ship detection and tracking | ship tracking | 95 | ≤ [20~100], NOTE 2 | NA | NA | NA | ≤ [10~100] | NA | 5000 | 1 | 5 | 5 | Outdoor （ocean, river, coast, port...) |
| NOTE 1: The typical size (Length x Width x Height) of ship is 270 m x 30 m x 30 m according to reference [91], NOTE 2: The KPI values are referred to FFS, |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Nineth Change * * * *

## 7.9 Use case on advanced modern city transportation system

### 7.9.1 Description

New modes of transportation such as eVTOLs (Electric Vertical Take-Off and Landing) and UAVs integrate into our daily life. The eVTOL Aircraft market size is projected to grow from USD 1.2 Billion in 2023 to USD 23.4 Billion by 2030, at a CAGR of 52.0% from 2023 to 2030 [92]. At the same time, the global autonomous vehicle market size is projected to grow from USD 1,921.1 billion in 2023 to USD 13,632.4 billion by 2030, exhibiting a CAGR of 32.3% during the forecast period [93]. It is believed that the advanced modern city transportation system includes the smart traffic system and low-altitude transportation system at least.

For smart traffic system, the number of transportation vehicles will increase significantly in future. These vehicles will become increasingly intelligent and traffic system will cover a larger space from two-dimensional to three-dimensional. So, the Intelligent Transport System（ITS） in the future will be an important cornerstone for comprehensive automatic driving. That is, ITS can leverage the sensing capabilities provided by the 6G system and other non-3GPP sensing data to obtain the current positioning, velocity and identifier information of specific participants in the traffic system, and should also be capable of foreseeing potential knowledge about future traffic conditions.

The 6G system could at least obtain a series of 3GPP sensing data and non-3GPP sensing data to have a view on transportation system, knowing the environmental information and state (such as position and velocity) of each vehicle. If the 6G network could predict the evolution of environment, and position and velocity for each vehicle in future, a smarter configuration of communication service, such as RAN parameters of modulation and coding scheme , could be adopted for better communication experience. Even more, the predicted position and velocity information could be transferred to s driver to avoid an accident. For example, if the predicted position and velocity information of an approaching truck could be told to one car who has a large blind area due to occlusion of buildings, the car could proactively slow down before seeing the approaching truck to avoid accident.

In conclusion, the prediction in 6G system is requested by a service consumer and derived by 3GPP sensing data or 3GPP sensing data and non-3GPP sensing data in history, describing the state in future of sensing target(s) and/or the environment object(s),  such as position and velocity in future.

Figure 7.9.1-1: Predicted future state of vehicles

In the above modern city transportation scenarios, when sensing operations are performing, it is always required to provide joint sensing operations with different sensing quality demands for each sensing operation, involving different sensing transmitter(s) and sensing receiver(s). As an example, one car driving on the road could request simultaneous positioning tracking and environment monitoring around the car. There is following issue for the request:

- The performance of tracking and environment monitoring could be better if the sensing operations of tracking and environment monitoring collaborate with each other. For example, the monitored states of environmental objects could be used to increase the accuracy of the estimated position of the car, by comparing with the stored digital city map.

In summary, the current description in TS 22.137 [6] would not be explicitly clear that a user can have several joint sensing operations collaborating which are involving different sensing transmitter(s) and sensing receiver(s) and with different sensing quality demands for each sensing operation. One user should be able to have more than one joint sensing operations connected to itself specifically with different granularity. For every single sensing operation, it would do the necessary sensing job and then better accuracy and new information could be obtained by collaborating jointly. For example, in Figure 7.9.1-2, the sensing operation by the base station is necessary for tracking the position of the UE, and the sensing operation by the UE is necessary for environment monitoring. The accuracy of position could be better and the precise global position of the truck in front of UE could be known by processing the sensing operations jointly.

Figure 7.9.1-2: Joint sensing operations of tracking and environment monitoring

### 7.9.2	Pre-conditions

In City A, Network Operator MM has deployed base stations, as an important part of ITS, which could utilize sensing results and prediction information to manage traffic. The deployed bases stations could transmit and receive sensing signal to generate 3GPP sensing data and collect non-3GPP sensing data from other sensors linked with the base stations.

Alex is an ordinary citizen of City A, and he subscribes to the sensing service from the 6G system. His car also accesses the ITS and could receive management information from the ITS.

### 7.9.3	Service Flows

1. Alex drives his car on the road of City A. He subscribes to the road sensing service from the 6G system, requiring tracking the position of his car, and monitoring the environment ahead of his car. At the same time, Alex allows 6G system to optimize his communication experience based on the sensing results.

2. When Alex begins to drive his car on the road, 6G system starts two joint sensing operations for Alex. The sensing operation#1 is assigned to base station M to track the position of Alex’s car, with performance requirements for tracking. The sensing operation#2 is assigned to the UE on Alex’s car to monitor the environment in front of the car, with performance requirements for environment monitoring.

3. Based on the 3GPP sensing data provided by base station M working for sensing operation#1, 6G network generates sensing results including the position and velocity information of Alex’s car at the current time point.

4. Based on the 3GPP sensing data provided by UE on Alex’s car working for sensing operation#2, 6G network could keep generating the sensing results about state of the other vehicles on the front of his car.

5. The 6G system merges the sensing data from sensing operation#1 and sensing operation#2 together. With the sensing data from operation#2, which consists of states of environment objects around Alex’s car, 6G network could optimize the positioning and velocity estimated results of sensing operation#1 with higher accuracy, by comparing the environment monitoring results and digital city map. What’s more, the 6G system would be able to generate the prediction information including the position and velocity information of Alex’s car for the next 10 seconds, by analyzing the potential actions of Alex facing the current environment given the current state of Alex’s car. 6. By joint sensing operation#1 and sensing operation#2, the 6G network could provide sensing results including measured positioning information and environment information to navigation application used by Alex, which fulfills the performance requirements of tracking and environment monitoring each other simultaneously.

7. Subject to regulatory requirements, operator’s policy and user’s consent, the 6G network transfers the prediction including the predicted position of Alex’s car at the next 10 s to the ITS. The ITS could tell Alex to slow down otherwise he would have high probability of colliding with an approaching truck located at blind area of Alex.

### 7.9.4	Post-conditions

Thanks to the ITS and the 6G system, traffic flows smoothly through an intersection used to be congested. Alex enjoys a safe driving experience.

### 7.9.5	Existing features partly or fully covering the use case functionality

In TS 22.137 [6], The basic procedure of sensing has been described, including capability reporting, data collection, data transmission, data processing, and capability exposure, listed below:

- The 5G system shall be able to provide sensing service to detect, and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s).

- Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

- Subject to operator’s policy and regulation, the 5G system shall be able to provide secure means for a trusted third-party to receive sensing results with contextual information.

- Sensing result: processed 3GPP sensing data requested by a service consumer.

- Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e., position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude.

- Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e., velocity) of the target object to its true velocity.

The definition of sensing result in TS 22.137 [6] is processed 3GPP sensing data requested by a service consumer, and sensing result related definition of accuracy of positioning estimate is the closeness of the measured sensing result (i.e., position) of the target object to its true position value. The sensing result related definition of accuracy of velocity estimate is velocity the closeness of the measured sensing result (i.e., velocity) of the target object to its true velocity. The word of “measured” and “true position/velocity value” means that the sensing result in TS 22.137 [6] describes state in history which has happened, not future.

In TS 22.137 [6], there is description as follows:

The 5G system shall be able to provide 5G wireless sensing service in a target sensing service area location using sensing transmitters and sensing receivers.

From the service perspective it is not crystal clear that a user should be able to have more than one joint sensing operations connected to itself specifically with different granularity and performance requirements.

### 7.9.6	Potential New Requirements needed to support the use case

[PR 7.9.6-1] Subject to operator’s policy and regulation, the 6G network shall be able to provide prediction of location and/or velocity of sensing target(s), based on 3GPP sensing data, or 3GPP sensing data and non-3GPP sensing data.

Editor’s Note: User consent in this req. is FFS.

[PR 7.9.6-2] Subject to user consent, regulation, and operator’s policy, the 6G network shall be able to provide secure means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party.

* * * Tenth Change * * * *

## 7.10	Use case on historical sensing data transfer

### 7.10.1	Description

Sensing data transfer, such as target detection and tracking, is certainly important within sensing useful scenarios. However, we cannot ignore the historical sensing data transfer scenarios due to the lack of connection between UE and gNB or gNB and CN or due to the specified service requirements.  In reality, there are numerous areas (outdoor and indoor) not covered by 6G signal, especially in disaster conditions, such as earthquakes, hurricanes, and so on.  And meanwhile, there is such scenario that do not have high requirements for sensing services. For example, in meteorological sensing, weather changes are a relatively slow process and data does not need to be obtained in real time. Another example is public safety, where a public safety organization may request sensing data for a certain area over a certain period of time. The same is true for traffic flow statistics. Accurate reflection of traffic conditions can be achieved through data accumulation over a period of time.

For example, sensing data transmission is a challenge when sensing service occur in wilderness areas or the basement of a building. During these scenarios, sensing services can’t be performed if the sensing service relies on sensing data transfer between sensing entities and the network. There is also sensing in earthquake rescue. The complex environment after the earthquake may cause network interruption. At this time, the historical sensing data is particularly important. And same as those activities like forest exploration. In the dense forest, network signals are difficult to cover. In these cases, there is no network coverage. Historical sensing data can be transferred and analysed when network conditions permit later.

The 6G network shall support the above mentioned historical sensing data transfer and reuse scenarios. Because in this way, the network can provide more diverse types of sensing services for different needs and meet the specific needs of various fields.

### 7.10.2	Pre-conditions

Figure  7.10.2-1: Historical sensing data transfer

The third party#1, third party#2 and third party#3 subscribe sensing service from the 6G network operator.

The 6G network operator provides the sensing service in an area by integrating some sensing transmitters and receivers (base stations and sensing terminal UEs).

The sensing transmitters and receivers (base stations and sensing UEs) can support historical sensing data transfer.

### 7.10.3	Service Flows

- Third party#1 is a real time map application, it requests the 6G network to provide sensing data in a certain area for road map reconstruction. Third party#2 is a public traffic management department, and it needs to have traffic flow statistics information.  Third party#2 also requests the 6G network to provide sensing data for a certain area over a certain period of time.

- Third party#3 is Natural Disaster Relief Center and authorizes some sensing transmitters and receivers to perform sensing operation when the sensing transmitters and receivers is offline.

- Based on the requirements from third party#1, the 6G network starts sensing operation by using a set of sensing transmitters and receivers (base station or UE) in appointed area and the sensing transmitters and receivers start to generate 3GPP sensing data and transports that data to the 6G network so that the 6G network can process the sensing data and transfer the sensing result for third party#1 in time. The collected sensing data can be store in 6G network for other usage purpose.

- Based on the requirements from third party#2, if the required sensing data is already stored in 6G network (e.g., collected when providing service for third party#1), the 6G network can provide the sensing result for third party#2 using the stored sensing data. Otherwise, the 6G network can collect the required sensing data when the network load is not high.

- Suddenly, the sensing transmitters and receivers lost connection with the 6G network (due to e.g. moving to the area out of 3GPP network coverage or the network is down because of natural disaster) and sensing transmitters and receivers that are performing sensing operation for third party#3 will start sensing operation to generate sending measurement data.

- When the connection between the sensing transmitters and receivers and the 6G network is back to normal, the sensing transmitters and receivers will transport historical sensing data to the 6G network for sensing result calculation.

- The 6G network starts to collect, process historical sensing data and calculate sensing result for third party#3 and send sensing result to third party#3.

### 7.10.4	Post-conditions

With support of 6G network, third party#1 can have sensing service in certain area and third party#2 and #3 can have sensing result based on historical sensing data in certain area.

### 7.10.5	Existing features partly or fully covering the use case functionality

None.

### 7.10.6	Potential New Requirements needed to support the use case

[PR 7.10.6-1] Subject to regulatory requirements and operator’s policy, the 6G network shall support suitable means to enable sensing service based on the historical sensing data.

Editor’s Note:  Historical sensing data is FFS.

* * * Eleventh Change * * * *

## 7.11	Use case on improving the credibility of visuals using sensing

### 7.11.1	Description

AI technology for image and video generation has been improving year by year. In the future, it may become difficult to distinguish between real camera footage and generated content, which could complicate analysis in criminal investigations and other areas. Therefore, it is beneficial to use sensing technology to implement mechanisms for determination of whether the footage/video is real or generated.

This use case proposes to use sensing and AI for verification of the credibility of visuals in the following steps:

- A sensing signal is transmitted from the sensing transmitter and received by the sensing receiver after being reflected by an object (assuming that the sensing transmitter and receiver are base stations or UE devices).

- The collected data is processed within the core network by using AI, to apply the required privacy measures and to filter out the usable data which can be shared/sent to a trusted 3rd party.

- Further data processing about the object's shape and speed is obtained through analysis performed by a third party.

### 7.11.2	Pre-conditions

- the police are undertaking criminal investigation at an incident location and need to verify some received unofficial footage and video,

- at the incident location, UEs and BSs supporting sensing, served by Network Operator A and has collected sensing data.

### 7.11.3	Service flows

- The police request footage and video verification from the incident location by using the collected sensing data by Network Operator A

- The collected sensing data being processed and filtered by Network Operator A, the result is sent to Police (or trusted 3rd party) to analyse the data and confirm the authenticity and the credibility of the footage and video.

- analysed results sent back confirming whether the footage and video are consistent with the actual scene or not.

### 7.11.4	Post conditions

The credibility of the footage and videos is enhanced, leading to more efficient analysis for the police criminal investigation

### 7.11.5	Existing features partly or fully covering the use cases functionality

TS 22.137 [6], defines under clause 5.2.4 the following requirements for Security and privacy:

The 5G system shall support encryption, integrity protection, privacy of the 3GPP sensing data, non-3GPP sensing data and sensing results, to protect the data inside the 5G system.

The 5G system shall provide a mechanism to protect identifiable information that can be derived from the 3GPP sensing data from eavesdropping.

These requirements related to privacy protection shall be also applied during sensing data processing and storage.

### 7.11.6	Potential New Requirements needed to support the use case

[PR 7.11.6-1]	Subject to regulation and operator policy, the 6G network shall ensure that only authorised entities are able to access the stored collected sensing data and results.

[PR 7.11.6-2]	Subject to regulation, operator policy and user consent, the 6G system shall support mechanisms to protect data privacy during the processing of sensing data.

NOTE:	The processing of sensing data is compliant with applicable regulations, and other pre-defined service criteria.

* * * Twelfth Change * * * *

## 7.12	Use case on enhanced XR user navigation

### 7.12.1	Description

While immersed in an XR application, a user cannot see the real environment or can only see the area covered by the camera or eyes' field of view. The XR application needs to ensure the user's safety by alerting them to avoid collisions with static and dynamic objects in the surroundings.

Current XR applications use visual detection, based on the device cameras, to estimate a map of the environment (static objects) and dynamically detect approaching objects, people, or animals, alerting the user in case of a possible collision.

The visual detection is limited to the cameras' or eyes' field of view (typically limited to the front area); hence, it would miss dynamic items that, for example, approach the user from the back. Moreover, the cameras' visual detection may have reduced performance in dark or overexposed lighting conditions; reflecting or transparent surfaces may also pose challenges to the visual detection, thus hindering user safety.

Wireless sensing can be used to provide additional detection, complementing the visual detection, and hence providing a more robust operation that is independent of the camera's field of view and lighting conditions.

### 7.12.2	Pre-conditions

A user is using an XR application, a VR training application in a factory workshop, with the user wearing an immersive Head Mounted Display (HMD) connected to a Non-Public Network which provides data connectivity and the Sensing Service.

The user’s surroundings are not physically bounded, hence e.g. Automated Guided Vehicles (AGV) in a factory. For this sensing service, it is expected that the sensing area around the user, and the detection should be continuous, highly reliable and low latency. Since the user/UE is the intended consumer of the Sensing Results, processing of the sensing data at the UE would ensure low latency for the sensing results.

### 7.12.3	Service Flows

- First the user enables the XR application, maps the area by using the HMD cameras and starts using the XR application. Then, the application invokes the sensing service to enable RF sensing for detecting moving objects in the user surroundings.

- Then, the network operator confirms that this user is authorized for the sensing service, configures the sensing service.

- The UE then performs sensing operations, acting as both the sensing transmitter and receiver (i.e. monostatic sensing) or the UE may act as the sensing receiver by receiving sensing signals from surrounding sensing transmitters (i.e. bistatic sensing).

- The UE processes the RF sensing data alone or jointly with the non-3GPP sensing data obtained from sensors in the HMD (e.g. visual detection data from cameras, accelerometer data, etc). During processing, the UE may leverage the info. provided by the network in deriving the sensing results.

- Also, the processing of the RF sensing data (and/or joint processing with non-3GPP sensing data) may leverage AI/ML algorithms for the detection. For example, the AI/ML model training may be performed offline in the network and the appropriate compressed model downloaded to the UE ahead of time. With the sensing data and AI/ML techniques, the sensing results are derived which in this case would be that the objects in the environment around the user are accurately detected even when out of view of the camera.

- An AGV approaching the user from the back is detected by the sensing service and notifies the XR application, which alerts the user to take the appropriate safety actions.

### 7.12.4	Post-conditions

Thanks to the Sensing Service, the user was alerted of a potential hazard not detected by the visual system, resulting in improved safety for the user.

### 7.12.5	Existing features partly or fully covering the use case functionality

Sensing: Release 19 ISAC requirements relating clauses 5.2.1 to 5.2.5 of TS 22.137 [6] can be leveraged for this service. Examples of existing requirements that apply include:

Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

Subject to user consent, regulation, and operator’s policy, the 5G system should support the joint processing of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

Positioning: The processing of the sensing data, especially in a bistatic setup, may require knowledge of the sensing transmitter and receiver positions. The positioning information can be derived from the 3GPP positioning service with requirements in clause 6.27.2 and high precision positioning in clause 7.3.2 of TS 22.261 [14].

### 7.12.6  Potential New Requirements needed to support the use case

[PR 7.2.6-2] The 6G system shall support the following KPIs:

Table 7.12.6-1: 	Performance requirements of sensing results for enhanced XR navigation

| Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing Resolution |  | Max sensing service latency [ms] | Refreshing Rate [s] | Missed Detection [%] | False Alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| 10 m2 | 99.9 (NOTE 2) | ≤ 0.5 (NOTE 1) | 0.5 (NOTE 1) | 0.5 | N/A | 0.1- 1 (NOTE 1,2) | 0.5 (NOTE 1) | ≤ 100 | 0.1 - 1 (NOTE 2) | 1% | 1% |
| NOTE 1: This positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the user. NOTE 2: The KPIs are based on the Hazard Prevention in Industrial Environments for “Prediction of workers’ and machines’ actions” and “Detection of worker’s location” in [94]. |  |  |  |  |  |  |  |  |  |  |  |

* * * Thirteenth Change * * * *

## 7.13	Use case on collaborative robots using digital twinning

### 7.13.1	Description

NOTE:	The use case described in this clause is based on [95].

Over recent years, Digital Twinning became significantly more important in complex scenarios where a digital representation of the physical world, in combination with processes to be automated in that physical world, allows to organise the sheer amount of data. Such capabilities eventually enable closed-loop automations where the Digital Twin (DT) application takes full control of the actions in the physical world without human interactions. The present use case focuses on such scenario where robots and assembly line modules collaboratively aim to solve a joint task, e.g. finding and transporting an object/person to a different location, investigating an area of interest or delivery of goods, which is conducted in an autonomous and closed-loop manner. While existing autonomous robots have a range of built-in sensors, e.g. video, LiDAR, radar, it is assumed in this use case that sensing based on signals from the 6G system  is mainly used to expose sensing results to the DT application for precise mapping of the environment to coordinate movements and actions of all robots on the ground. It is assumed that all robots act as a traditional U) and that they can reach the DT over the 6G network.

Figure 7.13.1-1 illustrates the proposed scenario. A DT application coordinates three robots on the ground, UE1, UE2 and UE3, and receives raw sensing data from their built-in sensors as well as the sensing results from the 6G system. The 6G system obtains the sensing results by performing a range of sensing operations which can be implemented using Monostatic, Bistatic or Multistatic sensing as described in [6]. These modalities of implementing sensing operations are differentiated by the number and relative deployment of the sensing transmitters and sensing receivers, as well as deployment relative to network entities (e.g. UEs, Transmitter Receiver Points (TRPs)). The goal is for the two robots (UE1 and UE2) to lift up a large steel bar from the ground and allow the third robot (UE3) to drill a hole in the wall through existing carved out holes in the steel bar. Upon completion, UE1 pushes bolts into the hole and tightens the bar with nuts. All the described actions are fully coordinated by a DT application with the robots only executing action commands sent by the DT application.

Figure 7.13.1-1: Illustration of the use case on collaborative robots based on digital twinning

In modern flexible factories, the assembly line frequently changes to accommodate the production of various products (e.g., different car parts, different car models). Therefore, it is crucial for the DT application to be aware of these changes. While 6G system-based sensing can help acquire this information, the sensing required for robot control differs from that needed to detect changes in the assembly lines. The former may necessitate a precision sensing processing capable of pinpointing the location of robots and the objects they handle, whereas the latter may require a holistic sensing processing capable of detecting large-scale changes in the assembly line and the factory floor. When distinct KPIs are used in relation to sensing data processing, interactions and synchronization are crucial for achieving flexible applications for factories. For instance, depending on the results of the holistic sensing processing, the DT application may need to update the parameters of the sensing operations and processing.

### 7.13.2	Pre-conditions

The DT application has been granted access to the exposure service for sensing results from the 6G network.

Furthermore, the following pre-conditions exist:

- Each robot acts as a UE and has the appropriate identification to attach to the 6G network

- Both TRPs are connected to the core network in the 6G system

- All UEs have successfully registered to the network

- UE1 and UE3 are attached via TRP1 and UE2 via TRP2

- The deployment provides Monostatic, Bistatic and Multistatic sensing operations in all its UE/TRP combinations.

- Any non-public information in this use case is removed before sensing results are exposed to the DT application.

### 7.13.3	Service Flows

The DT application requires a range of sensing results to control the collaborative task accurately. This includes a) the information about the environment where the robots will perform their collaborative task, b) location and orientation of the robots within the environment, and c) the location and orientation of the target object, i.e. the steel bar. To achieve this, it is foreseen that the DT application requests separate sensing services for a), b) and c). The sensing service coordinates and executes the requests and coordinates the UEs and TRPs involved, their sensing processing, sensing mode as well as KPI requirements. The sensing data from UEs and TRPs is used to produce the sensing results expected by the DT application. This includes the identification of objects, their categorisation as well as any high-layer abstraction of sensing data, e.g. 6 Degrees of Freedom (position and orientation).

### 7.13.4	Post-conditions

Based on the sensing results exposed from the 6G network, the DT application was successfully able to coordinate the robots to lift up the steel bar, drill the holes in the wall and mount the steel bar securely against the wall.

### 7.13.5	Existing features partly or fully covering the use case functionality

None.

### 7.13.6	Potential New Requirements needed to support the use case

New functional requirements for this use case are:

[PR 7.13.6-1] Subject to operator policies, the 6G System shall provide mechanisms for configuring a sensing operation with a single or multiple sensing modes from all sensing modes supported (e.g. bistatic, monostatic, multistatic).

Editor’s Note: It is FFS whether the PR above is already provided in 5G.

[PR 7.13.6-2] The 6G Network shall provide suitable mechanisms for the exposure of sensing results in a synchronised manner with other types of traffic (e.g. audio, video, haptics) to the sensing service consumer.

New performance requirements for this use case are:

[PR 7.13.6-3]	The 6G system shall be able to support the following KPIs:

Table 7.13.6-1: Proposed KPIs for collaborative robots using digital twinning

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Collaborative Robots (NOTE 1) | Indoor/Outdoor | [95] | [≤ 0.1] (NOTE 2) | [≤ 0.1] (NOTE 2) |  |  |  |  | N/A (NOTE 3) |  |  |  |

Editor’s Note: For “Accuracy of positioning estimate by sensing”, orientation accuracy across the x-y-z coordinates is FFS and may result in additional KPIs. For “Max sensing service latency” it is FFS whether/how algorithmatic offset impacts on values are to be captured in the table.

NOTES for the above table are missing and need to be provided.

* * * End of Changes * * * *