3GPP TSG SA WG 1 Meeting #112 Ad Hoc - e	S1-260077r1

12-16 January 2026, electronic meeting	(revision of S1-254477)

Source:	Ericsson

pCR Title:	Solving two editor’s notes in 6.38 AI for disability

Draft Spec:	3GPP TR 22.870 1.0.1

Agenda item:	1.3

Document for:	Agreement

Contact:	Daniel.lonnblad at ericsson.com

Abstract: This pCR introduce some discussion and explanation about the KPI’s in the table with the intent to solve the Editors notes.

1. Introduction

Two editors notes where added to the use case AI for disability support, Intention is to add some extra information how the KPI’s are estimated.

2. Reason for Change

The first Editor’s Note “The value of Application round trip latency are for FFS”.

The intended use case covers two possible enhancements, audio enhancements and video enhancements. Further more within these groups you can see different enhancements, but we proposae to split the kpi table into these two groups. This would clarify the kpi table specifically for the bit rate and latency.

The latency has been further clarified, it is divided into a communication part that is mainly valid for the video enhancements and a compute latency that needs to be fullfilled to give the expected experience.

If voice should be enhanced in a voice call the overall latency of the voice call including the enhancement should not be larger than 300 to 500ms to not break the overeall experience of the voice call. 300-500 ms can be seen as a upper bound for voice. This needs to take both the latency in the network (roundtrip) and the latency for the enhancement into acocunt. Estimating network latency to be 10ms to 50 ms in one direction leaving 200-400ms for the voice enhancement application.

If it instead is disability enhancement application to explain surrondings from a video feed somewhat higher latency can be expected. Latencies up to 1000ms could be accepted dependent on the actual type of enhancements. This then needs to cover video uplink, application and enahancement information of the content of the video in down stream.

For the second Editor’s Note “ The value of Bit rate uplink are FFS”

The UL data rate will be dependent on the type of support service the user intends to use. If a video stream should be analysed you need to  use  at least two greyscale cameras with 60fps to capture all the movements. This would result in 1920x1080x16x60 giving 2Gpbs uncompressed, compressed dependent on compression rate but 1-50Mbps is expected.

3. Conclusions

The two Editor Notes can be removed

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

We also suggest to remove the now empty box at the top of the table. It is not possible to do with change bars.

* * * First Change * * * *

6.38.6	Potential New Requirements needed to support the use case

The following requirement for AI for Disability support are derived from the use case above.

[PR 6.38.6-1] Based on operator policy, local regulation and subscriber permission, the 6G system should enable the IMS services enhanced by AI capability, to enhance the audio and video stream for disability support.

NOTE:	The disability support can be e.g. describing and explaining the surroundings, audio aid (modification of the audio stream i.e. enhancements, clarifications, translations, speech to text, text to speech, etc.), visual aid (modification of the visual stream).

[PR 6.38.6-2] Based on operator policy, local regulation and subscriber permission, the 6G system may be able to enhance the IMS emergency communication service with AI capability, to enhance the audio and video stream for disability support.

[PR 6.38.6-3] The 6G system, including IMS, shall support the following performance requirements for AI Disability support:

Table 6.38.6-1: Performance requirements for AI Disability support

| Profile | Bit rate downlink | Bit rate uplink |  | End to End Communication latency | AI inference/compute latency | Total latency | Communication service availability | Overall user density | Activity factor | UE speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AI for Disability support Audio enhancements (NOTE 4) (NOTE 7) | [1-2] Mbit/s | N/A |  | N/A | 200-400ms | 400-500 ms (NOTE 6) | Dense Urban [99.9]% Urban [99.9]% Rural [99]% | Dense Urban [2 500] / km2 Urban [1 000] / km2 Rural [10] / km2 (NOTE 2) | [50]% | 5 km/h |
| AI for Disability support Video enhancements (NOTE 5) (NOTE 7) | [1-2] Mbit/s | [1-50] Mbit/s (NOTE 1) |  | [10-50] ms | [400- 800] ms for video support (NOTE 3) | 900ms (NOTE 8) | Dense Urban [99.9]% Urban [99.9]% Rural [99]% | Dense Urban [2 500] / km2 Urban [1 000] / km2 Rural [10] / km2 (NOTE 2) | [50]% | 5 km/h |
| NOTE 1: Compression ratio may change dependent on use case, Assumption is two greyscale cameras, 60fps giving 2Gbps uncompressed. NOTE 2: It is assumed that 10% of all users are using AI for Disability support NOTE 3: Video support can be “Fast” (e.g., instance sign language interpretation, scene interpretation) and “Slow” (e.g., orientation, object-recognition). NOTE 4: The KPIs related to the traffic related to this use case are in addition to the KPIs associated to the ongoing IMS session. NOTE 5: The video enhancement service may not use IMS video call NOTE 6: The overall one-way latency needs to be below 400-500 ms for user acceptance as can be seen in [Y] NOTE 7: The KPIs are applicable to both indoor and outdoor scenarios. NOTE 8: A total latency for communication and compute / AI inference should be kept below 900ms[X] |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

2	References

[X] Delay Threshold for Social Interaction in Volumetric eXtended Reality, Carlos Cortes et al., ACM, 2024.

[Y] ITU-T Recommendation G.114 (05/2003), One-way transmission time.