3GPP TSG-SA WG1 Meeting #112 	S1-254261

Dallas, USA, 17-21 November 2025	(revision of S1-25xxxx)

Source:	Apple

Title:	Definition of sensing KPIs

Agenda item:	8.1.4.2 Integrated Sensing and Communications

Document for:	Discussion

Contact:	Mona Mustapha at apple dot com

Abstract: This contribution highlights that the sensing KPIs have not been defined and suggests that the terms used in TS 22.137 are adopted.

1. Discussion

TS 22.137 includes a number of definitions for the sensing KPI tables:

Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e., position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude.

-	Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e., velocity) of the target object to its true velocity.

-	Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy.

-	Sensing Resolution describes the minimum difference in the measured magnitude of target objects (e.g., range, velocity) to be allowed to detect objects in different magnitude.

-	Missed detection probability describes the conditional probability of not detecting the presence of target object/environment when the target object/environment is present. This probability is denoted by the ratio of the number of events falsely identified as negative, over the total number of events with a positive state. It applies only to binary sensing results.

NOTE 1:	An event with a positive state refers to the presence of the characteristics of a target object or environment, including the event falsely identified as being negative and truly identified as being positive.

-	False alarm probability describes the conditional probability of falsely detecting the presence of target object/environment when the target object/environment is not present. This probability is denoted by the ratio of the number of events falsely identified as being positive, over the total number of events with a negative state. It applies only to binary sensing results.

NOTE 2:	An event with a negative state refers to the non-presence of the characteristics of a target object or environment, including the event falsely identified as being positive and truly identified as being negative.

-	Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface.

-	Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results.

2. Proposal

It is proposed that these definitions would also apply for the KPI tables in TR 22.870.

The definitions could be included e.g. in the consolidation clause for Integrated Sensing and Communications