3GPP TSG-SA WG1 Meeting #112 	S1-254217r1

Dallas, USA 17-21 November 2025	(formerly S1-253356)

Source:	Xiaomi

pCR Title:	pCR for New use case UE sensor groups

Draft Spec:	TR 22.870 v0.4.1

Agenda item:	8.1.4.2

Document for:	Approval

Contact:	Gordon Young <gordonpetery@xiaomi.com

R1#112 update

The PRs 1 and 2 are updated to express the sensors utilization is independent and clarify that it is not their respective control that is intended, since the sensor may be non-3GPP sensor and therefore control is outside of the 3GPP sensing service.

To clarify PR1 – discovery of sensors, to remove user consent,

general concerns with this use case from E///,

sensing entity to be clarified.

Abstract: this paper proposes a new use case for a sensor group for a single UE, wherein a vehicle has connected two independently operated sensors, each providing a different sensing view of its surrounding environment.

<FIRST CHANGE>

## 7.X	Autonomous driving service with a UE using multiple sensors

### 7.X.1	Description

Vehicles equipped with a UE supporting 3GPP sensing service may have one or more sensors installed in a distributed manner which may or may not be a 3GPP sensor e.g. camera or LiDAR, in order to support better coverage of the vehicle’s sensed local environment.

A UE when registering for a sensing service will indicate to the 6G network the capabilities of its available sensors independently, so that the 6G system is able to determine whether to select it for sensing operation for a particular target sensing service area. In some instances when the UE has more than one sensor, each sensor can be selected for sensing operation on an individual basis, and each may have different view of its environment, including the target area.

Figure 7.X.3-1. Truck with two sensors, front and rear facing

In some instances the vehicle may be able to pre-process the respective sensors’ measurement data creating a fused sensing result for reporting from the UE. The UE reports the respective sensor measurement data for each sensor or fused measurement result in accordance with the sensing service configuration.

The view of the sensing service target area (TSSA) for a sensor is directly related to the sensor’s position, the antenna aperture and the orientation of the sensor relative to the true position of the UE. In the case of a vehicle with 2 sensors each sensor may be positioned on the vehicle so that it provides distinct or different view from the vehicle, of the target sensing service area, e.g. on a car or transport vehicle sensors may provide forward facing and rear facing views.

For road users operating in a TSSA the activation of a UE sensor is determined by the 6G network controlling the authorized sensing service, with the acceptance of the UE connected to the sensors.

In this case the sensors could be considered as a sensor group, wherein they have a known proximity to one UE which is responsible for managing the sensors, and communicating with the sensing service, as in [9] section 5.19. However in the scenario presented here, the sensors are connected to one UE and not each associated with separate UEs with individual sensors.

During 6G system discovery of sensing entities (where a sensing entity is an entity capable of acting as a sensing receiver or sensing transmitter, or both, as configured for a sensing service), to support a sensing service the MNO P sensing service identifies available sensing capable UEs capable of supporting sensing for the TSSA. As part of this process the 6G system  should be aware that a UE/vehicle has more than one sensor, potentially with different views of the target sensing service area and the 6G system may select one or more of the sensors to support the sensing service.

In order to ensure network resource efficiency the 6G system should be able to select one or more of the sensors individually for reporting . For example in some instances one of the sensors may not be facing the target sensing service area. Alternatively one of the sensors, e.g. acting as a sensing receiver, may not be facing in the direction of a sensing transmitter (bistatic configuration) in order to provide meaningful sensing measurement data.

In this case the sensor connections to the UE are considered to be ideal (no loss) and can be wireless or wired.

As described in this scenario where a vehicle with two sensors moves through a target sensing service area, depending on the 6G network configuration, the UE may be configured to utilise one or other or both of the available sensors independently, and report the measurement data to the 6G network individually. This sensing configuration may relate to the direction of travel and be changed as the vehicle passes across a target sensing service area. At some instance the vehicle could be configured to utilise both connected sensors, and additionally report measurement data for both sensors separately, or as an alternative if sensing data fusion is supported at the vehicle UE, it may be configured to report a fused sensing report combining sensing measurements received independently from each sensor by the UE.

As it moves away from the target sensing service area, the 6G network may decide to change the sensors utilisation independently e.g. from one, or both, of the vehicle sensors in order to maintain an overall efficient sensing service configuration, e.g. considering other available sensing entities (e.g. sensing receivers or sensing transmitters) and the resources at its disposal for sensing the target sensing service area.

### 7.X.2	Pre-conditions

Vehicles A, B, C and D are registered subscribers to a third party autonomous driver service X which provides V2X services and sensing services of different levels of autonomy or functionality to subscribers in a sensing service area e.g. mapping, warning and sensing services to aid drivers of vehicles A, B, C and D.

Autonomous driving service X has an agreement with MNO P for provision of sensing service and V2X services including localisation and user privacy for reported service results.

Vehicles A, B, C and D are registered with MNO P and are in network coverage and connected.

Vehicle A has 2 sensors fitted 1 front and 1 rear. Service X and MNO P’s 6G sensing service control function is aware of the two sensors and vehicle a has provided this capability information to sensing service of MNO P. Initially only the forward sensor on vehicle A is utilized for the sensing service and vehicle A is reporting sensing measurement information from this forward sensor for the configured target sensing service area.

Vehicle D is stationary.

### 7.X.3 	Service Flows

- The vehicles A, B, C and D are travelling in a rural area and approaching a T-junction. Traffic is approaching from both the left and right of the junction and vehicles A and B need to turn into the road passing the T-junction safely, with vehicle A turning left and vehicle B turning right (traffic drives on the left of the road in this scenario).

- The visibility at the junction is poor due to high hedges and poor light etc., and vehicles A and B are reliant on the service X for safe passage, and avoiding of traffic.

Figure 7.X.3-2 Vehicles approaching T-junction and after vehicle A has turned with both sensors active

- Approaching the junction the sensing service X, aware of the sensing capabilities of vehicle A, e.g. vehicle A supports both a forward facing and a rear facing sensor, determines that it wishes to utilise the rear facing sensor on vehicle A in addition to the forward facing sensor, so that it can have aview towards vehicle C, after vehicle A has entered the same lane of traffic. Also the rear view will give a clear view of vehicle D as vehicle A passes it, e.g. capturing its size and confirming if it starts moving. Hence informing the sensing service of the status of vehicle D and it potentially causing a problem for vehicle B which is intending to turn in the opposite direction into the same lane as vehicle D.

- Vehicle A provides sensor measurement data for both sensors to the MNO P sensing service being operated and delivered to service provider X.

- The MNO P sensing service with knowledge regarding the rear sensor orientation (field of view) is able to fuse all the sensing data from vehicle A with the other received sensing data from vehicles A, B, C and D to create the required sensing result for the TSSA.

- Vehicle D moves off, gaining speed and approaches the junction.

- MNO P forwards the sensing result in accordance with the sensing service requirements to service provider X.

- Sensing service provider X delivers autonomous driving service combined with sensing results to vehicles A, B, C and D.

### 7.X.4 	Post Conditions

Sensing service provider detects and determines the size of vehicle D early as vehicle A approaches and passes it, and is able to ensure vehicles A and B navigate the junction safely.

### 7.X.5	Existing features partly or fully covering the use case functionality

The following requirements exist for sensing entity configuration, but do not consider a UE with more than one available sensor for independent operation, considering their respective view of the target sensing service area.

TS 22.137 [6] section 5.1.

5.1 General

…

It is important to consider energy efficient sensing operations which can include temporarily disabling sensing transmitters and receivers that are not involved in sensing and communication operations or adjusting the sensing operation parameters (e.g., sensing frequency) to minimize energy consumption. …

5.2 Requirements

5.2.1 General

…

The 5G system shall be able to provide 5G wireless sensing service in a target sensing service area location using sensing transmitters and sensing receivers.

Subject to regulation and operator policy, the 5G network shall be able to activate, configure, and deactivate 5G wireless sensing based on parameters such as location and network conditions (e.g., network load). …

The location of the UE in the scope of this use case does not account for the distributed sensors with distinct and separate views. The location of the UE in itself is not conclusive for assessment of whether the UE and its associated sensing receivers and sensing transmitters should be activated, configured or deactivated.

5.2.2	Configuration and authorization

Subject to regulation and operator’s policies, the 5G network shall be able to configure and/or authorize or revoke authorization of sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service.

NOTE 1:	Such configuration and authorization can be based on sensing transmitter or sensing receiver location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data, etc.

NOTE 2:	Such configuration and authorization can also include the selection of multiple sensing transmitters/receivers for 5G wireless sensing services. …

In addition relating to sensing groups or use of multiple sensors in proximity to a UE, TR 22.837[9] section 5.19 specified the need to identify a sensing group (of sensing entities) in proximity to a UE requesting a sensing service. This enabled the establishment of a distributed group of sensors for a target sensing service area. Groups are established based on group members locations known to the 6G system being in proximity to a UE requesting a sensing service and it is only captured for the handling of delivery of a sensing service to a UE requesting the sensing service within the sensing group. This was not carried on to normative requirement for 5GA sensing.

[9] CPR 7.1.3-2	Subject to operator’s policy, the 5G network shall be able to provide secure means to enable trusted third-party to request discovering a sensing group in the proximity of the UE that is requesting a 5G wireless sensing service from application server.

NOTE:	[9] Network Exposure CPR 7.1.3-2 represents PR 5.19.6-2

### 7.X.6	Potential New Requirements needed to support the use case

[PR 7.X.6-1] Based on third-party request and subject regulatory requirements, operator’s policy and the user consent, the 6G system shall support a UE with sensing capabilities including more than one independent sensor in order to support a sensing service.

NOTE 1:	An independent sensor is a sensor that may be activated either on its own or in conjunction with another sensor when supporting 3GPP Sensing service by a UE, which can be identified and activated individually by the 6G system.

[PR 7.X.6-2] A 6G system shall be able to configure the activation of sensors of a UE independently where UE has capability supporting more than one sensor.

NOTE 2:	This assumes that activation of a second sensor is independent of a first sensor also connected to the same UE by the 6G network.

NOTE 3:	Sensors connected to a UE are assumed to have an ideal connection, and not utilizing MNO spectrum.

[PR 7.X.6-3] A 6G System shall be able to configure a UE to provide efficient sensing measurement reporting including reconfiguration following an update to an ongoing configuration to activate, configure or deactivate a sensing transmitter or sensing receiver connected to the UE.

NOTE 4:	 this assumes a UE has capability with more than one sensor, and UE may change reporting to efficiently handle sensing measurement reporting.