3GPP TSG-SA WG1 Meeting #58 	S1-121153

# Seville, Spain, 7 - 11 May 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.101 | CR | 0404 | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NOVES Modality Support Indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  |  | Date: |  | 2012-04-30 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Use case 10 of TR 22.871 requires that a 3-way video call is established between the emergency caller, PSAP, and sign language interpreter. Current stage 2 and 3 assume that a person in the PSAP would recognise the emergency caller’s special needs and set up a 3-way call with an interpreter. However this may not meet the requirements of users in need of modality support. Since emergency calling must be as equal to everyday calling as possible, this indicator must be available in all calls. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirement for an indicator from the UE to the network and PSAP that the caller or called user has needs for specific modalities in the call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A person in the PSAP has to deduce that the emerency caller has special needs and may not know what action to take. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 24.008 |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 10.4.2	IMS Multimedia Emergency Sessions

#### 10.4.2.1	General

For IMS emergency calls towards IP PSAPs, other media types may be supported by the UE and the IMS, subject to regulatory requirements.

The media types that may be supported during an IMS MES include:

-	Real time video (simplex, full duplex), synchronized with speech if present;

-	Session mode text-based instant messaging;

-	File transfer;

-	Video clip sharing, picture sharing, audio clip sharing;

-	Voice; and

-	GTT.

NOTE 1: An IMS MES need not contain voice or GTT.

To avoid interworking issues, a UE and IMS that supports text based instant messaging shall support a common session mode text-based instant messaging protocol.

IMS MES does not include support for legacy store and forward messaging such as the Short Messaging Service (SMS).

Calls from non-human associated devices (e.g. fire alarms) are outside the scope of this specification.

Adding, removing and modifying individual media to/from an IMS MES shall be supported.

An IMS MES is not a subscription service. A UE capable of IMS emergency calls and capable of supporting the other media types should also be able to support initiation of an IMSMES.

An IMS MES from an unauthenticated UE (as far as the IM CN is concerned) shall be supported by the IM CN subsystem, if required by local regulation.

An IMS MES shall support providing the location of the UE, in a manner similar to IMS emergency voice calls.

An IMS UE that supports IMS MES shall identify an emergency number dialled by the end user as a valid emergency number utilizing the same mechanisms as used for IMS emergency voice calls as defined in subclause 10.1.1.

NOTE 2:	This capability supports the general public, including facilitating emergency communications by individuals with disabilities (e.g. persons who are deaf, deaf-blind, hard of hearing, or have a speech disability).

An originating network and UE may support some or all of these other media types, and support of any specific media by an originating network or UE may be subject to regulatory requirements.

Voice call continuity per clause 21 shall be supported when a UE with an active IMS MES with voice and other media moves out of IMS voice coverage and voice call continuity is supported by the UE and network. The remaining media (i.e. voice call) then becomes a CS emergency call e.g. TS12 call for 3GPP systems as defined in 3GPP TS 22.003 [14].

Other media shall be dropped when a UE with an active IMS MES moves out of IMS voice coverage, irrespective of whether or not there is an active voice session.

10.4.2.2	UE Requirements

When IMS MES are supported by the UE, the following apply:

-	An IMS UE that supports IMS MES shall also support IMS emergency voice calls.

-	Once a UE is aware that an IMS MES has been initiated, the UE shall be able to (subject to user configuration) avoid drawing unnecessary attention to the user (e.g., playing audible tones or flashing brightly) and should confirm this to the user in as private a manner as is reasonable e.g. using text on the screen or audio if headphones are already connected. UE behaviour in an IMS MES  may need to be different relative to the normal configuration.

-	The UE should clearly differentiate IMS emergency session-mode text based instant messaging from IMS non-emergency session-mode text based instant messaging on the user display.

-	The IMS UE supporting video transfer during an IMS MES should be able to deliver recorded media in a form that allows progressive playback. (It is desirable that all pre-recorded media sent during an emergency session be progressively viewable.)

-	When an IP PSAP attempts to add additional media to an existing IMS MES, the user shall be made aware of this. When additional media is requested by the PSAP, the user shall be able to permit or deny it.

-	The UE shall provide an indication to the user  for each requested media, whether it was successfully or unsuccessfully established.

-	Further notifications of added and removed media shall be indicated to the user while the IMS MES is active.

-	If none of the media requested by the UE is successfully established, the IMS MES will fail and an IMS MES failure indication shall be provided to the user.

-	In handover of an IMS MES where other media is dropped when IMS MES is not supported, the UE shall indicate to the end user that the other media is not supported in this area.

The following requirements for IMS emergency voice calls also apply when an IMS MES is supported by the UE:

-	An IMS UE that supports IMS MES shall indicate to the network that the call is an IMS emergency call as is done for an IMS emergency voice call.

-	An IMS UE that supports IMS MES shall be able to receive an IMS call-back from a PSAP per clause 10.1.3 with voice, GTT or other media per clause 10.4.2.1.

-	An IMS UE that supports MES shall utilize the same trust and security mechanisms for the other media as utilized for an IMS emergency voice call.

-	When roaming, a UE shall originate an IMS MES in the serving network in the same  manner as for IMS emergency voice calls.

-    An IMS UE that supports MES shall be able to send a modality indicator set by the end user.  This indicator is intended to provide information on communication modalities that the user prefers to use for transmission and perception (e.g., video with sign language, real time text, voice …).

10.4.2.3	Originating Network Requirements

When an IMS MES is enabled by the originating network, the following apply:

-	Other media shall only be supported in packet-based networks that support IMS emergency voice calls.

-	The originating network shall deliver all media to the same IP PSAP throughout the duration of the IMS MES.

-	The network shall indicate to the UE, for each requested media, whether it was successfully or unsuccessfully established.

-	Further notification of added and removed media shall be provided to the UE while the IMS MES is active.

-	If none of the media requested by the UE is successfully established, the IMS MES will fail and an IMS MES failure indication will be provided to the UE.

The following requirements for IMS emergency voice calls also apply when IMS MES is supported by the network:

-	Subject to regional regulatory requirements, the network shall be able to authenticate the UE using the same procedures as for IMS emergency voice calls.

-	The originating network shall provide the capability to enable an IMS UE supporting IMS MES to obtain local emergency numbers or other emergency address(es) (e.g. destination address) utilizing the same mechanism as used for IMS emergency voice calls.

-	An IMS MES shall be provided in the local serving network.

-	For an IMS MES, any kind of emergency addressing (e.g. SIP URIs, Tel URIs) and special indications for emergency sessions shall be treated in the same manner as IMS emergency voice calls.

-	The originating network should detect all IMS MESs regardless of the UE emergency call indication. According to operator policy, the originating network may either inform the UE to enable re-origination as an IMS MES or support origination of the initial call.

-	The originating network shall be responsible for routing the IMS MES towards the appropriate destination (e.g., based on emergency service type, location, or policy), which can be a PSAP or a relay service (e.g. translation service) that provides access to a PSAP.

-  When selecting the destination of an IMS MES, the originating network shall have the capability to take into account the modality indicator together with other criteria (e.g. preferred language).

-	The network shall be able to provide integrity protection, and/or privacy for other media similar to that provided for IMS emergency voice calls.

-	An IMS MES shall utilize the same priority mechanisms as IMS emergency voice calls.

-	Detailed log records of the IMS MES shall be generated by the originating network in a similar manner to IMS emergency voice calls and subject to regulatory requirements.

-	All media content within the IMS MES shall be carried with an indication of the source, in a similar manner as for IMS emergency voice calls.