3GPP TSG-SA WG1 Meeting #71 ad hoc on SMARTER (5G)	S1-153056

Vancouver, Canada, 19 – 21, October, 2015

Title:		New SMARTER Use Case on support for broadcast service

Agenda Item:	3.5	New use cases

Source:		Dish Network

Contact: 	Ojas Choksi (ojas.choksi@dish.com)

Abstract: This submission proposes a new use case related to broadcast/multicast service delivery via a dedicated radio carrier

5.x Broadcast/Multicast Services using a Dedicated Radio Carrier

5.x.1 Description

The massive growth in mobile broadband services over the last few years has caused regulatory bodies across the world to consider re-allocating some of the UHF spectrum, because of its superior propagation characteristics, for mobile broadband services. This could potentially cause displacement of smaller/rural TV broadcasters and make them look at alternate delivery models (e.g. channel sharing, delivery via mobile broadband, etc.).

Additionally, as a potential new business opportunity, wireless operators could deploy an overlay 3GPP network using dedicated spectrum for the broadcast service to serve the customers affected by displacement of smaller broadcasters within a geographic area.

Alternatively, a new wireless entrant could deliver a stand-alone broadcast/multicast only service over a large geographic area by deploying less number of sites with greater coverage area.

Consider a use case where Operator A has deployed a 3GPP system using frequency f1 in urban geographic area where demand for the mobile broadband service, and therefore the capacity need is the highest.  Operator A decides to introduce a new broadcast/multicast service either on its own or in partnership with another broadcast/multicast service provider.  The service is expected to be made available over a much wider area than its existing 3GPP system.

Wireless operator A deploys an overlay over its existing 3GPP system to create a single frequency network with few sites, each covering a wide geographic area.  In order to ensure it can accommodate a wide variety of broadcast/multicast content simultaneously, wireless operator A uses a dedicated frequency, f2, to deploy the broadcast/multicast service.

As a result of this overlay deployment, users in urban area X can simultaneously receive existing broadband data services (on f1) as well as the new broadcast/multicast service (on f2).  Users in suburban/rural areas Y and Z only receive the new broadcast/multicast service.

5.x.2 Potential Service Requirements

- Operator shall be able to deploy a stand-alone 3GPP based broadcast/multicast system (MBMS) by forming a multicast/broadcast single frequency network using multiple high power/tall sites, each with a coverage radius of up to 100 km.

- Operator shall have the flexibility to allocate 0% to 100% of radio resources of a radio carrier for the delivery of broadcast/multicast content.

- Depending on the capability of the terminal, the user shall be able to receive the broadcast/multicast content via the broadcast/multicast radio carrier while a concurrent data session is going on over another radio carrier.

- The MBMS operation shall efficiently utilize the available resources to maximize the amount of content that can be delivered over a dedicated radio carrier.

5.x.3 Potential Operational Requirements