3GPP SA WG1 Meeting #96e 	S1-214125

Electronic Meeting, 8 - 18 Nov 2021	(revision of xx-yyxxxx)

# Source:	China Telecom

# Title:	New SID on Enhanced Relay Communication with High Reliability and High Data Rate

# Document for:	Approval

# Agenda Item:	xxx

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Enhanced Relay Communication with High Reliability and High Data Rate

Acronym:	FS_eReCoRD

Unique identifier:	xxxxx

Potential target Release: Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 720005 | New Services and Markets Technology Enablers (SMARTER) | Previous normative work includes one-hop relay |
| 840034 | Enhanced Relays for Energy Efficiency and Extensive Coverage (REFEC) | Pervious normative work includes multi-hops relay |

Dependency on non-3GPP (draft) specification:

None

3	Justification

With the development of radio technologies, high frequency (e.g. mmWave) would become more widely used. Experiments show that the higher the frequency, the smaller the coverage. If using the existing base stations for higher frequency, the user experience such as service continuity would be heavily affected. To solve this problem, operators need to invest more on base stations.

Having relays is obviously a good way to extend coverage with less cost. If relays are used to extend the coverage, operators could reduce their investment on infrastructure to have more investment on research and development to improve user experience. Both operators and users could take advantages. Moreover, relay owners could benefit from providing their UE as a relay as well, e.g. operators would offer incentive scheme for the relay owners.

With one-hop relay or multi-hop relay, if the relay in one-hop relay or one of the relays in the multi-hop relay is down or out of coverage, the connection would be failed. However, the communication could have high reliability with multi-path relay. If one of the relays is down or moves, the data could either transmit through other paths or other available devices nearby. Also, with multiple paths transmitting simultaneously, user could have higher data rate.

In the previous releases, requirements related to one-hop relay and multi-hop relay were introduced. Also, in the previous study FS_REFEC, there was one use case involving relays with multiple paths. In that use case, the scenario when different data packets from one UE transmitted through different paths is not taken into account.

With multi-path relay, the connection between the remote UE and gNB has multiple paths. Within each path, there could be one-hop relay or multi-hop relays. In this study, only multi-path one-hop relay is considered. In conclusion, there is not enough use cases on multi-path one-hop relay. This study will investigate the use cases and potential requirements to support multi-path one-hop relay.

NOTE: The scenario when involving both path with relay in between the remote UE and gNB and path without relay is not considered in this study. Only scenario when all the paths have relay in between the remote UE and gNB is considered.

4	Objective

The objectives of this study are to define:

- Use cases and corresponding requirements on multi-path one-hop relay,

- Other aspects, including charging and security,

- Gap between the identified requirements for multi-path one-hop relay and the existing requirements for 5G system.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 22.XXX | Study of enhanced relay communication with high reliability and high data rate | TSG#97 | TSG#98 | Zhang, Yuying, China Telecom, zhangyy45@chinatelecom.cn |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A | N/A | N/A | N/A |

6	Work item Rapporteur(s)

Zhang, Yuying, China Telecom, zhangyy45@chinatelecom.cn

7	Work item leadership

SA1

8	Aspects that involve other WGs

SA2, SA3, SA5

9	Supporting Individual Members

Supporting IM name

China Telecom

CATT