3GPP TSG-SA WG1 Meeting #98e 	S1-221046

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Source:	Apple

Title:	Emergency services and UE rejected with cause #78

Document for:	Discussion

Contact:	Mona Mustapha

Abstract: This document provides additional background on the use case for emergency call attempts on a satellite NG-RAN that had previously rejected UE registration with "PLMN not allowed to operate in the country of the UE’s location" when no other network is available

1. Introduction

CT1 sent an LS to SA1 on Emergency services and UE rejected with "PLMN not allowed to operate in the country of the UE’s location" in C1-223045. CT1 asked for further clarification of an earlier SA1 LS in S1-211319 in which SA1 indicated that:

"If the PLMN indicates it is not allowed to operate in the country of the UE’s location, the UE should not (re)-attempt emergency calls."

This document explains the scenarios indicated by CT1 and proposes an SA1 view based on the draft responses provided in S1-221045.

2. Discussion

The intent of the SA1 response above was that to ensure that the UE would not attempt emergency calls on networks that indicate they are not allowed to operate in the country of the UE's location, to ensure that the UE would attempt emergency calls on other available networks.

However, if the only available network(s) is(are) not allowed to operate in the country of the UE's location, then the UE should be allowed to attempt further emergency calls on that(those) network(s). This is because the emergency call may be attempted a period of time after the rejected registration attempt, and the UE may have moved to another location, which means there is a chance that the registration attempt will be successful.

It is understood that there are currently no mechanisms defined in Stage 3 for the UE to block emergency call attempts towards networks that reject UE registration. This was due to the assumption that emergency calls are manually triggered by the user or by certain events (e.g., car accident) and there would not be any automatic re-tries from the user side. Therefore, it is expected that the network would reject any emergency calls as necessary.

3. Proposal

A draft response to the CT1 LS is provided in S1-221045 based on the proposed views above.