3GPP TSG-SA WG1 Meeting #98e 	S1-221103

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Source:	vivo, China Unicom

pCR Title:	Pseudo-CR on use case of mobility for non-N2 shared network

Draft Spec:	3GPP TR 22.851v0.0.0

Agenda item:	7.5

Document for:	Approval

Contact:	Yanchao Kang <kangyanchao@vivo.com>

Abstract: introduce the mobility scenarios and potential requirements for the non-N2 shared network in TR22.851.

1. Introduction

Study FS_netshare aims to study use cases and potential requiremnts for the non-N2 shared network.

2. Reason for Change

Mobility scenarios and potential requirements needs to be discussed and agreed for the non-N2 shared network.

3. Conclusions

none

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.851.

* * * First Change * * * *

5.A.1	Description

It is worth mentioning that 5G networks have been designed to provide shared facilities at the beginning of the deployment. This means that if the 4G network has independent E-UTRAN coverage and shared E-UTRAN coverage at the same time, there will be more than one types of operator's wireless access technology simultaneously supporting coverages:

- Independent E-UTRAN coverage,

- Shared E-UTRAN coverage,

- Independent 5G NR coverage,

- Shared 5G NR coverage.

Therefore, interoperability with existing technologies needs to be considered when introducing non-N2 network sharing.

5.A.2	Pre-conditions

1. OP1 use same PLMN-ID for existing 4G and 5G; and OP2 use same PLMN-ID for existing 4G and 5G. only OP1 5G RAN is "shared" with OP2.

NOTE: The existing 4G and 5G are deployed with independent and shared wireless access technology.

- OP1 5G shared RAN broadcast 1 with non-N2 direct connection.

3. PLMN-ID of shared RAN: Different from home PLMN-IDs of OP1 and OP2.

3. OP1 shared 5G coverage may overlap with OP2 4G coverage; may ALSO overlap with OP2 5G coverage (e.g. at OP1 border).

4. following coverage scenarios are possible:

- OP1-5G only.

- OP1-5G (shared) + OP2-4G (OP1 5G preferred for UE of OP2),

- OP1-5G (shared) + OP2-5G (OP2-5G preferred for UE of OP2),

- OP2 5G only.

NOTE: OP2 UE should stay on OP1, if under OP1 coverage, unless OP2-5G is available.

Further:

1.UE subscribes to OP2’s home PLMN.

2.UE register successfully from OP1’s PLMN in shared 5G area.

5.A.3	Service Flows

- R_OP2_4G

- R_OP2_5G

- R_OP2_5G

- R_OP1_5G

- R_OP1_5G

- R_OP1_5G

②

①

Figure X Non-N2 shared network mobility scenario

The UE moves between the OP1 shared network and the OP2 network(home).

NOTE 1: R_OP1_5G may be Non-N2 shared network.

NOTE 2: R_OP2_5G may be MOCN networks or independent network.

NOTE 3: R_OP2_4G may be a MOCN network or independent network.

Scenario 1: shared 5G   - >  home (OP2)5G

-UE with or without services, and moves from shared network to home 5G network. As shown as ① in figure, that is R_OP1_5G(shared)  - > R_OP2_5G.

Scenario 2: shared 5G - > home (OP2) 4G

-UE with or without services, and moves from shared network to home 4G network. As shown as ② in figure, that is R_OP1_5G(shared)  - > R_OP2_4G;

Scenario 3: home (OP2) 4G - > shared 5G

-UE with or without services, and moves from home 4G network to shared network. As shown as ② in figure, that is R_OP2_4G - >  R_OP1_5G(shared) ; for voice call, UE may quickly accesses to the shared 5G;

Scenario 4: home (OP2) 5G - > shared 5G

-UE with or without services, and moves from home 5G network to the shared network. As shown as ① in Figure .

More detailed access technology mobility is shown in the table 5.A-1.

Table 5.A-1: Mobility for non-N2 network sharing

| Scenario | 1 (OP1-shared - > OP2-5G) | 2 (OP1-shared- > OP2-4G) [no OP2-5G] | 3 (OP2-4G - > OP1-shared) [no OP2-5G] | 4 (OP2-5G - > OP1-shared) |
| --- | --- | --- | --- | --- |
| a) I-PLMN sel | Yes | No | No | No |
| b) I-PLMN cell resel | Yes (via ePLMN + I-freq priority) | Yes (via ePLMN), only at OP1 border | Yes (via PLMN + IRAT priority) | Yes (via ePLMN), only at OP2 border |
| c) I-PLMN HO | Yes (via ePLMN) | Yes (via ePLMN), only at OP1 border | Yes (via ePLMN) | Yes (via ePLMN), only at OP2 border |

5.A.4	Post-conditions

5.A.5	Existing feature partly or fully covering use case functionality

5.A.6	Potential New Requirements needed to support the use case

Mobility shall be supported if a user crosses the border between the shared network managed by another operator and the home operator’s network.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>