3GPP TSG SA WG 1 Meeting 98e		 S1-221052r8

Electronic Meeting, 9 – 19 May 2022

Title:	Reply LS on emergency service support over ProSe Relays

Response to:	S2-2203130 from SA2

Release:	Release 18

Work Item:	FS_5G_ProSe_Ph2

Source:	SA1

To:	SA2

Cc:	-

Contact person:	Mona Mustapha at Apple dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 CR0584 to TS22.101

# 1	Overall description

SA1 thanks SA2 for their LS in S2-2203130.

SA1 has agreed the attached CR in connection with the SA2 questions. One new general requirement is the following:

Subject to local regulatory requirements, the UE shall first attempt to make an emergency session via any available network. If the UE fails to find a network that allows emergency sessions, the UE shall attempt an emergency session via a relay UE.

Another requirement is also added, as indicated in the SA1 answers below.

Please note that the term “Emergency session” used in this LS and attached CR, covers both IMS emergency voice calls as well as IMS Multimedia Emergency Sessions (MES), as defined in TS22.101.

Q1: When a 5G ProSe UE-to-Network Relay offers emergency service support, does it imply that the ProSe UE-to-Network Relay needs to be responsible for Remote UE's emergency service according to local regulation, as indicated in TS 22.101, i.e. to support Remote UEs without SIM/USIM/ISIM, or for valid Ues only?

SA1 response to Q1:

SA1 notes that the following existing requirement in TS22.101 applies to all UEs, regardless if indirect network connection (via relay UE) or direct network connection is used:

"Emergency calls shall be supported by the UE without a SIM/USIM/ISIM being present."

In case of emergency session via a relay UE, it is assumed that a remote UE is connected to a relay UE that is registered to a PLMN, i.e. the relay UE has a SIM/USIM/ISIM.

Q2:  Since the 5G ProSe UE-to-Network Relay may be roaming, under which conditions would it be required that the emergency call is routed to PSAP of the same country as the Remote UE?

SA1 response to Q2:

Please refer to the new requirement, as per attached CR.

Q3: Should the 5G ProSe UE-to-Network Relay and Remote UE be required to belong to the same PLMN?  For example, in case of network sharing where Remote and Relay Ues may be connected via different CN PLMNs.

SA1 response to Q3:

SA1 interprets that 'belong to' means that the remote and relay UEs are connected to the same CN PLMN. In this case, there is no restriction on whether the 5G ProSe UE-to-Network Relay and Remote UE need to be connected to the same CN PLMN. How the call is connected towards the PLMN is not defined in Stage 1, as long as emergency session capabilities are maintained during and after the emergency session in the same way as for a direct UE to network emergency session. For network sharing, the UEs may be connected to different Core Networks.

Q4: While a Relay UE is providing emergency connectivity services support for a Remote UE, if the Relay UE needs to make emergency access of its own, what if any requirements for the Relay UE to continue to provide emergency access to the Remote UE?  SA2 assumes that a Relay UE may provide emergency service access to only one Remote UE at any time.

SA1 response to Q4:

There are no service requirements on relative priority between concurrent emergency sessions.

# 2	Actions

To SA2:

ACTION: 	SA1 asks SA2 to take the above clarification into account.

# 3	Dates of next TSG SA WG 1 meetings

SA1#99e	22 Aug – 1 Sep 2022		Electronic Meeting

SA1#100	07-11 Nov 2022			TBD, North America (fallback: Electronic Meeting)