3GPP TSG SA WG 1 Meeting #99e 	S1-222108

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Source:	Nokia, Nokia Shanghai Bell

pCR Title:	Pseudo-CR on consolidated potential KPIs for sensing scenarios

Draft Spec:	3GPP TR 22.837

Agenda item:	7.2

Document for:	Approval

Contact:	Hideaki Takahashi, hideaki.takahashi@nokia.com

Abstract: This contribution proposes consolidated KPIs applicable to all use cases for integrated sensing and communications.

1. Introduction

At SA1 #98e, some KPIs were proposed, which could be regarded as use case or application specific, and so hard to be evaluated from 3GPP functionality point of view. Therefore, this paper proposes a set of KPIs which is generic and can be applied for all services leveraging sensing functinalities.

2. Reason for Change

The following KPIs can be considered for all sensing use cases. The rationale behind each propose KPI is provided below, from the viewpoint of potential requirements for NR radio interface where applicable.

-	Minimum sensing distance; Decisive factor on the bandwidh and time duration required for sensing signals, as well as the necessity of full-duplex capabilities.

-	Maximum sensing distance; Impact on transmit power of sensing signals, receiver sensitivity and carrier frequency.

-	Spatial resolution; Impact on the bandwidth range and beam width of radiated sensing signals.

-	Detectable Radio Cross Section (RCS); Closely relates to the maximum sensing distance. Likewise, it impacts on transmit power of sensing signals, receiver sensitivity and carrier frequency.

-	Measurement latency; The minimum requirement of measurement latency has been specified in 3GPP standard, e.g. for UE as in TS 38.133. Given that the required measurement latency might be different amongst different sensing use cases, it is worthwhile identifying the required value for each use case and checking if the existing requirement can cover it.

-	Reliability; can be measured by the probability of missed detection of false alarm. The maximum sensing distnace, spatial relation, detectable RCS and measurement latency could affect these probabilities.

Needless to say, different use case would have different value for each KPI, which are to be discussed for further.

3. Conclusions

By defining the generic set of KPIs, the other WGs will be able to perform feasibility study from 3GPP functionality point of view.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 and for every use case to use the same KPI table.

* * * First Change * * * *

# 7	Consolidated potential requirements and KPIs

## 7.1	Consolidated potential requirements

## 7.2	Consolidated potential KPIs

Table 7.2-X	Performance requirements for integrated sensing and communication scenarios

| Scenario | Minimum sensing distance (note 1) | Maximum sensing distance (note 2) | Spatial resolution (note 3) | Detectable radio cross section (note 4) | Measurement latency (note 5) | Reliability (missed detection/false alarm) (note 6) |
| --- | --- | --- | --- | --- | --- | --- |
| NOTE 1: The minimum distance between the transmitter of sensing signals and a taget object NOTE 2: The maximum distnace between a transmitter of sensing signals and a target object NOTE 3: The size of the smallest target object that can be detected by NR based sensing NOTE 4: A measure on detectability of a target object at the receiver [dBsm] NOTE 5: Latency of obtaining sensing measurement results NOTE 6: Probability of missed detection and false alarm |  |  |  |  |  |  |