| 3GPP TR 23.700-56 V0.2.0 (2025-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on system architecture enhancement for next generation real time communication phase 3; Stage 2 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	VOID	9

3.3	Abbreviations	9

4	Architectural Assumptions and Requirements	9

4.1	Architectural Assumptions	9

5	Key Issues	9

5.1	Key Issue #1: ADC establishment in alerting phase for early media	9

5.2	Key Issue #2: Enhancements to capability exposure framework for eMMTel services	10

5.3	Key Issue #3: Supporting dynamic configuration of MDC2 endpoints	10

6	Solutions	11

6.0	Mapping of Solutions to Key Issues	11

6.1	Solution #1: ADC establishment in alerting phase initiated by the UE	11

6.1.0	High-level solution Principles	11

6.1.1	Description	11

6.1.2	Procedure	11

6.1.3	Impacts on Services, Entities and Interfaces	15

6.2	Solution #2: Support for NW initiated data channel application in early media session with subscription	15

6.2.1	High level principles	15

6.2.2	Description	15

6.2.3	Procedures	17

6.2.4	Impacts to Services, Entities and Interfaces	20

6.3	Solution #3: IMS ADC establishment during alerting phase when initial INVITE does have BDC offer	20

6.3.0	High-level solution Principles	20

6.3.1	Description	20

6.3.2	Procedures	21

6.3.3	Impacts on Services, Entities and Interfaces	23

6.4	Solution #4: IMS ADC establishment during alerting phase when initial INVITE does not have BDC offer	24

6.4.0	High-level solution Principles	24

6.4.1	Description	24

6.4.2	Procedures	25

6.4.3	Impacts on Services, Entities and Interfaces	27

6.5	Solution #5: I Have No Title	27

6.5.0	High-level solution Principles	27

6.5.1	Description	27

6.5.2	Procedure	27

6.5.3	Impacts on Services, Entities and Interfaces	29

6.6	Solution #6: Support Alerting phase P2A ADC establishment through Autolaunch Parameter	30

6.6.0	High-level solution Principles	30

6.6.1	Description	30

6.6.2	Procedure	31

6.6.2.1	Application Download Procedure	31

6.6.2.2	Application Data Channel Establishment Procedure	32

6.6.3	Impacts on Existing Nodes and Functionality	32

6.7	Solution #7: I Have No Title Again	33

6.7.0	High-level solution Principles	33

6.7.1	Description	33

6.7.2	Procedure	34

6.7.3	Impacts on Services, Entities and Interfaces	36

6.8	Solution #8: Subscription and notification of DC application downloading events	36

6.8.0	High-level solution principles	36

6.8.1	Types of DC application downloading events	36

6.8.2	Procedure of DC application downloading event subscription and notification	37

6.8.3	Impacts on services, entities and interfaces	38

6.9	Solution #9: Capability exposure of IMS DC events supported by DCSF	39

6.9.0	High-level solution Principles	39

6.9.1	Description	39

6.9.2	Service definition	39

6.9.2.1	General	39

6.9.2.1.1	HSS Service	39

6.9.2.1.2	DCSF Service	41

6.9.3	Procedure	42

6.9.3.1	DCSF instance registration in HSS	42

6.9.3.2	DCSF address discovery by AF	43

6.9.3.3	Subscription for IMS DC events supported by DCSF	44

6.9.4	Impacts on Services, Entities and Interfaces	45

6.10	Solution #10: DC3/DC4 Enhancement to Support Subscription, Notification and Instruction for P2A and P2A2P DC establishment.	46

6.10.0	High-level solution Principles	46

6.10.1	Description	46

6.10.2	Service definition	46

6.10.2.1	General	46

6.10.2.2	Ndcsf_EventExposure	46

6.10.2.2.1	General	46

6.10.2.2.2	Ndcsf_EventExposure_Subscribe	47

6.10.2.2.3	Ndcsf_EventExposure_Unsubscribe	47

6.10.2.2.4	Ndcsf_EventExposure_Notify	47

6.10.2.3	Ndcsf_MediaControl	47

6.10.2.3.1	General	47

6.10.2.3.2	Ndcsf_MediaControl_MediaInstruction	47

6.10.3	Procedure	48

6.10.3.1	Service Subscription Procedure	48

6.10.3.2	P2A Application Data Channel Establishment Procedure	48

6.10.3.3	P2A2P Application Data Channel Establishment Procedure	50

6.10.4	Impacts on Services, Entities and Interfaces	51

6.11	Solution #11: DC3/DC4 based on DCSF services	51

6.11.0	High-level solution principles	51

6.11.1	Description	52

6.11.2	Service definition	52

6.11.2.1	General	52

6.11.2.2	Ndcsf_SessionEvent	53

6.11.2.2.1	General	53

6.11.2.2.2	Ndcsf_SessionEvent_Subscribe	53

6.11.2.2.3	Ndcsf_SessionEvent_Unsubscribe	53

6.11.2.2.4	Ndcsf_SessionEvent_Notify	53

6.11.2.3	Ndcsf_ADCControl	54

6.11.2.3.1	General	54

6.11.2.3.2	Ndcsf_ADCControl_Create	54

6.11.2.3.4	Ndcsf_ADCControl_Delete	54

6.11.3	Procedure	54

6.11.3.1	Person-to-Application (P2A) Application Data Channel Setup	54

6.11.3.2	Person-to-Application-to-Person (P2A2P) Procedure	56

6.11.4	Impacts on services, entities and interfaces	58

6.12	Solution #12: Support dynamic configuration of MDC2 endpoints with new DCSF services	59

6.12.0	High-level solution Principles	59

6.12.1	Description	59

6.12.2	Procedures	59

6.12.2.1	P2A DC establishment procedure	59

6.12.3	DCSF Services	61

6.12.3.1	Ndcsf_MDC2ConnectionRequest	61

6.12.3.1.1	General	61

6.12.3.1.2	Ndcsf_MDC2ConnectionRequest_Create service operation	61

6.12.3.1.3	Ndcsf_MDC2ConnectionRequest_Update service operation	61

6.12.3.1.4	Ndcsf_MDC2ConnectionRequest_Delete service operation	62

6.12.4	Impacts on Services, Entities and Interfaces	62

6.13	Solution #13: DC3/DC4 enhancements with new media management	62

6.13.1	Description	62

6.13.2	Service definition	63

6.13.2.1	General	63

6.13.2.2	Ndcas_Media_Management	63

6.13.2.2.1	General	63

6.13.2.2.2	Ndcas_Media_Management_Create	63

6.13.2.2.3	Ndcas_Media_Management_Update	64

6.13.2.2.4	Ndcas_Media_Management_Delete	64

6.13.2.3	Nimsas_ImsParameterProvision Service API	64

6.13.2.3.1 	Type: ImsPpDataEntry	64

6.13.2.4	Nimsas_SessionEventControl Service API	64

6.13.2.4.1	Type: SessionEventNotification	64

6.13.3	Procedure	65

6.13.3.1	Person-to-Application (P2A) Application Data Channel Setup	65

6.13.3.2	Person-to-Application and Application-to-Person (P2A2P) Procedure	67

6.13.4	Impacts on Services, Entities and Interfaces	69

6.14	Solution #14: Dynamic MDC2 endpoint configuration IMS Data Channel	70

6.14.0	High-level solution Principles	70

6.14.1	Description	70

6.14.2	Service definition	71

6.14.2.1	General	71

6.14.2.1.1	NEF Service	71

6.14.2.1.2	HSS Service	72

6.14.2.1.3	DCSF Service	74

6.14.2.2	Ndcsf_SessionEvent	75

6.14.2.2.1	General	75

6.14.2.2.2	Ndcsf_SessionEvent_Subscribe	75

6.14.2.2.3	Ndcsf_SessionEvent_Unsubscribe	75

6.14.2.2.4	Ndcsf_SessionEvent_Notify	75

6.14.3	Procedure	76

6.14.3.1	DCSF instance registration in HSS	76

6.14.3.2	DCSF address discovery by DC AS	76

6.14.4	Impacts on Services, Entities and Interfaces	77

6.X	Solution #X: <Solution Title>	78

6.X.0	High-level solution Principles	78

6.X.1	Description	78

6.X.2	Procedures	78

6.X.3	Impacts on Services, Entities and Interfaces	78

7	Interim agreements	79

7.1	Agreed Principles	79

7.1.Y	Agreed Principles for KI#Y	79

7.2	Topics for further consideration	79

7.2.Z	Topics for further consideration for KI#Z	79

8	Conclusions	79

Annex A:	Change History	80

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document investigates further enhancements to the capabilities of IMS data channel (DC), specifically in the areas of ADC establishment in alerting phase, IMS capability exposure and dynamic configuration of MDC2 endpoints.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 26.114: "Media handling and interaction".

[4]	3GPP TS 23.203: "Policy and charging control architecture".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[7]	3GPP TS 23.392: "Application enablement aspects for MMTel".

[8]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[9]	3GPP TS 29.175: "IP Multimedia Subsystem (IMS) Application Server (AS) Services Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	VOID

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A2P	Application to Person

ADC	Application Data Channel

API	Application Programming Interface

DC	Data Channel

DCAR	Data Channel Application Repository

DCMF	Data Channel Media Function

DCMTSI	Data Channel Multimedia Telephony Service for IMS

DCSF	Data Channel Signalling Function

IMS	IP Multimedia Subsystem

MF	Media Function

MRF	Media Resource Function

NEF	Network Exposure Function

NF	Network Function

OMA	Open Mobile Alliance

P2A	Person to Application

P2A2P	Person to Application to Person

P2P	Person to Person

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

The following Architectural Assumptions are applicable to this Study:

-	The IMS and IMS DC architecture defined in TS 23.228 [2] is used as the basis for any enhancements.

-	An AS may be trusted or untrusted.

-	The functionality of existing standardized APIs (e.g. OMA APIs) exposing MMTEL services may be used where applicable when the MMTEL service does not depend on IMS DC media.

-	OMA APIs are not extended to support Data Channel and the principles outlined in TS 23.228 [2] Annex AD (IMS Subscribe/Notify framework for event monitoring).

# 5	Key Issues

## 5.1	Key Issue #1: ADC establishment in alerting phase for early media

This key issue will investigate how to support IMS application data channel establishment in alerting phase to provide early media for a normal IMS DC session.

This Key Issue will study:

-	Whether and how the capability of supporting ADC establishment in alerting phase is exchanged between UE and IMS network.

-	How the use of ADC during alerting phase is controlled.

-	Whether and how to download a specific list of DC applications that can be used during alerting phase.

-	How to establish and terminate an ADC during alerting phase.

## 5.2	Key Issue #2: Enhancements to capability exposure framework for eMMTel services

This Key Issue will investigate how to enhance IMS capability exposure framework to expose IMS capability for eMMTel services (voice/video) and related events besides Data Channel, supporting IMS session establishment, modification and release to enable eMMTel services that may require enhancing audio/video services with DC capabilities.

Solutions should describe how to enhance Rel-19 IMS event exposure framework and event definition to support IMS DC events supported by DCSF (e.g. application downloading event) and non-DC events, including subscriber specific and non-subscriber specific supported by IMS AS.

NOTE 1: 	This Key Issue will analyse relevant use cases, the gap between OMA and 3GPP APIs and specify exposure functionality missing from OMA APIs.

NOTE 2:	This Key Issue does not intend to transfer the responsibility of OMA API(s) to 3GPP.

This Key Issue will study:

-	Identification of new events and/or capabilities to be exposed, in addition to those already listed in Table AD.2.5.3-1 of TS 23.228 [2].

-	Assessment of whether existing OMA APIs support the exposure of the newly identified events and/or capabilities.

-	Possible enhancement of exposure functionality where the assessment above identifies gaps in supporting exposure of the newly identified events and/or capabilities.

## 5.3	Key Issue #3: Supporting dynamic configuration of MDC2 endpoints

This Key Issue will investigate whether and how to define interfaces between DCSF and NEF/AF to complete existing P2A/P2A2P Data Channel procedures by specifying DC3/DC4 interfaces to enable dynamic configuration of MDC2 endpoints.

This Key Issue will study:

-	the services and information elements that are exposed via DC3/DC4 for UE initiated P2A and P2A2P ADC session establishment.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 | X |  |  |
| #5 | X |  |  |
| #6 | X |  |  |
| #7 |  | X |  |
| #8 |  | X |  |
| #9 |  | X |  |
| 10 |  |  | X |
| 11 |  |  |  |
| 12 |  |  |  |
| 13 |  |  |  |
| 14 |  |  |  |

## 6.1	Solution #1: ADC establishment in alerting phase initiated by the UE

### 6.1.0	High-level solution Principles

This solution addresses Key Issue #1: DC establishment in alerting phase for early media.

The solution reuses Rel-19 TS 23.228 [2] with the addition that

-	Based on the UE request (via a specific root URL) , the DSCF sends to the UE(s) an application list related with the alerting phase-	The calling party receive BDC from the terminating network during alerting phase.

-	The UE(s) decide whether and which application to download (as for Rel-19 specifications).

-	The UE(s) decide when to release an application that was running in alerting phase; they may decide to continue this application after the alerting phase has finished. (as for Rel-19 specifications).

The solution may e.g. be used for the case where the originating UE1 calls a help centre and is immediately provided with an App that the user will be using when interacting with the call centre.

### 6.1.1	Description

This solution provides a procedure on how to establish a P2A ADC in alerting phase.

Editor's note:	Current version of the solution supports P2A ADC establishment in alerting phase. it is FFS whether it can support other forms of ADC, i.e. P2A2P and P2P.

### 6.1.2	Procedure

The pre-condition of ADC establishment in alerting phase is the capability negotiation during the IMS registration phase on whether the UE supports the DC in alerting phase.

Based on the IMS DC capability negotiation defined in clause AC.7.0 of TS 23.228 [2], when the UE supporting DC in alerting phase and the UE is allowed to use DC in alerting phase, it includes the media feature tag of supporting DC in alerting phase in the Contact header field of the initial REGISTER request and any subsequent REGISTER request to allow the home IMS network to discover its capability of supporting DC in alerting phase.

If the IMS network supports DC in alerting phase, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting DC in alerting phase in the 200 OK response to the initial and any subsequent REGISTER request, which is used by the UE to discover the IMS data channel capability of its home IMS network.

The UE may receive a Feature-Caps header field indicating IMS's capability of supporting DC in alerting phase in the 200 OK response to a subsequent REGISTER request when the network starts supporting DC in alerting phase after successful initial registration of the UE.

When the UE supporting DC in alerting phase initiates an IMS session and if the UE is allowed to use DC in alerting phase, it includes the media feature tag in the Contact header field of the initial INVITE message, regardless of data channel media being part of the SDP or not.

The UE shall not include the media feature tag in the Contact header field and data channel media description in the SDP offer of the initial INVITE request or any subsequent re-INVITE request message, if the S-CSCF has not included the capability indication of supporting DC in alerting phase in the Feature-Caps header field in the 200 OK response either to the initial REGISTER or a subsequent REGISTER request message.

If the UE is not configured whether to use DC in alerting phase, it is up to UE implementation whether or not to include the media feature tag in the Contact header field.

With a successful capability negotiation about the support of DC in alerting phase, the procedure is illustrated in Figure 6.1.2-1, which is based on clause AC.7.2.2 in TS 23.228 [2].

Figure 6.1.2-1: Procedure of ADC establishment in alerting phase

The steps in the procedure are as follows:

1.	Same as step 1-14 in clause AC.7.1 of TS 23.228 [2], UE#1 initiates a SIP INVITE towards the IMS AS. After validating the user's subscription, the IMS AS notifies the DCSF about DC support. The DCSF provides the IMS AS with instructions and media information for establishing BDC. The IMS AS allocates resources with MF accordingly and updates the INVITE with the negotiated media. The updated SIP INVITE is forwarded to the terminating network/UE#2 for terminating network negotiation.

2-3.	UE#2 and terminating network returns an 18x response with the SDP answer to BDC to originating network. According to the received SDP answer, MF may update data channel media resource information for UE#2.

4.	The IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, Session ID, Media Info List) to DCSF.

5.	The DCSF responds to the Nimsas notification request.

6.	The SIP message18x is forwarded to UE#1 which indicates the BDC has been established and that the alerting phase has started.

7.	The originating network P-CSCF executes QoS procedure for BDC as specified in TS 23.203 [4] and TS 23.503 [5].

8.	PRACK and 200 OK for PRACK procedure are performed.

9-10.	When the BDCs have been established between terminating MF and UE#1/UE#2, the DC application list for alerting phase and DC application for alerting phase is requested and downloaded to UE#1 and UE#2 from terminating DCSF.

The DCSF provides via MF the application list for alerting phase from the received specific URL , further to UE#1 and UE#2, based on their data channel capabilities and their choices.

The UE(s) use a specific URL related with the fact that they request a list of applications available in the alerting phase

Editor's note:	It is FFs how the UE is alerted to initiate the download.

11.	UE#1 initiates a P2A ADC for the selected DC application for alerting phase. UE#1 sends SIP UPDATE with an updated SDP to IMS AS via originating network P-CSCF and S-CSCF. The updated SDP contains the BDC information, as well as the requested ADC and the associated DC application binding information.

12.	Same as step 3-14 of clause AC.7.2.2-1 in TS 23.228 [2], the IMS AS notifies the DCSF about DC support. The DCSF instructs the IMS AS to establish MF anchoring via MDC2. The IMS AS provisions the necessary media resources at the MF and returns the information to the DCSF. The DCSF then initiates ADC setup toward the DC Application Server via DC3/DC4. After receiving the DC AS's response with its own MDC2 media resource, the DCSF instructs the IMS AS to update the MF resource accordingly. The IMS AS completes the MF configuration and notifies the DCSF.

13-14.	IMS AS sends the SIP UPDATE to remote network side and UE#2, via the originating S-CSCF, which does not include a request in the SDP for the ADC.

15-17.	UE#2 and terminating network returns a SIP UPDATE 200 OK response with SDP answer for audio/video/bootstrap.

18.	IMS AS notifies the DCSF about the successful result of media confirmation.

19.	DCSF replies to the notification.

20.	The IMS AS includes SDP answer for ADC to UE#1 in SIP UPDATE 200 OK response that it sends towards the UE 1 via the S-CSCF and P-CSCF.

21.	The originating network P-CSCF executes QoS procedure for ADC media based on the SDP answer information from the 200 OK response.

22.	SIP UPDATE 200 OK forwarded to UE#1 with ADC SDP answer.

23.	The ADC between UE#1 and DC AS is established via MF. MF forwards data channel traffic between UE#1 and DC AS based on MDC2 media point information.

24-25.	UE#2 answers the call and SIP INVITE 200 OK response is sent to the IMS AS via I/S-CSCF.

26.	The IMS AS notifies the DCSF, via Nimsas_SessionEventControl_Notify (Session ID) of the call state change.

27.	DCSF replies to the notification.

28-29.	SIP INVITE 200 OK forwarded to UE#1 which indicates the call is answered and ACK procedure is performed.

30.	Both UE#1 and UE#2 retrieve a new application list via BDC since call is answered. The new list supersedes the previously downloaded list. The UEs send request to MF to request application list by providing a root URL, via the established BDC with its data channel capabilities. The DCSF provides the application list from the root URL provided by UE#1 via MF, further to UE#1 and UE#2, based on their data channel capabilities and their choices through MF.

31.	Subsequent procedures for DC application, as defined in clause AC.7.2 of TS 23.228 [2]. Depending on the application and user interaction, the UE(s) determine when to release the application initiated in alerting phase but this aspect is out of scope of specifications and does not require a new UE behaviour.

### 6.1.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

-	supporting retrieving DC application list and DC application for alerting phase; this includes providing a specific URL to get from DCSF the application list for alerting phase,

-	supporting ADC establishment in alerting phase.

IMS AS:

-	supporting negotiation of ADC in alerting phase.

Editor's note:	Impacts on IMS AS and DCSF are FFS.

DCSF:

-	providing DC application list based on the specific URL received from the UE and associated with the application list for alerting phase.

## 6.2	Solution #2: Support for NW initiated data channel application in early media session with subscription

### 6.2.1	High level principles

This solution has the following high level principles, in order to support IMS data channel application in early media session:

-	Data Channel Application in Alerting Phase (DCAP) service can be configured by the user and stored in the HSS as part of the subscription data.

-	IMS AS triggers the application data channel establishment based on UE subscription data when receives INVITE from remote UE.

-	IMS AS performs as a B2BUA to update the SDP for data channel in early session and media flow for the main session.

-	IMS AS activates the application data channel upon receiving 180 Ringing from the serving UE.

-	The application data channel may coexistent with other early session services, e.g. CAT, CRS.

### 6.2.2	Description

This is a solution for Key issue #1 " DC establishment in alerting phase for early media".

The user of UE#2 is able to subscribe to the Data Channel Application in Alerting Phase (DCAP) service, including activate (or de-activate) the service and update the settings, e.g. to change by configuration the active data channel application. The DCAP subscriber is able to refine the data channel application selection behaviour with configured rules, e.g. time, calling party's location, called party's location, the identity of the calling and called party. The DCAP service is able to select the appropriate data channel application according to the rules.

DCAP is a terminating network service, but can also have an originating network functional component. That is, DCAP can be selected on behalf of the called subscriber for presentation to the calling party, but the calling (IMS) subscriber can also subscribe to and activate the DCAP service.

### 6.2.3	Procedures

The overall procedures of data channel establishment in early media phase are illustrated in figure 6.2.3-1.

Figure 6.2.3-1: ADC establishment in early media phase

0.	The user of UE#2 subscribes Data Channel Application in Alerting Phase (DCAP) service to his/her serving IMS network. The data channel application to be initiated in alerting phase is selected by the user and configured in UE#2's subscription data.

1.	UE#1 sends a SIP INVITE targeting UE#2. The initial SDP contains offers for audio/video flow, bootstrap data channel. UE#1 indicates support for preconditions, reliable provisional responses and indication of early-session supported.

2.	Base on UE#2’s subscription data for DCAP service, the IMS AS decides to trigger the pre-configured data channel application to UE#1 in early media session. The IMS AS sends Nimsas_SessionEventControl_Notify to the selected DCSF to notify the session establishment, including the application data channel (inactive) and bootstrap data channel. The IMS AS may add the remote bootstrap data channel to UE#1 (BDC from UE#1 to the DCSF in the terminating network) if not presented.

3.	The DCSF decides whether the requested data channels are allowed and determines the control policy and media information for the application data channel and bootstrap data channels.

4.	The DCSF invokes the Nimsas_MediaControl_MediaInstruction instructing the IMS AS how to set up the application data channel and bootstrap data channels with MF both for originating and terminating side.

5.	The IMS AS instruct the MF to allocate required bootstrap data channels. The IMS AS indicates to the DC AS that the media stream of ADC is inactive.

6.	The IMS AS responds to the MediaInstruction request received in step 4.

7.	The DCSF indicate the DC AS reserving the media resource for the application data channel and MDC2 end points to the MF. The DCSF indicates to the DC AS that the media stream of ADC is inactive.8.	The DCSF responds to the Notify Request received in step 2.

9.	The IMS AS sends the INVITE to UE#2.

10.	UE#2 determines the subset of the media flows and responds with an Offer Response message back to the IMS AS.

11.	The IMS AS forwards the Offer Response message to UE#1.

12.	The IMS AS initiates an UPDATE message to UE#1, which includes the SDP for the and remote bootstrap data channel and inactive application data channel for early session. The IMS AS may add a remote bootstrap data channel with UE#1 if not request in the INVITE message.

13.	Based on UE implementation and local policy, UE#1 may alert the user that an application data channel in early media is triggered. The user can decide to accept or reject the application.

14.	If accepted by the user, UE#1 reserves the media resource for the application data channel and bootstrap data channel. UE#1 sends the200 OK, which acknowledges the UPDATE message to the IMS AS.

15.	The BDC between UE#1 and the DCSF is established as part of the SDP Offer in the early session. If the requested data channel application is not locally available, UE#1 downloads the data channel application via the established BDC and update the early session with the IMS AS.

16.	The IMS AS forwards the PRACK message to UE#2.

17-18.	UE#2 reserves resource and sends a SIP 200 (OK) response for the SIP PRACK request to UE#1. The application data channel between the DC AS and UE#1 is established and UE#1.

19.	UE alerts the user and sends Ringing indication to the IMS AS.

20.	The IMS AS decides to activate the pre-configured data channel application to UE#1.

21.	The IMS AS instructs the MF to activate the media resource for the application data channel. The IMS AS may notify the DC AS to activate the application towards UE#1.

22.	The IMS AS updates the IMS session with UE#1 to activate the application data channel in early session.

23.	UE#1 confirms the activation of the application data channel.

24.	The interaction between UE#1 and the DC AS via application data data channel in early session is started.

25.	UE#2 accepts the session and sends 200 OK to the INVITE.

26-27.	The IMS AS updates UE#1 with SDP offer to initiate the media flow of the main session and terminate the application data channel in the early session.

28.	The IMS AS instructs the MF to update the media resource and terminate the application data channel.

29-30.	The IMS AS forwards 200 OK to UE#1. UE#1 sends ACK to UE#2.

31.	The media flow of the main session between UE#1 and UE#2 starts. The requested BDCs for both UE#1 and UE#2 in the main session are established.

### 6.2.4	Impacts to Services, Entities and Interfaces

UE:

-	Indicates supporting of early media session in SIP INVITE message, as specified in existing specification, e.g. TS 24.229 [6].

-	Supports application data channel in the early session.

-	Notifies user the requested data channel application in early session.

HSS:

-	Supports management of user subscription for Data Channel Application in Alerting Phase (DCAP) service.

-	Supports the user configuration of data channel application(s) in alerting phase.

IMS AS:

-	Triggers the application data channel establishment when receiving INVITE from the originating UE.

-	Activates the established application data channel when receiving 180 Ringing from the terminating UE.

-	Terminates the application data channel when the call accepted by the terminating UE.

-	Performs as B2BUA to update the media flow in the SDP exchanged with UE.

## 6.3	Solution #3: IMS ADC establishment during alerting phase when initial INVITE does have BDC offer

### 6.3.0	High-level solution Principles

The main principle of this solution for support of application data channel establishment during alerting phase of normal IMS DC session as follows:

-	The calling party will receive bootstrap data channel offer from the terminating network during alerting phase.

-	The calling party will establish the bootstrap data channel, download the application and based on the interest, trigger application data channel establishment by selecting one particular application from the terminating network.

-	The called party after getting notification about the application running status in the calling party device, will establish bootstrap data channel, download the application and once accepts the session, the call will be established.

### 6.3.1	Description

This paper proposes solution for supporting application data channel establishment during alerting phase when calling party has offered bootstrap data channel in the initial INVITE.

In this proposal, the calling party and the called party will establish bootstrap data channel(s), download application or application list and establish application data channel while call has not yet been established.

### 6.3.2	Procedures

The procedure of establishing application data channel during alerting phase.

Figure 6.3.3-1: ADC establishment in early media phase

1.	UE-A sends an initial SIP INVITE request to the originating IMS AS through P-CSCF and S-CSCF with an initial SDP offer for audio/video and for the bootstrap data channel establishment with bootstrap DC stream ID. The SDP contains bootstrap data channel offers for both the originating and the terminating side. P-CSCF may include the P-Early-Media header field to indicate the support for early media authorization in the initial SIP INVITE request.	NOTE: The UE is assumed to support P-Early-Media.

2-13.	Steps 2-13 of clause AC.7.1 in TS 23.228 [2] apply.

14.	UE-B returns a SIP 18X response with an SDP answer for bootstrap data channel to the originating network. The terminating IMS AS includes P-Early-Media header field in the SIP 18X response indicating early media is allowed based on the SDP answer not including any application data channel.

15.	The bootstrap data channel has been established between originating MF and UE-A. The UE shall request for a data channel application or data channel applications list from the MF via the established bootstrap data channel. The DCSF provides the data channel application or the data channel application list to UE-A through MF. UE-A may download the data channel application or the data channel application list through the established bootstrap data channel..

16.	The bootstrap data channel has also been established between terminating MF and UE-A.

17.	UE-A sends a SIP PRACK request towards the terminating side before or during step 15.

18.	UE-B returns a SIP 200 OK response for the PRACK. Simultaneously, UE-B may alert the terminating user that the incoming session is a normal IMS DC session.

19.	After the DC application is downloaded, UE-A initiates a SIP UPDATE request to update the IMS session for adding the application data channel and to inform UE-B about the associated DC application binding information. The IMS AS shall notify DCSF about the media change request event and request MF to allocate media resources for the application data channels based on instructions received from the DCSF if MF is anchoring application data channel. Once acknowledgement is received from the DCSF, the IMS AS shall send the update request to the terminating side and UE-B through S-CSCF. The SIP UPDATE request with the modified SDP offer includes the negotiated bootstrap data channel media description, requested application data channel media description and the associated data channel application binding information.

The IMS AS includes P-Early-Media header field in the SIP UPDATE request to indicate that early media is allowed based on the previously received SDP answer not indicating any application data channel.

20-22.	The bootstrap data channel has been established between the originating MF and UE-B. The bootstrap data channel has also been established between the terminating MF and UE-B. UE-B may alert the terminating user that the data channel application is running in UE-A and to download the data channel application if not already available in UE-B. UE-B shall request for the data channel application and the DC application list from the MF for downloading via the established bootstrap data channels if needed.

23-29.	When the DC application is available in UE-B, then the terminating network sends a 200 OK response to the originating side based on which the IMS AS notifies DCSF of the successful bootstrap DC and application DC establishment. Once the DCSF responds to the session establishment request, 200 OK response is forwarded to the UE-A. The originating network P-CSCF executes QoS procedures for bootstrap data channel. The IMS AS may include P-Early-Media header field in the 200 OK response to the SIP UPDATE request to indicate that early media is allowed. UE-B alerts the terminating user for running the DC application.

30.	UE-B returns a SIP 180 response indicating that UE-B is ringing.

31.	Once UE-B accepts to run the DC application, UE-B answers the IMS session. UE-B sends 200 OK to the initial SIP INVITE request and subsequent procedures follow.

Editor's note:	The consideration on originating user consent of NW initiated ADC during alerting phase is FFS.

### 6.3.3	Impacts on Services, Entities and Interfaces

Editor's note:	Impacts are FFS.

## 6.4	Solution #4: IMS ADC establishment during alerting phase when initial INVITE does not have BDC offer

### 6.4.0	High-level solution Principles

The main principle of this solution for support of application data channel establishment during alerting phase of normal IMS DC session as follows:

-	The calling party will receive bootstrap data channel offer from the terminating network during alerting phase.

-	The calling party will establish the bootstrap data channel, download the application and based on the interest, trigger application data channel establishment by selecting one particular application from the terminating network.

-	The called party after getting notification about the application running status in the calling party device, will establish bootstrap data channel, download the application and once accepts the session, the call will be established.

### 6.4.1	Description

This clause proposes a solution for supporting application data channel establishment during alerting phase when calling party has not offered bootstrap data channel in the initial INVITE.

In this proposal, one scenario is explained how the called party (e.g. restaurant customer service) network is providing bootstrap data channel offer to the calling party during alerting phase so that the calling party can establish bootstrap data channel, download the application or application list and then session is established once called party user accepts it.

Scenario.

-	The calling party triggers a normal call to restaurant customer service with intention of reserving seats for lunch.

-	The called party has taken an interactive ringback tone service which means whenever any incoming session is received for that restaurant, the network will offer bootstrap data channel and list of applications provided by the restaurant.

-	The calling party will establish the bootstrap data channel, download the list of applications and select "seat reserve" application.

-	The called party based on the notification of running application on the calling party side will download the application if not available and then accepts the session.

### 6.4.2	Procedures

The procedure of establishing application data channel during alerting phase.

Figure 6.4.3-1: ADC establishment in early media phase

1-2.	UE-A sends an initial SIP INVITE request to the terminating side through P-CSCF and S-CSCF with an initial SDP offer for audio/video. P-CSCF may include the P-Early-Media header field to indicate the support for early media authorization in the initial SIP INVITE request.

NOTE:	The UE is assumed to support P-Early-Media.

3.	UE-B returns a SIP 18X response with an SDP answer for audio/video session to the originating network. The terminating IMS AS includes P-Early-Media header field in the SIP 18X response indicating early media is allowed.

4-5.	UE-A sends a SIP PRACK request towards the terminating side and UE-B returns a SIP 200 OK response for the PRACK.

6.	UE-B sends a SIP UPDATE request to the terminating IMS AS through P-CSCF and S-CSCF with an SDP offer for audio/video and bootstrap data channel. The SDP contains bootstrap offer for the terminating side. Steps 2-10 of clause AC.7.1 in TS 23.228 [2] apply in which the terminating IMS AS determines to notify the terminating DCSF based on DC subscription and UE-B DC capability and the DCSF instructs the IMS AS and MF to establish the resource required for the terminating side. In this scenario, the DCSF allows for application selection from the remote UE, even before the application has been selected by the peer UE.

7.	IMS AS sends the modified SIP UPDATE request including the SDP offer for audio/video and bootstrap data channel adding media information of MF to the originating side and UE-A.

8.	Originating IMS AS determines to notify DCSF based on DC subscription and UE DC capability. If the network does not support DC service or UE-A does not have DC capability, then the originating network sends an SDP answer to setup only audio/video session and remove any bootstrap data channel media information.

9.	The bootstrap data channel has been established between terminating MF and UE-A. The UE shall request for a data channel application or data channel applications list from the MF via the established bootstrap data channel. The DCSF provides the data channel application or the data channel application list to UE-A through MF. UE-A may download the data channel application or the data channel application list through the established bootstrap data channel.

Editor's note:	The consideration on originating user consent of NW initiated ADC during alerting phase is FFS.

10.	Once the bootstrap data channel has been established, UE-A sends 200 OK response for the SIP UPDATE request to the terminating side.

11.	UE-B alerts the terminating user to indicate an incoming IMS session.

12.	After the DC application is downloaded, UE-A initiates a SIP UPDATE request to update the IMS session for adding the application data channel and to inform UE-B about the associated DC application binding information. The IMS AS shall notify DCSF about the media change request event. Once acknowledgement is received from the DCSF, the IMS AS shall send the update request to the terminating side and UE-B through S-CSCF. The SIP UPDATE request with the modified SDP offer includes the negotiated bootstrap data channel media description, requested application data channel media description and the associated data channel application binding information.

The IMS AS includes P-Early-Media header field in the SIP UPDATE request to indicate that early media is allowed based on the previously received SDP answer not indicating any application data channel.

13-14.	The bootstrap data channel has been established between the terminating MF and UE-B. UE-B may alert the terminating user that the data channel application is running in UE-A and to download the data channel application if not already available in UE-B. UE-B shall request for the data channel application and the DC application list from the MF for downloading via the established bootstrap data channels if needed.

15-21.	When the DC application is available in UE-B, then the terminating network sends a 200 OK response to the originating side based on which the IMS AS notifies DCSF of the successful bootstrap DC and application DC establishment. Once the DCSF responds to the session establishment request, 200 OK response is forwarded to the UE-A. The originating network P-CSCF executes QoS procedures for bootstrap data channel. The IMS AS may include P-Early-Media header field in the 200 OK response to the SIP UPDATE request to indicate that early media is allowed.

22.	UE-B alerts the terminating user for running the DC application.

23.	The UE-B returns a SIP 180 response to indicate the UE-B is ringing.

24.	Subsequent procedures continue. When the terminating user accepts to run the DC application for communication, the UE#2 answers the IMS session.

Editor's note:	How to handle the both the existing ring back tone and interactive ring back tone together is FFS.

Editor's note:	The BDC and ADC procedure should be executed before or after 180 ringing is FFS.

### 6.4.3	Impacts on Services, Entities and Interfaces

This solution may have the following impacts to existing entities and interfaces:

IMS AS:

-	Terminating IMS AS will provide a new offer for IMS bootstrap data channel based on the interactive ringback tone service subscription of called party user.

Editor's note:	Additional Impacts are FFS.

## 6.5	Solution #5: I Have No Title

Editor’s note:	Solution Title to be provided.

### 6.5.0	High-level solution Principles

This solution addresses Key Issue #1: ADC establishment in alerting phase for early media.

The solution focuses on the DC based Customized Alerting Tones (CAT) service which is subscribed by the called party and it will provide customized content to the calling party as an early media during the calling process.

The solution has the following principles:

-	The called UE B has subscribed to the DC based CAT service.

-	The calling UE A and called UE B have the DC based early media capability and both of them have negotiated with the serving network regarding this capability.

-	The solution needs an enhancement of the subscribe/notify framework for event monitoring procedure to notify the DC AS the calling event which needs an A2P DC to deliver the early media

-	The solution reuses the IMS DC capability exposure procedure to establish an A2P DC which is used to transmit the early media to the UE.

-	The IMS-AS serving the called UE B notifies the calling event to the DC AS; the DC AS triggers the A2P DC establishment process to the calling UE A.

### 6.5.1	Description

This solution provides a procedure on how to establish an A2P ADC in alerting phase to provide customized DC content to the calling party as an early media.

### 6.5.2	Procedure

The pre-condition of the following procedure is that the called UE B has subscribed to the DC based CAT service and the calling UE A and called UE B have the DC based early media capability and both of them have negotiated with the serving network regarding this capability. It assumes that the DC based CAT service will not coexists with the legacy CAT service; it means the UE will subscribe only one of the CAT services.

Figure 6.5.2-1: Procedure of A2P ADC establishment in alerting phase

The steps in the procedure are as follows:

1.	UE A initiates SIP INVITE including Audio/Video/BDC SDP; step 1-13 of Figure AC.7.1-1 in TS 23.228 [2] for BDC establishment apply.

2.	BDC has been established for UE A and UE B and they may download the application list.

3.	UE B is alerting and 180 Ringing is sent to IMS AS-2.

Editor's note:	Whether the following steps 4~7 are performed before or after the 180 Ringing signal in Step. 3 is FFS.4. Based on the subscription of UE B, IMS AS-2 decides to notify the DC AS this calling event.

NOTE 1:	It assumes that the DC AS has subscribed the calling event of UE B.5. The IMS AS-2 sends the Nimsas_ImsEE_Notify Request to the NEF the calling event including the calling UE A and the called UE B information, as well as IMS Session ID.

6.	The NEF sends the Nnef_ImsEE_Notify Request to the DC AS the calling event including the corresponding information.

7.	The DC AS decides to establish A2P ADC to UE A to deliver the DC early media.

8.	The DC AS establishes A2P ADC with UE A; and step 1-21 of Figure AG.2.1.1-1 in TS 23.228 [2] for adding ADC to an existing IMS session apply with the following differences.

-	The A2P ADC is added to an IMS session in establishment phase not an existing IMS session.

-	The SIP signal UPDATE is used instead of re-INVITE in the procedure of AG.2.1.1 in TS 23.228 [2].

-	The destination of A2P ADC is UE A rather than UE B in the terminating network.

UE A may download the application used in the alert phase.

9.	The UE B/terminating network and UE A/originating network exchange 180 Ringing, PRACK and 200OK (PRACK) SIP signals.

10.	The A2P ADC between UE A and the DC AS which goes through the MF is activated and the early media (customized alerting tones) of this IMS session is delivered from the DC AS to UE A. The user of UE A experiences with the early media.

11.	The user of UE B picks up the phone and the UE B/terminating network and UE A/originating network exchange 200 OK (INVITE) SIP signal.

12.	The IMS AS-2 notifies the DC AS the IMS session establishment event.

13.	The DC AS releases the A2P ADC with UE A.

14.	Subsequent procedures continue.

NOTE 2:	The detailed steps for interaction between the IMS-AS and the DCSF, the IMS-AS and the MF are not illustrated in the procedure, which reuse the corresponding steps in Figure AC.7.1-1 and Figure AG.2.1.1-1.

Editor's note:	The procedure for Customized Ringing Signal (CRS) service is FFS.

### 6.5.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

-	supporting ADC establishment in alerting phase.

IMS AS:

-	supporting report of IMS session establishment events to DC AS;

-	supporting A2P DC establishment for delivering early media.

MF:

-	receiving early media from DC AS and providing it to UE through A2P DC.

DC AS:

-	supporting receiving IMS session establishment events from IMS AS through NEF;

-	supporting initiating and releasing A2P DC for delivering early media.

Editor's note:	Additional Impacts are FFS.

## 6.6	Solution #6: Support Alerting phase P2A ADC establishment through Autolaunch Parameter

### 6.6.0	High-level solution Principles

This solution supports application data channel establishment during the alerting phase. This solution is based on the following high-level solution principles:

-	The bootstrap data channel media negotiation will follow the existing procedure defined in clause AC.7 of TS 23.228 [2] and that the bootstrap data channel will be established before SIP 180 message from the terminating party.

-	The subscriber will not be able to select any data channel application from the application list. Instead, the autolaunch parameter defined in TS 23.392 [7] will be used by the UE to automatically launch the application that need to be used during the alerting phase. The configuration of data channel application profile is defined in TS 23.392 [7].

NOTE:	Allowing the subscriber to select the data channel application during the alerting phase violates the basic principle of alerting phase, which is to inform the user of incoming call. Any subscriber-requested DC application could be used after the call establishment.

-	Only P2A application data channel is supported. Any P2P or P2A2P application data channel should be established after the 200 OK of initial SIP INVITE.

### 6.6.1	Description

This solution proposes to support application data channel establishment during the alerting phase to enhance specific subscription-based services. Some examples of such services include:

-	Customized Alerting Tone (CAT)

-	Customized Ringing Signal (CRS)

-	Originating Identification Presentation (OIP)

-	Terminating Identification Presentation (TIP)

This solution supports establishment of 4 types of P2A application data channel during the alerting phase:

-	From originating network to originating subscriber. (e.g. CAT, TIP)

-	From originating network to terminating subscriber. (e.g. CRS, OIP)

-	From terminating network to originating subscriber. (e.g. CAT, TIP)

-	From terminating network to terminating subscriber. (e.g. CRS, OIP)

The overall procedure is as following:

1.	UE-A initiates the session. The bootstrap data channel is established before the 180 ringing.

2.	The DCSF sends the data channel application profile contained in the root application/application list. The profile contains the Autolaunch parameter based on configuration or subscription. UE-A/UE-B automatically download and launch the application based on the Autolaunch parameter.

3.	The application established the ADC.

### 6.6.2	Procedure

#### 6.6.2.1	Application Download Procedure

The TS 23.392 [7] defines the Autolaunch parameter in the data channel application profile, which could be used to automatically download and launch data channel application used during the ringing phase.

NOTE 1:	The privacy and user consent issue should comply with the definition in SA6. The security aspect should be considered in SA3.

Figure 6.6.2.1-1: Application Download Procedure

1.	Steps 1-14 of clause AC.7.1 of TS 23.228 [2] applies to establish IMS session and bootstrap data channel.

2.	UE-A/UE-B sends HTTP Get request to download the root application/application list.

3.	DCSF-A/DCSF-B sends the root application/application list through the HTTP get response. The root application/application list contains the data channel application profiles, which one of the profile may contain the Autolaunch parameter. The configuration of data channel application profile is defined in TS 23.392 [7].

Editor’s note:	How the SA6 architecture of application and application profile download maps to the existing SA WG2 architecture is FFS. Whether the autolaunch parameter could be used during early session is FFS. Further collaboration with SA6 might be needed.

Editor’s note:	Whether and how the UE informs the user about the application to be downloaded before the step 4 is FFS.

NOTE 2:	Steps 2-3 are out of scope of SA WG2.

NOTE 3:	It is also possible that UE-A sends HTTP Get request to the DCSF-B and UE-B sends HTTP Get request to the DCSF-A. The procedure will follow the existing specification.

4.	Optionally, UE-A/UE-B can automatically download and launch a given data channel application if it contains Autolaunch parameter in its data channel application profile. The definition and usage of the parameter are defined in TS 23.392 [7].

NOTE 4:	How the UE authorize the download are based on the existing procedure defined in SA6 and UE implementation.

#### 6.6.2.2	Application Data Channel Establishment Procedure

The UE-A/UE-B may initiate application data channel establishment procedure after the corresponding data channel application has been downloaded and launched. The depicted procedure starts after the UE-B sends the 180 ringing to the UE-A.

Figure 6.6.2.2-1: Application Data Channel Establishment Procedure

1.	UE-B sends 180 ringing message indicating it starts to ring.

2.	UE-A sends SIP Update to establish P2A application data channel for application downloaded with the procedure specified in Clause 6.6.2.1.

3.	Steps 2-21 of clause AC.7.2.2 of TS 23.228 [2] applies to establish P2A ADC with the difference that the SIP Update instead of SIP re-INVITE has been used.

4.	The IMS AS forwards the 200 OK response of SIP Update from terminating UE to the UE-A.

5.	The terminating network sends 200 OK of SIP INVITE, indicating the session has been established.

6.	The UE-A sends ACK.

Editor's note:	The terminating side procedure is FFS.

Editor's note:	Whether and how the application data channel should be terminated after 200 OK is FFS.

Editor's note:	The impact to the ongoing application download during the alerting phase if the 200 OK is received is FFS.

### 6.6.3	Impacts on Existing Nodes and Functionality

No impact.

Editor's note:	Any further impact is FFS.

## 6.7	Solution #7: I Have No Title Again

Editor's note:	Solution Title to be provided.

### 6.7.0	High-level solution Principles

This solution addresses Key Issue #2: enhancements to capability exposure framework for eMMTel services, especially how to support IMS DC events supported by DCSF.

The solution has the following principles:

-	The DCSF can expose some IMS DC events to NEF or AF.

-	The exposure interfaces will reuse DC3 between DCSF and NEF and DC4 between DCSF and AF.

-	The AF supports subscribing the events exposed by the DCSF and receiving the IMS DC events from the DCSF.

### 6.7.1	Description

This solution provides a procedure on how to support IMS DC events supported by DCSF.

The table AD.2.5.2-1 List of events for monitoring capability in clause AD.2.5.2 of TS 23.228 [2] is enhanced to support the IMS DC events supported by DCSF.

Table AD.2.5.2-1: (from TS 23.228 [2]) List of events for monitoring capability

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Bootstrap Data Channel established | Detected when a Bootstrap data channel is established. Network detects that a Bootstrap data channel is established when the SDP for an established IMS DC includes the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76]. | IMS AS |
| Bootstrap Data Channel Release | Detected when an established Bootstrap data channel is released. Network detects that a Bootstrap data channel is released when an IMS DC that includes an SDP with the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76] is released, or an IMS DC is updated to remove the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76]. | IMS AS |
| Application Data Channel established | Detected when an Application Data Channel is established. Network detects that an Application data channel is established when the SDP for an established IMS DC includes the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76]. The Application DC stream ID can be explicit or any stream ID. | IMS AS |
| Application Data Channel Release | Detected when an Application data channel is released. Network detects that an Application data channel is released when an IMS DC that includes an SDP with the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76] is released, or an IMS DC is updated to remove the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76]. | IMS AS |
| Application Data Channel requires interworking is established | Detected when an Application Data Channel that requires interworking is in the process of establishment. Network detects that an P2A Application data channel is in process of establishment and the MDC2 resources are assigned for the Application data channel. | IMS AS |
| Application operation | Detected when an application is operated. Network detects that an application is operated, e.g. downloading. | DCSF |

NOTE:	Only "Application Operation" is added to the above table - other entries already exist in TS 23.228 [2].

### 6.7.2	Procedure

The procedure for DCSF event exposure is very similar to the subscribe/notify procedure for non-subscriber specific IMS events in clause AD.3.3 of TS 23.228 [2]. The following differences apply:

-	the subscribing destination is DCSF rather than IMS AS;

-	the subscribing request from AF is non-subscriber specific.

Editor's note:	It is FSS how to address the subscribing request from AF when it is subscriber specific.

Figure 6.7.2-1: Procedure for subscribe/notify of events supported by DCSF

The steps in the procedure are as follows:

1.	The AF subscribes to NEF initiating the Nnef_imsEE_Subscribe Request for non-subscriber specific IMS DC events supported by the DCSF. If it’s the request for subscriber specific events, the IMPU of the subscriber is included.

2.	If the subscription to the non-subscriber specific IMS event is authorized by the NEF, the NEF locates all DCSF instances that support the requested IMS DC events via NRF or by local configuration.

3.	For each DCSF instance that supports the requested IMS DC event, NEF subscribes to the DCSF for the requested IMS event via DC3.

Editor's note:	It is FFS whether the subscription request is sent to the DCSF directly or via the IMS AS.

4.	The DCSF returns to the NEF the Subscription Response via DC3.

5.	The NEF returns to AF the Nnef_ImsEE_Subscribe Response.

6.	At some point, the requested IMS DC event is detected by the DCSF.

7.	The DCSF sends a notification to the NEF.

8.	The NEF sends Nnef_ImsEE_Notify Request to the AF.

NOTE:	For the trusted AF, the AF may send the subscription request to the DCSF and receive the event notification from the DCSF via DC4 directly.

### 6.7.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DCSF:

-	supporting detecting IMS DC events which are subscribed by an AF;

-	supporting IMS DC event subscription and notification.

NEF:

-	supporting transferring the subscription request from AF to DCSF;

-	supporting transferring the event notification from DCSF to AF.

AF:

-	supporting subscription and receiving IMS DC events detected by DCSF.

## 6.8	Solution #8: Subscription and notification of DC application downloading events

### 6.8.0	High-level solution principles

This solution targets KI2.

This solution enables an AF (a DC AS) to subscribe to and being notified about events related to the downloading of DC applications. Four types of events are supported: these events can be related to specific or any UE(s), as well as independent or dependent on application(s) (see clause 6.8.1). Subscription requests may optionally include event filters, which can be negotiated with the IMS network, to control the granularity and frequency of notifications. The AF (DC AS) can use the corresponding notifications to assess the popularity of applications (especially P2P applications).

Once the IMS AS has received a subscription request from the AF as defined in Annex AD.3 of TS 23.228 [2] and when a BDC is established, the IMS AS, based on the type of the DC application downloading events (see clause 6.8.1), provides the subscription to the selected DCSF handling the BDC (this assumes the definition of an IMS AS event requiring the IMS AS is to forward to the selected DCSF). The solution does not assume a 1-1 mapping between IMS AS and DCSF. The selected DCSF is responsible for detecting these events and reports the relevant information directly to the AF/DC AS (or to the NEF).

### 6.8.1	Types of DC application downloading events

To support enhanced exposure capabilities, the IMS DC exposure framework provides mechanism for a DC AS to subscribe to events related to the downloading of DC applications. Four types of DC application downloading events are supported:

-	Type 1: Subscriber-specific and DC application-specific event: The subscription request relates to one or more UEs (identified by their IMPUs) and one or more DC application identifiers. A notification shall be triggered only when one of the UE(s) downloads one of the DC applications.

-	Type 2: Subscriber-specific and DC application-agnostic event: The subscription request relates to one or more UEs, without indicating any specific DC application identifier. The DC AS identifier shall be included in the subscription request. A notification shall be triggered when one of the specified UE(s) download any DC application associated with the subscribing DC AS. Notifications for DC applications associated with other DC ASs shall not be provided.

-	Type 3: Subscriber-agnostic and DC application-specific event: The subscription request relates to one or more DC application identifiers, without specifying any UE. A notification shall be triggered when a UE downloads the specified DC application(s).

-	Type 4: Subscriber-agnostic and DC application-agnostic event: The subscription request shall not specify any UE or DC application identifier. The DC AS identifier shall be included in the subscription request. A notification shall be triggered when a UE downloads any DC application associated with the subscribing DC AS. Notifications for DC applications associated with other DC ASs shall not be provided.

NOTE 1:	It is assumed that the DCSF can determine that a DC application is associated with a DC AS either via configuration or via proper application naming convention.

Subscription requests may optionally include event filters to enable the DC AS to specify criteria for event reporting, such as periodic notification preventing excessive signalling for individual events. The IMS network may negotiate the event filter parameters with the DC AS to ensure alignment with network capabilities, resource availability and operational policies.

The event subscription provided to the DCSF contains the AF Id of the DC AS allowing the DSCF to control whether to send a notification related to a given application to the DC AS.

NOTE 2:	This is to ensure that a DC AS can receive notifications only about "its own applications".

### 6.8.2	Procedure of DC application downloading event subscription and notification

Figure 6.8.2-1 depicts a signalling flow diagram for DC application downloading event subscription and notification.

Figure 6.8.2-1: Procedure of DC application downloading event subscription and notification.

1.	The AF (DC AS) sends a subscription request (see clause 6.8.1) to the DC application download event(s) to IMS AS via the NEF. For Type 1 or Type 2 event targeting specific UE(s), clause AD.3.2 of TS 23.228 [2] is followed. For Type 3 or Type 4 event targeting any UE, clause AD.3.3 of TS 23.228 [2] is followed.

The IMS AS instance(s) locally store the subscription request(either globally for subscription targeting Any UE or in association with the IMS registration for subscription targeting an individual UE). The IMS AS needs to understand the event type and the targeted UE(s), but does not need to understand other information about this event in the subscription request.

NOTE 1:	This corresponds to a new IMS AS event requiring the IMS AS to forward the corresponding event subscription to the selected DCSF.

2.	The UE requests BDC establishment as defined in clause AC.7.1 of TS 23.228 [2]. During this process, the IMS AS selects a DCSF.

3.	The IMS AS on behalf of the NEF/DC AS forwards the event subscription request received in step 1 to the selected DCSF for: when the IMS AS notifies the DCSF of the session establishment event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, etc.) it provides the DC AS / NEF subscription request to the DCSF.

4.	The DCSF responds to the IMS AS, indicating successful subscription.

5.	The UE downloads a DC application via the established BDC.

6.	The DCSF checks whether the downloaded DC application is associated with the DC AS having issued the subscription request and whether the Event is authorized for this UE. If it does not, the event shall not be notified.

NOTE 2:	This is to ensure that a DC AS can receive notifications only about "its own applications". To be able to detect whether the downloaded DC application is associated with the DC AS having issued the subscription request either the DSCF can be configured with the mapping between DC applications and the AF ID associated with the DC AS or the application Id (negotiated with the DC AS) contains a reference to this AF ID,

7-8.	The DCSF notifies the NEF of the DC application downloading event. The NEF responds to the notification.

9-10.	The NEF notifies the DC AS of the DC application downloading event. The DC AS responds to the notification.

### 6.8.3	Impacts on services, entities and interfaces

DC AS:

-	Definition of new event about DC application downloading.

NEF:

-	Definition of new event about DC application downloading.

IMS AS:

-	Forwarding of new event subscription request to DCSF based on event type and target UE(s).

-	definition of an IMS AS event requiring the IMS AS to forward the corresponding subscription to the selected DCSF.

DSCF:

-	Receiving new event subscription request.

-	checking the association between DC application and DC AS.

-	Notifying of the event to DC AS.

## 6.9	Solution #9: Capability exposure of IMS DC events supported by DCSF

### 6.9.0	High-level solution Principles

This solution addresses Key Issue #2: Capability exposure of IMS DC events supported by DCSF.

The main principle of this solution for support capability exposure of IMS DC events supported by DCSF as follows:

-	The trusted AF or untrusted AF via NEF shall discover and select the DCSF.

-	To discover DCSF address, DC AS subscribe with HSS and HSS notifies once DCSF register its address in HSS.

-	Similar to third party registration done by S-CSCF to IMS AS, IMS AS does third party registration with DCSF and then DCSF register its address in HSS.

-	Upon successful discovery and selection, DCSF subscribes for the establishment of supported events with DCSF and get notified when DCSF detects the events.

Editor's note:	This solution involves DCSF during registration of IMPU while it was invoked during session earlier. The implications of this approach is FFS.

### 6.9.1	Description

This solution proposes how the trusted AF and untrusted AF via NEF discovers the DCSF and subscribes to get notification for the supported events by DCSF.

Discovery of DCSF:

The trusted AF can directly discover the DCSF with the help of HSS and in case of untrusted AF, the discovery happens through NEF. For this purpose, HSS can expose one service operation using which DC AS/NEF will subscribe and get notification about the DCSF address for the UE.

When UE registers with IMS network, S-CSCF does third party registration with IMS AS and then IMS AS registers its address with HSS. Similarly, this solution proposed IMS AS does third party registration with DCSF and then DCSF registers its address with HSS. Once HSS comes to know the DCSF address, it notifies the NEF/AF.

Subscription and Notification with DCSF:

Upon successful discovery, AF/NEF shall subscribe with DCSF for the supported events and get notified when DCSF detects the event.

Table 6.9.1-1 List of events supported by DCSF

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Successful Application download to UE | Detected when an Application Data Channel is established. DCSF detects that UE has been successfully downloaded one specific application | DCSF |

### 6.9.2	Service definition

#### 6.9.2.1	General

The following table shows the required service operation.

##### 6.9.2.1.1	HSS Service

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1 (from TS 23.228 [2]): IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
|  | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| ImsSubscriber Data | Subscribe | Subscribe/Notify | S-CSCF, AS |
| Management (_ImsSDM) | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
|  | Registration | Request/Response | S-CSCF, IMS AS, DCSF |
|  | DeregistrationNotification | Subscribe/Notify | S-CSCF |
|  | Deregistration | Request/Response | S-CSCF, IMS AS |
| Ims UE Context | Authorize | Request/Response | I-CSCF |
| Management (_ImsUECM) | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
|  | AsInfoGet | Request/Response | NEF |
|  | DcsfInfoGet | Subscribe/Notify | NEF, AF |
|  | DcsfInfoUnsubscribe | Subscribe/Notify | NEF, AF |
|  | DcsfInfoNotify | Subscribe/Notify | NEF, AF |
| ImsUE Authentication (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure | Subscribe | Subscribe/Notify | NEF, Trusted AF |
| (_ImsEE) | Unsubscribe |  | NEF, Trusted AF |

NOTE:	The modifications to the above table from clause AA.2.1.1 in TS 23.228 [2] are only to the service Ims UE Context Management (_ImsUECM), in the following ways, DCSF is added to the Example Consumers of the Registration Service Operation and new Service Operations DcsfInfoGet, DcsfInfoUnsubscribe, & DcsfInfoNotify are added.

6.9.2.1.1.0	Nhss_ImsUECM_Registration service operation (modified)

Clause AA.2.1.2.1 of TS 23.228 [2] is modified to:

Service operation name: Nhss_ImsUECM_Registration

Description: If S-CSCF is the consumer, this service operation registers the serving S-CSCF assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

If IMS AS is the consumer, this service operation registers the IMS AS instance assigned to an IMS user.

If the DCSF is the consumer, this service operation registers the DCSF instance assigned to an IMS user.

Inputs, Required: Public Identity, S-CSCF name and Registration Type (e.g. Initial Registration, Unregistered) if the S-CSCSF is the consumer, IMS AS instance Id if the IMS AS is the consumer, DCSF instance Id is the DCSF is the consumer.

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

NOTE:	The modifications to the above service description from TS 23.228 [2] are "If the DCSF is the consumer, this service operation registers the DCSF instance assigned to an IMS user" and "DCSF instance Id is the DCSF is the consumer" is a required output.

6.9.2.1.1.1	Nhss_ImsUECM_DcsfInfoGet service operation

Service operation name: Nhss_ImsUECM_DcsfInfoGet

Description: Service consumer like NEF or DC AS subscribes with HSS to receive the DCSF address assigned for the UE. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the UAS NF/NEF

Inputs, Required: Public Identity.

Inputs, Optional: Private Identity.

Outputs, Required: Result.

Outputs, Optional: None.

6.9.2.1.1.2	Nhss_ImsUECM_DcsfUnsubscribe

Service operation name: Nhss_ImsUECM_DcsfUnsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

6.9.2.1.1.3	Nhss_ImsUECM_DcsfNotify

Service operation name: Nhss_ImsUECM_DcsfNotify

Description: This service operation enables the HSS to notify the consumer with the updated DCSF address where there is a change of DCSF happens for the UE.

Inputs, Required: DCSF address.

Inputs, Optional: IMPU, IMPI.

Outputs, Required: Result.

Outputs, Optional: None.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.9.2.1.2	DCSF Service

6.9.2.1.2.1	Ndcsf_SessionEvent

6.9.2.1.2.1.1	General

Service description: This service enables the consumer to subscribe with the supported events by DCSF and get notified when the event is detected.

The following operations are available for this service:

-	Subscribing and unsubscribing with events supported by DCSF.

-	Notifying after event is detected to consumers, e.g. NEF, AF.

6.9.2.1.2.1.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Notification Target address, Event ID

Only one Event ID is currently defined: successful download of specific application by UE.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application ID

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (otherwise an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

6.9.2.1.2.1.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

6.9.2.1.2.1.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, DC Application ID.

Inputs, Optional: Calling party and called party ID.

Outputs, Required: Result.

### 6.9.3	Procedure

#### 6.9.3.1	DCSF instance registration in HSS

Figure 6.9.3.1-1depicts a signalling flow diagram for an DCSF registering in HSS the DCSF instance assigned to a registering UE.

Figure 6.9.3.1-1: DCSF Instance Registration in HSS

1.	UE performs initial IMS Registration (see clause 5.2).

Editor's note:	It is unclear which clause is being referenced here.

2.	S-CSCF performs third party Registration with the IMS AS instance assigned to the registering UE.

3.	IMS AS performs third party Registration with the DCSF instance assigned to the registering UE. This is over http and not SIP.

4.	DCSF assigned to the UE/IMPU registers in HSS using Nhss_ImsUECM_Registration service operation or using Sh interface.

NOTE:	Figure 6.9.3.1-1 shows only SBA services. Corresponding Sh procedure is not included in the Figure.

#### 6.9.3.2	DCSF address discovery by AF

Figure 6.9.3.2-1 depicts a signalling flow diagram for discovering DCSF address by AF.

.

Figure 6.9.3.2-1: DCSF address discovery by DC AS

1.	DCSF address is already registered as per Fig1 solution.

2.	AF discovers DCSF by sending Nnef_ImsUECM_DcscfInfoGet service operation to NEF.

3.	NEF discovers DCSF by sending Nhss_ImsUECM_DcscfInfoGet service operation to HSS

4.	HSS provides DCSF address in Nhss_ImsUECM_DcscfInfoGet response service operation to NEF.

5.	NEF provides DCSF address in Nnef_ImsUECM_DcscfInfoGet response service operation to AF.

Editor's note:	It is FFS how to address the scenario where the IMPU which is not registered or deregistered but queried by DC AS.

#### 6.9.3.3	Subscription for IMS DC events supported by DCSF

Figure 6.9.3.3-1depicts a signalling flow diagram for subscriptions for IMS DC events supported by DCSF.

Figure 6.9.3.2-1: DCSF address discovery by DC AS

1.	AF subscribes with NEF using Nnef_ImsEE_Subscribe request.

2.	NEF provides Nnef_ImsEE_Subscribe Response.

3.	Based on the requested events, NEF decides that the request need to be sent to DCSF. It is assumed that NEF has discovered DCSF based on clause 6.9.2. NEF then sends the Ndcsf_ImsEE_Subscribe request with IMPU and Application ID, event ID as input.

4.	DCSF provides Ndcsf_ImsEE_Subscribe Response.

5.	DCSF detected that the application which was subscribed by NEF has been downloaded by the UE.

4.	DCSF sends Ndcsf_ImsEE_Notify to NEF.

7.	NEF sends Nnef_ImsEE_Notify to AF.

### 6.9.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

AF/NEF:

-	subscribing with DCSF for specific application download by UE event and get notified when the event is detected by DCSF.

-	subscribing with HSS to discover the DCSF address.

DCSF:

-	receiving third party registration from IMS AS.

-	registering DCSF address in HSS.

-	receiving subscription for specific successful application download event from NEF or AF and notifying when UE successfully downloads the application.

HSS:

-	receiving subscription from AF/NEF for discovering DCSF address.

-	notify with DCSF address once DCSF register its address to HSS.

## 6.10	Solution #10: DC3/DC4 Enhancement to Support Subscription, Notification and Instruction for P2A and P2A2P DC establishment.

### 6.10.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints. This solution enhances the current P2A and P2A2P ADC establishment procedure defined in clauses AC.7.2.2 and AC.7.2.3 of TS 23.228 [2].

### 6.10.1	Description

This solution requires the DC AS to subscribe to the call events from DCSF so that DC AS can give media instruction after receiving the notification from DCSF. This solution defines two services to enable DC AS to subscribe to the DCSF and to instruct DCSF.

This solution also enhances and unifies the timing of notifying DC AS in the current P2A and P2A2P ADC establishment procedure defined in clauses AC.7.2.2 and AC.7.2.3 of TS 23.228 [2].

### 6.10.2	Service definition

#### 6.10.2.1	General

The following table shows the DCSF Services defined.

Table 6.10.2-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
|  | Subscribe | Subscribe/Notify | NEF, DC AS |
| Ndcsf_EventExposure | Unsubscribe | Subscribe/Notify | NEF, DC AS |
|  | Notify | Subscribe/Notify | NEF, DC AS |
| Ndcsf_MediaControl | MediaInstuction | Request/Response | NEF, DC AS |

#### 6.10.2.2	Ndcsf_EventExposure

##### 6.10.2.2.1	General

Service description: This service enables the consumer to subscribe and to be notified when the given event is detected.

##### 6.10.2.2.2	Ndcsf_EventExposure_Subscribe

Service operation name: Ndcsf_EventExposure_Subscribe

Description: This service operation enables consumers to subscribe to specific events.

Inputs, Required: Notification Target address, Event ID

Three Event ID is currently defined: application data channel establishment request, application data channel establishment success, application data channel establishment failure.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the notification will be sent to the consumer for all users.

When no application binding information is given, the notification will be sent to the consumer for all IMS DC applications.

Outputs, Required: Result indication, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

Editor's note:	Whether the explicit subscription is needed is FFS.

Editor's note:	Whether the service operation type is Subscribe/Notify (and provided by DCSF) or Request/Response (and provided by DC AS) is FFS.

##### 6.10.2.2.3	Ndcsf_EventExposure_Unsubscribe

Service operation name: Ndcsf_EventExposure_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the subscription.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 6.10.2.2.4	Ndcsf_EventExposure_Notify

Service operation name: Ndcsf_EventExposure_Notify

Description: This service operation enables the DCSF to notify the consumer about the subscribed event.

Inputs, Required: Session ID, Event ID

Inputs, Optional: User IDs of calling party and called party, application binding information.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.10.2.3	Ndcsf_MediaControl

##### 6.10.2.3.1	General

Service description: This service enables the consumer to give media instruction to DCSF. In this release, the instruction is only used to establish P2A and P2A2P data channel.

##### 6.10.2.3.2	Ndcsf_MediaControl_MediaInstruction

Service operation name: Ndcsf_MediaControl_MediaInstruction

Description: This service operation enables the consumer to provide instruction to the DCSF.

Inputs, Required: Session ID, Media information of MDC2 interface.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

### 6.10.3	Procedure

#### 6.10.3.1	Service Subscription Procedure

Figure 6.X.3.1-1: service subscription procedure

1.	DC AS sends Ndcsf_EventExposure_Subscribe to DCSF to subscribe to specific ADC establishment event.

2.	DCSF sends 200 OK.

Editor's note:	How the DC AS find the right DCSF is FFS.

#### 6.10.3.2	P2A Application Data Channel Establishment Procedure

Figure 6.10.3.2-1 depicts the P2A application data channel establishment procedure.

Figure 6.10.3.2-1: P2A ADC establishment procedure

1.	Steps 0-3 of clause AC.7.2.2 of TS 23.228 [2] applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the subscription from DC AS.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received.

4.	The DC AS sends 200 OK indicating it receives the notification.

5.	The DC AS sends Ndcsf_MediaControl_MediaInstruction to instruct the DCSF about the MDC2 related media information.

6.	The DCSF sends 200 OK indicating it receives the instruction.

NOTE:	It is up to operator's configuration and implementation that how the DCSF knows that the DC AS will send instruction to the DCSF. A possible implementation is that the DCSF will continue the following steps after a fix timer is expired.

7.	Steps 11-20 of clause AC.7.2.2 of TS 23.228 [2] applies.

8.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that the application data channel establishment success event. The DCSF will also forward MDC2 related information from the MF to the DC AS.

9.	The DC AS sends 200 OK indicating it receives the notification.

10.	Steps 21-27 of clause AC.7.2.2 of TS 23.228 [2] applies.

#### 6.10.3.3	P2A2P Application Data Channel Establishment Procedure

Figure 6.10.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.10.3.2-1: P2A2P Application Data Channel Establishment Procedure.

1.	Steps 0-3 of clause AC.7.2.3 of TS 23.228 [2] applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the subscription from DC AS.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received.

4.	The DC AS sends 200 OK indicating it receives the notification.

5.	The DC AS detects that this is a P2A2P ADC so it allocates two sets of MDC2 endpoints. The DC AS sends Ndcsf_MediaControl_MediaInstruction to instruct the DCSF about the MDC2 related media information.

6.	The DCSF sends 200 OK indicating it receives the instruction.

NOTE:	It is up to operator's configuration and implementation that how the DCSF knows that the DC AS will send instruction to the DCSF. A possible implementation is that the DCSF will continue the following steps after a fix timer is expired.

7.	Steps 6-8 of clause AC.7.2.3 of TS 23.228 [2] applies. The DCSF shall send all the MDC2 media related information received from the DC AS to the MF. In this case, the MF receives two sets of MDC2 endpoints.

8.	Steps 10-18 of clause AC.7.2.3 of TS 23.228 [2] applies. The DCSF will instruct the MF via IMS AS to update the media related information after receiving SDP answer from the terminating side.

9.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that the application data channel establishment success event. The DCSF will also forward MDC2 related information from the MF to the DC AS. In the P2A2P scenario, two sets of endpoint information shall be sent to the DC AS.

10.	The DC AS sends 200 OK indicating it receives the notification.

11.	Steps 19-26 of clause AC.7.2.3 of TS 23.228 [2] applies.

### 6.10.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	sending subscription request to the DCSF.

-	receiving notification from the DCSF which may include MDC2 related media information.

-	sending DC AS side MDC2 related media information to DCSF.

DCSF:

-	receiving subscription from DC AS.

-	notifying the session events to the DC AS.

-	sending MF side MDC2 related media information to DC AS.

-	receiving DC AS side MDC2 related media information from DC AS.

## 6.11	Solution #11: DC3/DC4 based on DCSF services

### 6.11.0	High-level solution principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints.

The DC AS can subscribe to events related with MDC2 channels, targeting specific UE(s) or any targeting any UE. The subscription can be either implicitly or explicitly established. For an implicitly establishment, the subscription is configured at the DCSF. For an explicitly established, the subscription request is delivered to IMS AS by following the procedure defined in clause AD.3.2 of TS 23.228 [2] or in clause AD.3.3 of TS 23.228 [2]. Once, the UE(s) initiates a P2A/P2A2P ADC establishment, the IMS AS selects a DCSF instance for handling the ADC as defined in clauses AC.7.2.2 and AC.7.2.3 of TS 23.228 [2] and forwards the DC AS subscription information to the selected DCSF instance. The subscription at DCSF lasts only as long as the corresponding end user session.

The solution does not assume a 1-1 mapping between IMS AS and DCSF.

Upon receipt of a P2A/P2A2P ADC establishment request meeting the subscription filter conditions, the DCSF invokes a new Ndcsf_SessionEvent_Notify to notify the DC AS of the event that MDC2 needs to be established. This notification includes MF-side information required for MDC2 configuration and a Resource URL where the DCSF expects further instructions from the DC AS.

Based on the received notification from the DCSF, the DC AS derives the DC AS-side MDC2 configuration information and transfers it to the DCSF via a new Ndcsf_ADCControl_Create operation towards the Resource URL received in Ndcsf_SessionEvent_Notify.

### 6.11.1	Description

This solution specifies the DCSF services exposed over the DC3/DC4 interface to support UE-initiated P2A and P2A2P ADC establishment. Specifically, the following services are defined: Ndcsf_SessionEvent and Ndcsf_ADCControl. The associated service parameters and corresponding procedures are defined in clause 6.11.2.

The DC AS subscribes to (or is subscribed to) events notifications related with MDC2 channel establishment need . Such an event subscription can be explicitly initiated by the DC AS or implicitly configured in the DCSF. Events can be UE related (i.e. targeting specific UE(s)) or UE unrelated (i.e. targeting any UE) and associated with filter conditions such as P2A/P2A2P ADC associated with specific applications.

Upon receipt of a P2A or P2A2P ADC establishment request meeting the subscription filter conditions, the DCSF invokes Ndcsf_SessionEvent_Notify to notify the DC AS of the MDC2 channel establishment need event. This notification includes MF-side information required for MDC2 configuration. Based on the notification received, the DC AS derives the DC AS-side MDC2 configuration information and transfers it to the DCSF via the Ndcsf_ADCControl_Create service operation. The DCSF subsequently proceeds with the MDC2 configuration based on the information received from the DC AS.

This enhancement to the DC3/DC4 interface is crucial for ensuring consistent and up-to-date MDC2 media resources between the DCSF and the DC AS. The DC AS and the MF may dynamically deploy/insert MDC2 terminations and thus use the dynamically determined IP address. The DC port and/or DC SCTP port may be dynamically associated to the session by the MF and/or the DC AS. Furthermore, the network (DCSF) may need to provide the DC AS with the UE certificate fingerprint received from the UE over IMS signalling.

The current P2A/P2A2P data channel procedures lack a mechanism for MDC2 media resource negotiation, leading to potential inconsistencies of MDC2 media resource between the two MDC2 endpoints. This proposed enhancement, by enabling the MDC2 media negotiation and dynamic configuration, significantly improves the robustness and flexibility of these procedures, preventing failures due to outdated or missing MDC2 media resource information.

### 6.11.2	Service definition

#### 6.11.2.1	General

The following table shows the DCSF Services and DCSF Service Operations.

Table 6.11.2-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
|  | Subscribe | Subscribe/Notify | IMS AS |
| Ndcsf_SessionEvent | Unsubscribe | Subscribe/Notify | IMS AS |
|  | Notify | Subscribe/Notify | NEF, Trusted AF/DC AS |
|  | Create | Request/Response | NEF, Trusted AF/DC AS |
| Ndcsf_ADCControl | Update | Request/Response | NEF, Trusted AF/DC AS |
|  | Delete | Request/Response | NEF, Trusted AF/DC AS |

Editor's note:	Further information, e.g. QoS, charging, monitoring, is for FFS.

Editor's note:	The content of UE's and DC AS's certificate fingerprint for end-to-end security between UE and DC AS shall be aligned with SA WG3.

#### 6.11.2.2	Ndcsf_SessionEvent

##### 6.11.2.2.1	General

Service description: This service enables the consumer to subscribe to and to be notified about events related with MDC2 channels.

The following operations are available for this service:

-	Subscribing and unsubscribing to receive event related with MDC2 channels.

-	Notifying the event to consumers, e.g. NEF, trusted DC AS.

##### 6.11.2.2.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event related with MDC2 channels (in this release only event is defined “the establishment of a MDC2 channel is needed”).

Inputs, Required: Notification Target address, Event ID

NOTE: 	The Notification Target address contains the Notification target address of the NEF or of the subscribing trusted DC AS.

Only one Event Id is currently defined: establishment of a MDC2 channel is needed.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (as an alternative, to be decided by stage 3, an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

##### 6.11.2.2.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from an event .

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.11.2.2.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), Resource URL(for any Ndcsf_ADCControl operation that would correspond to the same session), DC application ID

One or two MF-side MDC2 addressing information are provided depending on whether the session is established in P2A or P2A2P mode.

Inputs, Optional: Calling party and called party IDs, application ID, security Setup if UE initiates the DTLS negotiation, , certificate fingerprint of UE for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

.Outputs, Required: Result.

Outputs, Optional: None.

#### 6.11.2.3	Ndcsf_ADCControl

##### 6.11.2.3.1	General

Service description: This service enables the consumer to create, update or terminate an ADC.

##### 6.11.2.3.2	Ndcsf_ADCControl_Create

Service operation name: Ndcsf_SessionEvent_Create

Description: This service operation enables the consumer to create information used by the DCSF for ADC establishment in an IMS session.

Inputs, Required: Session ID, DC AS-side MDC2 endpoint information (e.g. IP address, port number),.

One or two MF-side MDC2 addressing information are provided depending on whether the session is established in P2A or P2A2P mode.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation, certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.11.2.3.4	Ndcsf_ADCControl_Delete

Service operation name: Ndcsf_ADCControl_Delete

Description: This service operation enables the consumer to provide information used by the DCSF for ADC termination in an IMS session

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

### 6.11.3	Procedure

#### 6.11.3.1	Person-to-Application (P2A) Application Data Channel Setup

Figure 6.11.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.11.3.1-1: P2A ADC establishment with a trusted DC AS.

0.	A subscription of the DC AS about the need for establishment of a MDC2 channel is created; this subscription is either implicitly configured at the DCSF (e.g. based on the DC Application), or explicitly provided to the IMS AS following the exposure framework defined in Annex AD of TS 23.228 [2]

1.	IMS session and BDC are established. The IMS AS selects a DCSF as specified in Figure AC.7.1-1 step 2 of TS 23.228 [2]

2-9.	UE#1 requests the establishment of P2A ADC establishment. The IMS AS forwards the DC AS subscription request to the selected DCSF instance as part of a modified step 3 of Figure AC.7.2.1-1 of TS 23.228 [2]: when the IMS AS notifies the DCSF of the session establishment event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, etc.), the IMS AS provides the DC AS subscription request to the DCSF. The DCSF determines the DC control policy and indicates the IMS AS to request MF for reserving the originating media resources.

NOTE 1:	The IMS AS does not need to know whether the DC is requested for P2A ADC establishment. It sends the subscription via Nimsas_SessionEventControl_Notify and it is up to DCSF to determine whether the subscription applies (as the DC is requested for P2A ADC establishment)

10.	The DCSF notifies DC AS about the need to establish a MDC2 channel (indicating the session is established in P2A mode) using Ndcsf_SessionEvent_Notify message. The message includes calling and called parting id, application id, session id, MF-side MDC2 media resource info, Resource URL to be used by DC AS to send subsequent ADC Control message and the certificate fingerprint of UE. The DC AS responds to (acknowledges) the notification message.

11.	The DCSF waits for ADC control instruction (Ndcsf_ADCControl_Create). If such instructions are not provided then DCSF has the ADC establishment procedure aborted (Nimsas_MediaControl_MediaInstruction).

12.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS sends Ndcsf_ADCControl_Create message with the allocated DC AS-side MDC2 media resource information. The message includes session id, DC AS-side MDC2 media resource info and may include the certificate fingerprint of DC AS.

NOTE 2:	Work on the handling of security information like certificate fingerprints is to be co-ordinated with SA WG3.

13.	The DCSF sends response of Ndcsf_ADCControl_Create to DC AS.

14-17.	The DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF. This follows already specified interactions

18-29.	IMS AS performs SDP negotiation with terminating network. With a successful SDP negotiation, the application data channel is established. This follows already specified interactions

#### 6.11.3.2	Person-to-Application-to-Person (P2A2P) Procedure

Figure 6.11.3.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application to person use case.

Figure 6.11.3.2-1: P2A2P ADC establishment with a DC AS.

0.	As in step 0 of Figure 6.11.3.1-1.

1.	As in step 1 of Figure 6.11.3.1-1.

2-9.	UE#1 requests the establishment of P2A2P ADC establishment. The IMS AS forwards the DC AS subscription request to the selected DCSF instance: when the IMS AS notifies the DCSF of the session establishment event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, etc…), the IMS AS provides the DC AS subscription request to the DCSF.

NOTE 1:	The IMS AS does not need to know whether the DC is requested for P2A2P ADC establishment. It sends the subscription via Nimsas_SessionEventControl_Notify and it is up to DCSF to determine whether the subscription applies (as the DC is requested for P2A2P ADC establishment).

The DCSF determines the DC control policy and indicates the IMS AS to request MF for reserving the originating and terminating media resources.

10.	DCSF notifies DC AS about the session establishment (indicating the session is established in P2A2P mode) using Ndcsf_SessionEvent_Notify message. The message includes calling and called parting id, application id, session id, MF-side MDC2 media resource info, Resource URL used by DC AS to send subsequent ADC Control message and the certificate fingerprint of UE. DC AS responds to (acknowledges) the notification message.

11.	DCSF waits for ADC control instructions (Ndcsf_ADCControl_Create). If such instructions are not provided then DCSF has the ADC establishment procedure aborted (Nimsas_MediaControl_MediaInstruction).

12.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS sends Ndcsf_ADCControl_Create message to the DCSF with the allocated DC AS-side MDC2 media resource information. The message includes session id, DC AS-side MDC2 media resource info and may include the certificate fingerprint of DC AS.

NOTE 2:	Handling of security information like certificate fingerprints is to be co-ordinated with SA WG3.

13.	The DCSF sends response of Ndcsf_ADCControl_Create to DC AS.

14-21.	IMS AS performs SDP negotiation with terminating network. And the IMS AS notifies the DCSF about the successful result of the SDP negotiation. . This follows already specified interactions

22.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information and the media resource information of the terminating network. Then, the IMS AS sends response to DCSF. This follows existing procedures e.g. Steps 8 of Figure AC.7.2.3-1 of TS 23.228 [2].

23-29.	IMS AS continues performing SDP negotiation with the originating network. With a successful SDP negotiation, the application data channel is established. . This follows already specified interactions.

### 6.11.4	Impacts on services, entities and interfaces

The solution has the following impacts:

DC AS:

-	subscribing to the event of "establishment of a MDC2 channel is needed" and receiving corresponding notifications containing MF-side information for MDC2 configuration;

-	sending DC AS-side information for MDC2 configuration;

-	sending/receiving certificate fingerprints for end-to-end security between UE and DC AS,

DCSF:

-	receiving subscription to the "event of establishment of a MDC2 channel is needed" and sending corresponding notification, containing MF-side information for MDC2 configuration;

-	receiving DC AS-side information for MDC2 configuration;

-	sending/receiving certificate fingerprints for end-to-end security between UE and DC AS,

IMS AS:

-	this solution assumes the definition of an IMS AS event requiring the IMS AS is to forward to the selected DCSF;

-	sending subscription request to the DCSF instance.

## 6.12	Solution #12: Support dynamic configuration of MDC2 endpoints with new DCSF services

### 6.12.0	High-level solution Principles

This solution addresses KI#3: Supporting dynamic configuration of MDC2 endpoints.

This solution takes the existing Rel-18 procedures of P2A and P2A2P DC establishment as the basis and updates the existing steps between DCSF and DC AS by defining new service of DCSF.

### 6.12.1	Description

This solution defines a new service of DCSF which is used to request to establish, update or delete a MDC2 connection. This service exchanges the MDC2 media points information on MF and DC AS to each other. The connection ID is allocated by DCSF and is used to correlate the MDC2 connection.

The solution also clarifies that the DCSF selects the correct DC AS and get the address of the selected DC AS URL based on binding information which indicates which DC AS is serving the application. The mapping of the app id and the DC AS URL is locally configured on DCSF or is retrieved from the application data in the HSS.

### 6.12.2	Procedures

#### 6.12.2.1	P2A DC establishment procedure

This procedure depicts the procedure of UE initiated P2A DC establishment which reuses the P2A DC establishment procedure in clause AC.7.2.2 of TS 23.228 [2] as the basis and updates original steps 9 and 10 to clarify the interaction between DCSF and DC AS.

Figure 6.12.2.1-1

Editors Note:	This Figure has no Title.

The steps in the procedure are as follows:

Step 0-8 is the same with step 0-8 in clause AC.7.2.2 of TS 23.228 [2];

9.	The DCSF selects the DC AS serving this session based on the binding information of this P2A DC. The DCSF gets the DC AS URL of the selected DC AS by local configuration or retrieves it from DCSF service specific data stored in HSS.

10.	The DCSF invokes Ndcsf_MDC2ConnectionRequest_Create service operation as defined in clause 6.12.3.2 to request the selected DC AS to establish MDC2 connection with the MF for the P2A DC. The service operation includes the MDC2 media endpoint information on the MF.

11.	The DC AS responds to Ndcsf_MDC2ConnectionRequest_Create service operation with its MDC2 media endpoint information for the MDC2 connection.

12-28.	The same with step 11-27 in clause AC.7.2.2 of TS 23.228 [2].

Editor's note:	The procedure of establishing P2A2P DC is similar to the above procedure and will be added later.

### 6.12.3	DCSF Services

This solution defines DCSF services to support interaction between DCSF and DC AS for establishing the P2A or P2A2P DC.

Table 6.12.3.1-1: DCSF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_MDC2ConnectionRequest | Create | Request/Response | DC AS |
|  | Update | Request/Response | DC AS |
|  | Delete | Request/Response | DC AS |

Editor's note:	Whether the service operation type is Subscribe/Notify (by DCSF) or Request/Response (by DC AS) is FFS.

#### 6.12.3.1	Ndcsf_MDC2ConnectionRequest

##### 6.12.3.1.1	General

Service description: This service enables the consumer to be requested to establish, update or delete MDC2 connection towards the MDC2 endpoint on the MF.

##### 6.12.3.1.2	Ndcsf_MDC2ConnectionRequest_Create service operation

Service operation name: Ndcsf_MDC2ConnectionRequest_Create

Description: This service operation enables DCSF to request the consumer to establish a MDC2 connection with the MF towards the MDC2 endpoint on the MF and responds with its own MDC2 endpoint information.

Inputs, Required: Session ID, Connection ID, remote MDC2 media endpoint information.

Session ID is the identity of the IMS session to which the MDC2 connection belongs.

Connection ID is the identity of the MDC2 connection. It is allocated by the DCSF.

Remote MDC2 media endpoint information contains the IP address and port of the MDC2 media endpoint on MF.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. The Calling ID and Called ID can be used by DC AS to decide whether a P2A2P DC should be further established.

Outputs, Required: Result indication.

Outputs, Optional:  Local MDC2 media endpoint information if the result is successful.

##### 6.12.3.1.3	Ndcsf_MDC2ConnectionRequest_Update service operation

Service operation name: Ndcsf_MDC2ConnectionRequest_Update

Description: This service operation enables DCSF to request the consumer to update an existing MDC2 connection.

Inputs, Required: Session ID, Connection ID, updated remote MDC2 media endpoint information.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional:  Local MDC2 media endpoint information if the result is successful.

##### 6.12.3.1.4	Ndcsf_MDC2ConnectionRequest_Delete service operation

Service operation name: Ndcsf_MDC2ConnectionRequest_Delete

Description: This service operation enables DCSF to request the consumer to delete an existing MDC2 connection.

Inputs, Required: Session ID, Connection ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

### 6.12.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DCSF:

-	Supports to select the DC AS serving the session based on the binding information of this P2A DC.

-	supports Ndcsf_MDC2ConnectionRequest service to interact with DC AS to establish, update or delete MDC2 connection for P2A or P2A2P DC.

DC AS:

-	supports Ndcsf_MDC2ConnectionRequest service to interact with DCSF to establish, update or delete MDC2 connection for P2A or P2A2P DC.

## 6.13	Solution #13: DC3/DC4 enhancements with new media management

### 6.13.1	Description

This solution addresses KI#3.

This solution specifies the DCAS services exposed over the DC3/DC4 interface to support UE-initiated P2A and P2A2P ADC establishment. Specifically, the following new service function is defined: Ndcas_Media_Management. the associated service parameters and corresponding procedures are also defined.

The DC AS provisions the service triggering (triggering information) criteria for establishing P2A/P2A2P ADC at the DCSF using the Nimsas_ImsParameterProvision_Create service operation (see clause AA2.4.6 of TS 23.228 [2], v19.0.0). Such provisioning can be explicitly initiated by the DC AS or implicitly configured per DC AS in the IMS AS and/or in the service document in HSS (Repository data). Triggering can be user specific or user unrelated (i.e. PSI) and associated with filter conditions such as P2A/P2A2P ADC associated with specific applications. Upon receipt of a P2A or P2A2P ADC establishment request meeting the triggering filter conditions, the IMS AS invokes Nimsas_SessionEventControl_Notify to notify the DCSF of a media change request event and the availability of an P2A/P2A2P ADC. This notification includes the SDP information for the P2A/P2A2P ADC representation on the Mb interface. Based on the notification received, the DCSF will request anchoring of media in the MF for the originating MF-side of the MDC2 interface. The DCSF will then provide the originating MF-side of the MDC2 to the DCAS and request the DCAS-side for the MDC2 via the Ndcas_Media_Management_Create service operation. The DCSF will provide the received media information from the DCAS to IMS AS over the Nimsas_mediaControl service with the requested MDC2 configuration based on the information received from the DC AS.

Note that the proposal utilizes the principle that configuration/trigger can be sent to any (not all) IMS AS, according to operator choice and the data will be then stored in HSS over N71 as repository data (using the same mechanisms as defined in clause AF.4 of TS 23.228 [2]) and can subsequently be used by any IMS AS serving specific subscriber or non-subscriber specific triggers.

The use of IMS Nimsas_SessionEventControl_Notify was purposefully done in order to reuse (thus no need for new interfaces in DCSF, etc.). The provisioning paradigm used is a request-resp interface and it is not connected to IMS events (as covered in KI#2) ensuring that all IMS ASs can avail of the data even if not always applicable only to a specific subscriber. These triggers are static in nature and are similar to iFC triggers.

This enhancement to the DC3/DC4 interface is crucial for ensuring a time sensitive and 3GPP SBA principle compliant consistent handling of MDC2 media resources between the DCSF and the DC AS. The DC AS and the MF may dynamically deploy/insert MDC2 terminations and thus use the dynamically determined IP address. The DC port and/or DC SCTP port may be dynamically associated to the session by the MF and/or the DC AS. Furthermore, the network (DCSF) may need to provide the DC AS with the UE certificate fingerprint received from the UE over IMS signalling using the Dcas_Media_Management_Update service operation.

The current P2A/P2A2P data channel procedures lack a 3GPP SBA service self-containment complaint mechanism for MDC2 media resource negotiation, leading to potential inconsistencies of MDC2 media resource between the two MDC2 endpoints. This proposed enhancement, by enabling the MDC2 media negotiation and dynamic configuration, significantly improves the robustness and flexibility of these procedures, preventing failures due to outdated or missing MDC2 media resource information. The DCSF availing of the DC AS offered / provided service, Dcas_Media_Management ensures that it is the resource control entity when interworking with DC AS.

Furthermore, this solution ensures that DC AS can be selected by DCSF (for establishing P2A/P2A2P ADC) on a per session basis without creating long lived dependencies between the entities.

### 6.13.2	Service definition

#### 6.13.2.1	General

The following table shows the DC AS Services and DC AS Service Operations.

Table 6.13.2-1: NF services provided by the DC AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcas_Media_Management | Create | Request/Response | DCSF |
|  | Update | Request/Response | DCSF |
|  | Delete | Request/Response | DCSF |

Editor's note:	Whether the service operation type is Subscribe/Notify (and provided by DCSF) or Request/Response (and provided by DC AS) is FFS.

#### 6.13.2.2	Ndcas_Media_Management

##### 6.13.2.2.1	General

Service description: This service enables the consumer (DCSF) to create, update or terminate a media resource controlled by the producer (DCAS).

##### 6.13.2.2.2	Ndcas_Media_Management_Create

Service operation name: Ndcas_Media_Management_Create

Description: This service operation enables the consumer to create a media resource (e.g. ADC (P2A, P2A2P) or other media resources like audio/Video. The information is used by the DCAS to create a resource and establish a media resource in an IMS session.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode, trigger Information.

Inputs, Optional:

Outputs, Required: Result. The result contains the following data elements: session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent operations.

Outputs, Optional: None.

##### 6.13.2.2.3	Ndcas_Media_Management_Update

Service operation name: Ndcas_Media_Management_Update

Description: This service operation enables the consumer to update a media resource information used by the DCAS.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), destination URL

Inputs, Optional: certificate, fingerprint of the UE, trigger Information

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.13.2.2.4	Ndcas_Media_Management_Delete

Service operation name: Ndcas_Media_Management_Delete

Description: This service operation enables the consumer to provide information used by the DCAS for media termination in an IMS session

Inputs, Required: Session ID, mediaInformation, destination URL

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.13.2.3	Nimsas_ImsParameterProvision Service API

##### 6.13.2.3.1 	Type: ImsPpDataEntry

Table 6.5.6.2.2-1: Definition of type ImsPpDataEntry, TS 29.175 [9]

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rcdServerInformation | RcdServerInfo | O | 0..1 | The RCD server information provisioned to the IMS AS. |
| triggerinformation | TriggerInfo | O | 0..1 | Triggering information to invoke for example dcas media management |

Editor’s note:	The TriggerInfo data type needs to be defined as common data and is for FFS.

NOTE:	An additional data type is required (existing defined operation only contains RCD related info) to trigger the service to invoke DCSF (and locate the correct DCAS) for this session.

#### 6.13.2.4	Nimsas_SessionEventControl Service API

##### 6.13.2.4.1	Type: SessionEventNotification

The parameters that are new are: afInstance, triggerinformation

Table 6.1.6.2.2-1: Definition of type SessionEventNotification TS 29.175 [9]

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notificationEvent | NotificationEvent | M | 1 | The NotificationEvent corresponds to a report on one subscribed event. |
| sessionId | SessionId | M | 1 | The session ID is the identity of the IMS session for which the event relates to. This IE contains the information in the Call-ID header which is the typical header of SIP message. |
| sessionInfo | SessionInfo | C | 0..1 | The sessionInfo is the IMS session related information, e.g. the calling and called identity. It shall be contained when the attribute eventType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "MEDIA_CHANGE_REQUEST", "EXTERNAL_SESSION_CREATE", "SESSION_ESTABLISHMENT_SUCCESS" (NOTE). |
| mediaInfoList | map(MediaInfo) | C | 1..N | Media info list includes a list of media related information, e.g. media identity, media type and media specifications. The mediaId attribute within the MediaInfo data type is the key of the map. It shall be contained when the attribute eventType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "SESSION_ESTABLISHMENT_PROGRESS", "SESSION_ESTABLISHMENT_ALERTING", "SESSION_ESTABLISHMENT_SUCCESS", "MEDIA_CHANGE_REQUEST", "MEDIA_CHANGE_SUCCESS", "MEDIA_CHANGE_FAILURE", or "MEDIA_CHANGE_CANCEL". |
| psDataOff | PsDataOffStatus | C | 0..1 | It indicates the active/inactive status of PS Data Off for the served UE. It shall be present if the attribute eventType of the notificationEvent is set to one of the following: - "SESSION_ESTABLISHMENT_REQUEST", when IMS session establishment is invoked; - "PS_DATA_OFF_STATUS_CHANGE", when the status change of PS Data Off happens without any other changes to the IMS session; It may be present in other cases if the change of PS Data Off status happens together with other events (e.g. media changes). |
| mediaInfoSet | array(MediaInfoExternal) | C | 0..1 | Media information set includes a set of media information. It shall be contained when the attribute eventType of the notificationEvent is one of the "EXTERNAL_SESSION_UPDATE" "EXTERNAL_SESSION_CREATE" |
| afInstance | string | O | 0..1 | The string identifying the served DCAS service instance |
| triggerinformation | TriggerInfo | O | 0..1 | Triggering information to invoke dcas media management |
| NOTE: The sessionInfo shall be present when the attribute eventType of the notificationEvent is set to "SESSION_ESTABLISHMENT_SUCCESS" and the originating IMS AS received the redirected-to party identity via the History-Info header field in the 200 OK responses to the initial INVITE request. |  |  |  |  |

### 6.13.3	Procedure

#### 6.13.3.1	Person-to-Application (P2A) Application Data Channel Setup

Figure 6.13.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.13.3.1-1: P2A ADC establishment with a trusted DC AS

1.	The DCAS provisions service triggers using the Nimsas_ImsParameterProvision.

NOTE:	The IMS Parameter Provisioning flow is simplified for illustration purposes. The IMS Parameter Provisioning flow is defined in clause AF.4 of TS 23.228 [2] and uses an IMS AS instance that does not need to be the one assigned to the specific IMS user. Since the provisioned parameters are stored as repository data in HSS, the IMS AS assigned to the IMS user will retrieve the DCAS provided service triggers from HSS.

Editor's note:	How the IMS Parameter Provisioning procedure can support provisioning of service triggers for any UE is FFS.

2.	The IMSAS responds the request to the DC AS confirming the provisioned data stored for the DCAS.

3.	IMS session and BDC has been established.

4-11.	UE#1 requests the establishment of P2A ADC establishment. IMSAS evaluate the service triggers provided in step 1. IMSAS provide the service triggers and the DCAS instance to the DCSF. DCSF determines the DC control policy and request media endpoint for the MDC2 from the IMSAS. IMS AS to request MF to reserving the originating media resources.

12.		DCSF request the DC AS to establish a media endpoint for the MDC2 via Ndcas_Media_Management_Create message. The message includes calling and called parting id, application id/DCAS instance, session id, MF-side MDC2 media resource info. source URL used by DC AS to send subsequent ADC Control message and the certificate fingerprint of UE.

13.		DC AS allocates DC AS-side MDC2 media resource for this session. DC AS respond to the DCSF with the DC AS endpoint for the MDC2 via Ndcas_Media_Management_Create response. The message contains: result, session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent Ndcas_Media_Management messages to update or terminate the connection. Update might contain e.g. the certificate fingerprint of UE.

14-16.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF.

17-29.	IMS AS performs SDP negotiation with terminating network. With a successful SDP negotiation, the application data channel established.

#### 6.13.3.2	Person-to-Application and Application-to-Person (P2A2P) Procedure

Figure 6.13.3.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.13.3.2-1: P2A2P ADC establishment with a trusted DC AS.

1.	The DCAS provisions service triggers using the Nimsas_ImsParameterProvision.

NOTE:	The IMS Parameter Provisioning flow is simplified for illustration purposes. The IMS Parameter Provisioning flow is defined in TS 23.228 [2] Annex AF.4 and uses an IMS AS instance that does not need to be the one assigned to the specific IMS user. Since the provisioned parameters are stored as repository data in HSS, the IMS AS assigned to the IMS user will retrieve the DCAS provided service triggers from HSS.

Editor's note:	How the IMS Parameter Provisioning procedure can support provisioning of service triggers for anyUE is FFS.

2.	The IMSAS responds the request to the DC AS confirming the provisioned data stored for the DCAS.

3.	IMS session and BDC has been established.

4-11.	UE#1 requests the establishment of P2A ADC establishment. IMSAS evaluate the service triggers provided in step 1. IMSAS provide the service triggers and the DCAS instance to the DCSF. DCSF determines the DC control policy and request media endpoint for the MDC2 from the IMSAS. IMS AS to request MF to reserving the originating media resources.

12.	DCSF request the DC AS to establish a media endpoint for the MDC2 via Ndcas_Media_Management_Create message. The message includes calling and called parting id, application id / DC AS ID, session id, MF-side MDC2 media resource info. source URL used by DC AS to send subsequent ADC Control message and the certificate fingerprint of UE.

13.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS respond to the DCSF with the DC AS endpoint for the MDC2 via Ndcas_Media_Management_Create response. The message contains: result, session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent Ndcas_Media_Management messages to update or terminate the connection. Update might contain e.g. the certificate fingerprint of UE.

14-16.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF.

17-24.	IMS AS performs SDP negotiation with terminating network. And the IMS AS notifies the DCSF about the successful result of the SDP negotiation.

25.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information and the media resource information of the terminating network. Then, the IMS AS sends response to DCSF.

26-33.	IMS AS continues performing SDP negotiation with the originating network. With a successful SDP negotiation, the application data channel established.

### 6.13.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	provisioning service triggers for the UE-initiated P2A/P2A2P ADC establishment;

-	receiving MF-side information for MDC2 configuration;

-	receiving the certificate fingerprint of UE for end-to-end security between UE and DC AS;

-	sending DC AS-side information for MDC2 configuration;

-	sending the certificate fingerprint of DC AS for end-to-end security between UE and DC AS.

DCSF:

-	receiving subscription of the event UE-initiated P2A/P2A2P ADC establishment;

-	notifying about the event of UE-initiated P2A/P2A2P ADC establishment;

-	sending MF-side information for MDC2 configuration;

-	sending the certificate fingerprint of UE for end-to-end security between UE and DC AS;

-	sending request to create DC AS-side for the MDC2;

-	receiving DC AS-side information for MDC2 configuration;

-	receiving the certificate fingerprint of DC AS for end-to-end security between UE and DC AS;

-	requesting IMS AS for MDC2 configuration.

IMS AS:

-	receiving provisioning of trigger information from the DCAS;

-	receiving MDC2 configuration request from DCSF;

-	instructing MF for MDC2 configuration.

MF:

-	receiving MDC2 configuration instructions from IMS AS;

-	performing MDC2 configuration.

## 6.14	Solution #14: Dynamic MDC2 endpoint configuration IMS Data Channel

### 6.14.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints.

The main principle of this solution for support of dynamic MDC2 endpoint configuration for IMS Data Channel during UE initiated P2A and P2A2P session as follows

-	The DC AS shall discover and select the DCSF handling the UE for P2A or P2A2P session

-	To discover DCSF address, DC AS subscribe with HSS and HSS notifies once DCSF register its address in HSS.

-	Similar to third party registration done by S-CSCF to IMS AS, IMS AS does third party registration with DCSF and then DCSF register its address in HSS.

-	Upon successful discovery and selection, DCSF subscribes for the establishment of P2A or P2A2P events with DCSF and get notified when DCSF detects the events.

-	After receiving notification, If DC AS accepts P2A application data channel establishment request, DC AS sends an MDC2 reserved media resource in the newly proposed Ndcsf_MediaControl service operation message.

-	DCSF after receiving MDC2 endpoint information from DC AS, invoke IMS AS to provide the MDC2 endpoint address of DC AS and request for MDC2 endpoint from MF side.

-	DCSF provides the MF side MDC2 endpoint address to DC AS in response to Ndcsf_MediaControl service operation.

Editor's note:	This solution involves DCSF during registration of IMPU while it was invoked during session earlier. The implications of this approach is FFS.

### 6.14.1	Description

This solution proposes how the DC AS discovers the DCSF and subscribes to get notification for the UE initiated P2A and P2A2P session and then shares the MDC2 endpoint addresses. This will overcome the existing gaps of the P2A and P2A2P session which does not provide any flexibility in dynamically exchanging MDC2 endpoint addresses between DC AS and MF.

Discovery of DCSF by DC AS:

The trusted DC AS can directly discover the DCSF with the help of HSS and in case untrusted DC AS, the discovery happens through NEF. For this purpose, HSS can expose one service operation using which DC AS/NEF will subscribe and get notification about the DCSF address for the UE.

When UE registers with IMS network, S-CSCF does third party registration with IMS AS and then IMS AS registers its address with HSS. Similarly, this solution proposed IMS AS does third party registration with DCSF and then DCSF registers its address with HSS. Once HSS comes to know the DCSF address, it notifies the NEF/ DC AS.

Subscription with DCSF by DC AS:

Upon successful discovery, DC AS shall subscribe with DCSF for the UE initiated P2A or P2A2P session establishment events.

Providing MDC2 endpoint address by DC AS:

Upon notified by DCSF about establishment of P2A or P2A2P call, DC AS provides its MDC2 endpoint address with DCSF.

Table 6.14.1-1 List of events supported by DCSF

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| P2A pr P2A2P Application Data Channel established | Detected when an Application Data Channel is established. Network detects that an Application data channel offer takes DC Application Server as target endpoint and requires to anchor in the MF | DCSF |

### 6.14.2	Service definition

#### 6.14.2.1	General

The following table shows the required updates in TS 23.502 [8].

##### 6.14.2.1.1	NEF Service

The NEF services for IMS Event exposure is already defined in table 5.2.6.1-1 of TS 23.502 [8].

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_ImsEventExposure | Subscribe | Subscribe/Notify | AF, DC AS |
|  | Unsubscribe | Subscribe/Notify | AF, DC AS |
|  | Notify | Subscribe/Notify | AF, DC AS |

NOTE 1:	The enhancement to Table 5.2.6.1-1 is the addition of DC AS, as an Example Consumer to the Nnef_ImsEventExposure service.

NOTE 2:	Additionally the service descriptions from TS 23.502 [8] are updated to include DC AS in the example NF consumer.

5.2.6.39.2	Nnef_ImsEventExposure (ImsEE) service

5.2.6.39.2.1	Nnef_ImsEE_Subscribe service operation

Service operation name: Nnef_ImsEE_Subscribe

Description: The NF consumer (for e.g. AF or DC AS) subscribes for a specific IMS event.

Inputs, Required: Event-ID(s) representing the event of interest, Event Reporting Information as defined in Table 4.15.1-1.

Inputs, Optional: List one or more IMS Public Subscriber identity, Event Filters, Notification Target address, Notification Correlation Information, Expiry time, Subscription Correlation ID (in the case of modification of the event subscription), AF Identifier.

IMS Public subscriber identity entry is only present for subscriber specific IMS events.

NOTE 1:	The semantics of the Event ID and Event Filters are transparent to the NEF.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can expire based on the operator's policy).

Outputs, Optional: If applicable to the event type, the current status related to the subscribed event type is included, if Immediate Reporting Flag is included in the request within Event Reporting Information and if available (see clause 4.15.1).

NOTE:	If the current status related to the subscribed event type is available and included in the response, it is not considered as an event report (i.e. it is not considered a one-time notification on event detection). That is, it is not deducted from the maximum number of reports if included in the request within the Event Reporting Information.

##### 6.14.2.1.2	HSS Service

The following shows the required updates in TS 23.228 [2].

AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| ImsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| Ims UE Context | Registration | Request/Response | S-CSCF, IMS AS, DCSF |
| Management (_ImsUECM) | DeregistrationNotification | Subscribe/Notify | S-CSCF |
|  | Deregistration | Request/Response | S-CSCF, IMS AS, DCSF |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
|  | AsInfoGet | Request/Response | NEF |
|  | DcsfInfoGet | Subscribe/Notify | NEF, DC AS |
|  | DcsfInfoUnsubscribe | Subscribe/Notify | NEF, DC AS |
|  | DcsfInfoNotify | Subscribe/Notify | NEF, DC AS |
| ImsUE Authentication (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure | Subscribe | Subscribe/Notify | NEF, Trusted AF |
| (_ImsEE) | Unsubscribe |  | NEF, Trusted AF |

NOTE:	The table is updated to include DCSF as an example consumer of the Registration and Deregistration Service operations of the Ims UE Context Management (_ImsUECM) service and to add the new DcsfInfoGet, DcsfInfoUnsubscribe and DcsfInfoNotify service operations to the Ims UE Context Management (_ImsUECM) service.

AA.2.1.2.1	Nhss_ImsUECM_Registration service operation

Service operation name: Nhss_ImsUECM_Registration

Description: If S-CSCF is the consumer, this service operation registers the serving S-CSCF assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

If IMS AS is the consumer, this service operation registers the IMS AS instance assigned to an IMS user.

If the DCSF is the consumer, this service operation registers the DCSF instance assigned to an IMS user.

Inputs, Required: Public Identity, S-CSCF name and Registration Type (e.g. Initial Registration, Unregistered) if the S-CSCSF is the consumer, IMS AS instance Id if the IMS AS is the consumer, DCSF instance Id is the DCSF is the consumer.

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

NOTE:	The above service operation is modified from the existing text in TS 23.228 [2] by adding the text “Ims UE Context Management (_ImsUECM) service” and adding “DCSF instance Id is the DCSF is the consumer” to required Inputs.

6.14.2.1.2.1	Nhss_ImsUECM_DcsfInfoGet service operation

Service operation name: Nhss_ImsUECM_DcsfInfoGet

Description: Service consumer like NEF or DC AS subscribes with HSS to receive the DCSF address assigned for the UE. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the UAS NF/NEF.

Inputs, Required: Public Identity.

Inputs, Optional: Private Identity.

Outputs, Required: Result.

Outputs, Optional: None.

6.14.2.1.2.2	Nhss_ImsUECM_DcsfUnsubscribe

Service operation name: Nhss_ImsUECM_DcsfUnsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

6.14.2.1.2.3	Nhss_ImsUECM_DcsfNotify

Service operation name: Nhss_ImsUECM_DcsfNotify

Description: This service operation enables the HSS to notify the consumer with the updated DCSF address where there is a change of DCSF happens for the UE.

Inputs, Required: DCSF address.

Inputs, Optional: IMPU, IMPI.

Outputs, Required: Result.

Outputs, Optional: None.

NOTE:	The above clauses 6.14.2.1.2.1, 6.14.2.1.2.2 and 6.14.2.1.2.3 describe the new service operations added.

##### 6.14.2.1.3	DCSF Service

The following table shows the new DCSF Services and new DCSF Service Operations.

Table AA.2.4.1-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_ADCControl_MDC2 | Create | Request/Response | NEF, DC AS |
|  | Update | Request/Response | NEF, DC AS |
|  | Delete | Request/Response | NEF, DC AS |

6.14.2.1.3.1	Ndcsf_ADCControl_MDC2 service operation

Service operation name: Ndcsf_ADCControl_MDC2

6.14.2.1.3.2	General

Service description: This service enables the consumer to create, update or terminate an ADC.

6.14.2.1.3.3	Ndcsf_ADCControl_MDC2Create

Service operation name: Ndcsf_SessionEvent_Create

Description: This service operation enables the consumer to create information used by the DCSF for ADC establishment in an IMS session.

Inputs, Required: Session ID, DC AS-side MDC2 endpoint information (e.g. IP address, port number), certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation.

Outputs, Required: Result.

Outputs, Optional: None.

6.14.2.1.3.4	Ndcsf_ADCControl_MDC2Update

Service operation name: Ndcsf_ADCControl_Update

Description: This service operation enables the consumer to update information used by the DCSF for ADC updating in an IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

6.14.2.1.3.5	Ndcsf_ADCControl_MDC2Delete

Service operation name: Ndcsf_ADCControl_Delete

Description: This service operation enables the consumer to provide information used by the DCSF for ADC termination in an IMS session

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.14.2.2	Ndcsf_SessionEvent

##### 6.14.2.2.1	General

Service description: This service enables the consumer to subscribe with the supported events by DCSF and get notified when the event is detected.

The following operations are available for this service:

-	Subscribing and unsubscribing with events supported by DCSF.

-	Notifying after event is detected to consumers, e.g. NEF, AF.

##### 6.14.2.2.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Notification Target address, Event ID

Only one Event Id is currently defined: establishment of a MDC2 channel is needed

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (otherwise an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

##### 6.14.2.2.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.14.2.2.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), Resource URL(for any Ndcsf_ADCControl operation that would correspond to the Notification), DC application ID, certificate fingerprint of UE for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Inputs, Optional: Calling party and called party IDs, application ID, securitySetup if UE initiates the DTLS negotiation.

Outputs, Required: Result.

### 6.14.3	Procedure

#### 6.14.3.1	DCSF instance registration in HSS

Figure 6.14.3.1-1depicts a signalling flow diagram for an DCSF registering in HSS the DCSF instance assigned to a registering UE.

Figure 6.14.3.1-1: DCSF Instance Registration in HSS

1.	UE performs initial IMS Registration (see clause 5.2).

Editor's note:	This reference is unclear

2.	S-CSCF performs third party Registration with the IMS AS instance assigned to the registering UE.

3.	IMS AS performs third party Registration with the DCSF instance assigned to the registering UE. This is over http and not SIP.

4.	DCSF assigned to the UE/IMPU registers in HSS using Nhss_ImsUECM_Registration service operation or using Sh interface.

NOTE:	Figure 6.14.3.1-1 shows only SBA services. Corresponding Sh procedure is not included in the Figure.

#### 6.14.3.2	DCSF address discovery by DC AS

Figure 6.14.3.2-1depicts a signalling flow diagram for discovering DCSF address by DC AS.

.

Figure 6.14.3.2-1: DCSF address discovery by DC AS

1.	DCSF address is already registered as for the Figure 6.14.3.1-1 solution.

Editor's note:	Add correct clause reference

2.	DCAS discovers DCSF by sending Nnef_ImsUECM_DcscfInfoGet service operation to NEF.

3.	NEF discovers DCSF by sending Nhss_ImsUECM_DcscfInfoGet service operation to HSS.

4.	HSS provides DCSF address in Nhss_ImsUECM_DcscfInfoGet response service operation to NEF.

5.	NEF provides DCSF address in Nnef_ImsUECM_DcscfInfoGet response service operation to DC AS.

Editor's note:		It is FFS how to address the scenario where the IMPU which is not registered or deregistered but queried by DC AS.

The P2A and P2A2P call flow is to point to "solution x" from S2-2509275 during pCR implementation.

Editor's note:	Update to add reference text to solution 11.

### 6.14.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS/NEF:

-	subscribing the event of UE-initiated P2A/P2A2P ADC establishment.

-	receiving notification about the event of UE-initiated P2A/P2A2P ADC establishment.

-	receiving MF-side MDC2 endpoint information in the notify message.

-	sending DC AS-side MDC2 endpoint information in a new service operation message.

-	subscribing with HSS to discover the DCSF address.

DCSF:

-	receiving third party registration from IMS AS.

-	registering DCSF address in HSS.

-	receiving subscription for UE-initiated P2A/P2A2P ADC establishment event from NEF or DC AS and sending MF-side MDC2 endpoint information in notification message.

-	receiving DC AS-side MDC2 endpoint information in a new service operation message.

HSS:

-	receiving subscription from AF/NEF for discovering DCSF address.

-	notify with DCSF address once DCSF register its address to HSS.

## 6.X	Solution #X: <Solution Title>

Editor's note:	This Rapporteur is picky - please use unique descriptive titles for all solution proposals.

### 6.X.0	High-level solution Principles

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

Documentation of more than one solution from the same company for the same KI is allowed but it shouldn't be expected that all of them will be documented in the TR.

### 6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Further (sub-)clause(s) may be added to capture details.

### 6.X.2	Procedures

Editor's note:	This clause will describe the high-level procedures and information flows for the solution.

### 6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#Y

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

## 7.2	Topics for further consideration

### 7.2.Z	Topics for further consideration for KI#Z

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future release.

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Annex A:
Change History

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA2#170 | S2-2507417 | - | - | - | Skeleton approved at SA2#170 | 0.0.0 |
| 2025-08 | SA2#170 | S2-2507417, S2-2507546, S2-2507547, S2-2507551, S2-2507610, S2-2507611, S2-2507612, S2-2507614, S2-2507615, S2-2507648, S2-2507655, S2-2507656 | - | - | - | Baseline Doc, Scope, References, Architectural Assumptions Key issue 1, 2 &3, Solution 1, 2, 3, &4 | 0.1.0 |
| 2025-10 | SA2#171 | S2-2509268, S2-2509270, S2-2509274, S2-2509275, S2-2509276, S2-2509277, S2-2509598, S2-2509599, S2-2509600, S2-2509621, S2-2509622 |  |  |  | Solution #2 update & add new solutions (#6, #7, #8, #9, #10, #11, #12,#13 and #14) | 0.2.0 |