SA WG2 Meeting #113	S2-160927

25 - 29 January 2016, St. Kitts & Nevis, KN	e-mail revision 2 of S2-160868

Source:	NTT DOCOMO, ZTE, LG Electronics, Nokia Networks, Alcatel-Lucent, NEC, Ericsson

Title:	Evaluation of Key Issue #3: PLMN-ID determination at IMS Entities

Document for:	Approval

Agenda Item:	6.6

Work Item / Release:	FS_V8, REL-14

Abstract of the contribution: Provides evaluation of the different solutions for obtaining VPLMN-ID at IMS entities.

# Proposal

The evaluation of Solution #1 is based on updates provided by NEC to this meeting in S2-160124

The following change on TS.23.749 is proposed.

************* START OF CHANGE *************

# 7	Overall Evaluation

## 7.x Key Issue #3: Determination of the ID of the Visited PLMN at IMS Entities in HPLMN

Assumption made for this evaluation:

- Serving PLMN ID retrieval is NOT done at IMS re-registration

- Serving PLMN changes are noticed at the IMS level only when UE performs IMS registration as a result of IP address changes.

The following solutions have been captured in the TR for this key issue:

-	Solution #3 Determination of PLMN ID via HSS interaction:

HSS performs roaming restriction check based on PLMN-ID from access network. HSS provides PLMN-ID to S-CSCF in Cx interface at IMS registration. PLMN-ID added to SIP message from S-CSCF to P-CSCF.

-	Solution #6 P-CSCF query PLMN ID in IMS registration:

P-CSCF retirives PLMN-ID on Rx interface. The PCRF subscribes to PLMN change in access network. PCRF provides PLMN-ID on Rx to P-CSCF.

-	Solution #7 P-CSCF query PLMN ID in IMS registration and session establishment:

P-CSCF uses Netloc and subscribes to access-network information on Rx. Netloc supported in VPLMN and detailed location (eg. ECGI) provided to PCRF via PGW. PCRF provides PLMN-ID to P-CSCF.

The following table captures the key evaluation criteria and evaluations of these three solutions.

Table 7.x.x Evaluation Criteria Table for Key Issue #3

| Evaluation Criteria | Solution #3 (HSS based) | Solution #6 (New Rx query for PLMN-ID change) | Solution #7 (ULI based reporting) |
| --- | --- | --- | --- |
| Impact on Nodes and Interfaces | HSS: (i) On Cx interaction with I-CSCF needs to differentiate between S8HR and LBO. (ii) Needs to provide VPLMN-ID to S-CSCF on IMS registration. S-CSCF: Needs to forward VPLMN-Id to I-CSCF and then onto the P-CSCF P-CSCF: Needs to process VPLMN-ID from SIP message. Cx and SIP protocol | PCRF, P-CSCF: Needs to support new Rx procedure. Rx: New procedure to request for PLMN-ID retrieval. | None (Standard Netloc functionality) VPLMN needs to support ULI reporting (i.e. comply to Rel-11) |
| Amount of additional signalling | None. (VPLMN ID is carried via existing SIP signalling interaction) | P-CSCF requests for serving PLMN ID at initial IMS registration for both roaming and non-roaming UEs served by the P-CSCF, which may increase the Rx signalling traffic. (For example; when IP can type retrieval is needed, Rx signalling traffic is not increased). | P-CSCF requests for serving PLMN at every IMS registration, and hence doubling the Rx interaction at registration with PCRF. ULI information sent on IMS bearer establishment, modification, deletion. |
| Support for Lawful Interception | Since PLMN-ID is not available when SIP Register Request arrives at the P-CSCF but only when SIP response message from S-CSCF is received at the P-CSCF, the decision for LI is delayed till receipt of SIP response message from S-CSCF. The SIP Register message will need to be buffered at the P-CSCF till this point. | Supported. | Supported |
| Limitations of the Solution | The solution cannot work if the UE access EPS simultaneously via more than one access network which use different PLMN-IDs | None | None |

************* NEXT CHANGE *************

# 8	Conclusions

For Key Issue #3: Determination of the ID of the visited PLMN at IMS Entities in HPLMN:

-	Solution #6 as captured in Section 6.6 is the selected solution.

Editor’s note:	This conclusion may be challenged in SA2#114 if it can be shown that the solution does not meet requirements for key issue #3 or it can be shown that there is strong need for multiple solution.

************* END OF CHANGE *************